3GPP TSG-WG2 Meeting #132	R2-2508613

Dallas, TX, USA, November 17 – 21, 2025

Agenda Item:	10.3.3.2

Source:	Ericsson

Title:	Energy Efficiency for 6G: Network and UE Perspectives

Document for:	Discussion, Decision

# 1	Introduction

As stated in the RP-251881 6G WG SID, it is no longer sufficient to simply evolve the existing technologies to meet the future demands of performance, flexibility and scalability. The study and standardization effort should focus on revisiting the radio system design focusing on the growing requirements for user experience and performance excellence, while maintaining efficiency and simplicity as key design principles. Furthermore, optionality and configuration complexity should be reduced to minimize the cost and energy use of the user devices. The objectives of the 6G study include a separate objective for energy efficiency:

The detailed objectives of the study are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Following this, in RAN2#131-bis, the aspect of energy efficiency was discussed and following was captured in the chair notes:

Design goal 1. Strive to develop solutions that consider network energy efficiency and UE power saving

RAN2 also agreed the following in the same meeting: For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution we will focus on the importance of applying a clean slate approach and taking only the best learnings aiming at a minimalistic “single feature for a single purpose”-approach instead of directly building onto and merging the existing 5G NR technologies in a complex way that may not necessarily be worth the effort.

# 2		 	Discussion

In 5G NR releases, energy saving techniques were standardized for both UE and NW concerning the RRC CONNECTED state, namely, UE C-DRX and Cell DTX/DRX. UE C-DRX has control over when the UE may turn off its receiver to save battery life, based on the configured timers and DRX cycles. Cell DTX/DRX, on the other hand, has control over gNB’s transmissions and receptions to enable energy savings on the network side. When combined with UE C-DRX, Cell DTX can be viewed as “a mask on top of a mask.” This is because UE C-DRX already limits periods UE is available, and Cell DTX at the gNB side further restricts (or “masks”) those same C-DRX occasions, i.e., effectively applying an additional layer of masking.

Therefore, to maintain simplicity, enhance flexibility, and avoid duplication of features serving the same purpose, a mechanism that can achieve energy savings already through configuration and alignment of UL and DL transmissions is favourable over a rather rigid masking mechanism specified via multiple features. Furthermore, note that to fully realize the energy-saving potential of Cell DTX/DRX, alignment of UE C-DRX masks across the UEs and the alignment of UE C-DRX masks with a Cell DTX mask is necessary (c.f., “The configured cellDTX-DRX-Cycle is an integer multiple of configured drx-longCycle or vice versa” in 3GPP TS 38.331).

Standardizing both Cell DTX/DRX-like and UE C-DRX-like features duplicates efforts to provide similar functionality at a cost of increased complexity of feature inter-working.

Enabling adaptive configuration and alignment of UL and DL transmission occasions is desirable to maximize energy savings both at the UE and the NW.

In the rest of the contribution, we discuss how to configure flexible and adaptable DL transmissions to ensure maximum UE energy savings. Furthermore, we discuss how UL occasions should be configured and how UL and DL occasions configured across UEs could be jointly aligned to achieve maximum NW energy savings while at the same time enabling responsiveness to change in load (to balancing energy savings and UE performance).

## 2.1	UE Energy Savings

### 2.1.1	UE C-DRX and PDCCH monitoring

In NR, Connected Mode DRX (C-DRX) operation specified in the MAC specification enables the UE to save energy by periodically turning off its receiver. During the C-DRX active time (i.e., onDuration, drx-InactivityTimerRunning, and retransmission periods), UE monitors PDCCH to receive in the search space occasions which can be preconfigured and/or later adapted via PHY commands (e.g., SSSG switching, BW adaptation, etc.). Outside the active time, the UE may omit PDCCH reception. In other words, the UE’s effective DRX activity can be viewed as a two-layer interaction, where PHY-layer’s PDCCH configuration is masked by the MAC-layer defined C-DRX active time.

Figure 1: UE’s effective DRX activity can be viewed as a two-layer interaction, where the PHY-layer’s PDCCH configuration is masked by the MAC-layer defined C-DRX active time.

NR defines multiple overlapping mechanisms across MAC and PHY for PDCCH monitoring control, resulting in redundant and partly conflicting behaviour.

In NR, the effective UE DRX activity is determined by both MAC-layer C-DRX timers and PHY-layer PDCCH monitoring configurations.

The C-DRX configuration remains relatively rigid, being semi-statically configured via RRC, without support for setting different parameters (e.g., different OnDuration lengths and/or drx-InactivityTimer values) for the short and long cycles. Furthermore, the possibility for dynamic adaptation of parameters is limited, typically allowing the network to terminate the remainder of potentially ongoing active time or to switch the UE between short and long DRX cycles.

NR does not support pre-configuration of multiple PDCCH monitoring sets tied to DRX states (e.g., different monitoring patterns/bandwidths for OnDuration vs. drx-InactivityTimer). Such sets must be dynamically (de-)activated via PHY-layer DCI signalling.

The PHY-layer specifications introduced mechanisms to adjust PDCCH occasions and scheduling patterns via DCI-based commands (e.g., SSSG switching, enhanced cross-slot scheduling, or BWP switching) and associated timer-based configuration changes. Compared to the timer-based DRX schemes, the DCI controlled approach has the benefit that the NW can e.g. allow the UE to monitor the PDCCH in a sparse mode even upon reception of some DL data. However, the DCI signalling may lead to excessive control overhead, particularly when many UEs are in the connected mode. In addition, the PDCCH dynamics may in practice be subject to increased delays, as the network may wish to ensure successful DCI reception by the UE and therefore wait for reception confirmations for scheduled “dummy” data transmissions accompanied by these DCI commands.

Frequent DCI-based reconfigurations (e.g., PDCCH monitoring adaptations) may increase control channel overhead, particularly when managing large UE populations.

It would be beneficial to introduce a framework, combining the strengths of C-DRX and DCI controlled PDCCH adaptations. In such a framework, different windows analogous to “onDuration” and “drx-inactivityTimer” could be individually preconfigured with corresponding individual PDCCH monitoring configurations (e.g., Coreset, search space, bandwidth, TimeDomainResourceAllocations TDRA including cross-slot vs same-slot, etc.) via RRC. Both the network and the UE would then switch PDCCH configurations depending on which window is active, thereby reducing signalling and associated delays. For example, during the “onDuration”-alike window, the UE could operate over a narrower bandwidth and use a sparse search space and/or cross-slot scheduling. Upon data activity, the UE could transition to an “drx-inactivityTimer”-alike window with a wider bandwidth, denser search space, and/or same-slot scheduling.

Figure 2: UE DRX framework including windows of PDCCH configurations. UE switches from one window to another based on data activity.

For 6GR, support low-overhead transitions between preconfigured PDCCH monitoring sets without requiring DCI-based signalling for each switch, thereby reducing load and latency.

For 6GR, allow configuration of multiple DRX window types in connected mode (analogous to NR C-DRX "OnDuration", "drx-InactivityTimer"), each associated with separate PHY-layer configurations (e.g., search space, TDRA configuration, bandwidth) preconfigured via RRC.

Nevertheless, it would still be desirable to maintain the ability for additional PDCCH adaptation control when necessary and invoke “onDuration” with a WUS.

Figure 3: UE DRX framework including windows of PDCCH configurations. WUS is used to trigger the UE to activate a window for monitoring PDCCH. The PDCCH configuration can additionally be adapted by lower layer signalling.

For 6GR, retain the ability for additional lower-layer control (e.g., search space, enhanced cross-slot, bandwidth) when necessary, enabling a combination of preconfigured monitoring patterns across different windows and dynamic adaptability.

For 6GR, allow WUS-based triggering of specific "OnDuration" window configurations, enabling energy-efficient receiver reactivation with appropriate PHY setting.

# 2.2	Network Energy Savings

## 2.2.1	Discontinuous UL/ DL transmissions for UEs in connected mode

In this section, we discuss the energy-efficient configurations of UL occasions, and how the NW can create transmission and reception gaps that offer greater opportunities for network energy savings.

### 2.2.1.1	UL transmissions

In 5G NR, preconfigured UL occasions in connected mode (e.g., Scheduling Request (SR) and grant-free PUSCH opportunities) are periodically repeated and therefore spread in time. These periodic patterns are typically configured with different offsets across UEs for optimal performance in case there would be high-load scenarios. Consequently, the opportunities for the NW to enter energy-saving states on the UL are limited, even during periods of low UL traffic.

The Cell DRX feature, introduced in 5G NR Rel-18 as part of the Cell DTX/DRX NW energy-saving enhancements, allows for muting of certain UL occasions by applying a Cell DRX mask. During Cell DRX non-active periods, UEs are prohibited from transmitting on SRs and Configured Grants (CGs). However, Cell DRX comes with several limitations. First, Cell DRX affects only SRs and CGs and does not address the other types of preconfigured UL occasions such as PRACH. Second, Cell DRX only offers limited flexibility for the NW to control the UL occasions on a per-UE basis or to adapt the preconfigured UL resource configurations to varying UL load conditions. Consequently, during Cell DRX non-active duration, it is not possible to make exceptions and allow specific type of UL transmissions on preconfigured occasions for particular UEs with, for example, delay-sensitive traffic. Finally, when both Cell DRX and Cell DTX are configured simultaneously, according to 5G NR Rel-18, the configuration of Cell DRX cycle is constrained by the configuration of Cell DTX cycle, further restricting the NW’s capability to create UL energy-saving opportunities.

To support adaptive common signal and channel transmissions 5G NR Rel-19 introduced time-domain adaptation of PRACH where a baseline sparse set of PRACH occasions can be configured to be always active, and flexibly additional preconfigured PRACH resources can be activated on demand. We believe that applying similar mechanisms to other preconfigured UL occasions for connected mode UEs can significantly improve flexibility for enabling energy-saving opportunities during periods of low network load while maintaining responsiveness as load increases.

Using a mechanism similar to the 5G NR Rel-19 time-domain adaptation of RA occasions for other preconfigured UL occasions (e.g., SR and grant-free PUSCH opportunities) can provide additional network energy savings.

One effective approach to increase NW energy efficiency is to introduce greater flexibility in configuring UL occasions. This can be achieved by supporting energy-efficient configurations. Examples of energy-efficient configurations include configurations of sparse periodic UL occasions and configurations that group UL occasions into clusters with silent intervals between them. Furthermore, to accommodate diverse UE requirements and handle fluctuating uplink load, the NW should support per-UE/per-UE-group control and dynamic adaptation of preconfigured UL occasions. As such, the NW will then also have a flexibility to make exceptions and allow UL transmissions on preconfigured occasions for particular UEs with delay sensitive traffic. Such mechanisms would enable fast, efficient transitions to and from energy-efficient configurations, balancing energy reduction with responsiveness and performance. An example illustrating this solution from the point of view of a single UE shown in Figure 4.

Example a

Example b

Figure 4: Single UE perspective: dynamic switching between (Example a) periodic preconfigured UL resource patterns with sparse and dense UL resource opportunities, and (Example b) clustered preconfigured UL resource pattern and periodic preconfigured UL resource pattern.

For 6GR, allow energy-efficient configurations of UL transmission occasions and dynamic transitions to and from these configurations.

### 2.2.1.2		Alignment of energy-efficient UL and DL occasions for UEs/UE-groups in the same cell

To maximize energy efficiency for both the NW and UE and fully realize the benefits of the energy-efficient DL and UL occasion configurations discussed in the previous sections, the NW should be able to align these transmission/reception occasions across a group of UEs or across all UEs within a cell. In the case of time-domain alignment, the NW can create transmission and reception gaps that offer greater opportunities for network energy savings. Time-domain alignment can be implemented by the NW through appropriate selection of the values for configuration parameters such as offsets and periodicities. For example, UEs may be configured with different sets of parameters/configurations to support different traffic conditions and enable e.g., a high load operation mode as well as energy-saving operation. For example, when the UL load falls below a certain threshold, the NW may activate energy-efficient UL occasion configurations with identical offset values and periodicities that are integer multiples of each other for a group of UEs or for all UEs in the cell. In a similar way, the DL occasion configurations can be adjusted to correspond to the change in DL load.

Furthermore, when DL and UL energy-saving mechanisms are used together, the NW should be able to jointly control the alignment between DL and UL occasions to maximize overall energy savings. This involves coordinating timing and configuration parameters to ensure that DL and UL transmission and reception windows are synchronized, thereby optimizing the energy efficiency in both directions.

An example illustrating the alignment of DL and UL occasions in case of two UEs is shown in Figure 5.

Figure 5: NW perspective: alignment of DL as well as UL occasions, and coordination of the relative timing between UL and DL occasions in case of two UEs.

Alignment of DL and UL transmission occasions, and coordination of timing between those UL and DL occasions for UEs/groups of UEs in a cell, should be possible by configuration.

# 3 Conclusion

In this contribution we have focused on the importance of applying a clean slate approach and taking only the best learnings aiming at a minimalistic “single feature for a single purpose”-approach instead of directly building onto and merging the existing 5G NR technologies in a complex way that may not necessarily be worth the effort. We have made the following observations:

Observation 1	Standardizing both Cell DTX/DRX-like and UE C-DRX-like features duplicates efforts to provide similar functionality at a cost of increased complexity of feature inter-working.

Observation 2	Enabling adaptive configuration and alignment of UL and DL transmission occasions is desirable to maximize energy savings both at the UE and the NW.

Observation 3	NR defines multiple overlapping mechanisms across MAC and PHY for PDCCH monitoring control, resulting in redundant and partly conflicting behaviour.

Observation 4	In NR, the effective UE DRX activity is determined by both MAC-layer C-DRX timers and PHY-layer PDCCH monitoring configurations.

Observation 5	NR does not support pre-configuration of multiple PDCCH monitoring sets tied to DRX states (e.g., different monitoring patterns/bandwidths for OnDuration vs. drx-InactivityTimer). Such sets must be dynamically (de-)activated via PHY-layer DCI signalling.

Observation 6	Frequent DCI-based reconfigurations (e.g., PDCCH monitoring adaptations) may increase control channel overhead, particularly when managing large UE populations.

Observation 7	Using a mechanism similar to the 5G NR Rel-19 time-domain adaptation of RA occasions for other preconfigured UL occasions (e.g., SR and grant-free PUSCH opportunities) can provide additional network energy savings.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For 6GR, support low-overhead transitions between preconfigured PDCCH monitoring sets without requiring DCI-based signalling for each switch, thereby reducing load and latency.

Proposal 2	For 6GR, allow configuration of multiple DRX window types in connected mode (analogous to NR C-DRX "OnDuration", "drx-InactivityTimer"), each associated with separate PHY-layer configurations (e.g., search space, TDRA configuration, bandwidth) preconfigured via RRC.

Proposal 3	For 6GR, retain the ability for additional lower-layer control (e.g., search space, enhanced cross-slot, bandwidth) when necessary, enabling a combination of preconfigured monitoring patterns across different windows and dynamic adaptability.

Proposal 4	For 6GR, allow WUS-based triggering of specific "OnDuration" window configurations, enabling energy-efficient receiver reactivation with appropriate PHY setting.

Proposal 5	For 6GR, allow energy-efficient configurations of UL transmission occasions and dynamic transitions to and from these configurations.

Proposal 6	Alignment of DL and UL transmission occasions, and coordination of timing between those UL and DL occasions for UEs/groups of UEs in a cell, should be possible by configuration.