3GPP TSG-RAN2#132	TDoc R2-2508614

Dallas, Texas, USA, Nov. 17th-21st 2025

Agenda Item:	10.3.2.2 RRC Structure and (re)configuration

Source:	Ericsson

Title:	RRC ASN.1 structure for 6G

Document for:	Discussion, Decision

# 1	Introduction

In this document we present our view on the RRC signalling structure for the 6G radio interface. We analyse problems observed in previous generations and suggest high level principles to overcome those issues in 6GR. We also outline a few concrete solutions as basis for further discussion.

# 2	Discussion

## 2.1	Flatten the ASN.1 hierarchy

LTE’s and NR’s RRC ASN.1 messages use a tree-like data structure for configuring protocols, channels, and signals. RAN2 tried to group those functional elements by their (seemingly) common properties such as frequency domain position/width, synchronization, numerology, duplex mode (FDD/TDD), or protocol relations (PDCCHPDSCH).

Due to the different architecture options (EN-DC, NR-SA, NR-DC, etc.), the support for dual connectivity, carrier aggregation and bandwidth parts (BWP) as well as due to the large number of configuration parameters for the physical layer, NR’s RRC ASN.1 messages became large and deeply nested (many levels of hierarchy for CellGroups, ServingCells and BWPs). The division into common and dedicated parameters as well as the separation of uplink and downlink fields created additional hierarchy levels.

At a first glance, such grouping seems beneficial since it gives a structure which helps understand the system. However, the hierarchy can also make it very difficult to comprehend the configuration. For example, PDSCH parameters are configured and inherited from IEs on several hierarchical levels (serving cell common/dedicated, BWP common/dedicated) which is hard to understand from the IEs themselves. Similarly, the relation of common and dedicated IEs and the parameters therein remained ambiguous in many cases.

The grouping may in some cases also reduce signalling overhead since certain properties are configured only on the top-level IE. For example, the network indicates the bandwidth of the BWP and the physical channels contained in that group inherit that configuration parameter - except PDCCH CORESET for which the network configures the bandwidth separately anyway.

Furthermore, RAN2 used the grouping to enforce configuration constrains. However, this also turned out to be one of the big drawbacks. The grouping hinders evolution when a formerly prohibited configuration is meant to be allowed in a later release. This happened already in the later phase of Rel-15 when the introduction of BWPs did not fit well with the original separation into common and dedicated parameters nor to the structure of the Serving Cells. Ultimately, this led e.g. to the problem that BWPs could not be used to switch a RedCap UE between several narrow channels within a wider carrier. The reason was that the “channel BW” configuration landed outside the BWP configuration and inside the ServingCellConfigCommon branch that could not be modified by dedicated signalling. Similarly, the introduction of various multi-TRP schemes required configuring additional PDCCHs and/or PDSCHs which were in the same frequency band but (sometimes) associated with a different cell, i.e., with an another SSB carrying another PCI. At this point one may note that the Serving Cell lost most of its original meaning. Channels and signals that are grouped in a Serving Cell don’t share the same frequency-domain bandwidth/position, the same numerology nor the same SSB as root QCL reference.

While hierarchical groups (Serving Cells, BWPs, …) intended to inherit common properties to their child elements (channels, signals, …) there are typically exceptions when those default inheritance isn’t applicable.

Grouping in multiple hierarchical levels also complicated cross-referencing between IEs in different branches, which is needed especially when functionality deviates from the hierarchy. Examples can e.g. be found for cross carrier scheduling where PDCCH CORESETs and SearchSpaces in different Serving Cells had to be linked to each other. Since IDs were declared unique only within the scope of a serving cell, references had to comprise of two IDs.

Based on the discussion and examples above we observe the following:

Hierarchically structured configuration messages hinder the introduction of new functionality and thereby drive complexity and development costs.

To avoid running into the same problems in 6G, we suggest not to group/structure the building blocks of the 6G Uu interface (physical channels & signals, protocol entities, …) by any physical- or protocol-related properties. In other words, we suggest not to squeeze functionality into “cell groups”, “serving cells” or “bandwidth parts” but rather to seek for a signalling structure that ensures efficient signalling.

6G’s ASN.1 structure should not group configuration IEs (for physical channels/signals, measurement reports and protocol entities) by physical- or protocol-related properties. Instead, 6G’s ASN.1 signalling structure should strive for signalling efficiency, usability and extensibility.

As a concrete example we could envision a structure for the configuration message (RRCSetup, RRCReconfiguration) which groups elements only by their high-level types and allows configuring those in lists:

ConnectionConfiguration ::= SEQUENCE { referenceSignalSetList ReferenceSignalSetList OPTIONAL, reportConfigList ReportConfigList OPTIONAL, rateMatchingPatternSets RateMatchingPatternSetList OPTIONAL, physicalChannelList PhysicalChannelList OPTIONAL, harqEntityList HarqEntityList OPTIONAL, radioBearers RadioBearerList OPTIONAL, ... }

Elements within these lists express their properties either explicitly or by referencing another element in the same or a different list.

For example, a PDSCH configured in the physicalChannelList could define its position in frequency domain explicitly (ARFCN, SCS, PRB offset). It could also refer to an SSB configured in the referenceSignalSetList as its QCL reference. A PDCCH (also configured in the physicalChannelList) should in addition refer to a PDSCH that it will schedule.

Since this flat structure does not group elements by physical properties (frequency-domain positions, QCL reference signals, …) or protocol relations (PDCCHPDSCH), it allows configuring functionality that was cumbersome to realize with NR’s hierarchical structure. For example, …

- If the UE supports “CA”, the network may configure two PDSCH entries with different frequency domain positions. In case of “inter-cell multi-TRP” there could be two PDSCHs with the same frequency domain positions but referring to different SSBs (with different PCIs) in the referenceSignalSetList.

- To configure cross-carrier scheduling a PDCCH configuration has a different frequency domain position than the PDSCH that it schedules. If a PDCCH is configured at the same frequency domain position as the PDSCH that it schedules this resembles to traditional same-carrier scheduling.

A flat ASN.1 structure (no hierarchical grouping by seemingly common properties) allows configuring features/functionality flexibly, in accordance with the functionality that the UE supports.

Since there is intentionally no hierarchy, dependencies must be expressed by references to other elements in the same or different list. NR made heavy use of such IDs (used both for AddMod lists but also for cross-referencing). We will discuss how to further improve this in the section about delta signalling.

## 2.2	Critical Extensions

Another weakness of LTE’s and NR’s configuration signalling is the almost exclusive use of non-critical extensions, i.e., the addition of parameters to the end of a previously introduced information element. To facilitate that, RAN2 defines most fields in an IE as OPTIONAL which allows omitting the field when it becomes obsolete for the functionality of a later release.

After a couple of releases many of the information elements (PDSCH-Config, PDCCH-Config, PUSCH-Config, …) comprise of many optional fields which suggests a great configuration flexibility. However, it is often ambiguous which combination of values is legitimate. RAN2 tries to capture constraints in field descriptions, condition tables or procedural text. But those are typically not complete and not easy to test and validate.

Continued non-critical extensions of large IEs tend to make it ambiguous which combination of parameters and values is valid.

The use of a critical extension could be a viable alternative in such cases. The new version of an IE type (e.g. PDSCH-Config-r22) would only contain the fields required to configure this new PDSCH version.

However, critical extensions had far-reaching consequences in NR’s ASN.1 structure. One reason is that we typically configure the original IE type in AddMod-lists:

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet OPTIONAL,

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId OPTIONAL,

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace OPTIONAL,

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId OPTIONAL,

Therein, each element was associated with an ID that was used within the original IE type:

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

Had we defined a critical extension of NR’s IE (e.g. SearchSpace-r19) one would have had to define at least a new AddMod list for that new type. UE and NW would then need to handle those parallel lists in parallel including special case where an element is released from one list and added to the other or where it may mistakenly exist in both lists.

Furthermore, RAN2 tends to restrict the size of the IDs to the bare minimum required for the original version of the IE type. Consequently, also a new ID type had to be introduced in some cases (e.g. ControlResourceSetId-r16 replacing the original ControlResourceSetId). This can have far reaching impact on other parts of the configuration structure if other IEs (e.g. SearchSpace) used the original ID to refer to the original ControlResourceSet. In such cases, those other IEs (e.g. SearchSpace) would need to use (also) the new ID type (ControlResourceSetId-r16) to refer to instances of the new IE type. This would cause additional complexity and ambiguity in those other IE types, too.

We suspect that these were the primary reasons why RAN2 used critical extensions of IEs only rarely despite the potential benefits outlined above.

For 6G RAN2 should develop a configuration messages structure that is easy to use and extend. For example, RAN2 could prepare the later use of critical extensions by defining “abstract super-types” of typical configuration IEs (e.g. PDSCH-Config) already in the first version of the specification. In NR and LTE such types were introduced primarily on the top level of messages. But critical extensions on that level would have far reaching impact on product implementations and were hence not used in practice.

If sufficiently large IDs refer to those super-type elements, critical extensions (new variants of the super-types) can be introduced later without affecting other parts of the specification.

The following ASN.1 snippet shows how this could be realized for a PDSCH configuration IE:

PDSCH ::= CHOICE { pdsch-r21 PDSCH-r21, pdsch-r23 PDSCH-r23, spare1 NULL, ... } PDSCH-Id ::= INTEGER(0..31) PDSCH-List ::= SEQUENCE(SIZE(1..32)) OF PDSCH

6G’s ASN.1 configuration structure should accommodate for critical extensions of lower-level configuration IEs. Whether to extend critically or non-critically shall be decided on a case-by-case basis.

## 2.3	Delta signalling

To reduce the downlink signalling overhead, LTE and NR supported delta signalling. Traditionally, RAN2 assumes that the use of delta signalling is the default for reconfigurations within a gNB but also for mobility across gNBs.

RAN2 specifies delta signalling by capturing what a UE shall do if ASN.1 fields were previously configured but are absent in a subsequent RRC reconfiguration message. Those rules are captured in need codes (comments in the definitions in the ASN.1 syntax), field descriptions, condition tables and procedural text. While this seems simple at a first glance, it turns out to be ambiguous and impossible to automate and hence the use of delta signalling results in errors in implementations and IODT testing.

LTE’s and NR’s ASN.1 delta signalling cannot be automated since rules are captured in need codes, field descriptions, condition tables and procedural text.

Those challenges led to the unfortunate situation that delta signalling can only be used in limited scenarios in practice. Especially for inter-node mobility where small configuration messages would have been particularly beneficial delta signalling is challenging. The reason is that inter-node support would require that a target gNB parses an incoming UE configuration produced by the source node and determines unambiguously which of the child branches/fields therein it needs to release. While this would be possible for fields corresponding to functionality supported by this target gNB, it isn’t feasible for all other fields that this gNB does not support but that a source node might have used.

NR’s delta signalling isn’t feasible for inter-node reconfigurations since the target node would need to support releasing features which it does not support itself.

It should be noted that a UE does not face the same problem since it may assume that a gNB configures only those fields that the UE supports according to its UE capabilities. Nevertheless, implementing NR’s or LTE’s delta signalling is extremely challenging also for UEs. And in cases where we try to use delta signalling for (intra-gNB) reconfigurations we have noticed that there are often UEs/chipsets that fail to process those messages correctly.

NR’s way of supporting delta signalling by means of need-codes is error-prone and results in error cases in live networks despite extensive IODT.

6G should therefore use a simple yet efficient in-line definition of delta signalling. It is vital that the delta signalling rules (which fields to release, which fields to keep) are embedded in the ASN.1 itself so that they survive the ASN.1 compilation, i.e., that they remain visible in the resulting source code. This would allow UE- and network implementations to automate the handling of delta signalling and thereby remove today’s ambiguity. Special attention should also be given to forward compatibility, i.e., to the case where the source gNB configured fields for a feature which the target node does not implement. Preferably, delta signalling should be specified such that the target gNB can still generate a delta configuration that is syntactically correct and by which the UE releases those features.

6G’s delta signalling should be specified in a machine-readable manner to avoid ambiguity and implementation errors. 6G’s delta signalling should support inter-node reconfigurations with reasonable implementation complexity for UEs and NWs.

While this sounds ambitious at a first glance, we believe that those goals could be achieved with only a few changes compared to NR’s and LTE’s delta signalling:

- Don’t use need codes in comments, field descriptions, conditions and procedural text since those made delta signalling non-automatable and error prone.

- Instead, specify that the UE releases all OPTIONAL fields if they were configured before and are absent in a subsequent reconfiguration. In other words, the UE treats all fields like NR’s “Need R” and hence there is no need to tag fields by ASN.1 comments. This simple rule ensures that the UE releases all parts of the source configurations unless the target node instructs it explicitly to keep them (see next bullet).

- Introduce parameterized types (e.g. one for individual elements, one for (large) lists) which contain a CHOICE structure by which the network can instruct the UE to keep the previous configuration or set/change it. Since the CHOICE’s options “survive” the ASN.1 compiler, the NW’s and UE’s application logic could detect which branches of the configuration are subject to delta signalling and automate the handling thereof.

For individual fields the above-mentioned parameterized type could look as follows:

-- A parameterized type to enable delta signalling of individual fields.

SetKeep {ElementType} ::= CHOICE {

keepElementUnchanged NULL,

configureElement ElementType

}

The ElementType could e.g. be a “PDSCH-Config” or a “RadioBearerConfig”. If the network sets the CHOICE to "keepElementUnchanged", the UE keeps the field as it was previously configured. If network sets the CHOICE to "configureElement", the UE applies the given configuration.

If this parameterized type is applied to an OPTIONAL field and if that field is absent the UE releases the entire field if it was configured before (like for all optional fields, see bullet 2 above).

The SetKeep appears similar to the SetupRelease structure which NR used in combination with fields marked “Need M”. While the NR gNB had to instruct the UE explicitly to release the previously configured element (by including the element and setting the CHOICE to “release”), the 6G UE would release the optional field by default if the field is absent. The 6G NW must tell the UE explicitly to keep the contained element by setting the SetKeep CHOICE to “keepElementUnchanged”.

The signalling overhead will be similar:

- NR: 1 bit for keeping the element, 2 bit for releasing it

- 6G: 2 bit for keeping the element, 1 bit for releasing it

Delta signalling is considered important for lists with many and/or large elements in which the network needs to add, change or remove elements. In NR this was handled by so-called AddMod/Release lists. A dedicated section in the RRC specification describes how they should be used. But due to the way the ASN.1 syntax was built, every occasion required case-specific handling anyway.

For 6G RAN2 should consider introducing a parameterized type which aligns the realization of such lists and allows automating an implementation thereof. An example could look as follows:

SetKeepList {ElementType, INTEGER:maxNrofElements, ElementIdType} ::= CHOICE {

keepListUnchanged NULL,

configureElements SEQUENCE (SIZE (1..maxNrofElements)) OF SEQUENCE {

elementId ElementIdType,

element ElementType OPTIONAL

}

}

Like in the single-element example above, the ElementType determines the IE type that may be configured in a specific instance of this list (e.g. be PDSCH-Config). maxNrofElements determines the maximum number of elements in the list and the ElementIdType should be set to a (INTEGER) type by which the ElementType can be referenced, e.g. PDSCH-ConfigId.

If the network sets the CHOICE to "configureElements" the SEQUENCE refers to one or more elements by their "elementId". The UE sets or modifies the elements for which the field "element" is present. The UE removes previously stored elements for which the field "element" is absent. The UE keeps elements unchanged if they were configured before and if their elementId is absent in the given SEQUENCE.

If the network sets the CHOICE to "keepListUnchanged", the UE keeps all previously configured elements unchanged.

And as explained in bullet 2 above, the UE shall release the entire list (all previously configured elements) if the OPTIONAL field that uses this parameterized type is absent.

As explained above, we aim for a flat structure (see section 2.1) in which we group configurations of individual components only on a logical basis. For example, one could group all physical channels in a common list. To be able to introduce new physical channels as well as critical extensions of existing physical channels (see section 2.2) in later releases, one could define an abstract super-type “PhysicalChannel” as follows:

PhysicalChannel ::= CHOICE {

pdcch PDCCH-Config,

pdsch DynamicPDSCH-Config,

pusch-CPOFDM NULL,

pusch-DFTS NULL,

spare3 NULL,

spare2 NULL,

spare1 NULL,

...

}

PhysicalChannelId ::= INTEGER(0..127)

PhysicalChannelList ::= SetKeepList {PhysicalChannel, 128, PhysicalChannelId}

As explained before, the SetKeepList associates each PhysicalChannel configuration (regardless which specific type) with a PhysicalChannelId. The ID is used for delta signalling but may also be used to refer to a PhysicalChannel from other parts of the configuration (e.g. the PDCCH configuration may include the PhysicalChannelId of the PDSCH that it schedules).

Note that the PhysicalChannelList has already been used in the flat configuration structure exemplified in section 2.1. We repeat it here for convenience:

ConnectionConfiguration ::= SEQUENCE { referenceSignalSetList ReferenceSignalSetList OPTIONAL, reportConfigList ReportConfigList OPTIONAL, rateMatchingPatternSets RateMatchingPatternSetList OPTIONAL, physicalChannelList PhysicalChannelList OPTIONAL, harqEntityList HarqEntityList OPTIONAL, radioBearers RadioBearerList OPTIONAL, ... }

The other lists (ReferenceSignalSetList, ReportConfigList, …) could be defined according to the same principle as the PhysicalChannelList.

## 2.4	xxxCommon vs. xxxDecicated

In IDLE and INACTIVE state, the UE operates in accordance with the configuration that it obtained from MIB and SIB1 (ServingCellConfigCommonSIB).

In NR the UE specific RRCSetup/Reconfiguration builds on top of the IDLE/INACTIVE mode configuration that the UE obtained via MIB/SIB1

When the UE enters CONNECTED state, the gNB provides a UE specific configuration in the RRCSetup message and revises that later by an RRCReconfiguration. However, those UE specific configurations were built around/on-top of the configuration that the UE obtained from SIB1. For some of those parameters, the network has the possibility to override them by UE specific parameters. But other parameters such as the root SSB’s PCI can only be changed by a “handover”, i.e., if the NW includes the reconfigurationWithSync field which triggers also several other actions. Inside this field the network provides the ServingCellConfigCommon IE which is similar (but different) to the ServingCellConfigCommonSIB that the UE acquired from SIB1. Here, the gNB seemingly gets the opportunity to override cell-specific parameters on a UE specific basis. However, the UE would override those again when reading SIB1 subsequently.

In NR the dependencies between the common (IDLE) and dedicated (CONNECTED) mode configuration are partially ambiguous and tend to hinder UE specific configurations.

In 6G there may still be cases where a connected UE should obtain updated configuration parameters from SIB1. But to avoid the ambiguities discovered above, we suggest that the dedicated configuration for a connected UE is by default independent of the IDLE mode configuration broadcast in SIB1. If the network wants the UE to use a parameter that might be updated in SIB1, it should explicitly indicate that in the dedicated UE configuration. E.g. the gNB could configure an SSB (time/frequency position, PCI, …) in the UE specific configuration message and tell the UE that it shall (re-)read SIB1 to obtain the SSB’s up-to-date TX power level.

By default, the connected mode configuration should be independent of the UE’s IDLE mode configuration obtained via MIB/SIB1. The network may configure the UE explicitly to acquire selected parameters from system information and to re-acquire it if system information changes.

## 2.5	Modularization

Several companies mentioned the need for better modularization of the RRC signalling in 6G. We agree that NR’s RRC ASN.1 modules grew too large. But depending on the actual pain points, the solutions may be different.

It may for example be beneficial to break the ASN.1 definitions into several/many smaller *.asn files. This would be valuable especially if 3GPP decides to capture specifications in tailor-made file formats (markdown- ASN, JSON, …) rather than in DOCX containers. If the ASN.1 definitions are meant to be split into several files, it seems advisable to defined one ASN.1 module per file. Thereby, the ASN.1 syntax and validation of required imports can be validated automatically (no need to pre-merge files before ASN.1 syntax checking). We agree that such kind of modularization would ease the specification development.

Defining the RRC ASN.1 structure in many smaller ASN.1 modules and capturing them in individual *.asn files simplifies their development and maintenance.

However, the top level ASN.1 messages (e.g. ConnectionConfiguration defined in one module) will reference to the lower-level configuration IEs that might be defined in other modules (e.g. ReferenceSignalSetList, PhysicalChannelList). Hence, the source code generated by the ASN.1-compiler will anyway contain the classes for (almost) all ASN.1 defined across the different modules. In other words, the footprint of the generated source code and the ultimately compiled binary will likely not benefit much from such modularization. RAN2 should investigate whether there are anyway means to force an ASN.1 compiler to skip selected IE types that are known not to be supported by a specific UE- or NW implementation. Alternatively, IE types of other modules could not be referenced explicitly but rather only within an OCTET STRING. This may be feasible for selected cases but the additional overhead would likely prohibit such approach in lower level lists.

Splitting the ASN.1 structure into many modules may not result in smaller binaries if the top-level messages import and reference all lower-level IE types anyway. RAN2 should investigate this further.

In theory RAN2 could also design and maintain several variants of the top-level messages (e.g. RRCReconfiguration-MBB, …-MIOT, …-XR, …-simple, …-advanced). Those variants could import only those child IE types that seem relevant for the envisioned purpose of the specific top-level type. Depending on the technical realization, this may reduce the binary-footprint of the variants that supports (significantly) fewer configuration options. Technically, this approach would be similar to a critical extension of top-level messages and therefore it would inherit the same problems: Networks would need to implement and evolve all variants in parallel and configure UEs according to the configuration options allowed by that signalling structure and by the UE capabilities. More importantly, 3GPP would continuously discuss for each feature and for each enhancement in which of the different top-level messages this feature could be configured and in which one it could not. Those discussions would likely be time-consuming since the proponents of a new feature would likely insist that the feature could be used by any UE whereas others might want to limit it to a specific top-level type. It seems likely that restrictions would later be revisited so that an older feature (or variant thereof) could be used via another top-level configuration IE. Obviously, this would result in market fragmentation and make it even less likely that features of later releases are deployed in practice.

Creating several top-level configuration IEs (e.g. variants of RRCReconfiguration) that support/preclude different functionality, would consume a lot of discussion time in 3GPP, a lot of implementation overhead in products and lead to further market fragmentation which makes it less likely that features are implemented in practice.

Discuss which practical problems arise from NR’s large ASN.1 modules and thereafter seek for a solution that does neither cause market fragmentation nor increases development efforts.

# 3	Conclusion

In this paper we have reached these observations:

Observation 1	While hierarchical groups (Serving Cells, BWPs, …) intended to inherit common properties to their child elements (channels, signals, …) there are typically exceptions when those default inheritance isn’t applicable.

Observation 2	Hierarchically structured configuration messages hinder the introduction of new functionality and thereby drive complexity and development costs.

Observation 3	A flat ASN.1 structure (no hierarchical grouping by seemingly common properties) allows configuring features/functionality flexibly, in accordance with the functionality that the UE supports.

Observation 4	Continued non-critical extensions of large IEs tend to make it ambiguous which combination of parameters and values is valid.

Observation 5	LTE’s and NR’s ASN.1 delta signalling cannot be automated since rules are captured in need codes, field descriptions, condition tables and procedural text.

Observation 6	NR’s delta signalling isn’t feasible for inter-node reconfigurations since the target node would need to support releasing features which it does not support itself.

Observation 7	NR’s way of supporting delta signalling by means of need-codes is error-prone and results in error cases in live networks despite extensive IODT.

Observation 8	In NR the UE specific RRCSetup/Reconfiguration builds on top of the IDLE/INACTIVE mode configuration that the UE obtained via MIB/SIB1

Observation 9	In NR the dependencies between the common (IDLE) and dedicated (CONNECTED) mode configuration are partially ambiguous and tend to hinder UE specific configurations.

Observation 10	Defining the RRC ASN.1 structure in many smaller ASN.1 modules and capturing them in individual *.asn files simplifies their development and maintenance.

Observation 11	Splitting the ASN.1 structure into many modules may not result in smaller binaries if the top-level messages import and reference all lower-level IE types anyway. RAN2 should investigate this further.

Observation 12	Creating several top-level configuration IEs (e.g. variants of RRCReconfiguration) that support/preclude different functionality, would consume a lot of discussion time in 3GPP, a lot of implementation overhead in products and lead to further market fragmentation which makes it less likely that features are implemented in practice.

We propose:

Proposal 1	6G’s ASN.1 structure should not group configuration IEs (for physical channels/signals, measurement reports and protocol entities) by physical- or protocol-related properties. Instead, 6G’s ASN.1 signalling structure should strive for signalling efficiency, usability and extensibility.

Proposal 2	6G’s ASN.1 configuration structure should accommodate for critical extensions of lower-level configuration IEs. Whether to extend critically or non-critically shall be decided on a case-by-case basis.

Proposal 3	6G’s delta signalling should be specified in a machine-readable manner to avoid ambiguity and implementation errors. 6G’s delta signalling should support inter-node reconfigurations with reasonable implementation complexity for UEs and NWs.

Proposal 4	By default, the connected mode configuration should be independent of the UE’s IDLE mode configuration obtained via MIB/SIB1. The network may configure the UE explicitly to acquire selected parameters from system information and to re-acquire it if system information changes.

Proposal 5	Discuss which practical problems arise from NR’s large ASN.1 modules and thereafter seek for a solution that does neither cause market fragmentation nor increases development efforts.

Note that this is just meant as an example. It may be beneficial to specify frequency domain positions in another list and to refer to those entries from various physical channels.