3GPP TSG-RAN WG2 Meeting #132	R2-2508616

Dallas, USA, 17th –21st November, 2025

Agenda Item:	10.2.1

Source:	Huawei, HiSilicon

Title:	Discussion approach for 6G UE capability

Document for: Discussion and Decision

1	Introduction

In last RAN2 meeting, there are following agreements on 6GR UE capability framework [1].

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this contribution, we first analyse the issues for existing UE capability framework in NR, and discuss the potential directions to address the issues in 6GR.

2	Discussion on issues related UE capability

2.1	UE capability signalling framework

Over the years, the UE AS capability size has increased. In 5G NR, at most 16 RRC segments for UE capability information can be transferred over the air interface. The large capability size is a burden for storage for the RAN and the CN, as well as considerable signalling overhead on the N2/Xn interface. The processing complexity for RAN is also not negligible, especially considering the increasing number of bands/band combinations (BC). In addition, when the maximum number of segments allowed by the network is smaller than the size supported by the UE, the UE is not able to reports its full capability. For example, the UE has to cut off the reported capability from 3-band combinations to 2-band combinations. It would result in degradation of user experience and network/UE performance.

In a word, a large capability size will not only bring a burden to the network on storage, transmission and processing, but also restricts the performance improvement on high-end UEs and networks.

Observation 1: The large UE capability size burdens the network in terms of storage, transmission, and processing, limiting performance improvements for high-end UEs and networks.

Based on the analysis of the UE NR capability in field, following issues on capability signalling framework need to be addressed in 6GR.

- Issue#1: Over-classification of capabilities into finer granularity brings signalling overhead.

One issue on signalling framework is the granularity of capabilities. In 5G, various granularities are defined in the specification, such as per UE, per band, per BC, per band per BC and per CC. These granularities are a good starting point for 6GR. However, when classifying the granularity for a specific feature (including RAN1/RAN2/RAN4 features), it is important to consider that a finer granularity can offer better flexibility for different UE implementations but can also lead to increased signalling overhead. Unnecessarily classifying capabilities into finer granularity can result in additional signalling overhead.

In some cases, multiple granularities are defined together for a feature (e.g., per band and per BC, or, per CC and per BC). Such capability signalling with multiple granularities have been used in 5G for some MIMO features and baseband capability (i.e. aggregated bandwidth). It brings benefit to provide better flexibility. However, it provides some complexity on usage of these capabilities, e.g. how to understand the prerequisite, relationship between each other, etc. A clear and unified principle should be defined from the beginning, to avoid any ambiguity and potential non-backward compatibility issues.

Observation 2: Balancing UE flexibility and signaling overhead is crucial when determining the granularity of capabilities. Over-classifying capabilities into smaller units results in increased signaling overhead.

Besides, we observed the capability related to band combinations occupy more than 90% of the whole message size. To figure out where the overhead comes from, we summarize different categories of band combinations according to the capability information collected in the field.

Table-1 Percentage of different categories of BCs in the total number of BCs

| UE model | CA combinations with different DL bands(e.g. band A+band B, band A+band C, band B+band C) | CA combinations with same DL bands but paired with different UL bands (e.g. band A(DL 1CC/UL 1CC)+Band B(DL 2CC), Band A(DL 1CC)+Band B(DL 2CC/UL 1CC) | Different fallback CA combinations | Uplink Tx switching combinations |
| --- | --- | --- | --- | --- |
| Model#1 | 56% | 26% | 18% | + 25% (additionally calculated on top of CA combinations) |
| Model#2 | 62% | 25% | 13% |  |
| Model#3 | 43% | 28% | 29% |  |

Based on the table above, we identified the issues below on signalling framework of band combinations.

- Issue#2: The number of CA band combinations exploded with the increased number of frequency bands.

According to the table above, 40% to 60% of the band combination entries differ from the DL CA band combinations specified by DL bands and DL bandwidth classes as defined in TS38.101-1. As RAN4 introduces more frequency bands and more bands in one band combination, the number of enumerated CA band combinations increases.

Observation 3: The growing number of frequency bands listed for carrier aggregation has led to an explosion in the number of band combinations.

- Issue#3: The pairing between DL and UL capability cannot be indicated flexibly.

According to the table above, over 20% of band combinations come from pairing of DL and UL bands. For example, in cases where the DL CA combination is associated with a single UL band, the pairing of that UL band with each individual DL band needs to be indicated explicitly. However, the NR signalling structure requires coupling of DL feature set and UL feature set for a band, and some UL capabilities defined per-BC level (e.g. SRS capability) also restrict the flexibility to reuse the same BC entry. These issues lead to additional BC reporting to indicate asymmetric capability between DL and UL.

Observation 4: The lack of flexibility in the DL and UL capability structure, such as the coupling of DL/UL feature sets and UL-specific capabilities at the per BC level, leads to increased BC reporting requirements.

- Issue#4: Separate BC lists for CA and uplink Tx Switching results in redundancy.

In 5G, separate BC lists for CA and uplinkTxSwitching are defined to prevent non-backward compatibility issues. However, the relationship between these two BC lists is not specified. While most features are equally supported across the BC lists, additional BC entries must be included in the capability information for uplink Tx Switching. These additional band combinations can increase the number of BC entries by over 20% compared to the number of CA BCs. The support of different types of spectrum aggregation introduces redundancy in capability reporting in 5G.

Observation 5: Separate BC lists for CA and uplink Tx Switching results in redundancy in capability reporting.

Based on the analysis above, we believe that addressing the signalling structure of band combinations should be the primary focus in order to reduce signalling overhead in 6G. Any enhancement on capability size should be studied with the objective of making signalling efficient.

2.2	RACS

UE capability ID (RACS) was introduced in 5G to reduce signalling overhead. There are two kinds of capability ID specified: manufacturer-assigned capability ID and PLMN-assigned capability ID. The idea of the RACS is to use one identifier for the set of capabilities of a UE model. However, RACS is not deployed in 5G networks. One reason is that the number of capability sets that more than one capability ID would be needed due to reasons such as different user settings, different software versions or other implementation-specific factors (e.g. multi-SIM, over-heating). Therefore, for a single UE model, multiple capability sets are needed. The table-2 shows our analysis on the potential number of capability sets for a single UE model.

In addition, we observed that for capability sets of the same UE model, there is little variation in the supported band combinations (BCs). The main variations are in non-BC capabilities, such as voiceOverNR and ROHC profiles.

Table-2 A potential number of capability sets for a single UE model

|  | User settings (e.g. low-power mode, high-performance mode) | Software versions | Other implementation-specific factors(e.g. MU-SIM, over-heating) | Total number of capability sets for one model in theory |
| --- | --- | --- | --- | --- |
| Number of capability sets | 4 | 4 | 6 | 96 (4*4*6) |

Observation 6: A single UE model can have multiple capability sets due to factors like user settings, software versions, or other implementation-specific factors (e.g. multi-SIM). The primary differences among these sets lie in the non-BC capabilities.

Besides, obtaining the manufacturer-assigned UE capability ID requires effective collaboration among operators, core network vendors, radio network vendors, UE vendors, and chipset vendors, which is difficult to achieve in practice. Managing and maintaining a large amount of capability IDs is also a significant difficulty. In comparison, the PLMN-based capability ID, allocated by the core network (i.e. UCMF) for identical UE capability sets, does not require such offline collaboration. However, this mechanism is considered less valuable in reality because full capability reporting cannot be avoided after each UE registration, and the filter-based capability reporting mechanism reduces the possibility of reusing the same capability ID for different UEs. In addition, the number of capability sets for one UE model can be substantial based on our evaluation above.

Observation 7: RACS is not supported in commercialization due to the challenges in managing and maintaining capability IDs among operators, core/radio network vendors, UE vendors and chipset vendors.

In our opinion, the difficulty is in commercializing RACS, regardless of its support in the initial 5G release. Rather than focusing on RACS, RAN2 should study a closed-loop mechanism in 3GPP, with a limited number of capability IDs.

2.3	UE capability update

Capability restriction/update based on UE Assistance Information (UAI) was specified in NR for scenarios like overheating, power saving and MU-SIM. However, this solution has not yet been successfully commercialized. The main reason for this lack of success is that capability restriction may have a negative impact on communication performance, resulting in limited network interest in supporting this feature. To address UE-related issues like overheating in 6G, it is necessary to consider the mutual interests (e.g. power saving) of both UEs and networks.

On the other hand, since UE behaviour regarding such capabilities is usually implementation-dependent, without specification requirements mandating UEs to only request capability restriction/updates for specific use cases (e.g., overheating or MU-SIM), the network cannot determine how important the request of a UE is.

Observation 8: The commercialization of capability restriction/updates based on UE Assistance Information (UAI) was not successful in 5G, due to the following issues:

- Lack of consideration of the mutual interests of both UEs and networks.

- The inability to verify the real cause of the UE’s behaviour when requesting capability restrictions, which could lead to misuse, where UEs improperly reduce their capabilities.

- Capability restriction/updates may significantly affect communication performance, making it difficult to meet the QoS requirements in some scenarios.

3	Discussion approach for UE capabilities

Based on the observations above, we believe RAN2 should first study the issues related to the UE capability framework and identify existing drawback to avoid repeating the same issues of 5G. In addition, since the discussion on the UE capability framework relies heavily on the analysis of 6G features and functions across all the RAN working groups, the development of mechanism and solutions for capability enhancement can be deferred until after substantial progress has been made on 6G features.

Proposal: For UE capability study at current stage, problems/issues identification are more important than debating on solutions/enhancements:

- RAN2 should initially focus on identifying issues observed in NR UE capabilities (see above 8 observations).

- Capability enhancement discussions can be postponed to a later phase in the 6G study, once 6G feature studies have made sufficient progress.

# 4	Conclusion

This contribution makes following observations and proposal for UE capability:

UE capability signalling framework

Observation 1: The large UE capability size burdens the network in terms of storage, transmission, and processing, limiting performance improvements for high-end UEs and networks.

Observation 2: Balancing UE flexibility and signaling overhead is crucial when determining the granularity of capabilities. Over-classifying capabilities into smaller units results in increased signaling overhead.

Observation 3: The growing number of frequency bands listed for carrier aggregation has led to an explosion in the number of band combinations.

Observation 4: The lack of flexibility in the DL and UL capability structure, such as the coupling of DL/UL feature sets and UL-specific capabilities at the per BC level, leads to increased BC reporting requirements.

Observation 5: Separate BC lists for CA and uplink Tx Switching results in redundancy in capability reporting.

RACS

Observation 6: A single UE model can have multiple capability sets due to factors like user settings, software versions, or other implementation-specific factors (e.g. multi-SIM). The primary differences among these sets lie in the non-BC capabilities.

Observation 7: RACS is not supported in commercialization due to the challenges in managing and maintaining capability IDs among operators, core/radio network vendors, UE vendors and chipset vendors.

UE capability update

Observation 8: The commercialization of capability restriction/updates based on UE Assistance Information (UAI) was not successful in 5G, due to the following issues:

- Lack of consideration of the mutual interests of both UEs and networks.

- The inability to verify the real cause of the UE’s behaviour when requesting capability restrictions, which could lead to misuse, where UEs improperly reduce their capabilities.

- Capability restriction/updates may significantly affect communication performance, making it difficult to meet the QoS requirements in some scenarios.

Proposal: For UE capability study at current stage, problems/issues identification are more important than debating on solutions/enhancements:

- RAN2 should initially focus on identifying issues observed in NR UE capabilities (see above 8 observations).

- Capability enhancement discussions can be postponed to a later phase in the 6G study, once 6G feature studies have made sufficient progress.

# 5	References

- Draft Meeting Report for RAN2#131-bis