3GPP TSG-RAN WG2 Meeting #132	R2-2508618

Dallas, USA, 17th –21st November, 2025

Agenda Item:	10.3.2.2

Source:	Huawei, HiSilicon

Title:	Discussion on RRC signalling design

Document for: Discussion and Decision

1	Introduction

In the last meeting, RAN2 made some progress on RRC signalling design for 6G and achieved the following agreements.

Agreements on RRC signalling design and reconfigurations:

1.       ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

2.       Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

3.       Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g., improvements to delta signaling or no delta signaling).

4.       RAN2 will study modular design of RRC for 6G. Further study how to modularize RRC e.g., based on features, functions, verticals, etc.

In this contribution, we further discuss the 6G RRC signalling design and reconfiguration, focusing on the RRC modular design and delta configuration.

2	Discussion

2.1	RRC modular design

During the last meeting, RAN2 reached consensus on studying modular design of RRC for 6G, though details (e.g., granularity) require further study. The core principle of RRC modular design is to group RRC configurations belonging to the same "granularity" into a single module. In this way, the network can include only the configurations of specific modules in UE-specific messages, avoiding unnecessary overhead due to other undesired modules (e.g., the presence bits for optional fields). Even when taking into account the overhead for modularization (e.g., the length indications for each module), the overall RRC signalling overhead is still reduced, particularly considering that only roughly 1/10 features are commercially deployed in NR. Naturally, RRC module design should also strive to minimize the overhead introduced by the modularization itself. RRC modular design should focus on how the RRC configurations are organized, and should not have any negative impacts on features or functions themselves, such as their performance. For instance, RRC design for coverage enhancement module should not impose any limitations on the coverage performance.

Proposal 1:	RRC modular design should not have negative impacts on the performance of features.

The current RRC signaling design interweaves IEs for different features, as illustrated in Figure 1:

Fig.1 Interwoven structure of NR IEs

The drawbacks we observed in the current RRC interwoven structure in NR are listed below:

- Implementing RRC IEs for one specific feature necessitates understanding of the entire RRC structure, including IEs for non-relevant features. For example, in order to implement the RRC IEs for feature Z (e.g., IE#3, IE#6 and IE#7), the transmitter/receiver must support the encoding/decoding operations for preceding IEs, including IE#1 and IE#4 for feature X, and IE#2 and IE#5 for feature Y, since the RRC IEs are encoded/decoded sequentially. This increases RRC implementation complexity and memory cost, particularly for low-end UEs which are cost-sensitive. High-end UEs support only about 10-20 out of 150 commercial features in NR, while low-end UEs may support much fewer features. The additional cost in RRC memory due to the interwoven structure of IEs is considerable. In additional, to signal IEs for a set of features, many additional bits are needed to indicate the absence of IEs for non-relevant features. This also introduces non-negligible costs.

- The interwoven structure degrades handover performance. Currently, if the UE configuration from the source gNB contains unrecognized configurations (e.g., due to version difference), the target gNB must include the full configuration in the handover command. This arises because under the interwoven structure, the target gNB cannot interpret the unknown configurations and thus cannot indicate the UE to release these configurations. As a result, full configuration must be used, leading to packet loss and service interruption.

- From the compatibility perspective, one of the lessons learned from NR is that compatibility issues cannot be completely avoided. During NR discussion, non-backward compatible changes in RRC signaling are only allowed in the very initial stage of each release of specifications (e.g., before -vXX.3.0). For a number of features which are not deployed in commercial networks, non-backward compatible changes can still be beneficial after the first frozen version because they are easier to read and with less signaling overhead. However, in the current interwoven structure, non-backward compatible changes for these features impact other features (e.g., commercialized features) in the same release and then need to be completely avoided after the first frozen version.

Observation 2-1:	The interwoven structure of NR RRC signalling design causes the following issues:

- Implementing IEs for a specific feature requires understanding of IEs for other features. Consequently, it is difficult for the UE to only implement IEs of supported features.

- The target gNB has to use full configuration when some IEs in the source configuration are not known during handover.

- Non-backward compatible changes are not possible after the first version, even the changes are specific for some non-commercial features.

When studying RRC modular design for 6G, RAN2 should learn lessons from NR and strive to avoid these issues from the beginning.

Proposal 2:	The RRC modular design for 6G should aim to:

- Support implementing only IEs of features to be developed in UE implementation;

- Reduce the use of full configuration in inter-BS handover cases due to version mismatch between BSs;

- Avoid compatibility issue from impacting across modules and support flexible updates and extensions of signaling.

One open issue from the last meeting is how to modularize RRC e.g., based on features, functions, verticals, etc. This needs further study, as companies provided various proposals during the last meeting.

Nevertheless, certain pieces of configuration will remain common regardless of the modularization principles and granularity. For example, the configuration for basic functions (e.g., basic cell/BWP/DRB configurations) must be supported by all devices regardless of the UE capabilities, so they can be included in the basic module. Then, the configuration for functions (e.g., multiple carriers, MIMO) used in typical deployment scenarios could also be included in the basic module. Other modules can add module specific configurations based on the basic module, as illustrated in Figure 2:

Fig. 2 Structure of RRC modular design

This basic module could be used to reduce the full configuration during inter-BS handover. As a possible solution, the target BS could fall back to the basic module if some pieces of configuration are not known by the target BS. The basic module defined in the initial version would remain stable and avoid unnecessary modifications. This would ensure that the basic module can consistently provide essential connectivity, even in failure cases such as reconfiguration failures.

Proposal 3:	For a modular design of RRC, RAN2 should study the design of a basic module that can contain the common configurations for other modules.

Regarding configurations beyond the basic module, how to split modules requires careful consideration.

One possibility is to split modules based on device types. However, UEs belonging to the same device type may support different features. For instance, different eMBB devices can support a significantly different sets of features. Therefore, RRC modular design based on different device types is not sufficient for reducing RRC memory cost.

Another possible option is to define RRC modules based on features, like NTN, XR or sensing, which could base on WIs in each release. Considering the way of RRC implementation, feature-based modular RRC design best aligns with the requirement to completely avoid unnecessary ASN.1 complexity and memory cost. However, there might also be functions used by multiple features so this may result in duplication configuration across features.

Therefore, another granularity to consider is function-based modules. RRC module specific to a given function (e.g., PDCP duplication, DRX, configured grants) can be beneficial for unified configuration and redundancy reduction, as multiple features may refer to the same configuration for a specific enhanced function.

In terms of signalling overhead, the granularity of modularization should consider both the benefit and the overhead introduced by modularization itself. For instance, modules only containing few fields/IEs may be aggregated into a single module, to reduce the modularization overhead. Also, as previously mentioned, the basic module can be used to provide a default configuration, enabling fallback in case of reconfiguration failure.

The next-level question would be how to group the configurations for different modules. Firstly, we assume there would a group of UE capabilities for a specific feature like today in [1] from RAN1 and RAN4. These UE capabilities can be explicitly mapped to configuration fields/IEs, and certain fields/IEs can be grouped into a module.

For later releases, we can group the configurations for different modules according to per-WI organization. There can be one or multiple new modules defined by a single WI, or some new configuration can be put into modules defined in previous releases.

However, for the first release in 6G, this would require some inter-WG inputs for RAN1/RAN4 specific IEs i.e., RAN1/4 needs to explicitly indicates the IEs and associated capabilities in their LS to RAN2 for capturing these IE.

Proposal 4:	For configurations beyond the basic module, RAN2 to study RRC modular design based on features, functions, or group of them. RAN2 to study the aspects including signaling overhead, UE memory requirement and coupling of features. The detail modular split depends on 6G feature design and needs coordination with RAN1/RAN4 for detail design in a later stage.

2.2	Delta configuration

Delta configuration is widely used in LTE and NR to reduce signalling overhead. The benefit of delta configuration is quite high when only limited configurations need to be updated in a large RRC message (e.g., RRCConnectionReconfiguration message in LTE and RRCReconfiguration message in NR). Similarly, 6G also need delta configuration for the same purpose. Otherwise, the network would need to provide full configurations every time, even when only a few updates are needed. Such full configuration operations not only increase signalling overhead, but may also introduce additional interruption time and data loss during configuration procedure.

Observation 5-1:	Delta configuration is still useful in 6G to reduce signalling overhead.

Delta configuration requires fields to be specified as OPTIONAL in the ASN.1. In NR, need codes, as ASN.1 comments or in conditional presence tables, are used to indicate the need for fields to be present in a message or an abstract type. The Figure 3 below shows the use of need codes, using ServingCellConfig IE in NR RRCReconfiguration message as an example.

Fig.3 The use of need codes in ServingCellConfig IE

Need codes and conditions can be categorized into the following two types:

- Type 1: The associated UE behaviour is explicitly defined only by the need codes. For example:

  - Need M: The field is stored by the UE (i.e., maintained) when absent.

  - Need R: The field is released by the UE when absent.

  - Need N: The UE shall take no action when the field is absent.

- Type 2: The associated UE behaviour also depends on additional constraints, e.g., whether the network behaviour is valid or not, or whether another field is configured properly. For example:

  - Need S: The additional constraint on defaultDownlinkBWP-Id is that the UE uses the initial BWP as default BWP when the field is absent. This kind of constraint introduces reference relationships with other fields (e.g., initialDownlinkBWP), requiring additional UE behaviour handling.

  - Conditions: The additional constraints defined in conditions normally restrict the network to configure or not configure the field in certain scenarios, i.e., the guideline of configuration for network. Possibly, this kind of constraint also introduces the dependency between fields. For example, the condition description of SyncAndCellAdd not only increases dependency between firstActiveDownlinkBWP-Id and other fields (e.g., reconfigurationWithSync), but also adds multiple requirements for checking validity of network configurations, from both UE side and network side.

| SyncAndCellAdd | This field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync and upon RRCSetup/RRCResume. The field is optionally present for an SpCell, Need N, upon reconfiguration without reconfigurationWithSync. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M. |
| --- | --- |

The second type is the main reason of the implementation complexity of delta configuration. In both UE and network implementation, extensive effort has been devoted to handling the Type 2 need codes/conditions, to avoid compatibility issues. However, evidence suggests that completely avoiding such compatibility issues is impractical.

Observation 5-2:	The need code introducing additional restraints (e.g., Need S) and conditions are the main causes of implementation complexity and compatibility issues in delta configuration.

In 6G RRC design supporting delta configuration, ASN.1 design should strive to minimize delta signalling processing complexity to allow the processing could be near-complete automation, minimizing compatibility risks introduced by human involvement. For example, RAN2 could try to decouple:

- The control of optional signalling (e.g., Need codes, the default value usage), which should remain in ASN.1 to explicitly indicate whether to keep or release the field; and

- Configuration guideline (e.g., explanation for the conditions) for network, e.g., the scenarios that the network should or should not configure particular fields, which could be specified beyond ASN.1 comments or left to network implementation.

Proposal 5:	RAN2 to study the improvement of delta configuration to reduce ambiguities and implementation complexity — for example, decoupling the handling of optional signaling (e.g., Need codes) from the configuration guideline information (e.g., explanations for relevant configuration conditions).

3	Conclusion

This contribution includes the following observations and proposals:

RRC modular design

Observation 2-1:	The interwoven structure of NR RRC signalling design causes the following issues:

- Implementing IEs for a specific feature requires understanding of IEs for other features. Consequently, it is difficult for the UE to only implement IEs of supported features.

- The target gNB has to use full configuration when some IEs in the source configuration are not known during handover.

- Non-backward compatible changes are not possible after the first version, even the changes are specific for some non-commercial features.

Proposal 1:	RRC modular design should not have negative impacts on the performance of features.

Proposal 2:	The RRC modular design for 6G should aim to:

- Support implementing only IEs of features to be developed in UE implementation;

- Reduce the use of full configuration in inter-BS handover cases due to version mismatch between BSs;

- Avoid compatibility issue from impacting across modules and support flexible updates and extensions of signaling.

Proposal 3:	For a modular design of RRC, RAN2 should study the design of a basic module that can contain the common configurations for other modules.

Proposal 4:	For configurations beyond the basic module, RAN2 to study RRC modular design based on features, functions, or group of them. RAN2 to study the aspects including signaling overhead, UE memory requirement and coupling of features. The detail modular split depends on 6G feature design and needs coordination with RAN1/RAN4 for detail design in a later stage.

Delta configuration

Observation 5-1:	Delta configuration is still useful in 6G to reduce signalling overhead.

Observation 5-2:	The need code introducing additional restraints (e.g., Need S) and conditions are the main causes of implementation complexity and compatibility issues in delta configuration.

Proposal 5:	RAN2 to study the improvement of delta configuration to reduce ambiguities and implementation complexity — for example, decoupling the handling of optional signaling (e.g., Need codes) from the configuration guideline information (e.g., explanations for relevant configuration conditions).

4	References

- 3GPP TR 38.822: " NR; User Equipment (UE) feature list".