# 3GPP TSG RAN WG2 Meeting #132                                                            R2-2508624

# Dallas, USA, Nov. 17th-21st, 2025

Agenda item:	10.2.1

Source:	Ofinno

Title:	UE Capability Framework in 6G

Document for:	Discussion and decision

# Introduction

This document discusses the problems and potential areas to consider when designing 6G capability framework.

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

# Discussion

From a RAN2, architectural, and protocol standpoint, a foundational design principle for 6G involves creating a single, unified Radio Access Technology (RAT) that natively accommodates a wide spectrum of device types from its initial release. The 6G UE capability framework should overcome the complexities and market fragmentation caused by the thousands of optional features seen in NR. The 6G must natively support an even broader range of devices, including low performance (e.g., LPWA, RedCap) up to high performance UEs (e.g., IIOT, XR, aerial/vehicular, NTN).

6G UE capability framework aims to be scalable, unified, and forward-compatible for the broad range of devices and services while being simplified (in comparison to NR).

The following summarizes concerns or issues identified from 5G capability framework:

- Large size of the UE capability message

There is a signaling burden linked to UE capability reporting which has grown drastically considering the optionality associated to each feature and band combination (i.e., excessive signaling overhead). This also leads to having to segment the UECapabilityInformation message when exceeding the transport limitations which also adds latency and processing burden in UE and network.

- Inefficient update of UE capability

In 5G, a UE requires to enter RRC_IDLE for any capability change which leads to an interruption of the ongoing connection. This model would not be preferable when supporting devices with dynamic hardware or operational states (e.g., foldable smartphones, thermal restrictions, multi-SIM interactions). A workaround was used by using UE Assistance Information (UAI); however, network behavior upon reception of UAI is left up to implementation which can lead to inconsistent and unreliable operations.

- Complex structure/framework of UE capability

The 5G capability framework provides a large flexibility (granularity) which also leads to the excessive signaling volume (just discussed) and a complex structure/framework to maintain and keep consistent across features and releases.

- Lack of unified UE capability framework for diverse device types

5G did not consider specialized device categories from the beginning (e.g., IoT, NTN, AI) resulting in compatibility challenges and market fragmentation. The absence of a scalable baseline framework made it difficult to accommodate diverse device types defined in future releases.

- Limited success of Radio Capability Signaling Optimization (RACS)

RACS was defined in Rel-16 aiming to reduce signalling of a UE’s radio capability by using a compact identifier (UE Radio Capability ID) instead of repeatedly transmitting the full capability container over the air and network interfaces. This feature seems to have a limited success due to its per-UE identifier model, large database requirements, and coordination difficulties between operators and vendors. However, it introduced a good conceptual goal reducing signaling overhead through pre-identified identifier or capability sets which is relevant.

- Weak cross working group (WG) coordination

5G UE capability discussions usually started late (more towards the end of the WI) and cross-WG alignment can always be challenging considering their different requirements, progress, and timelines among all WGs, highlighting e.g., RAN1/2/3/4 and SA2/SA3/CT1. Moreover, relevant content is scattered across multiple specifications, e.g. RAN capabilities are captured in TS 38.306, TS 38.331, TR 38.822, complicating maintenance and cross-reference.

The design principles just explained for the 6G capability framework may be enabled by creating a more modular and scalable signaling (e.g., considering for example, compressed and/or group/feature based), by standardizing mechanism to change UE capabilities in any RRC state with none/minimal service interruption where this may be driven by a UE-request or network-request, by creating a hierarchical capability structure that allows reusability, DL/UL decoupling, and streamlined representation, by minimizing redundancy while considering the capabilities associated to the different layers (across features and WGs). Moreover, to ensure strong economies of scale and streamlined signaling, the framework should aim to define a mandatory common baseline of features that all UEs (or specific groups) must support. This approach focuses on achieving a simple, unified framework with minimal signaling overhead and forward compatibility for diverse device types.

The concerns or issues identified in 5G should be the driving factors for 6G UE capability framework, such as, large size of the UE capability message, inefficient update of UE capability, complex structure/framework of UE capability, lack of the unified UE capability framework for diverse device types/features, limited success of RACS, and weak/late cross WG coordination.

- The design of 6G capability framework should aim to (a) reduce signalling overhead (associated to the exchange over the air and across network nodes), (b) allow an efficient update of UE capabilities, (c) design a  simple structure, (d) unified/common for diverse device types/features, (e) being forward scalable/compatible, and (f) enable cross WG coordination from the beginning of a WI.

# Conclusion

The observations captured are the following:

Observation 1.	6G UE capability framework aims to be scalable, unified, and forward-compatible for the broad range of devices and services while being simplified (in comparison to NR).

Observation 2.	The concerns or issues identified in 5G should be the driving factors for 6G UE capability framework, such as, large size of the UE capability message, inefficient update of UE capability, complex structure/framework of UE capability, lack of the unified UE capability framework for diverse device types/features, limited success of RACS, and weak/late cross WG coordination.

The proposals captured are the following:

Proposal 1.	The design of 6G capability framework should aim to (a) reduce signalling overhead (associated to the exchange over the air and across network nodes), (b) allow an efficient update of UE capabilities, (c) design a  simple structure, (d) unified/common for diverse device types/features, (e) being forward scalable/compatible, and (f) enable cross WG coordination from the beginning of a WI.