3GPP TSG RAN WG2 Meeting #132                                                                   R2-2508629

# Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:	10.3.1.3

Source:	Ofinno

Title:	6G Scheduling and (re-)transmission

Document for:	Discussion and decision

# Introduction

The following agreements for user plane were made in RAN2#131bis meeting:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

Based on these agreements, especially on the highlighted ones, this contribution provides our views on the 6G SR/BSR, HARQ/ARQ, and LCP procedure.

# Discussion

## SR and BSR procedure

The SR and BSR procedures are the primary trigger chain for uplink scheduling in NR:

- UL Data arrival → triggers BSR procedure → trigger SR transmission.

- UL Grant allocation → report BSR information.

- Network scheduler determines resource allocation based on received BSR information.

- UL data transmission on PUSCH by the scheduled UL resources.

Figure 1: 5G SR and BSR

Limitations in 5G SR and BSR

While effective for traditional eMBB traffic, this multi-step interaction creates significant delay, which becomes a bottleneck for latency-critical or bursty traffic envisioned in 6G (e.g., AI-assisted traffic, delay-critical XR traffic, or sensor-driven services).

- Excessive uplink access latency: the SR/BSR procedure involves multiple signaling exchanges before actual data transmission:

- UE waits for a valid SR opportunity.

- UE transmits SR → waits for dynamic UL grant.

- UE transmits BSR → waits for another UL grant for data.

Even under optimal conditions, this chain introduces tens of milliseconds of delay, dominated by:

- SR opportunity periodicity (commonly 10–20 ms).

- UL grant processing and BSR handling delays.

- Scheduling delay before uplink transmission.

- Separation and redundancy between SR and BSR Procedures: The SR and BSR are designed as separate MAC procedures, even though they are logically interdependent. The SR signals “buffer non-empty” status but conveys no buffer size information. The BSR provides buffer size but typically only after a grant is issued in response to an SR. This separation leads to redundant signaling and multiple round-trip delays before the network obtains full buffer visibility.

- High signaling overhead and power consumption: The multi-step SR/BSR chain increases control signaling load and requires repeated PUCCH/PUSCH transmissions, which in turn increase UE energy consumption. For power-sensitive devices such as sensors or wearables, maintaining periodic SR resources or transmitting frequent BSRs becomes unsustainable.

Study on 6G SR and BSR

The fundamental goal of the proposed 6G mechanism is to bypass the initial steps (e.g., SR wait time and subsequent small grant RTT) and allow the UE to immediately send a BSR upon data arrival, e.g., by contention-based BSR via shared PUSCH.

Study aspects:

- Always-available resources: The proposal involves introducing a pool of UL resources that are always available and shared by several UEs.

- Contention-based transmission: When UL data arrives and no valid grant is available, the UE sends a BSR and its identity (e.g., C-RNTI) on this shared resource pool in a contention-based fashion.

- Resource allocation: The shared PUSCH allocation would be small but frequently recurring. It would use a robust MCS to minimize decoding errors, leaving enough capacity for a detailed BSR, potentially within 1–2 PRBs.

- Contention management: The shared resource pool must be carefully designed to balance achieving low collision probability while keeping resource overhead minimal.

- Collision handling: Even if collisions occur, the network could still potentially decode multiple colliding PUSCH transmissions by using features like random DMRS selection from a small set of configured DMRS sequences. UEs that do not receive a timely response are expected to re-attempt the BSR transmission, similar to NR's SR retransmissions.

- Scheduler coordination: The proposal requires early liaison with RAN1, as it involves the design of physical layer aspects. The network's blind decoding effort must be kept reasonably low, even though all UEs would use the same small set of DMRS sequences

Expected Benefits:

- Latency reduction: This approach eliminates the waiting time for the dedicated SR resource and one RTT, which significantly reduces the UL latency.

- Spectral efficiency and performance: Simulations suggest that sending BSRs on a frequently recurring, contention-based PUSCH improves the performance of UL- and DL-heavy applications. This method consumes fewer UL resources than continuously reserved dedicated SR resources on PUCCH and scales better with increasing system load.

The multi-step SR/BSR procedure in 5G introduces substantial uplink access delay and signaling redundancy, as the UE sequentially sends SR and BSR before receiving transmission grants. This design leads to tens of milliseconds of latency, redundant control exchanges, and increased power consumption.

RAN2 to study how to allow UE to transmit BSR/DSR as soon as possible upon arrival of UL data (i.e., without waiting for a SR or a dedicated UL grant).

## HARQ and ARQ

In the 5G NR user plane protocol architecture, data reliability is primarily achieved through the combined use of HARQ at the MAC/PHY layer and ARQ at the RLC layer. HARQ operates in the time-critical domain close to the physical layer, performing retransmissions based on instantaneous ACK/NACK feedback from the receiver. Its design relies on short feedback loops and soft-combining mechanisms that allow incremental redundancy versions to be aggregated. ARQ, by contrast, functions at the RLC AM sublayer and provides an additional level of reliability through explicit positive or negative acknowledgments. When packets are not confirmed by the peer, the transmitter retains copies and re-sends them after timer expiry or upon receipt of a status report. The design of HARQ/ARQ in 5G was optimized for throughput efficiency, not deterministic latency. In 6G, these mechanisms must evolve toward a hardware-friendly, energy-efficient, and latency-bounded framework that can flexibly support diverse service types — from ultra-reliable industrial data to best-effort or inference traffic.

Figure 2: 5G HARQ and ARQ

Limitations in 5G HARQ and ARQ

- Decoupled and Uncoordinated Operation: HARQ and ARQ currently operate in isolation, with no standardized interface for sharing delivery status information. When a HARQ process fails repeatedly, the RLC entity remains unaware of these failures until its own timer expires. Conversely, when HARQ eventually succeeds after several retransmissions, RLC may still retransmit the same packet because it cannot know that the data was successfully delivered at the physical layer. This lack of cross-layer coordination results in redundant transmissions, duplicated buffering, and inefficient use of radio and memory resources.

- High Buffering and Memory Duplication: Because HARQ and ARQ maintain independent retransmission buffers, the same data unit may be stored and referenced multiple times across protocol layers. HARQ soft-combining requires the storage of multiple redundancy versions, while RLC simultaneously keeps copies of the same PDU for ARQ recovery. Each layer also performs its own reordering and segmentation operations, leading to duplicated data movement between memory blocks. This results in increased memory footprint, higher power consumption, and complex hardware data paths, especially in high-throughput or distributed baseband implementations.

Study on 6G HARQ and ARQ

- Integrated or Tightly-Coordinated HARQ/ARQ Framework

A key enhancement for 6G is to integrate HARQ and ARQ into a unified or tightly coordinated retransmission framework, replacing the existing layered isolation. The fundamental idea is that the HARQ entity’s instantaneous feedback information (e.g., ACK, NACK, or soft-failure indication) which could be exposed in near real-time to the RLC entity. This allows the RLC layer to make retransmission decisions based on actual link performance rather than static timers.

Study aspects: Define new cross-layer signaling that deliver HARQ status to RLC. When persistent HARQ failures are detected (e.g., multiple consecutive NACKs for the same process), RLC can immediately initiate retransmission without waiting for t-Reassembly expiry. When HARQ successfully delivers a packet, the RLC transmitter can release its stored copy without further acknowledgment, avoiding redundant transmission. Study shared retransmission buffer architectures that allow both HARQ and ARQ to reference the same physical memory, eliminating duplicate storage.

Expected Benefits: This approach can achieve sub-millisecond retransmission latency, substantially reduce jitter, and lower memory usage. It also simplifies control logic, as retransmission timing becomes directly tied to physical-layer success indications rather than independent timer cycles.

- Unified Status and Acknowledgment Reporting

Combine HARQ feedback and ARQ status report into a single HARQ/ARQ report to minimize signaling redundancy and improve packet tracking across layers.

Study aspects: HARQ provides physical-layer ACK/NACK results to RLC. RLC provides delivery sequence acknowledgment and reordering information. The single HARQ/ARQ report aggregates both into a compact message shared between MAC and RLC, allowing both layers to maintain a consistent view of packet delivery state.

Expected Benefits: Simplifies feedback management, provides full visibility of end-to-end delivery, and reduces overhead in both uplink and downlink directions.

In 5G, HARQ and ARQ operate independently without coordination or shared status visibility, leading to redundant retransmissions, duplicated buffering, and inefficient memory use. This decoupled design increases implementation complexity and processing overhead.

RAN2 to study a coordinated HARQ/ARQ framework  by considering  sharing of HARQ results for ARQ, e.g., to trigger retransmission or discard of SDU/PDUs.

## LCP procedure

The Logical Channel Prioritization (LCP) procedure in 5G NR MAC is a core function that determines which logical channels (LCHs) are selected for data transmission when the UE has limited uplink or downlink resources. The LCP process ensures that higher-priority logical channels are served first and that QoS differentiation configured by RRC is reflected at the MAC layer. In uplink operation, the procedure executes after a UL grant is received. The UE performs the following: (1) Selects which logical channels to serve based on priority. (2) Determines how many bytes to include from each channel according to available grant size. (3) Builds the MAC PDU with headers and subheaders, concatenating selected SDUs and CEs. The NR LCP design assumed moderate traffic diversity, relatively stable QoS priorities, and predictable grant cycles — conditions suitable for eMBB and SRB traffic. However, as 6G aims to support highly dynamic, low-latency, and service-aware traffic (e.g., XR, real-time control, AI inference), the limitations of the current LCP procedure have become significant.

Figure 3: 5G LCP

Limitations in 5G LCP procedure

- Head-of-Line (HOL) Blocking in Mixed-Priority Traffic: In NR, LCP allocates resources per-UL grant and constructs one MAC PDU per transport block. When the UE’s buffer contains both large lower-priority data and small high-priority packets, the large data fragments may occupy most of the UL grant. Consequently, urgent data — even if of higher priority — may not fit into the remaining grant space, forcing it to wait for the next opportunity. This results in HOL blocking and latency inflation for delay-critical traffic. The issue is amplified in scenarios where multiple service types (e.g., XR, haptic feedback, sensor updates) are multiplexed on a single DRB. Because LCP operates at the LCH level, not at the QoS flow or stream level, fine-grained isolation between concurrent services is not possible.

- MAC CE Contention with User Data: During LCP, both user data and MAC CEs compete for the same UL grant. If the UE must transmit higher priority MAC CEs, they may preempt available space and delay the actual data transmission. For small UL grants, a few MAC CEs can completely consume the transport block, especially when concatenation or segmentation alignment constraints apply. This increases delay for small but urgent packets.

- Static and Coarse Priority Model: LCP priority values are configured by RRC as static integers.
This approach does not reflect real-time service state, packet urgency, or contextual importance. For instance: A low-priority logical channel carrying an imminent XR frame or control signal may still be scheduled after best-effort data. Dynamic service adaptation (e.g., changing frame rate or latency tolerance) requires reconfiguration through RRC, introducing delay and signaling overhead. As 6G aims to support dynamic and adaptive service behavior, the static LCP model limits its responsiveness and scalability.

Study on 6G LCP procedure

- Intra-LCH prioritization

In 5G NR, LCP operates only at the LCH level, where all PDUs belonging to a DRB share one queue and one static priority value. However, emerging 6G traffic such XR, industrial control, and AI traffic often multiplex multiple application streams within the same DRB. Because these flows can have drastically different delay and reliability requirements, a single-priority model causes HOL blocking and suboptimal resource use.

Study aspects: Study an LCP procedure supporting prioritization within a logical channel. Each logical channel can maintain multiple sub-queues; each associated with a stream or a QoS flow. LCP first selects between logical channels, then between sub-queues within a selected LCH according to latency, survival time, or reliability metrics.

Expected Benefits:

- Eliminates HOL blocking across QoS flows sharing a DRB.

- Supports multi-stream applications with diverse latency and reliability requirements.

- Improves uplink efficiency and fairness for mixed traffic.

- Dynamic and event-driven LCP priority updates

In NR, priorities are statically configured by RRC, and changes require reconfiguration — unsuitable for dynamic, latency-critical services. 6G services demand real-time adaptability where the UE can temporarily raise or lower the priority of a channel or stream based on instantaneous context.

Study aspects: Study and define event-driven LCP priority adaptation, allowing the UE to locally modify logical channel or stream priorities without full RRC signaling. For example, the triggering events could be arrival of delay-sensitive data (e.g., new XR frame, control packet), approaching QoS deadline or survival time expiry, transition between service phases (e.g., low-motion → high-motion XR scene), and/or HARQ retransmission or congestion conditions that require reprioritization. Further study how to indicate temporary priority adjustments.

Expected Benefits:

- Immediate adaptation to real-time service needs.

- Avoids RRC signaling delays for temporary priority changes.

- Enables adaptive, context-aware scheduling responsive to application conditions.

- Separation of control and data scheduling

In NR, MAC CEs and data compete within the same UL grant. For small grants, even a few bytes of MAC CE can consume the entire transport block, blocking latency-critical data. Conversely, deferring importance MAC CEs (e.g., BSR) risks uplink inefficiency.

Study aspects: Establish explicit separation rules between control and data during LCP procedure. For example,  uplink grant specifically for control signaling or reserve a small fixed portion of each UL grant for control signaling, preventing MAC CE starvation while keeping data uninterrupted. Study MAC CE deferral policy. For example, allow LCP to postpone non-urgent MAC CE transmissions (e.g., periodic BSR) when high-priority data is queue Study preemptive MAC CE Transmission, e.g., for time-critical MAC CEs, LCP may preempt ongoing data selection.

Expected Benefits:

- Avoids HOL blocking between MAC CEs and user data.

- Maintains stable uplink latency for small high-priority packets.

- Simplifies scheduler decision-making under mixed control/data loads.

In NR, LCP operates at the logical channel level and builds one MAC PDU per UL grant, which can cause large low-priority packets to occupy most of the transport block. As a result, small but delay-sensitive packets are forced to wait for the next transmission opportunity, creating HOL blocking and increased latency for mixed-service scenarios such as XR or haptic traffic.

RAN2 to study an intra-LCH prioritization mechanism that enables prioritization of traffic within a single LCH.

LCP priorities in NR are statically configured by RRC and do not reflect real-time service conditions or packet urgency. This rigidity prevents adaptive prioritization when service characteristics (e.g., frame rate, latency tolerance) change, limiting responsiveness and scalability for 6G’s dynamic and context-aware traffic patterns.

RAN2 to study a dynamic and/or event-driven LCH priority update mechanism that enables UE and/or NW to temporarily adjust LCH priorities.

During LCP, MAC CEs and user data share the same UL grant, leading to contention when control signaling preempts payload data. In small grants, a few MAC CEs can consume the entire transport block, delaying urgent user data and degrading performance for latency-critical services.

RAN2 to study a control–data separation mechanism in uplink scheduling to prevent contention between MAC CEs and user data using the same UL grant.

# Conclusion

Based on the analysis above, we have the following observations and proposals:

- The multi-step SR/BSR procedure in 5G introduces substantial uplink access delay and signaling redundancy, as the UE sequentially sends SR and BSR before receiving transmission grants. This design leads to tens of milliseconds of latency, redundant control exchanges, and increased power consumption.

Proposal 1	RAN2 to study how to allow UE to transmit BSR/DSR as soon as possible upon arrival of UL data (i.e., without waiting for a SR or a dedicated UL grant).

In 5G, HARQ and ARQ operate independently without coordination or shared status visibility, leading to redundant retransmissions, duplicated buffering, and inefficient memory use. This decoupled design increases implementation complexity and processing overhead.

Proposal 2	RAN2 to study a coordinated HARQ/ARQ framework  by considering  sharing of HARQ results for ARQ, e.g., to trigger retransmission or discard of SDU/PDUs.

In NR, LCP operates at the logical channel level and builds one MAC PDU per UL grant, which can cause large low-priority packets to occupy most of the transport block. As a result, small but delay-sensitive packets are forced to wait for the next transmission opportunity, creating HOL blocking and increased latency for mixed-service scenarios such as XR or haptic traffic.

Proposal 3	RAN2 to study an intra-LCH prioritization mechanism that enables prioritization of traffic within a single LCH.

LCP priorities in NR are statically configured by RRC and do not reflect real-time service conditions or packet urgency. This rigidity prevents adaptive prioritization when service characteristics (e.g., frame rate, latency tolerance) change, limiting responsiveness and scalability for 6G’s dynamic and context-aware traffic patterns.

Proposal 4	RAN2 to study a dynamic and/or event-driven LCH priority update mechanism that enables NW and/or UE to temporarily adjust LCH priorities.

During LCP, MAC CEs and user data share the same UL grant, leading to contention when control signaling preempts payload data. In small grants, a few MAC CEs can consume the entire transport block, delaying urgent user data and degrading performance for latency-critical services.

Proposal 5	RAN2 to study a control–data separation mechanism in uplink scheduling to prevent contention between MAC CEs and user data using the same UL grant.

# Reference

3GPP TS 38.322: “NR; Radio Link Control (RLC) protocol specification.”

3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification.”