3GPP TSG RAN WG2 Meeting #132                                                          R2-2508633

# Dallas, USA, Nov. 17th-21st, 2025

Agenda item:	10.3.3

Source:	Ofinno

Title:	Data Framework in 6G

Document for:	Discussion and decision

# Introduction

This document discusses data transfer and data model in 6G considering the following RAN2#131bis agreements:

1.  	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

- This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

- The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

- UE data collection should minimize impact to user traffic transmission and power saving features

These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.	Study model transfer/delivery requirements/functions.

5.	For next meeting companies can consider at least the following aspects: Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

For each type of data, study:

- Applicable use case(s)

- End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

- Data size in a single reporting and total data size

- Frequency of data reporting;

- QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

6.	Study 6G AI/ML LCM framework that supports at least the following functions/procedures

- UE AI/ML Capabilities Exchange;

- Applicable Functionality Reporting;

- Inference Configuration and Reporting;

- Performance Monitoring Configuration and Reporting;

- Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

# Discussion

## Diverse types of data in 6G

The evolution toward 6G introduces new data centric and intelligent services (e.g., SON/MDT, AI/ML, ISAC, XR, Digital Twin or other emerging applications). These services generate massive and heterogeneous data with varying characteristics (e.g., AI model parameters, sensing results, and network optimization data). 5G data transfer frameworks were not designed to accommodate this diversity and data-centric/intelligent services. For example, 5G protocol structure suffers from fragmentation, inefficiency, and static data handling mechanisms that limits the seamless integration of these new data types.

To effectively support diverse data in 6G, a unified, flexible and scalable data framework seems required. 6G framework would need to handle heterogeneous data flows with distinct requirements for volume, latency, reliability, and security. It should also enable flexible data routing and termination across different network entities (e.g., UE(s), RAN node(s), CN node(s) or even external nodes) while ensuring continuity during mobility.

6G framework needs to discuss whether a new data (and/or service) centric functionality is defined. This may require data (and/or service) centric mechanisms or protocol (layers) distinct from the traditional control and user planes. RAN2 would need to study how/whether this new data (and/or service) centric functionality provides flexible QoS configurations, data exchange between different start/end nodes, and adaptable start/end points for data transfer. For this, RAN2 would also need to discuss in conjunction with other WGs the data handling required e.g., segmentation, compression, routing, and reassembly across multiple nodes (including intermediate and end notes). By integrating data (and/or service) centric mechanisms or protocol (layers), 6G could provide a robust foundation for AI-native, sensing-aware, and globally connected communication systems. This transformation could mark a significant step toward realizing a data (or service) driven 6G system architecture capable of supporting the diverse and evolving requirements of next-generation networks.

Summarizing, during 6G study, RAN2 should consider any technical viable approach from new architectural elements to enhancements of the existing 5G control and user plane functions. When doing so. RAN2 needs to prioritize minimizing signalling overhead while creating data (and/or service) specific operation to enable lightweight mechanisms that allow managing, collecting and transferring high-volume data efficiently. On the security front, 6G will require robust mechanisms for user consent, data integrity, and privacy preservation (especially given the sensitive nature of some data e.g., AI and sensing).

5G system provides a limited, fragmented and rigid data transfer framework which does not seem efficient to support the heterogeneous and high-volume data generated by emerging data and intelligent centric services (e.g., SON/MDT, AI/ML, ISAC, XR, Digital Twin), etc.

- RAN2 should study a unified, flexible and scalable data centric framework for 6G. For this, RAN2 should study new data centric protocol layers (or mechanisms), as well as, data centric enhancements on top of the existing 5G system (considering both control and user planes).

## Requirements of the 6G data framework

The following summarizes the different requirements identified for study to support the diverse types of data in 6G.

Support for Diverse and Heterogeneous Data Types

The evolution toward 6G networks introduces unprecedented data diversity driven by the integration of intelligent and data-centric services, e.g. SON/MDT, AI/ML, ISAC, XR, Digital Twin, etc. These services generate heterogeneous data types (ranging from AI specific models, parameters and data sets to real time sensing outputs and user experience metrics) where each one may have very different transmission requirements (e.g., throughput, latency, reliability, and security). 6G data framework should be able to handle both structured data (e.g., models, metrics) as well as unstructured data (e.g., raw, sensing). Moreover, 6G data framework would need to coexistence and be integrated with the traditional kind of communications.

An efficient handling of both structured and unstructured data with varying transmission requirements (e.g., throughput, latency, reliability, and security) seems desirable.

Fine Granularity and adaptive QoS Management

A key requirement for the 6G framework is flexible support for heterogeneous and conflicting QoS demands. For example, AI training data transfers involve extremely large datasets but can tolerate delay, while inference or sensing data require ultra-low latency and near-zero packet loss. Another example is the updates associated to the services or applications (e.g. AI model updates) which may demand both reliability and integrity, whereas real-time data (e.g. XR or cooperative sensing traffic) depends on synchronization and deterministic delay. These contrasting characteristics reveal that 6G may not be able to rely on fixed/static approach. Instead, it might require a QoS with fine granularity and flexible (or adaptive) capabilities that can dynamically adapt to data characteristics and context. Furthermore, RAN2 needs to consider that data exchanges in 6G may not always align with a traditional session based communication (e.g., data may also be opportunistic, distributed, and asynchronous). RAN2 needs to also study how to enable or the impact in 6G system to support random data flows between a start and end points without a session.

Diverse, conflicting, and asynchronous data flows with varying latency, reliability, and synchronization are foreseen.

Flexible and Multi-End Data Routing

The 5G protocol architecture may be limited for 6G system. For example, CP-based data transfers (e.g., via RRC/NAS) face scalability issues (e.g., message size and segmentation), making them not ideal for high-volume data exchanges and distribution. Similarly, UP-based mechanisms (e.g., GTP-U sessions) lack the flexibility and visibility required for the data transfers between/in different nodes (e.g., UE(s), RAN node(s)). This session based design may limit efficient support for emerging use cases that could involve dynamic, cross-layer or node level intelligence data handling/interactions. 6G framework should aim to provide a simple, direct and efficient data exchange mechanism across layers and nodes, including support for distributed processing and adaptive operation (e.g., path selection).

Flexibility and multiple end points for the data handling across nodes and layers to be considered.

Unified and Modular Data Handling Architecture

5G fragmented design for data collection and management by developing independently different mechanisms across multiple releases and features (e.g., SON, MDT, QoE, and AI/ML) has led to redundancy, inefficiency, and specification complexity. Moreover, repeated collection of similar measurements by different network nodes or functions also results in a wastage of resources, and an increase of energy consumption and signaling overhead. For 6G, the number of data centric features or functions may multiply with all the emerging communications/technologies. Thus, there exist a need to define a forward looking, unified and modular framework capable of accommodating diverse data (and/or services) under a common coherent architecture or framework. This 6G data framework should aim to standardize the data handling (e.g., segmentation, compression, routing, and reassembly) while maintaining lightweight (minimal) signalling overhead, power consumption and resource utilization.

6G aims to develop a unified and modular architecture to efficiently manage diverse data, reduce redundancy, and minimize signaling, energy, and resource overhead while keeping complexity (cost) low.

Security, Privacy, and User Consent

6G data framework would increasingly involve the exchange of sensitive, context-aware, and user-related information. For this, it is imperative to provide a secure, private, and controllable environment. For example, AI and sensing data can expose user identity, mobility patterns, or environmental characteristics, requiring strong encryption, anonymity, and explicit user consent mechanisms. Moreover in RAN2#131bis, operators requested to have visibility and control over data transfers (including initiation, termination, and routing) to ensure compliance and prevent unauthorized use. These security and governance requirements would also need to be taken into consideration when studying the 6G data framework.

Data handling with robust security, privacy, and user consent is critical while providing operators control and protection over sensitive and context-aware data exchange/usage.

In summary, the diverse data framework for 6G must deliver flexibility, scalability, and intelligence far beyond the capabilities of 5G and aiming to accommodate future emerging data services. The aim should be to study how to provide an unify data handling across heterogeneous services considering adaptive QoS and routing, integrate distributed and multi-node data exchange, while also ensuring robust privacy and operator control.

- The 6G data framework requirements should consider developing a unified and modular data handling architecture, providing support of diverse and heterogeneous data types (structured and unstructured), enabling a fine-granularity and adaptive QoS management with a flexible and multi-end data routing, and ensuring robust security, privacy, and operator’s control.

## AI/ML framework

5G AI/ML framework, as detailed in technical reports TR 38.843 and TR 38.744, serves as a foundational element for discussions on 6G. This framework provides a structured approach to integrating AI/ML algorithms into RAN operations and encompasses several key components, including Life Cycle Management (LCM), model management, and specific procedures for data collection and performance monitoring. The table below summarizes 5G AI/ML framework:

| 5G AI/ML framework – LCM |  |
| --- | --- |
| Model Generation | This stage involves the offline processes of training, validating, and testing AI/ML models. |
| Model Inference | This is the process of using a trained model to generate outputs from a set of inputs. The 5G framework assumes NW-sided model inference is not visible to the UE, while for UE-sided models, specific configurations are used for inference configuration |
| Data Collection | The framework supports both UE-side and NW-side data collection for offline and online AI/ML model training. NW is responsible for allocating resources and indicating configurations to UE for both NW-side and UE-side data collection. |
| Performance Monitoring | For UE-sided models, UE-assisted performance monitoring is supported to ensure models operate effectively post-deployment. For NW-sided models, monitoring is handled by NW implementation. |
| Functionality-Based LCM | This manages AI/ML operations without requiring the network/UE to know the specifics of the model. This approach indirectly supports model selection, activation, deactivation, switching, and fallback to non-AI solutions. The process involves: - UE reporting its AI/ML capabilities. - NW providing configurations based on the reported capabilities to enable inference. - UEs reporting the applicability of these configurations, especially when network conditions change. |
| 5G AI/ML framework - Models |  |
| One-Sided Model | The entire AI/ML model inference is performed at either the UE (UE-sided model) or the network (NW-sided model). Model management for these is typically left to the implementation of the respective side (UE or NW). |
| Two-Sided Models | This involves a paired set of models where inference is performed jointly across both the UE and the network. This architecture is central to the AI-based CSI compression use case, which will be specified in Release 20 of 5G-Advanced. |

The 5G NR LCM framework, encompassing data collection, model training, model storage, management, and inference, serves as a baseline or starting point for 6G discussions. For 6G, a key requirement might be to develop a unified, extensible, and future-proof LCM framework that can accommodate diverse AI/ML use cases with minimal modifications to reduce signaling overhead, implementation complexity, and specification maintenance efforts. The unified data collection framework for diverse 6G services should aim avoiding duplicated data collection and reporting across different use cases or model types considering both standardized and non-standardized data formats.

For 6G, LCM framework aims to provide flexible functionality and model management, including online/offline and distributed training/inference, managing the model applicability, activation/deactivation, and switching between AI/ML models or also falling back to non-AI functions.

6G Training data is characterized by its diverse types, massive volume, and varying QoS requirements, which pose significant challenges to existing 5G data collection and transfer mechanisms. For offline model training, data volumes are typically large, with relaxed latency requirements and tolerance for some packet loss. For online training or real-time monitoring, latency requirements become more stringent. The data types may also include raw data, processed datasets, AI models and parameters, inference input/output, monitoring data, and assistance information among others.

Key requirements for training data collection and transfer in 6G include ensuring data security, integrity, and confidentiality. User data privacy, anonymity, and user consent must be respected, with user consent potentially configurable by OAM. MNOs require control over the standardized data collection process, including initiation, termination, and management, as well as full visibility of standardized data. Moreover, the design framework aims to be future-proof and scalable to accommodate new AI/ML applications and data.

5G NR LCM framework provides a baseline for 6G which aims to study a unified, extensible, and future-proof LCM framework to efficiently handle diverse AI/ML use cases while minimizing redundancy, signaling, and complexity.

- RAN2 aims to study how to develop a 6G AI/ML framework that supports flexible online/offline and distributed training/inference, avoiding duplicate data collection, and being scalable to accommodate emerging applications with minimal complexity.

# Conclusion

The observations captured are the following:

Observation 1.	5G system provides a limited, fragmented and rigid data transfer framework which does not seem efficient to support the heterogeneous and high-volume data generated by emerging data and intelligent centric services (e.g., SON/MDT, AI/ML, ISAC, XR, Digital Twin), etc.

Observation 2.	An efficient handling of both structured and unstructured data with varying transmission requirements (e.g., throughput, latency, reliability, and security) seems desirable.

Observation 3.	Diverse, conflicting, and asynchronous data flows with varying latency, reliability, and synchronization are foreseen.

Observation 4.	Flexibility and multiple end points for the data handling across nodes and layers to be considered.

Observation 5.	6G aims to develop a unified and modular architecture to efficiently manage diverse data, reduce redundancy, and minimize signaling, energy, and resource overhead while keeping complexity (cost) low.

Observation 6.	Data handling with robust security, privacy, and user consent is critical while providing operators control and protection over sensitive and context-aware data exchange/usage.

Observation 7.	5G NR LCM framework provides a baseline for 6G which aims to study a unified, extensible, and future-proof LCM framework to efficiently handle diverse AI/ML use cases while minimizing redundancy, signaling, and complexity.

The proposals captured are the following:

Proposal 1.	RAN2 should study a unified, flexible and scalable data centric framework for 6G. For this, RAN2 should study new data centric protocol layers (or mechanisms), as well as, data centric enhancements on top of the existing 5G system (considering both control and user planes).

Proposal 2.	The 6G data framework requirements should consider developing a unified and modular data handling architecture, providing support of diverse and heterogeneous data types (structured and unstructured), enabling a fine-granularity and adaptive QoS management with a flexible and multi-end data routing, and ensuring robust security, privacy, and operator’s control.

Proposal 3.	RAN2 aims to study how to develop a 6G AI/ML framework that supports flexible online/offline and distributed training/inference, avoiding duplicate data collection, and being scalable to accommodate emerging applications with minimal complexity.