3GPP TSG RAN WG2 Meeting #132                                                         R2-2508635

# Dallas, USA, Nov. 17th-21st, 2025

Agenda item:	10.3.3.3

Source:	Ofinno

Title:	Security for system information

Document for:	Discussion and decision

# Introduction

This document discusses the AS security aspects based on the agreed 6G SID [1].

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

# Discussion

In 5G, System Information (SI) are broadcast without security protection, leaving User Equipment (UEs) vulnerable to attacks from False Base Stations (FBS). This vulnerability can potentially lead to UEs camping on rogue cells, reselecting to incorrect neighboring cells, or even reacting to wrong SI notifications (e.g., responding to inexistent Public Warning System (PWS) messages). The SA1 6G study [2] has identified the need to increase the level of security between RAN entities and the UE during initial network selection. While SIB security was studied in 5G by SA3, as shown in TR 33.809 [3], it was not specified due to lack of timing and the potential complexity/impact foreseen of modifying a fundamental aspect of the legacy system design.

As 6G development commences without the constraints of backward compatibility, it presents a unique opportunity to address this fundamental security flaw from Day 1. This contribution outlines the problem, reviews candidate solutions, and proposes a way forward for RAN2 to address SIB security in 6G.

## SIB protection in SA3

SA3 acknowledges that the absence of security protection of the system information in 5G could allow attackers to broadcast rogue SI messages/configurations or replay previously captured ones. This can result in:

- Denial of Service (DoS) attacks on UEs: Hindering UEs' access to the network.

- Rogue services: Delivering unauthorized or unsolicited services to UEs.

- Public safety threats: Manipulation of ETWS and CMAS related SIBs (e.g., SIB 6-8) to transmit false alerts, causing social panic.

- Service attacks: Disruption of services like MBMS, V2X, ProSe, and NTN through manipulated SIBs (e.g., SIB 12, 13, 14, 19, 20, 24, 25).

SA3 6G Study Item (SI) on Security for the 6G System (WT#2,#3) [4] explicitly includes an objective on this, i.e. "SIB protection: Study security for SIBs to avoid FBS attacks" as it is identified as a critical aspect to enhance in 6G.

SA3 study captured in TR 33.809 [3] outlines solutions for Key Issue #2: Security protection of system information:

- Solution #7: Verification of authenticity of the cell.

- Solution #9: Using symmetric algorithm with assistance of USIM and home network.

- Solution #11: Certificate based solution against false base station.

- Solution #12: ID based solution against false base station.

- Solution #14: Shared key based MIB/SIBs protection.

- Solution #19: AS security based MIB/SIBs integrity information provided by gNB.

- Solution #20: Digital Signing Network Function (DSnF).

- Solution #21: Certificate based solution against false base station for Non-Public Networks.

- Solution #26: KI#2 with PKC-based and without tight time synchronization.

- Solution #27: Short-lived asymmetric key-based solution for protecting system information.

SA3 6G SI explicitly captures an objective to study the security of SIBs security to avoid FBS attacks and highlighting the need for RAN2 is also expected to study correspondingly secured system information delivery in 6G RAN.

## SIB protection in RAN2

SA3's 6G security study item [4] explicitly requires input from RAN on "architecture related requirements". RAN2 is tasked with studying the feasibility of SIB security in 6G in alignment with SA3. RAN2 should inform SA3 about the SIB contents causing security issues. Furthermore, SA3 will have a checkpoint with RAN WGs on AS security by June 2026 [4]. Key aspects for SA3 input, related to security of SIBs, may include the size of security information, latency requirements, frequency of transmission, and whether all or only parts of SIBs need protection.

Implementing SIB security introduces several technical considerations and potential impacts on RAN design:

- Overhead and SIB Size Limitation: Security parameters, such as verification fields for integrity protection or digital signatures, can range from tens of bits to thousands of bits. RAN2 needs to provide input to SA3 regarding the feasibility of including such extra bits.

- Scope of Protection: It may be impractical to apply integrity protection to all SIBs. A more pragmatic approach would be to protect only essential system information.

- Interaction with On-demand SIB (OD-SIB): 6G is exploring extensive use of on-demand SIB mechanisms for energy efficiency. The design of SIB security must consider how it applies to SIBs that are not always broadcast, including standalone OD-SIB1 and Cell A-assisted scenarios.

For broadcast SIBs, integrity protection offers a more practical security approach than encryption. While encryption would provide additional protection, the associated computational burden and latency impact on both UE and network elements make integrity protection the preferred solution for maintaining system efficiency.

Integrity protection mechanisms seem more critical for system information than encryption.

- RAN2 to study security aspects to enable system information protection in alignment with the objectives of SA3 6G SI. This study may include study of the granularity of system information that needs to be protected, information/ parameters to be provided to SA3.

# Conclusion

The observations captured are the following:

Observation 1.	SA3 6G SI explicitly captures an objective to study the security of SIBs security to avoid FBS attacks and highlighting the need for RAN2 is also expected to study correspondingly secured system information delivery in 6G RAN.

Observation 2.	Integrity protection mechanisms seem more critical for system information than encryption.

The proposals captured are the following:

Proposal 1.	RAN2 to study security aspects to enable system information protection in alignment with the objectives of SA3 6G SI. This study may include study of the granularity of system information that needs to be protected, information/ parameters to be provided to SA3.

# Reference

- RP-252912, Study on 6G Radio, SID, September 2025.

- 3GPP TR 22.870, “Study on 6G Use Cases and Service Requirements; Stage 1”

- TR 33.809, Study on 5G Security enhancement against False Base Stations (FBS)

- SP-251132, “Study on Security for the 6G System”