3GPP TSG-RAN WG2 Meeting #132						R2- 2508652

Dallas, USA, Nov. 17th - 21st, 2025

Agenda item:		9.2.2

Source:			Lenovo

Title:		Discussion for Topology 2 for Rel-20 Ambient IoT

Document for:	 	Discussion and Decision

Introduction

Rel-20 Ambient IoT Phase 2 WID [1] has been approved in RAN#108 meeting. At the same time, a new SID is also approved for Rel-20 Ambient IoT outdoor scenario for active device which is for RAN1 and RAN4 study. In the new WID, objective for Topology 2 will be studied firstly and in last RAN2 meeting, following agreements were reached for Topology 2

Agreements

- 1.  	gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

In this contribution, we continually discussed following aspects for A-IoT Topology 2 scenario, based on the SI conclusion in TR 38.769 [3] and agreements in above

- Reader-gNB Information Exchange

- Resource Allocation: Trigger by UE request

- Resource Validity for RRC CONNECTED mode (Scenario 1)

- Resource Validity for temporary out-of-connection (Scenario 2)

- UE Reader Selection

Discussion

Reader-gNB Information Exchange

In last RAN2 meeting [4], it was agreed that gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information. In Rel-19, AIoTF provide following assistance information to NG-RAN according to [5]

Table 1 Assistance information from AIoTF to NG-RAN

For Inventory or Command service operation, following Inventory Assistance Information is included in the Inventory Request from AIOTF to NG-RAN: a) AIoT service type (e.g. Inventory, Command). b) Optionally, approximate number of AIoT devices based on AF request. c) Size of the Inventory Response message from the AIoT Device. d) Optionally, time interval for AIoT Device response aggregation used by the NG-RAN as specified in clause 5.9. For Command service operation, following Command Assistance Information is included in the Command Request from AIOTF to NG-RAN: e) Size of the Command Response message from the AIoT Device.

Besides above assistance information, following information may also be provided

- AIoT Identification Information, i.e. paging id, to be included in the paging message

- Correlation ID

- follow on command indication in the Inventory Request message

- RAN AIoT Device NGAP ID

- AIOTF ID, for indirect connectivity between AIoTF and NG-RAN

For Topology 2, since UE reader is in charge of scheduling the AIoT interface transmissions and send paging message to the AIoT device, related AIoT information needs to be sent by the gNB to the UE reader. This includes following information

- AIoT Identification Information, i.e. paging id, to be included in the paging message

- follow on command indication in the Inventory Request message

- Assistance information

  - AIoT service type (e.g. Inventory, Command).

  - Optionally, approximate number of AIoT devices based on AF request.

  - Size of the Inventory Response message from the AIoT Device.

  - Size of the Command Response message from the AIoT Device.

Proposal 1: gNB will provide at least following AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN

- AIoT Identification Information, i.e. paging id, to be included in the paging message

- Follow on command indication in the Inventory Request message

- Assistance information

  - AIoT service type (e.g. Inventory, Command).

  - Optionally, approximate number of AIoT devices based on AF request.

  - Size of the Inventory Response message from the AIoT Device.

  - Size of the Command Response message from the AIoT Device.

For the time interval for AIoT Device response aggregation used by the NG-RAN, this is specified to decrease the signaling overhead between NG-RAN and AIoTF. For the UE reader, we believe similar information is also needed. But considering the UE reader will consume Uu interface resource to send AIoT data to the gNB, we think this parameter is better configured by the gNB for the UE reader, instead of directly send time interval for AIoT Device response aggregation used by the NG-RAN received from CN.

Proposal 2: gNB configures similar parameter of ‘time interval for AIoT Device response aggregation’ for UE reader, to reduce the RRC signalling overhead in Uu interface.

On the other hand, for correlation id, it is used in Topology 1 BS reader to generate transaction id. While for Topology 2, we think this may need further discussion whether the transaction id is generated by the gNB or the UE reader.

Proposal 3: RAN2 is suggested to further discuss transaction id is generated by the UE reader or the gNB in Topology 2.

On the last point, for RAN AIoT Device NGAP ID, which used to identify AIoT device between NG-RAN and AIoTF, BS reader maintains the mapping relationship between AS ID and RAN AIoT Device NGAP ID. While for Topology 2, it is not suitable to send RAN AIoT Device NGAP ID to UE reader directly. There needs an ID on Uu interface to identify AIoT device and correlates to RAN AIoT Device NGAP ID.

Proposal 4: An ID on Uu interface is needed to identify AIoT device between UE reader and gNB, and correlates to RAN AIoT Device NGAP ID in Topology 2.

Resource Allocation: Trigger by UE request

In last RAN2#131bis meeting [4], resource allocation for topology2 has been discussed and following agreements have been reached:

- gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. - The reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

Since RRC-based solution is used for AIoT upper layer information transfer for topology2, CN indicates the AIoT inventory/command request to the UE-reader and explicitly forwarded by the BS via NR Uu RRC message. Meanwhile, the AIoT resource information is allocated to the UE reader by using RRC signalling. Generally, the resource allocation can be triggered by CN request. However, in some cases, we understood that the UE reader may also transmit the assistance information associated with the resource allocation for the AIoT interface transmission to gNB. Specifically, UE reader may transmit the assistance information to gNB by considering following cases:

- Resource allocation with explicit request from gNB or unsolicited.

- Update the allocated/configured resource for AIoT interface transmission.

- Release the allocated/configured resource for AIoT interface transmission.

Proposal 5: UE reader may transmit assistance information to gNB in following cases:

- Resource allocation with explicit request from gNB or unsolicited.

- Update the allocated/configured resource for AIoT interface transmission.

- Release the allocated/configured resource for AIoT interface transmission.

For baseline procedure, BS may provide the AIoT resource information in the case of receiving the inventory/command request from CN, and the resource information may be transmitted with inventory/command request to UE reader via RRC signaling. However, since it is up to UE reader to determine the detailed scheduling parameters in AIoT paging message, e.g., the number of access occasions, the number of paging rounds, the number of time/frequency resource for each access occasion. While in some cases, gNB may need more information to determine the resource type or resource size. It may require the UE reader to provide specific assistance information, e.g., scheduling parameters for accurate resource allocation. Then the inventory/command request from gNB to UE reader may not contain the AIoT resource information or contain a request for assistance information, which means UE reader transmits assistance information with explicit request from gNB or unsolicited. In this case, the assistance information may include the scheduling parameters in assistance information to assist gNB to allocate the AIoT resource.

Proposal 6: The assistance information from UE reader may include the scheduling parameters determined by UE reader side to assist gNB to allocate the AIoT resource.

Considering gNB may not always to configure accurate resource to UE reader for AIoT transmission, UE reader may transmit the assistance information to gNB to update the allocated resource. For example, in Rel-19 AIoT, a more data indication is introduced to indicate whether there are more upper layer data to be send from the device or not since the allocated TBS may not exactly map the received data size, then from UE reader side, if UE reader receives a more data indication from the A-IoT device and there has no additional available resource at UE reader side, the UE reader may also report such information to the gNB, gNB to schedule more resource for the subsequent D2R transmission and sends it to UE reader. In this case, the assistance information from UE reader to gNB may include at least an indication for more resource, furthermore, expected resource type/size can also be included. On the other hand, for the different paging round, if the required resource for paging round is reduced, the UE reader can also indicate gNB to reduce the resource allocated to the AIoT interface.

Proposal 7: The assistance information from UE reader may include at least an indication to request gNB to configure more/less resource, FFS on the expected resource type/size is needed.

In another hand, to make the UE reader to perform inventory/command request efficiently, gNB may usually give sufficient resource to the UE reader to complete the AIoT services. Then in the case of the UE reader complete the AIoT service, it may send the assistance information to the gNB to release the unused resource, then gNB can make better use of the resource for other services transmission. In this case, the assistance information from UE reader may include a resource release indication if the UE reader has completed the AIoT transmission and there has unused resources.

Proposal 8: The assistance information from UE reader may include a resource release indication in the case of the UE reader has completed the AIoT transmission and there has unused resources.

Regarding how the UE reader to transmit the assistance information, the signaling details for the UE reader to transmit the assistance information is associated with what assistance information is transmitted from the UE reader to gNB. Based on above analysis, the assistance information transmission may be needed during the AIoT service request phase, e.g., for request resources for specific inventory/command request, or AIoT data transmission phase, e.g., to request more resources or to release resource. In NR Uu system, both RRC option and MAC options can be supported for such AIoT resource allocation request, for RRC options, the UE reader may transmit the assistance information by using RRC message, e.g., UE assistance information, etc. While for MAC option, the scheduling request (SR) is used for requesting UL-SCH resource for new transmission, while the buffer status reporting (BSR) is used to provide the serving gNB information about UL data volume in the MAC entity, at least in the case of the request more resource for AIoT data transmission, SR/BSR procedures shall still be workable. It is suggested RAN2 to discuss RRC option or MAC option is used for the UE reader to transmit the assistance information to gNB.

Proposal 9: RAN2 to discuss RRC option or MAC option is used for the UE reader to transmit the assistance information to gNB.

Resource Validity for RRC CONNECTED mode (Scenario 1)

In TR 38.769 [3], the resource validity is studied for scenario 1 and scenario 2, and it’s indicated that down-selection can be decided in WI phase. For scenario 1 (e.g., normal case), two options are as follows:

6.3.6 Topology 2 aspects on the interface between UE reader and RAN For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase): -Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: -Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or -Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

It can be observed that for both option 1 and option 2, gNB may release the allocated A-IoT radio resource by sending RRC reconfiguration message. But it’s still not clear how gNB determines to release the resource. From our point of view, AIoTF and/or UE reader may send some assistance information to gNB as they may know the exact status of the A-IoT procedure (e.g., whether the procedure is completed or not). The assistance information may be the resource release request, the explicit or implicit indication of the end of the procedure/service/paging rounds, etc. that has been discussed in last section.

Proposal 10: For resource validity in scenario 1, RAN2 to confirm that at least network explicitly releases radio resources by RRC reconfiguration message is supported.

In last RAN2 meeting, whether to use option 1 or option 2 to release the resource was discussed. However, no conclusion was reached since whether to use option 1 or option 2 relates the resource allocation method. For example, gNB may allocate periodic unlimited resource for the UE reader e.g. via configured grant, or a kind of resource pool for specific UE reader. On the other hand, gNB may allocate limited resource for the UE reader, e.g. by one shot resource allocation that several numbers of resources are allocated to the UE reader.

The latter resource allocation method, i.e. one-shot allocation with limited resource number, could be used for CFA procedure, which is for single device and required resource is expected not large.

Proposal 11: Support both resource allocation with unlimited resource number, and one-short resource allocation with limited resource number by the gNB in Topology 2.

Such resource allocation may relate to option 2. This depending on how to represent limited resource number e.g. whether use a timer to limit the resource number or not. But compared with a timer-based solution, we are more prefer to use resource number to limit how many resource are allocated. For example, configured grant like resource allocation with additional indication of allocated resource number. This is because the allocated resource may not be continuous, and it is difficult to use timer to control how many resources are used.

For periodic resource allocation with unlimited resources, it’s difficult for gNB to set proper time period when providing A-IoT resource configuration at the beginning as the actual duration of A-IoT procedure is dynamic. For example, the uncertain availability of A-IoT devices may cause multiple paging rounds, which may not be known by network in advance. And the response time of D2R messages is difficult to estimate accurately by network due to the uncertain size of command response, uncertain number of segments, etc.

Based on the above analysis, we prefer not to support option 2 for both unlimited resource allocation and limited resource allocation.

Proposal 12: For the resource validity in scenario 1, option 2 (timer-based release) is not supported.

Resource Validity for temporary out-of-connection (Scenario 2)

In TR 38.769 [3], it was agreed that in RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases), the details are shown as below:

6.3.6 Topology 2 aspects on the interface between UE reader and RAN -Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): -Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations. NOTE 1: The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.)

For Option 1 in Scenario 2, the UE reader considers the resources as temporarily invalid during the temporary out of connection (e.g., RLF and HO cases) when some conditions are satisfied. Some conditions in NR for RLF case and HO case can be reused for determining the resource validity. For Option 2 in Scenario 2, UE reader can determine the resource validity for A-IoT air interface at least based on timer-based mechanism, e.g., the UE reader considers that the resources remain valid for a time period. Under the timer-based mechanism, some timers defined in NR can be reused for the resource validity timer related to AIoT service. Based on the above analysis, both Option 1 and Option 2 in Scenario 2 may reuse conditions and timers in NR, and have less specification impact to support.

Proposal 13: RAN2 is suggested down-selection between the resource validity for a UE reader in temporary out of coverage scenario (e.g., RLF and handover) by considering both Option 1 (e.g., condition-based resource validity) and Option 2 (e.g., timer-based resource validity).

Then, we will analyze the temporary out of connection conditions for both RLF and HO cases. For the temporary out of connection with RLF case, UE reader determines the resources as invalid in recovery and re-establishment phases. For example, UE reader determines the resources as invalid until declaring RLF in recovery phase, and determines the resources as invalid upon successfully re-established to a new cell in the re-establishment phases. While for the temporary out of connection with HO case, UE reader considers the resource is invalid upon receiving HO command in handover preparation phase, and considers the resource as invalid until successfully handovered to a new cell in handover execution phases.

Proposal 14: If Option 1 for Scenario 2 is supported, UE reader determines the resources as invalid by considering any of the condition(s) defined in NR RLF case and HO case is satisfied, e.g., until declaring RLF, or receiving HO command, or successfully re-established or handovered to a new cell, etc.

Next, for the resource validity timer, two alternatives can be considered from RAN2 perspective. One is some timers defined in NR can be reused for the resource validity timer related to AIoT service, e.g., T310, T304, etc. For example, if T310 is reused, UE reader may consider to start the time upon receiving N310 consecutive "out-of-sync" indications from lower layers, and stop the time upon declaring RLF. If T304 is reused, UE reader considers to start the time upon receiving the handover command, and stop the time upon HO failure happens. Another alternative is to define a new resource validity timer for temporary out of connection scenario. RAN2 needs further discussion when to start and stop the defined timer.  For example, UE reader considers to start time upon quality (e.g., RSRP, RSRQ, or SINR) less than a threshold, and stop time upon UE reader successfully re-established or handovered to a new cell.

Proposal 15: If Option 2 for Scenario 2 is supported, RAN2 suggest to study the resource validity timer based on the following two alternatives:

- Alt1: Some timers can be reused for the resource validity timer, e.g., T310, T304, etc.

- Alt2: Define a new timer for temporary out of connection scenario.

For the RRC_CONNECTED UE reader, it may be handovered to a new cell due to mobility or re-established to a new cell due to RLF. When UE reader is handovered/re-established to a new cell, the AIoT procedures maybe still ongoing for the reason that some AIoT procedure (e.g., inventory of a larger number of devices) may take a long time to complete. However, in the WID, the resource is only valid in the serving cell. The incomplete AIoT procedure may be interrupted due to temporary out of connection happens, e.g., RLF or HO. To reduce latency and improve continuity, RAN2 needs to explore how to enable uninterrupted AIoT operation when UE reader enters temporary out of connection scenario.

Proposal 16: RAN2 further study how to support uninterrupted AIoT operation in the case that UE reader enters temporary out of connection scenario.

UE Reader Selection

In TR 38.769 [3], it has been concluded that for the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. Currently SA2 is still waiting for the conclusions from RAN3 on this issue. From RAN2 perspective, it is suggested to wait more conclusions on RAN3 and SA2 for the UE reader selection for Topology 2. In the case of BS selects the UE reader is supported, RAN2 can further check and discuss any radio conditions between UE and BS should be involved in the reader selection.

Proposal 17: RAN2 waits the conclusions on RAN3/SA2 for the UE reader selection in Topology2, and further identify if any RAN2 impacts.

Conclusion

In this contribution, we discussed several aspects for Rel-20 Ambient IoT Topology 2 and made the following observations and proposals:

Reader-gNB Information Exchange

Proposal 1: gNB will provide at least following AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN

- AIoT Identification Information, i.e. paging id, to be included in the paging message

- Follow on command indication in the Inventory Request message

- Assistance information

  - AIoT service type (e.g. Inventory, Command).

  - Optionally, approximate number of AIoT devices based on AF request.

  - Size of the Inventory Response message from the AIoT Device.

  - Size of the Command Response message from the AIoT Device.

Proposal 2: gNB configures similar parameter of ‘time interval for AIoT Device response aggregation’ for UE reader, to reduce the RRC signalling overhead in Uu interface.

Proposal 3: RAN2 is suggested to further discuss transaction id is generated by the UE reader or the gNB in Topology 2.

Proposal 4: An ID on Uu interface is needed to identify AIoT device between UE reader and gNB, and correlates to RAN AIoT Device NGAP ID in Topology 2.

Resource Allocation: Trigger by UE request

Proposal 5: UE reader may transmit assistance information to gNB in following cases:

- Resource allocation with explicit request from gNB or unsolicited.

- Update the allocated/configured resource for AIoT interface transmission.

- Release the allocated/configured resource for AIoT interface transmission.

Proposal 6: The assistance information from UE reader may include the scheduling parameters determined by UE reader side to assist gNB to allocate the AIoT resource.

Proposal 7: The assistance information from UE reader may include at least an indication to request gNB to configure more/less resource, FFS on the expected resource type/size is needed.

Proposal 8: The assistance information from UE reader may include a resource release indication in the case of the UE reader has completed the AIoT transmission and there has unused resources.

Proposal 9: RAN2 to discuss RRC option or MAC option is used for the UE reader to transmit the assistance information to gNB.

Resource Validity for RRC CONNECTED mode (Scenario 1)

Proposal 10: For resource validity in scenario 1, RAN2 to confirm that at least network explicitly releases radio resources by RRC reconfiguration message is supported.

Proposal 11: Support both resource allocation with unlimited resource number, and one-short resource allocation with limited resource number by the gNB in Topology 2.

Proposal 12: For the resource validity in scenario 1, option 2 (timer-based release) is not supported.

Resource Validity for temporary out-of-connection (Scenario 2)

Proposal 13: RAN2 is suggested down-selection between the resource validity for a UE reader in temporary out of coverage scenario (e.g., RLF and handover) by considering both Option 1 (e.g., condition-based resource validity) and Option 2 (e.g., timer-based resource validity).

Proposal 14: If Option 1 for Scenario 2 is supported, UE reader determines the resources as invalid by considering any of the condition(s) defined in NR RLF case and HO case is satisfied, e.g., until declaring RLF, or receiving HO command, or successfully re-established or handovered to a new cell, etc.

Proposal 15: If Option 2 for Scenario 2 is supported, RAN2 suggest to study the resource validity timer based on the following two alternatives:

- Alt1: Some timers can be reused for the resource validity timer, e.g., T310, T304, etc.

- Alt2: Define a new timer for temporary out of connection scenario.

Proposal 16: RAN2 further study how to support uninterrupted AIoT operation in the case that UE reader enters temporary out of connection scenario.

UE Reader Selection

Proposal 17: RAN2 waits the conclusions on RAN3/SA2 for the UE reader selection in Topology2, and further identify if any RAN2 impacts.

Reference

RP-251885, New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2

RP-252964, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

TR 38.769 V2.0.1 (2024-12), Study on solutions for ambient IoT (Internet of Things)

3GPP RAN2#131 bis meeting, chair notes.

TS 23.369, Architecture support for Ambient power-enabled Internet of Things; Stage 2