3GPP TSG-RAN WG2 Meeting #132	R2-2508653

Dallas, USA, 17th – 21st November 2025

Agenda Item:	8.7.2

Source:	Lenovo

Title:	Discussion on RLC-H02 and RLC-N02

Document for: Discussion and Decision

# 1	Introduction

In this contribution, we will discuss remaining RLC issues related to avoiding unnecessary retransmissions.

# 2. Discussion

RAN2#131 bis meeting has reached the following agreement on issues on RLC:

Agreements for RLC - (RLC-X01) We keep the spec as it is. - (RLC-V01) No need to indicate the SN of obsolete RLC SDUs to the receiving PDCP entity after t-RxDiscard expires. - (RLC-N01) We will clarify RX_Next state variable update when t-RxDiscard expires. FFS an exact wording and where to do the change (e.g. procedure or variable definition) - (RLC-N01) Try to capture it in the procedure “If t-RxDiscard is configured and expires, the receiving side of an AM RLC entity consider the corresponding SDUs as received.”, if problem is identified or companies have strong concern during CR phase, it will be postponed. - (RLC-N01) Add “or t-RxDiscard expires” at the end of definition of RX_Next. - (RLC-N02) In NOTE in 5.3.3.3 we add “If stopReTxDiscardedSDU is configured” at the beginning of the sentence. - (RLC-N02) FFS whether any other changes are needed, e.g. as in R2-2507159 - (RLC-E01) No change is needed. - (RLC-S01) No change needed as per the previous agreement - (RLC-N02) Companies will bring contribution in next meeting to explain the use cases and the TP. - (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it. - RLC-V02 is closed without any spec impact. - RLC-H01 is updated according to the CR in R2-2507016.

RLC-H02

The expiry of t-RxDiscard triggers both RX_Next to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Next is updated. RAN2 has confirmed that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. RX_Highest_Status holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. To maintain consistency in specification, RX_Highest_Status should be updated to be aligned with the definition of the “ACK_SN”, RX_Highest_Status should be updated, e.g., to the SN of the first RLC SDU with SN >= RX_Next for which not all bytes have been received, or the SN of the first RLC SDU with SN >= RX_Highest_Status for which not all bytes have been received, and which is not discarded due to the expiry of t-RxDiscard.

In this case, there is no RLC SDU that will be included as NACK RLC SDU in status report, and the ACK_SN is just the updated RX_Next, wherein the ACK_SN >= RX_Next > RLC SN of discarded RLC SDU. We also suggest to directly set the ACK_SN to the RX_Next when Status PDU is triggered by the expiry of t-RxDiscard as it is clearer and simpler.

Proposal 1: [RLC-H02] To maintain consistency in specification, update RX_Highest_Status upon the expiry of t-RxDiscard. E.g., Update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next for which not all bytes have been received in Appendix A.

Proposal 2: [RLC-H02] Set the ACK_SN to the RX_Next if Status PDU is triggered by the expiry of t-RxDiscard in Appendix A.

RLC-N02

There is one open issue from the last meeting which is related to the RLC polling function. There was further discussion in RAN2#131bis meeting on whether the Note in section 5.3.3.3 needs to be changed in order to accommodate the case when there is an RLC SDU which is negatively acknowledged and hence still under transmission. In this case, some companies argue that the transmitter should be able to keep polling and hence t-PollRetransmit should not be stopped.

NOTE: If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

In our understanding there is not a problem if t-PollRetransmit is stopped by the UE (RLC Tx) since the RLC transmitting entity has already obtained a status for each RLC SDU with SN up to POLL_SN, so retransmission of the poll is no longer needed. For ongoing retransmissions of RLC SDU(s)/ segments thereof a RLC status report will be anyway triggered by the receiving entity in case there are still further retransmission necessary, e.g. based on expiry of t-Reassembly. Therefore, there is no problem to stop and reset the running t-PollRetransmit even some RLC SDUs with SNs up to and including POLL_SN are already negatively acknowledged.

Hence, we think no further change of the NOTE is required.

Proposal 3: [RLC-N02] RAN2 to agree that no further change to NOTE in section 5.3.3.3 is needed.

# 3. Conclusion

This contribution makes the following proposals:

Proposal 1: [RLC-H02] To maintain consistency in specification, update RX_Highest_Status upon the expiry of t-RxDiscard. E.g., Update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next for which not all bytes have been received in Appendix A.

Proposal 2: [RLC-H02] Set the ACK_SN to the RX_Next if Status PDU is triggered by the expiry of t-RxDiscard in Appendix A.

Proposal 3: [RLC-N02] RAN2 to agree that no further change to NOTE in section 5.3.3.3 is needed.

# 4. Reference

[1] R2-250xxxx, List of open issues for Rel-19 XR 38.322 CR

[2] R2-2506338, Introduction of R19 XR enhancements for RLC spec.

- Appendix A

##### 5.2.3.2.x	Actions when t-RxDiscard expires

When t-RxDiscard expires, the receiving side of an AM RLC entity shall:

-	discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any;

-	update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received;

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next for which not all bytes have been received;

-	if RX_Next_Highest > RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-RxDiscard;

-	set RX_Next_Discard_Trigger to RX_Next_Highest.

### 5.3.4	Status reporting

…

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

NOTE X:	The expiry of t-RxDiscard triggers both RX_Next to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Next is updated.

…

When constructing a STATUS PDU triggered by the expiry of t-RxDiscard, the AM RLC entity shall:

-	set the ACK_SN to RX_Next in the resulting STATUS PDU.

Otherwise, when constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.