3GPP TSG-RAN WG2 Meeting #132	R2- 2508654

Dallas, USA, Nov. 17th – 21st, 2025

Agenda item:	   9.3.3

Source:		Lenovo

Title:                            Discussion on LCM for measurement event prediction

Document for:		Discussion and Decision

- Introduction

As instructed by the WID [1], the AI/ML for mobility WI will specify to support for RRM measurement prediction and measurement event prediction for UE sided models. In this paper, we discuss essential LCM functionality management aspects for measurement event prediction. During AI/ML for mobility for SI phase in TR 38.744 [2], some agreements have been reached on LCM, protocol and procedure aspects for RRM measurement event prediction, shown as below besides section 6.1.2.1 which is applicable for both RRM and event prediction:

### 6.1.3 Measurement event prediction It is up to UE’s implementation to decide on choice between indirect and direct event prediction methodology. On top of inference parameters captured in section 6.1.2.1.2, event-related parameters e.g. event type are part of inference configuration of measurement event prediction. UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. UE can report measurement results, predicted measurement event together with its timing related information. When UE is configured with intra-frequency temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact. For indirect event prediction, RSRP difference can be used as performance monitoring metric. NOTE 1: The spec impact captured in section 6.1.2.1 is applicable for measurement event prediction unless otherwise described explicitly in this section. NOTE 2: A single framework is aimed for direct and indirect event prediction. NOTE 3: There is no consensus on the feasibility of performance monitoring of direct event prediction. it should be concluded before proceeding with normative work.

In this contribution, we continually discuss following aspects for LCM functionality management aspects for measurement event prediction, based on the SI conclusion in TR 38.744 [2] in above:

- Applicability reporting

- Inference configuration

- Inference reporting

- Handover models

- Discussion

2.1. Applicability reporting

At the RAN2#131 meeting, it had discussed the applicability related issues for RRM measurement event prediction, e.g., applicability determination and applicability reporting, etc. It was agreed that “What the UE requires to the determine applicability is similar to RRM prediction.” [3]. For indirect measurement event prediction, measurement event prediction is performed based on RRM measurement prediction. The applicability determination or applicability reporting is the same as that for RRM measurement prediction. While for direct measurement event prediction, measurement event is predicted directly by an AI/ML model. Hence, except for the conditions used for applicability determination used for RRM measurement prediction, e.g., NW-side additional conditions (if provided, e.g., associated ID), UE-side additional conditions and functionality availability in the UE, RAN2 has agreed event-related parameters e.g. event type are part of inference configuration of measurement event prediction. Hence, except for the event type, RAN2 to further discuss whether other event-related parameters, e.g., hysteresis, TTT, etc. are also needed for applicability determination of an AI/ML model.

Proposal 1: RAN2 to discuss whether other event-related information except for event type, e.g., hysteresis, TTT, etc. are needed for UE determining the applicability of an AI/ML model.

2.2. Inference configuration

The events A1-A5, which are utilized to evaluate the signal quality of serving and neighbour cells located at intra/inter frequency, are designed for mobility purposes. When one of the events is met, UE is triggered to report the measurement results. Once the network receives the measurement results, the network will decide whether to prepare a handover and which neighbour cell can be selected as the target cell. Event A3 was agreed as a baseline since it is most relevant for mobility.

In current specifications, the entering condition and leaving condition are specified for each event. When the entry condition of an event is met during TTT, the event is determined as satisfied. This implies UE enters the cell edge and consequently, the measurement report is triggered. When the leaving condition of an event is met during TTT, it means UE returns to the cell centre from the cell edge. UE is triggered to report the measurement result if reportOnLeave is configured. Similarly, the network can configure UE to predict whether the entering condition or the leaving condition is fulfilled. The prediction of the leaving condition could be helpful for the network to decide a suitable time instance for handover execution.

UE receives a configuration for event prediction. Meanwhile, a prediction window may be provided together with the configuration, and UE is expected to predict whether an event occurs within the prediction window.

Proposal 2: UE is expected to predict whether an event occurs within the configured prediction window based on whether (a) entering condition or (b) leaving condition of a concerned RRM event will be met.

During RAN2#129 bis meeting, it was agreed that “The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference” [4]. This topic was further discussed in the following RAN2 #130 and RAN2#131 meetings, however, no consensus was reached regarding to this issue. There may be the following two options to solve this issue:

- Option 1: UE may start to perform inference based on the received inference configuration upon determining the AI/ML functionality as applicable.

- Option 2: UE may start to perform inference when a certain condition is satisfied, e.g., s-measure-like etc.

It is obvious that Option 1 has some disadvantages that UE may perform unnecessary prediction and infer unnecessary output, e.g., the outputs are from UE in cell centers, etc., which will consume huge UE power to do these useless predictions. RAN2 needs to discuss the solutions on when UE starts to perform inference.

Proposal 3: RAN2 to discuss when UE will start to perform inference with the following options:

- Option 1: UE starts to perform inference based on the received inference configuration upon determining the AI/ML functionality as applicable.

- Option 2: UE starts to perform inference when a certain condition is satisfied, e.g., s-measure-like.

2.3. Inference reporting

UE can be configured to report the prediction result for a configured event, which may be based on the configured prediction window. It has been agreed that UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). Moreover, for the content of UE reporting, it is also agreed that UE can report measurement results, predicted measurement event together with its timing related information. While for direct measurement event prediction, except for the predicted measurement event as well as the timing related information, UE needs to report the probability of event occurrence within PW. Hence, if the direct prediction is applied, the probability of event occurrence within PW may be reported as well.

Proposal 4: If the direct prediction is applied, the probability of event occurrence within PW may be reported.

2.4 Handover models

For RRM measurement event prediction with temporal domain, RAN2 had reached the consensus on how to decide on the time point to transmit handover command with the following options [2]:

The handover model is defined to facilitate SLS, where measurement event is predicted based on either intra-frequency temporal domain case A or intra-frequency temporal domain case B. For both cases, network starts with 40ms handover preparation once a predicted measurement event is received. A handover command will be transmitted at least after preparation is completed. After handover command, 40ms execution duration is assumed. If measurement event is predicted based on intra-frequency temporal domain case A, there are two options w.r.t. how to decide on the time point to transmit handover command: Option 1: Relying on legacy measurement event Option 2: Relying on predicted measurement event Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time, i.e. t0, measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier. If measurement event is predicted based on intra-frequency temporal domain case B, there is option 3 w.r.t. how to decide on the time point to transmit handover command: Figure 5.5.1-3: Handover model option 3 Option 3 is illustrated in Figure 5.5.1-3. Once a predicted measurement event e.g. A3 event is received network can transmit handover command immediately after handover preparation is completed. UE will report predicted measurement event at the time instance it is to be triggered.

For RRM measurement event prediction with temporal domain case A, UE can report measurement results, predicted measurement event together with its timing related information. After NW receives the reported results from UE, it is up to NW to decide when to transmit handover command, e.g., based on the legacy measurement event or predicted measurement event.

Proposal 5: RAN2 confirms it is NW implementation to decide when to transmit handover command, e.g., relying on legacy measurement event, or relying on predicted measurement event.

On the other hand, for NW to make the best decision on when to trigger the handover, after UE reports the predicted RRM event, we believe it is beneficial for UE to continue evaluating the validity of the reported RRM event prediction, and notify NW in case of any update/invalidity.

It may cause handover failure if a previous reported prediction is not precise and NW triggers the handover based on it, e.g., as in Handover Model Option 3. Taking Handover Model Option 2 as another example, between t0 and t2, the UE may predict (e.g., based on continuous prediction) or determine (e.g., based on monitoring) the RRM event will probably not happen at t1 any more, and triggering handover at t2 may cause problem. In those cases, NW may decide to fallback to non-AIML based handover triggering, e.g., handover model Option 1.

Proposal 6: To support optimal handover timing, mechanism is needed for the UE to continue evaluating the validity of a previously reported RRM event prediction, and notify NW in case of any update/invalidity.

- Conclusion

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

Proposal 1: RAN2 to discuss whether other event-related information except for event type, e.g., hysteresis, TTT, etc. are needed for UE determining the applicability of an AI/ML model.

Proposal 2: UE is expected to predict whether an event occurs within the configured prediction window based on whether (a) entering condition or (b) leaving condition of a concerned RRM event will be met.

Proposal 3: RAN2 to discuss when UE will start to perform inference with the following options:

- Option 1: UE starts to perform inference based on the received inference configuration upon determining the AI/ML functionality as applicable.

- Option 2: UE starts to perform inference when a certain condition is satisfied, e.g., s-measure-like.

Proposal 4: If the direct prediction is applied, the probability of event occurrence within PW may be reported.

Proposal 5: RAN2 confirms it is NW implementation to decide when to transmit handover command, e.g., relying on legacy measurement event, or relying on predicted measurement event.

Proposal 6: To support optimal handover timing, mechanism is needed for the UE to continue evaluating the validity of a previously reported RRM event prediction, and notify NW in case of any update/invalidity.

- References

[1] RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

[2] TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR (Release 19)

[3] RAN2 #131 Chairman Notes, India, Aug. 2025.

[4] RAN2 #129bis Chairman Notes, China, April 2025.