3GPP TSG-RAN WG2 Meeting #132	R2-2508660

Dallas, USA, Nov. 17th - 21st ,2025

Agenda item: 		9.7.2

Source:		Lenovo

Title:		Considerations on UP solution for voice support over IoT-NTN

Document for:		Discussion

- Introduction

The latest RAN2#131bis meeting made the following agreements regarding voice support over IoT-NTN:

Agreements regarding the reply LS to SA2: - Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope - Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues. - Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20. - Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet - We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

and

Agreements regarding the reply LS to SA4: - RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes. - For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size - For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size - RAN2 will provide further feedback on the expected average RoHC header size - We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

As per last meeting discussion and the reply LS to SA2, majority of companies in RAN2 prefer the UP over CP solution. Meanwhile, SA2 has added the following interim conclusion:

8.1 Interim conclusions for Key Issue #1 Editor's note 1: Other interim conclusions for Key Issue#1 is FFS. The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made: - The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

In our understanding although final decision is to be made in December plenary, it means that there is a greater possibility of UP solution than CP solution, or at least UP solution will be supported. In this contribution we discuss potential enhancement of NB-IoT-NTN to support IMS voice call over GSO, focusing on the further impacts and optimizations adopting UP-based solution (User Plane CIoT EPS/5GS optimization).

- Discussion

In cases without TN coverage, especially during disasters when TN infrastructure is unavailable, establishing real-time services including voice calls is essential. The NTN, in particular the GSO satellites, can provide wide, robust and sustainable coverage with sparser constellation. Meanwhile the NB-IoT devices with low complexity has much lower requirement for link budgets. As a result, support of voice over GSO for IoT NTN becomes an attractive way to facilitate global coverage of voice service especially for emergency situations.

To implement the support, SA2#171 have made an interim conclusion indicating that at least the UP-based solution will be supported.

The UP-based solution, namely User Plane CIoT EPS/5GS optimization, is part of the CIoT signalling reduction optimizations introduced in LTE to fulfill the requirements of small-size data transmission and low power consumption. The main differences between the CP and UP options are that the CP option uses RRC container message to carry small-size user data in NAS message, while the UP solution suspends connection instead of releasing it after data transmission so that the connection can be easily and quickly resumed with stored context.

## QoS differentiation and number of DRBs supported

To provide voice support in IoT NTN, it is expected that at least the QoS differentiation for SIP signaling and voice data transmission shall be guaranteed. This would be implemented by using two bearers, wherein for CP solution there is only one SRB with RLC AM mode in NB-IoT. As it cannot provide QoS differentiation between SIP signalling and voice data transmission, new SRB with RLC UM mode has to be supported. However, for UP solution things are much easier as two DRBs with configurable RLC mode has already been supported.

Proposal 1: For IMS voice over NB-IoT NTN, it is up to NW implementation to use the supported 2 DRBs for signaling and voice differentiation.

Beyond the fundamental support of 2 DRBs for QoS differentiation for SIP signaling and voice data transmission, there were also some discussions on support of more than 2 DRBs, e.g., in case that the data service needs to be carried simultaneously, and one more DRB can further implement service differentiation. However, the main challenge of supporting IMS voice over NB-IoT NTN come from the NTN characteristics which is irrelevant to the service pattern. Therefore from RAN2 perspective we see no essential needs to support more than 2 DRBs, unless there is new requirement (e.g. data service and voice at the same time) is identified.

Proposal 2: For IMS voice over NB-IoT NTN, there is no need to support more than 2 DRBs unless new requirement (e.g. data service other than voice at the same time) is identified.

## SPS configuration

As per SA2 study and discussion, two phases with significant difference are identified for voice packets, namely talk phase and silence phase. While SPS is considered to be useful for scheduling the resources for voice packets, this difference between phases has to be considered when SPS is configured, e.g., following the talk phase pattern results in over scheduling while following the silence phase results in under scheduling on the opposite. There were also proposals to have two sets of SPS configurations, but it will definitely increase the complexity of NB-IoT including additional configuration and switch between configurations.

Proposal 3: RAN2 to discuss how to configure SPS for IMS voice over NB-IoT NTN to minimize over/under scheduling, e.g., for the talk phase and the silence phase.

## RRC connection establishment

As per SA1 study and discussion, there is a performance requirement for the call set up time within 30s for satellite access. For IoT NTN considering the large RTT issue, it is expected to reduce the latency of RRC connection establishment when voice service is triggered. A straight-forward solution would be introducing a new establishment cause for the voice service so that the network can recognize and even prioritize it.

Proposal 4: RAN2 to discuss whether to introduce new establishment cause for voice call over NB-IoT NTN to reduce the call set up time.

- Conclusion

In this contribution we discuss potential enhancement of NB-IoT-NTN to support IMS voice call over GSO. We propose the following:

Proposal 1: For IMS voice over NB-IoT NTN, it is up to NW implementation to use the supported 2 DRBs for signaling and voice differentiation.

Proposal 2: For IMS voice over NB-IoT NTN, there is no need to support more than 2 DRBs unless new requirement (e.g. data service other than voice at the same time) is identified.

Proposal 3: RAN2 to discuss how to configure SPS for IMS voice over NB-IoT NTN to minimize over/under scheduling, e.g., for the talk phase and the silence phase.

Proposal 4: RAN2 to discuss whether to introduce new establishment cause for voice call over NB-IoT NTN to reduce the call set up time.