3GPP TSG-RAN WG2 Meeting #132	R2-2508662

Dallas, USA, 17 - 21 November 2025

Agenda Item:	8.12.2

Source:	Huawei, HiSilicon

Title:	RRC and Stage 2 issues for MIMO

Document for:  Discussion and Decision

# 1	Introduction

For Rel-19 MIMO, significant progress has been made in previous meetings. In RAN2 #131-bis, all the proposed RIL issues have been addressed, and several agreements for spec modification were achieved. Though the NR_MIMO_Ph5 work item was declared complete in RAN2, some necessary clarifications on RRC parameter definitions are needed as well as on Stage 2 details, for a better alignment with the expected functionality of the new feature.

This contribution provides additional analysis regarding the asymmetric mTRP and UEIBM procedure.

# 2	Discussion

## 2.1	RRC issues

### 2.1.1	[H407] startingBitOfFormat2-3-r19

In RAN1 #118-bis, about the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support

- Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is appliable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

The above agreements have been captured in the MIMO RRC CR, as shown below:

SRS-TPC-CommandConfig information element

-- ASN1START

-- TAG-SRS-TPC-COMMANDCONFIG-START

SRS-TPC-CommandConfig ::=               SEQUENCE {

startingBitOfFormat2-3                  INTEGER (1..31)                                                     OPTIONAL,   -- Need R

fieldTypeFormat2-3                      INTEGER (0..1)                                                      OPTIONAL,   -- Need R

...,

[[

startingBitOfFormat2-3SUL               INTEGER (1..31)                                                     OPTIONAL    -- Need R

]],

[[

startingBitOfFormat2-3-r19            INTEGER (1..45)                                                     OPTIONAL    -- Need R

]]

}

-- TAG-SRS-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

startingBitOfFormat2-3 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4). The network does not configure both startingBitOfFormat2-3 and startingBitOfFormat2-3-19 simultaneously.

The parameter startingBitOfFormat2-3-r19 uses the ‘-r19’ suffix. A ‘-r19’ suffix is normally reserved for defining a new version expecting to replace the old version. However, the range of the new field overlaps with the range of the existing field and the meaning is the same, regardless which version is used, so such a definition is wasting bits for nothing.

Proposal 1: The value range of startingBitOfFormat2-3-r19 be modified to integer (32..45), and its suffix be concurrently amended from -r19 to -v1900.

### 2.1.2	pathlossOffset

TCI-UL-State field descriptions

pathlossOffset Indicates the pathloss offset applied to the CI tate of an UL-only TRP (see TS 38.213 [13], clause 7). Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

According to the agreement, in the MIMO RRC CR, the field description for pathlossOffset under TCI-State and TCI-UL-State IEs has been generalized, clarifying that the field applies to TCI state of an UL-only TRP. However, the use of "UL-only TRP" is a little problematic. For one thing, the term "TRP" is being phased out in favor of more precise terminology (e.g., "cell," "beam," "panel") in many parts of the specification. Using "TRP" in new or updated parameter descriptions is inconsistent with this general direction and can lead to ambiguity. For another, The term "UL-only TRP" is not used in TS 38.300 and the corresponding Work Item description, where it is officially named as "UL TRP". In our understanding, one solution is just to remove the word "only", changing the current description from "UL-only TRP" to "UL TRP", another solution is completely remove the phrase "UL-only TRP". The corresponding field description of the second solution is shown in Annex A.

Proposal 2: In the field description of pathlossOffset, change the description from "UL-only TRP" to "UL TRP", or completely remove the phrase "UL-only TRP".

### 2.1.3	minimumPucch-PuschOffset

In the current RRC spec, the field description of minimumPucch-PuschOffset in CSI-ReportUE-Initiated is shown below. It states that “Value sym0 corresponds to 0, value sym1 corresponds to 1, and so on.” According to the legacy field description of “sym0” and “sym1”, the current description should be modified for alignment. Thus, the field description of “sym0” and “sym1” in minimumPucch-PuschOffset should be “Value sym0 corresponds to the first symbol, value sym1 corresponds to the second symbol, and so on.”

CSI-ReportUE-Initiated field descriptions

minimumPucch-PuschOffset Indicates the time offset in number of symbols for determining available transmission occasion of PUSCH from the PUCCH for mode-B UE initiated CSI reporting. Value sym0 corresponds to 0, value sym1 corresponds to 1 and so on.

SL-Unlicensed field descriptions

sl-StartingSymbolFirst Indicates the location of first starting symbol within a slot. Value sym0 corresponds to first symbol, value sym1 corresponds to the second symbol and so on. If the field is not configured, the UE shall use value sym0.

Proposal 3: Thus, the field description of “sym0” and “sym1” in minimumPucch-PuschOffset should be “Value sym0 corresponds to the first symbol, value sym1 corresponds to the second symbol, and so on.”

The corresponding TP is provided in the Annex A.

## 2.2	Stage 2

Regarding the Stage 2 level details of TS 38.300, the Rel-19 asymmetric DL/UL mTRP feature is described in clause 6.12.1:

### 6.12.1	Asymmetric DL single-TRP and UL multi-TRP

For asymmetric DL single-TRP and UL multi-TRP operation, the UL TRP may reduce or even disable DL transmission. Pathloss offsets between two TRPs can be configured by RRC and dynamically updated by Pathloss Offset Update MAC CE, which defined in 3GPP TS 38.321[6]. Each pathloss offset is explicitly indicated for the corresponding UL/Joint TCI state for PUSCH, PUCCH, and SRS transmission toward the UL TRP. The pathloss offset can also be indicated by a PDCCH order for a PDCCH order triggered PRACH toward the UL TRP, thereby facilitating pathloss calculation. In addition, up to two closed loop power control adjustment states can be supported for SRS separated from those used for PUSCH.

In our understanding, the current text in the specification, while technically correct, is ambiguous and lacks necessary precision for implementation. The core issue with the phrase "reduce or even disable DL transmission" is its inherent vagueness. In the context of network implementation, "reduce" is a relative term that requires a reference point. Does "reduce" mean transmitting only control channels? Does it imply a lower power? Without concrete examples, different equipment vendors and network operators could interpret these terms differently, leading to interoperability failures.

In RAN1 #119, it was agreed that

Agreement

The answer to the Question 1 in LS R1-2409353 is:

•	From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

This agreement explicitly defines a key use case for a "reduced" DL state. In this mode, the UL Transmission/Reception Point (TRP) maintains a minimal downlink presence solely for essential functions. Meanwhile, it would not transmit any user-plane or dedicated control-plane data, thus fulfilling the "asymmetric" nature of the operation where its primary role is uplink reception.

Therefore, we propose to add the two clear, unambiguous, and implementable examples “only SSB for DL transmission or totally without any DL” after the definition of asymmetric DL/UL mTRP.

Futhermore, regarding the two closed loop power control states added for SRS, we propose to clarify that one state applies to the DL TRP while the second state applies to an UL TRP, which is correspondent with the Work Item Description.

Based on the above analysis, we propose the Stage 2 clarifications in Annex B.

Proposal 4: Clarify the Rel-19 asymmetric DL/UL mTRP feature in TS 38.300 as Annex B.

# 3	Conclusion

From the analysis on Rel-19 MIMO study, we made the following proposals:

Proposal 1: The value range of startingBitOfFormat2-3-r19 be modified to integer (32..45), and its suffix be concurrently amended from -r19 to -v1900.

Proposal 2: In the field description of pathlossOffset, change the description from "UL-only TRP" to "UL TRP", or completely remove the phrase "UL-only TRP".

Proposal 3: Thus, the field description of “sym0” and “sym1” in minimumPucch-PuschOffset should be “Value sym0 corresponds to the first symbol, value sym1 corresponds to the second symbol, and so on.”

Proposal 4: Clarify the Rel-19 asymmetric DL/UL mTRP feature in TS 38.300 as Annex B.

# Annex A: TP for TS 38.331

### 6.3.2	Radio resource control information elements

#### –	SRS-TPC-CommandConfig

The IE SRS-TPC-CommandConfig is used to configure the UE for extracting TPC commands for SRS from a group-TPC messages on DCI.

SRS-TPC-CommandConfig information element

-- ASN1START

-- TAG-SRS-TPC-COMMANDCONFIG-START

SRS-TPC-CommandConfig ::=               SEQUENCE {

startingBitOfFormat2-3                  INTEGER (1..31)                                                     OPTIONAL,   -- Need R

fieldTypeFormat2-3                      INTEGER (0..1)                                                      OPTIONAL,   -- Need R

...,

[[

startingBitOfFormat2-3SUL               INTEGER (1..31)                                                     OPTIONAL    -- Need R

]],

[[

startingBitOfFormat2-3-v1900            INTEGER (32..45)                                                     OPTIONAL    -- Need R

]]

}

-- TAG-SRS-TPC-COMMANDCONFIG-STOP

-- ASN1STOP

SRS-TPC-CommandConfig field descriptions

fieldTypeFormat2-3 The type of a field within the group DCI with SRS request fields (optional), which indicates how many bits in the field are for SRS request (0 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its own TPC command bits. See TS 38.212 [17] clause 7.3.1 and , TS 38.213 [13], clause 11.4.

startingBitOfFormat2-3, startingBitOfFormat2-3-v1900 The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4). The network does not configure both startingBitOfFormat2-3 and startingBitOfFormat2-3-v1900 simultaneously.

startingBitOfFormat2-3SUL The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands for SUL carrier (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4).

#### –	TCI-State

TCI-State field descriptions

additionalPCI Indicates the physical cell IDs (PCI) of the SSBs when referenceSignal is configured as SSB for both QCL-Type1 and QCL-Type2. In case the cell is present, the additionalPCI refers to a PCI value configured in the list configured using additionalPCI-ToAddModList in the serving cell indicated by the field cell. Otherwise, it refers to a PCI value configured in a list additionalPCI-ToAddModList configured in the serving cell where the TCI-State is applied by the UE. When this field is present the cell for qcl-Type1 and qcl-Type2 is configured with same value, if present.

pathlossOffset Indicates the pathloss offset as specified in TS 38.213 [13], clause 7. Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

pathlossReferenceRS-Id The ID of the reference signal (e.g. a CSI-RS or an SS block) used for PUSCH, PUCCH and SRS path loss estimation. This field refers to an element in the list configured using pathlossReferenceRSToAddModList in the serving cell and UL BWP where the TCI State is applied by the UE.

qcl-Type1, qcl-Type2 QCL information for the TCI state as specified in TS 38.214 [19] clause 5.1.5.

tag-Id-ptr It indicates the TAG that is associated with this TCI state, value n0 means the TCI state associate with the TAG indicated by tag-Id, value n1 means this TCI state associated with the TAG indicated by tag2-Id. The tag-Id-ptr refers to the TAG of the serving cell where the TCI state is applied.

tci-StateId ID number of the TCI state.

ul-PowerControl Configures power control parameters for PUCCH, PUSCH and SRS for this TCI state. The field is present here only if ul-powerControl is not configured in any BWP-Uplink-Dedicated of this serving cell. This field refers to an element in the list configured using uplink-PowerControlToAddModList in the serving cell where the dl-OrJointTCI-StateToAddModList is configured. For each serving cell, ul-powerControl is either configured in each TCI-State configured or referred to in dl-OrJointTCI-StateList in each DL BWP, or it is not configured in any TCI-State configured or referred to in dl-OrJointTCI-StateList in any DL BWP.

#### –	CSI-ReportConfig

CSI-ReportUE-Initiated field descriptions

currentBeamReport If configured, the UE includes measurements of the current beam in the UE initiated CSI reporting.

eventDetectionTimeWindow Indicates the time window length for triggering event determination (see TS 38.214 [19], clause 5.2). Value ms4 corresponds to 4 milliseconds, value ms5 corresponds to 5 milliseconds and so on.

eventInstanceCount Indicates the minimum number of event instances for one same new beam within a configured time window that the UE can initiate SI report (see TS 38.214 [19], clause 5.2.1.5.4.1).

eventTypeUE-Initiated Indicates the event type for UE initiated CSI reporting and associated fields as specified in clause 5.2.1.5.4 of TS 38.214 [19]. When this field is set to event1, event1Threshold can only be configured with values from 16 to 113. The parameter conditionFulfillmentIndicator is only configured if eventDetectionTimeWindow is configured and nrofReportedRS-UE-Initiated is not set to n1.

minimumPucch-PuschOffset Indicates the time offset in number of symbols for determining available transmission occasion of PUSCH from the PUCCH for mode-B UE initiated CSI reporting. Value sym0 corresponds to first symbol, value sym1 corresponds to second symbol and so on.

# Annex B: TP for TS 38.300

### 6.12.1	Asymmetric DL single-TRP and UL multi-TRP

For asymmetric DL single-TRP and UL multi-TRP operation, the UL TRP may reduce or even disable DL transmission (i.e. only SSB for DL transmission or totally without any DL). Pathloss offsets between two TRPs can be configured by RRC and dynamically updated by Pathloss Offset Update MAC CE, which is defined in 3GPP TS 38.321[6]. Each pathloss offset is explicitly indicated for the corresponding UL/Joint TCI state for PUSCH, PUCCH, and SRS transmission toward the UL TRP. The pathloss offset can also be indicated by a PDCCH order for a PDCCH order triggered PRACH toward the UL TRP, thereby facilitating pathloss calculation. In addition, up to two closed loop power control adjustment states, one for the DL TRP and the other for UL TRP, can be supported for SRS separated from those used for PUSCH.