3GPP TSG-RAN WG2 Meeting #131 bis	R2-2508663

Dallas, USA, 17 - 21 November 2025

Agenda Item:	8.12.2

Source:	Huawei, HiSilicon

Title:	MAC issues for MIMO

Document for:	Discussion and Decision

# 1	Introduction

RP-242394, Revised Work Item: NR MIMO Phase 5 The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting.

The Rel-19 Mobility Enhancement WI was completed during the last two meeting. It was agreed that the overall UEIBM procedure should be captured in RAN1, while RAN2 would address the MAC impacts based on that procedure. RAN2 has already made several agreements for modification of MAC specification. According to the latest MAC running CR and MAC open issue list, there are some remaining issues that require further discussion.

This contribution provides additional analysis on the remaining MAC issue for MIMO.

# 2	Discussion

## 2.1	UEIBM

Summary: Based on comments so far, Option 2 seems acceptable for UE power saving in DRX, but needs to take into account the possible different UE implementations on whether continuously transmitting UEIRIs for a triggered UEI CSI report. Based on the current specification and suggested wording, Proposal 2: For DRX, after transmitting UEIRI on PUCCH, UE stays in active time until receiving PDCCH scheduling the mode-A UEI CSI report on PUSCH or until the next PUCCH occasion for UEIRI, whichever comes the first. TP baseline: When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]), a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received and the next PUCCH occasion to transmit UE Initiated Report Indication has not come.

According to the current MAC spec, it is mentioned that in DRX mode, the UE stays in active time until receiving PDCCH scheduling from the network. However, some companies argue that if the PDCCH for CSI reporting is never received (either because the PDCCH is not scheduled by the network after it receives the UEIRI, or because the network fails to receive the UEIRI on PUCCH) the UE will have to keep staying in DRX active time as the condition is always met, which would cause unnecessary power consumption.

Thus, option 2 has been accepted in the email discussion, and the suggested TP is shown above. However, the phrase “the next PUCCH occasion to transmit UE Initiated Report Indication has not come” may lead to different interpretations, implying “any PDCCH scheduling regardless of purpose.” To avoid such misunderstanding, it should be specified that the next occasion to transmit a UEIRI on PUCCH refers to the one used for the same mode-A CSI reporting as the PDCCH scheduling. The possible TP can be shown in the Annex.

Proposal 1: In DRX mode, the UE stays in active time until receiving PDCCH scheduling from the network and the next occasion to transmit a UEIRI on PUCCH for the same mode-A CSI reporting.

The corresponding TP is provided in the Annex.

# 3	Conclusion

In this paper, we have addressed some analysis on MAC remaining issue for UEIBM procedure. The text proposal is attached in Annex and the proposal of the document is shown:

Proposal 1: In DRX mode, the UE stays in active time until receiving PDCCH scheduling from the network and the next occasion to transmit a UEIRI on PUCCH for the same mode-A CSI reporting.

# 4	Reference

- RP-242394, WID revision: NR MIMO Phase 5;

- R2-2507938, Corrections for MIMO Phase 5

# 5	Annex – TP

3GPP TS 38.331

## 5.7	Discontinuous Reception (DRX)

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer, drx-InactivityTimer or lpwus-PDCCH-MonitoringTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or

-	there is an ongoing RACH-less LTM cell switch; or

-	there is an ongoing RACH-less handover in a terrestrial network; or

-	a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received and the next occasion to transmit a UEIRI on PUCCH for the same mode-A CSI report has not come yet after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]).