3GPP TSG-RAN WG2 Meeting #132	R2-2508667

Dallas, USA, Nov 17th-21st, 2025

Agenda item:	8.7.2

Source: 	Xiaomi

Title: 	Discussion on RLC-H02

Document for:	Discussion and Decision

# Introduction

During the previous meeting, an issue related to RX_Highest_Status was brought up and the following was agreed:

Agreements:

11.	(RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

In this contribution, we discuss further this topic.

# Discussion

## Background

The issue was discussed at RAN2#131bis (excerpt from [2])

RLC-H02 R2-2507056 Discussion on remaining issues for RLC for R19 XR Huawei, HiSilicon discussion NR_XR_Ph3-Core Proposal1: (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Specify the content of the RLC STATUS PDU triggered by the t-RxDiscard expiry in a separate condition. DISCUSSION: - Nokia thinks that this is already clear from other sections. - LGE thinks that according to current specification this case will not happen. Current spec is OK. - Fujitsu indicates that the variable is not updated when t-Rx discard expires, so there can be a problem. - Huawei and Lenovo think the situation can happen so it is better to clarify. - MTK also thinks this can happen. Discuss offline RLC-H02

[Rapp]: As mentioned, when RX_Next is larger than RX_Highest_Status, the condition of “RX_Next <= SN < RX_Highest_Status” is not satisfied. Therefore, the current specification means only ACK_SN will be included in the STATUS PDU, i.e. the current specification works well and we think no change is needed.

Discussion on RLC-H02

- LG would like to postpone it, but would like to update the RX_Next.

- Huawei think the issue should be ACKed. we could further discuss the solution. Lenovo agrees. vivo agree.

Offline agreement: - (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

## Solution

In our understanding, the condition “RX_Next <= SN < RX_Highest_Status” will always be satisfied, and when the issue occurs, the wrong SNs will be included in the STATUS REPORT. Hence, it is crucial to update RX_Highest_Status if RX_Next overtakes it.

Proposal 1: RX_Highest_Status is updated if overtaken by RX_Next at t-RxDiscard expiry

One option would be to update RX_Highest_Status to the future value of RX_Next (SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received). However, we believe it is also possible that RX_Highest_Status was more advanced than RX_Next_Discard_Trigger, and this might cause it to go backwards (see example in Annex A).

Hence, the condition should also ensure that RX_Highest_Status can only be updated forward.

We propose a TP in Annex B. There are 3 different options (in yellow). Options A1 and A2 operate before the update of RX_Next, while option B operates after the update.

Proposal 2: Agree one option from the TP in Annex

# Conclusion

In this contribution, we make the following proposals:

Proposal 1: RX_Highest_Status is updated if overtaken by RX_Next at t-RxDiscard expiry

Proposal 2: Agree one option from the TP in Annex

# References

- R2-2507056, Discussion on remaining issues for RLC for R19 XR	Huawei, HiSilicon

- R2-2507905 Discussion report on [AT131bis][501][XR] Discuss remaining RLC issues (vivo)	vivo

# Annex A – Example

This example considers:

T-Reassembly = 30ms, T-RxDiscard = 50ms

Odd SNs discarded, Even SNs received, every 10ms

(timer expiry assumed just after packet reception)

# Annex B – TP

##### 5.2.3.2.5	Actions when t-RxDiscard expires

When t-RxDiscard expires, the receiving side of an AM RLC entity shall:

-	discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any;

Option A1

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_ Next_Discard_Trigger and SN >= current RX_Highest_Status for which not all bytes have been received;

Option A2

-	if RX_Highest_Status is lower than the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_ Next_Discard_Trigger for which not all bytes have been received;

-	update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received;

-	if RX_Next_Highest > RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-RxDiscard;

-	set RX_Next_Discard_Trigger to RX_Next_Highest.

Option B

-	if RX_Highest_Status falls outside of the receiving window:

-	update RX_Highest_Status to RX_Next;