3GPP TSG RAN WG2 #132			R2-2508676

Dallas, USA, Nov. 17th – 21st, 2025

Source: 	Fainity Innovation

Title:	Views on 6G Data Framework and AI

Agenda item:	10.3.3.1

Document for:	Discussion

Introduction

Study on 6G Radio (6GR) with enumerated objectives in RP-251881 [1] was approved in RAN Plenary #108. As per the meeting content and agenda item in RAN2#131bis [2], 6G Common CP/UP includes security, AI, data framework, and energy efficiency.

The agreements for 6G common CP/UP in RAN2#131bis [2] are listed for data framework and for AI/ML.

- For data framework

- Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. - Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. - Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). - Study model transfer/delivery requirements/functions

- For AI/ML LCM

- Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality

According to the agreements, in RAN2#131bis some directions for study and deign are determined, yet other considerations or proposals are still welcomed. In this contribution, we discuss the data framework and AI/ML LCM. We will propose some aspects based on the directions which are agreed as well as the aspects with some other considerations.

Data Framework in 6GR

As discussed in RAN2#131bis, data framework at least includes data collection and data transfer. A few companies mentioned the end points of data framework, including the UE, the RAN, and the CN. Some companies also proposed to have a unified data framework for different use cases. In this section, we provide our views on these issues.

First, to enable the extensibility and flexibility of the framework, we think unified data framework for different use cases is pursued. When a new AI use case or a new use case which needs data collection and transfer, the framework should accommodate the new use cases with minimum changing efforts. That is, we only need to define the information associated with the new use cases in the signalling, instead of designing new signallings or new procedures just for the new use case.

Observation#1: A unified data framework avoids the additional complexity when a new use case is introduced.

The next issue will be how to fit different scenarios with variety of end point combinations into the unified data framework. As shown in Table 1, there are at least three combinations of end points for data collection and transfer.

Table 1. Scenarios for combinations of end points, corresponding protocols and use cases

|  | End Points | Protocols in 5G | Use cases |
| --- | --- | --- | --- |
| Scenario 1 | UE, RAN | RRC | Beam management, CSI compression, CSI prediction, RRM prediction, ISAC |
| Scenario 2 | UE, CN | NAS (e.g., LPP) | Positioning, ISAC |
| Scenario 3 | RAN, CN | GTP | AI-RAN, ISAC |

For example, AI-based beam management requires the involvement of the UE and the RAN, AI-based positioning requires the involvement of the UE and the CN, and the sensing use cases involves the RAN and the CN (and possibly the UE in 6G ISAC). In NR, different pairs of end points have their own interfaces, tunnels, and protocols, and the payload in an interface is transparent to the nodes which are not the end points. As a result, to unify the data framework, the integration of the interfaces, tunnels, and protocols between different pairs of end points is essential. The following two options can be considered:

  - Option 1: A unified protocol and tunnel is used for the data framework. This means other than the existing radio bearer, the PDU session, and the GTP tunnel, a new tunnel and protocol is designed specifically for the data framework (including collection and transfer). The new tunnel and protocol is commonly applied to the UE, RAN, and CN. Each end point can initiate the data collection and transfer to another designated end point according to the use case. The new protocol may resemble IP-like operation, indicating the destination of the data by the initiator. This can reduce the complexity of the existing tunnel and protocol design, but at the expense of introducing the complexity of a new protocol.

  - Option 2: Enable the visibility and controllability of the RAN. Since RAN is the intermediate node between the UE and the CN, the data collection and transfer can be smoothly done without congesting RAN if the RAN can control and schedule the collected data. This can avoid the introduction of a new protocol, but may impose changes on the existing protocols in order to enable the visibility and controllability.

Observation#2: A unified data framework can be achieved by a new IP-like protocol design, which has less impact on existing protocols but introduce the complexity of designing a new protocol.

Observation#3: A unified data framework can be compromised with the coordination of RAN via allowing visibility and controllability for RAN resource scheduling e.g., RAN-awareness QoS.

Proposal#1: RAN2 studies the feasibility to apply applications/service-awareness in the RAN for a unified data framework and decide whether to introduce a new protocol based on the studies.

As such, a data plane concept is proposed. In our consideration, the data plane allows a unified data collection and transfer framework by choosing one of the options above. If we consider data management as a service, data plane can be further integrated with model training and model delivery with user consent. This further enables the unification of UE-side data collection and NW-side data collection framework. With the controllability and visibility, RAN or CN can more easily collect different UE’s data and obtain a generalized model. In addition, considering an edge computing framework, the data plane can be further integrated with authenticated external model trainers (if the external server is only responsible for model training) or data servers (if the external server is responsible for data collection or processing); for example, a UE-trusted third-party external server or a UE-vendor-based external server. The external servers serve as the role similar to UE-side OTT server and can make the AI performance or LCM more robust. As different servers may train different models, the UE can select the most appropriate one for inference among the choices. Also, for monitoring aspects, the UE can have multiple applicable functions for a functionality, which somehow increases the resiliency of the functionality. The data plane can, based on the UE’s authentication and consent, control whether the collected data of a UE is exposed to the external servers.

Proposal#2: RAN2 is suggested to study a data plane and the functions of data plane includes the unification of data framework, UE’s authentication and consent management, visibility and controllability of RAN resource.

Another potential improvement is the group-based data collection, which is beneficial to reduce the signalling overhead. For model generalization, the data can be collected from different UEs, and the collected datasets are mixed to train a generalized model. The NW can broadcast or multicast the data collection configuration to all or a group of UEs, and the UEs can collect data accordingly. Similarly, the data request from the NW can be common to a group of UEs like paging, and the UEs which have available collected data then perform the report. With group-based data collection, the siganlling overhead is further reduced via the common configuration and shared request.

Observation#4: For model generalization among UEs, separate duplicate data collection configuration can lead to signalling overhead.

Proposal#3: RAN2 considers a group-based data collection framework for signalling overhead reduction for model generalization scenarios.

LCM for 6GR

In NR Release 19, AI framework was first introduced to enable the AI-awareness for both the UE and the network. LCM design, as shown in Figure 1 [3], is discussed during the meetings; however, the functional blocks of model training and model storage are considered to be implementation issue, so the discussion during the Release 19 meetings only focus on the inference, monitoring, and capability/applicability report which happens before entering LCM. Also, for the data collection procedure, during the meetings, the discussions mainly address the data collection procedure for training data collection because there was no identified issues regarding data collection for inference and monitoring. As a result, we think it is a good timing, as the start of 6GR, to further elaborate on the whole LCM functional blocks. We list some potential further enhancements, taking 5G LCM as a baseline.

- Data collection differentiation: the general data collection functional block can be further classified by characteristics according to their purpose. For example, training data features non-time-critical collection, while monitoring data and inference data features real time collection, with the difference between the existence of label (or intermediate KPI calculation). We think the data collection block should be further split into non-real-time (e.g., training data) collection and real time (e.g., monitoring and inference data) collection because the non-real-time data collection has a procedure and configuration between UE and NW different from the procedure of real time data collection. With the split, it will be helpful to clearly discuss the interaction between data collection and LCM in a comprehensive view.

- Specification of model training and storage: In Release 19 AI/ML, these two functional blocks are considered to be implementation issues and thus not discussed. However, for upcoming 6GR, considering both AI for network and network for AI, it is possible that the model training service of a UE-side model is trained by a network entity. In such case, the model training and storage functional blocks can still be implementation issues if UP solution without any RAN visibility and controllability is agreed; however, if a standardized solution can be approached, the whole LCM functional blocks can operate more smoothly because the NW can better manage the traffic flows with the awareness of AI/ML data/model transfer.

Fig. 1 LCM Framework in Rel-19 AI/ML

Proposal#4: RAN2 studies the following additional considerations for LCM in 6G: (1) differentiation of data collection types include offline/online training, and (2) model training and model storage and their functions and interaction with online training/model update in RAN.

In addition, in NR AI/ML, monitoring is configured along with inference and has its own monitoring period. To guarantee the model performance, the safest way is to periodically perform monitoring. In our view, the monitoring effort can be further reduced by an adaptive manner. That is, when the model’s performance is good enough, the monitoring is performed with a longer periodicity; when the performance is not that good but acceptable, the monitoring can be performed with a shorter periodicity; when the performance is below acceptable, decisions should be made, such as fallback or model switching. The adaptive monitoring allows the UE reduce the measurement, evaluation, and reporting efforts, and it still guarantees the model’s applicability (only unstable in case the model’s performance degrades fast within a monitoring period). As a result, we think adaptive monitoring can be considered for study.

Observation#5: Adaptive monitoring allows the UE to further reduce measurement efforts while guaranteeing the model’s performance.

Proposal#5: RAN2 studies the non-periodic monitoring mechanism, such as adaptive monitoring.

Furthermore, as proposed by a few companies in RAN2#131bis, online training is a promising solution in 6G as it allows on-device fine-tuning of the model with less calculation effort and adapts to drifting data characteristics dynamically. In our view, online training in 6G marks an important step from NR AI/ML.

Observation#6: Online training adapts the model to drifting data characteristics dynamically.

Before the design of online training, we may need to first consider whether online training can reuse the same LCM procedure or need a different LCM procedure specifically for online training. Some aspects can be considered:

- Data collection: The characteristic of data collection for online training is different from the known three types (i.e., legacy data collection for offline training, monitoring, and inference). Compared with offline training, data collection for online training requires real-time or near-real-time collection to fast fine-tune the model. Compared with inference, data collection for online training additionally requires the ground truth label for fine-tuning. The data collection for monitoring has the most similar characteristics, while it needs further study whether the frequency of monitoring is adequate for online training data collection.

- Model training (fine-tuning): In our understanding, model training includes the training (construction) of the model as well as the validation of the trained model. For offline training, the validation is done by the UE-side, so the network somehow is not involved in this functional block. As for online training, the network can be not involved, partially involved, or fully involved in the validation procedure. For no involvement case, just the same as legacy, the network can leave the validation effort to the UE. As for partial involvement case, the network can configure to the UE of a condition on validation result, and the UE determines whether validation is completed or not. As for full involvement, the network can request the UE to report the performance of the model after the validation and determine the next step on the model.

- Model storage: Another question is whether the fine-tuned model is stored in the model storage functional block or is directly used for inference. Since the online training features the real-time adaptation on model with on-device training, UE is supposed to directly use the fine-tuned model instead of transfer to the model storage (which may be a UE-side OTT server in general) and then download back to the UE. On the contrary, transfer to model storage has the benefit that the stored model is the up-to-date model, which may be a backup in case the UE needs the model again.

Proposal#6: RAN2 study the LCM functional blocks for online training with the following considerations: (1) data collection type, (2) level of NW involvement on validation, and (3) model storage for fine-tuned model.

Conclusions

In this contribution, we share our views and observations on 6G and propose:

Observation#1: A unified data framework avoids the additional complexity when a new use case is introduced.

Observation#2: A unified data framework can be achieved by a new IP-like protocol design, which has less impact on existing protocols but introduce the complexity of designing a new protocol.

Observation#3: A unified data framework can be compromised with the coordination of RAN via allowing visibility and controllability for RAN resource scheduling e.g., RAN-awareness QoS.

Observation#4: For model generalization among UEs, separate duplicate data collection configuration can lead to signalling overhead.

Observation#5: Adaptive monitoring allows the UE to further reduce measurement efforts while guaranteeing the model’s performance.

Observation#6: Online training adapts the model to drifting data characteristics dynamically.

Proposal#1: RAN2 studies the feasibility to apply applications/service-awareness in the RAN for a unified data framework and decide whether to introduce a new protocol based on the studies.

Proposal#2: RAN2 is suggested to study a data plane and the functions of data plane includes the unification of data framework, UE’s authentication and consent management, visibility and controllability of RAN resource.

Proposal#3: RAN2 considers a group-based data collection framework for signalling overhead reduction for model generalization scenarios.

Proposal#4: RAN2 studies the following additional considerations for LCM in 6G: (1) differentiation of data collection types include offline/online training, and (2) model training and model storage and their functions and interaction with online training/model update in RAN.

Proposal#5: RAN2 studies the non-periodic monitoring mechanism, such as adaptive monitoring.

Proposal#6: RAN2 study the LCM functional blocks for online training with the following considerations: (1) data collection type, (2) level of NW involvement on validation, and (3) model storage for fine-tuned model.

References

- RP- 251881, New SID: Study on 6G Radio

- RAN2#131bis Chair Notes

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface