3GPP RAN WG2 Meeting #132	R2-2508679

Dallas, USA, Nov 17th – 21st 2025

Agenda Item:	9.7.2

Source:	InterDigital

Title:	IMS voice over NB-IoT NTN

Document for:	Discussion, Decision

# Introduction

Support of IMS voice over NB-IoT NTN is required in Rel-20. The corresponding WID [1] has the following objective.

-	Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2]

-	Coordination with SA is required

-	The objective can be updated with further details based on the outcome of the above down-selection

This contribution discusses about the down-selection between the CP and the UP solution.

# Discussion

One RAN plenary document [2] shows the comparison between the CP and the UP solutions.

According to [2], the following observations were shown.

|  | CP solution | UP solution |
| --- | --- | --- |
| Spec impact | - Introduce new dedicated SRB for IMS voice - Introduce one or two dedicated SRB to distinguish the legacy NAS message; - Support UM RLC mode for SRB carrying voice packet. - QoS info from CN is needed to guarantee GBR of the dedicated SRB, priority of new SRB should be specified - Introduce NB-IoT eNB capability - indication in SI; | - Legacy DRB/QoS management for IMS voice in TN can be reused based on SA input; - Introduce NB-IoT eNB capability indication in SI; |
|  | Observation: CP solution has more RAN2 impact. |  |
| Protocol overhead | C-plane (NAS + RRC) overhead (13 bytes) is always present per packet. Protocol overhead ratio: Legacy: 82.4% Optimised: 55.6% | Protocol overhead ratio: without RoHC: 78.6% with RoHC: 36.8% |
|  | Observation: CP solution has more protocol overhead. |  |

Observation 1: CP solution has more RAN2 impact.

Observation 2: CP solution has larger protocol overhead.

In RAN2 #131-bis meeting [3], the following agreement was made and it’s captured in an LS to SA2 [4].

Agreements regarding the reply LS to SA2:

5.	We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

Observation 3: The majority of companies in RAN2 don’t see any benefit with the CP based solution over the UP one.

Proposal 1: RAN2 concludes to go for the UP based solution for the support of IMS voice over NB-IoT NTN.

# Conclusion

In this contribution the following observations and proposal are made regarding the down-selection between the CP and the UP solutions.

Observation 1: CP solution has more RAN2 impact.

Observation 2: CP solution has larger protocol overhead.

Observation 3: The majority of companies in RAN2 don’t see any benefit with the CP based solution over the UP one.

Proposal 1: RAN2 concludes to go for the UP based solution for the support of IMS voice over NB-IoT NTN.

# References

RP-252473	Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4	vivo

RP-251209	Support of voice call over GEO satellite based on NB IOT in 5G A Rel 20	vivo, CMCC, KT Corp Corp.,Verizon , Spreadtrum , UNISOC, ZTE Corporation, Sanechips

R2-2507703	Report from session on NES, NR NTN and IoT NTN	Session chair (ZTE)	Report

R2-2507784	Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC	 vivo