3GPP TSG-RAN WG2 Meeting #132	R2-2508693

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On remaining open issues for SF, Uplink Capacity enhancements and PWS for IoT-NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2-131bis meeting, several issues were discussed but not concluded. In this paper, we elaborate more and show our view on these issues.

# 2	Discussion

## 2.1	Store and Forward

Paging Skip Indication in SIB31

In RAN2-131bis RAN2 agreed to discuss further and conclude on indication on whether UE can skip the paging or not based on pending MT data when the cell operate in store and forward mode. For the relevant scenario when there is no MT data for triggering paging towards UE and if the satellite also decides that it does not have intention to trigger any system information modification via paging, the network can configure the ‘SF-Paging-off’ indication in SIB31.

Inclusion of this parameter in SIB31 allows the network to turn-off the ‘SF-Paging-off’ without further triggering paging for such changes. For example, if the network wants to trigger system information change for any parameter in SIB2/3/4 network can first disable the SF-Paging-off’ parameter in SIB31. Network can trigger the system information modification procedure including paging after the SIB31 validity timer expiry.  After completion of system information update network can enable ‘SF-Paging-off’ again in SIB31 directly without notification.

Observation 1:  Inclusion of ‘SF-Paging-off’ indication in SIB31 allows the network to configure (enable/disable) this indication without triggering paging notification.

Observation 2: With SIB31 indication, paging for other system information modification is possible after disabling the ‘SF-Paging-off’ in SIB31.

There could be delay in UE adapting to the changes in paging monitoring based on this parameter as the UE may acquire this parameter of SIB31 after validity timer expiry.  However if the network implementation can ensure that the change in this parameter and further paging transmissions are scheduled in proper timing this impact can be reduced.  If ‘SF-Paging-off’ is to be taken effect immediately at UE the network can optionally send this indication in paging message across all PO of DRX cycle.

Observation 3 :  Issues related to delay in UE realising the change in paging monitoring can be mitigated by network implementation related to change of this parameter and further scheduling related to paging.  Observation 4:  Extending the Indication in paging message when paging-skip is enabled is useful to quick update of this parameter at UE.

Based on the above observations, we propose that paging-skip indication can be introduced in SIB31 without impacting the paging reception and system information modification procedures. This parameter can be considered as assistance parameter for UE to decide on skipping the paging to minimise the specification impacts related to usage of this parameter.

Proposal 1A :  Proposal 1A :  New parameter ’SF-Mode-No-Paging’ is included in SIB31 if the network does not have stored DL data in store and forward mode operation. Usage of this parameter at UE for paging reception is left to UE implementation.

Proposal 1B :  The new parameter can also be optionally included in the Paging message for quicker update of this parameter at UE.

SF Mode Parameter for cell reselection measurement.

In RAN2-131bis , RAN2 agreed to discuss the proposals related to usage of SF mode timer value of serving cell to early start the cell reselection measurements.  The SF mode parameter is meant to indicate the UE application that the cell is operating in mode where there will not be any downlink feedback from applications. The application can determine what type of data it can transmit in this mode depending on the traffic type. This behaviour is applicable for UE capable of SF mode operation and also UE not capable of SF mode operation.

Extending this parameter to start early neighbour cell measurements is not needed as this parameter is not radio condition related. In case if there is no pending application data at UE there is no benefit for UE triggering the cell reselection earlier. Hence it should be left to UE implementation whether it should start early measurements or not to allow the UE to decide on this based on factors which is out of AS layer.

Proposal 2:  Use of the SF mode timer parameter for early triggering of cell reselection measurement is left to UE implementation.  No specification changes needed on cell reselection measurement procedure based on SF-mode timer value.

SF Mode operation and User-Plane solution

The chosen architecture option determines the solution option for user data transmission for IoT. TS 23.401 reflects this dependency via following NOTE in SF operation, Attach and TAU procedure in SF mode. From the below NOTE and the details in the Annexure, it is clear that UP solution for user plane data transfer is not supported in split MME option.  In Full EPC option data transmission via control plane EPS optimization and user plane is possible.

NOTE 3: Some deployments cannot support all system features, for example user plane establishment and user plane data transfer cannot be supported, when a satellite is operating in S&F Mode with the split MME deployment described in Annex O.

For the Full EPC option how the AS security keys are transferred across satellites and possibility of RRC Resume across satellites in SF mode was not analyzed in RAN2.  Earlier RAN2 concluded to wait for SA2 conclusion on the support for user plane solution. Now SA2 has completed the normative work and TS 23.401 does not provide clear normative procedures related to user plane data transfer and security context transfer details.  To support the user plane data transfer option along with RRC resume procedure across satellites further analysis needed along with close coordination with SA2. As the work item is already completed in RAN2, we will not have sufficient time to analyse and conclude on the RAN2 impacts. Hence, we propose that RAN2 confirm that user plane data transfer is not supported for SF mode operation in this release.

Proposal 3: RAN2 to confirm that user plane data transfer for SF mode operation is not supported in this release. Stage 2 specifications are updated to reflect these agreements. RAN2 to indicate the agreement to SA2 via LS.

## 2.2	UL capacity enhancement

In RAN2-131bis meeting, whether the “DCQR and AS RAI MAC CE” can be transmitted in CB-Msg3 for a NB-IoT UE was discussed but not yet concluded.

- CQI and AS RAI reporting in Msg3 R2-2507242 Issues on early termination of CB-Msg3-EDT without RRC message Google discussion Rel-19 IoT_NTN_Ph3-Core Observation 1 NB-IoT UEs are not allowed to transmit DCQR using “DCQR and AS RAI MAC CE” in CB-Msg3, which may hinder these UEs from informing the network of whether DL data following the UL data in CB-Msg3 is expected or not (i.e., may conflict with the RAN2#131 agreement). Proposal 1 NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB. Come back in the next meeting to decide whether NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB

The DCQR and AS RAI MAC CE was introduced in Rel-16 for both eMTC and NB-IoT. It enables the UE to report DL channel quality report (DCQR) and the Access Stratum Release Assistance Indication (AS RAI) to the eNB. Since the AS RAI may indicate that no subsequent DL and UL data transmission is expected, it helps the network to decide whether the connection can be released, thereby saving the UE’s power consumption.

36.321: 6.1.3.19 Downlink Channel Quality Report and AS RAI MAC Control Element DCQR and AS RAI MAC control element is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-2. A MAC PDU shall contain at most one DCQR and AS RAI MAC control element. <omit part> Figure 6.1.3.19-1: DCQR and AS RAI MAC control element Table 6.1.3.19-1: Values for AS RAI Codepoint/Index; Value. 00; No RAI information. 01; No subsequent DL and UL data transmission is expected. 10; A single subsequent DL transmission is expected. 11; Reserved.

For NB-IoT, according to the procedure as specified in TS36.321 5.25, the NB-IoT UE can only support a ”Regular DCQR” after receiving a DL DCQR Command MAC CE. Since the ”Regular DCQR” can only be sent in the UL grant scheduled via PDCCH for the MAC entity's C-RNTI, this implies the DCQR and AS RAI MAC CE can only be supported when the UE is in RRC Connected mode. In other words, in legacy, the DCQR and AS RAI MAC CE cannot be transmitted in Msg3.

Observation 5: According to current specification, an NB-IoT UE is not allowed to transmit the “DCQR and AS RAI MAC CE” to eNB in Msg3.

36.321: 5.25 Transmission of Downlink Channel Quality Report <omit part> If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur: - DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR"; - for BL UE or UE in enhanced coverage, transmission of DCQR in Msg3 is configured by upper layers in mpdcch-CQI-Reporting, in which case DCQR is referred below to as "Msg3 DCQR". <omit part> If "Regular DCQR" has been triggered: - if an uplink grant has been received on the PDCCH for MAC entity's C-RNTI: - instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19; - cancel the triggered "Regular DCQR". If "Msg3 DCQR" has been triggered: - if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI: - if the allocated resources can accommodate a DCQR and AS RAI MAC control element plus its subheader as a result of logical channel prioritization: - instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19; - else if the uplink grant is not for EDT: - if configured by upper layers in mpdcch-CQI-Reporting, use R and F2 fields in the MAC PDU subheader, to transmit the measurement outcome, as defined in clause 6.2.1; - cancel the triggered "Msg3 DCQR".

As discussed in RAN2-131bis meeting, the main motivation for proposing support of the “DCQR and AS RAI MAC CE” in Msg3 is to enable the UE to report the RAI to the eNB as early as possible, hence the eNB can early terminate the RRC connection when no further data transmission is expected. In our understanding, indeed AS RAI reporting is not supported in Msg3 for NB-IoT. The AS RAI Activation mechanism defined in Rel-14, controlled by the flag rai-Activation-r14, is only applicable to UEs in RRC Connected state, as this activation flag is configured by the eNB via dedicated RRC signalling instead of SIB.

However, for NB-IoT CP solution, the objective of early RRC termination can already be fulfilled through the NAS-level RAI reporting mechanism as specified in TS 23.401.

As illustrated in the call flow in Annex, in Step1, the UE may include NAS Release Assistance Information in the NAS PDU of the Msg3 RRC message RRCEarlyDataRequest, indicating whether no further Uplink or Downlink Data transmissions are expected, or only a single Downlink data transmission (e.g. Acknowledgement or response to Uplink data) subsequent to this Uplink Data transmission is expected.  Correspondingly, in Step11, if the NAS Release Assistance Information was received and indicated that no Downlink Data is expected, the MME sends S1AP UE Context Release Command to the eNB. The eNB can release the RRC connection accordingly.

Observation 6: According to current specification, for NB-IoT CP solution, NAS-level Release Assistance Information (NAS RAI) can be included in Msg3 to enable the eNB to early terminate the RRC connection when no further data transmission is expected.

Since the NAS-based RAI is only applicable for NB-IoT CP solution, for NB-IoT UP solution, we understand the both the AS RAI and NAS RAI cannot be included in Msg3.

Observation 7: According to current specification, for NB-IoT UP solution, neither AS RAI nor NAS RAI can be conveyed in Msg3.

Compared to legacy RACH procedure in NB-IoT, the CB-Msg3 procedure skips both Msg1 and Msg2. However, all the existing UP-EDT and CP-EDT RRC message and procedures are applicable to CB-Msg3-EDT.

RAN2 Agreement:

RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT

-	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

-	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

Therefore, for NB-IoT CB-Msg3 CP solution, we understand the NAS RAI based procedure can be reused as well.

Proposal 4: For NB-IoT CP solution, RAN2 to confirm the NAS-level Release Assistance Information (NAS RAI) can be included in CB-Msg3.

For NB-IoT CB-Msg3 UP solution, though both the NAS RAI and AS RAI  cannot be included in Msg3, the Rel-14 defined rai-Activation mechanism (by reporting BSR=0) as well as the Rel-16 defined DCQR and AS RAI MAC CE can be supported for PUSCH transmission when the UE enter RRC Connected state. However, if any enhancement is to be considered for RAI in Msg3, we think it should be discussed in legacy NB-IoT session instead of IoT NTN to align the NB-IoT UE behaviour in both TN and NTN. In this case, the most straightforward way is to apply the Rel-14 AS RAI reporting in Msg3 (as well as CB-Msg3).

Proposal 5: For NB-IoT UP solution, any enhancement to support AS RAI in Msg3 should be discussed in legacy NB-IoT session.

Based on above analysis, for NB-IoT NTN, we think there is no need to support the DCQR and AS RAI MAC CE in CB-Msg3.

Proposal 6: When rai-ActivationEnh is configured in SIB2-NB, the NB-IoT UE is not allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3

## 2.3	PWS

In RAN2-131bis meeting, the discussion on SI decoding accumulation was not concluded yet.

[S900] (Accumulate SI decoding across SI windows for PWS SIB) R2-2507284 RRC corrections related to [S900][S901] Samsung discussion Rel-19 IoT_NTN_Ph3-Core Late Proposal 1: Agree S900 – a scheduling configuration in SIB1 indicates whether a UE may accumulate SI decoding for PWS SIB. Agree text proposal in Annex A. Can come back in the next meeting

According to the legacy specification the NB-IoT UE is allowed, but not required to accumulate SI messages across windows:

TS36.331: Within the SI-window, the corresponding SI message can be transmitted a number of times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS.The UE acquires the detailed time/frequency domain scheduling information and other information, e.g. used transport format for the SI messages from schedulingInfoList field in SystemInformationBlockType1-NB. The UE is not required to accumulate several SI messages in parallel but may need to accumulate a SI message across multiple SI windows, depending on coverage condition.

Observation 8: The NB-IoT UE is allowed, but not required, to accumulate SI messages across SI windows.

This comes with the inherent risk of attempting to accumulate messages that do not have the same content, especially when considering that some SI message content can change without triggering the SI modification procedure. In the legacy specification, change of ETWS and CMAS information change is also exempt.

Change of system information (other than for ETWS, CMAS, EAB, UAC, and satellite assistance information parameters except for discontinuous coverage scenarios and for NB-IoT, other than for AB parameters and satellite assistance information parameters except for discontinuous coverage scenarios) only occurs at specific radio frames, i.e. the concept of a modification period is used. … E-UTRAN may not update systemInfoValueTag upon change of some system information e.g. ETWS information, CMAS information, RLOS indication (i.e., rlos-Enabled), regularly changing parameters like time information (SystemInformationBlockType8, SystemInformationBlockType16, hyperSFN-MSB in SystemInformationBlockType1-NB), EAB and AB parameters, UAC parameters, positioning system information blocks, or satellite assistance information. Similarly, E-UTRAN may not include the systemInfoModification within the Paging message upon change of some system information.

Observation 9: change of the ETWS and CMAS SI message content does not trigger the SI modification procedure.

Furthermore, the legacy specification does not prevent the UE from attempting to accumulate the PWS/CMAS SI messages across SI windows. In addition, there is no network indication to define whether the accumulation of SIBs is allowed. All the UE is required to do is to check the message Identifier and Serial Number when it has received a new SI message.

Since the legacy specification already allows SI message accumulation despite the risk of the UE attempting to accumulate messages with different content, we think it is best to follow the legacy procedure also for PWS.

Proposal 7: Allow the NB-IoT UE implementation to attempt to accumulate PWS (ETWS/CMAS) SI messages across SI windows. No specification impact is expected and no network indication regarding accumulation is needed.

In RAN2-131bis there were proposals submitted to introduce the PWS support indication in system information that to optimise the cell search for acceptable cell for UE capable of PWS reception.  The acceptable cell camping functionality is mainly introduced for emergency call support and PWS reception on these cells are additional benefits of this feature. In TN system when UE attempt for acceptable cell camping there is no differentiation of the cells based on its capability related to PWS.  Moreover, when the emergency call support is introduced in Rel-20 for acceptable cell there may be need to once again revisited the UE behaviour based on target cell capability for emergency call support.  Hence we propose that no additional indication on PWS capability is introduced in this release for acceptable cell camping.

Proposal 8:  System Information changes for indication of PWS capability of the cell is not supported in Rel-19.

# 3	Conclusion

This document has made the following observations and proposals

Store And Forward Operation

SIB Indication for paging monitoring relaxation

Observation 1:  Inclusion of ‘SF-Paging-off’ indication in SIB31 allows the network to configure (enable/disable) this indication without triggering paging notification.

Observation 2: With SIB31 indication, paging for other system information modification is possible after disabling the ‘SF-Paging-off’ in SIB31.

Observation 3 :  Issues related to delay in UE realising the change in paging monitoring can be mitigated by network implementation related to change of this parameter and further scheduling related to paging.

Observation 4:  Extending the Indication in paging message when paging-skip is enabled is useful to quick update of this parameter at UE.

Proposal 1A :  Proposal 1A :  New parameter ’SF-Mode-No-Paging’ is included in SIB31 if the network does not have stored DL data in store and forward mode operation. Usage of this parameter at UE for paging reception is left to UE implementation.

Proposal 1B :  The new parameter can also be optionally included in the Paging message for quick update of this parameter at UE.

SF Mode Parameter for cell reselection measurement.

Proposal 2:  Use of the SF mode timer parameter for early triggering of cell reselection measurement is left to UE implementation.  No specification changes needed on cell reselection measurement procedure based on SF-mode timer value.

CIoT UP Solution support for SF Mode.

Proposal 3: RAN2 to confirm that user plane data transfer for SF mode operation is not supported in this release. Stage 2 specifications are updated to reflect these agreements. RAN2 to indicate the agreement to SA2 via LS.

Uplink Capacity Improvement with CB-Msg3

Observation 5: According to current specification, an NB-IoT UE is not allowed to transmit the “DCQR and AS RAI MAC CE” to eNB in Msg3.

Observation 6: According to current specification, for NB-IoT CP solution, NAS-level Release Assistance Information (NAS RAI) can be included in Msg3 to enable the eNB to early terminate the RRC connection when no further data transmission is expected.

Observation 7: According to current specification, for NB-IoT UP solution, neither AS RAI nor NAS RAI can be conveyed in Msg3.

Proposal 4: For NB-IoT CP solution, RAN2 to confirm the NAS-level Release Assistance Information (NAS RAI) can be included in CB-Msg3.

Proposal 5: For NB-IoT UP solution, any enhancement to support AS RAI in Msg3 should be discussed in legacy NB-IoT session.

Proposal 6: When rai-ActivationEnh is configured in SIB2-NB, the NB-IoT UE is not allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3

PWS

Observation 8: The NB-IoT UE is allowed, but not required, to accumulate SI messages across SI windows.

Observation 9: change of the ETWS and CMAS SI message content does not trigger the SI modification procedure.

Proposal 7: Allow the NB-IoT UE implementation to attempt to accumulate PWS (ETWS/CMAS) SI messages across SI windows. No specification impact is expected and no network indication regarding accumulation is needed.

Proposal 8:  System Information changes for indication of PWS capability of the cell is not supported in Rel-19.

# Annex:

TS 23.401 Excerpt:

#### 5.3.4B.2 Mobile Originated Data Transport in Control Plane CIoT EPS Optimisation with P-GW connectivity Figure 5.3.4B.2-1: MO Data transport in NAS PDU 1. The UE establishes a RRC connection or sends the RRCEarlyDataRequest message as defined in TS 36.300 [5] and sends as part of it an integrity protected NAS PDU. The NAS PDU carries the EPS Bearer ID and encrypted Uplink Data. For IP PDN type PDN connections configured to support Header Compression, the UE shall apply header compression before encapsulating data into the NAS message. The UE may also indicate in a NAS Release Assistance Information in the NAS PDU whether no further Uplink or Downlink Data transmissions are expected, or only a single Downlink data transmission (e.g. Acknowledgement or response to Uplink data) subsequent to this Uplink Data transmission is expected. <omit part> 11. <omit part> If the NAS Release Assistance Information was received indicating no Downlink Data expected, it means that all application layer data exchanges have completed with the UL data transfer, then for this case, unless the MME is aware of additional pending MT traffic and unless S1-U bearers are established: - the MME sends S1AP UE Context Release Command either: - immediately after the S1AP DL NAS TRANSPORT (NAS Service Accept), in which case steps 12b and 14 are skipped, or - immediately after S1AP CONNECTION ESTABLISHMENT INDICATION, in which case steps 12a, 12b, 13, and 14 are skipped.