3GPP TSG-RAN WG2 Meeting #132				         			R2-2508700

Dallas, US, 17 – 21 November, 2025

Title: 	RRC corrections for NW-side data collection [H009/Z005]

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3

Document for:	Discussion/Decision

# Introduction

In this paper we discuss RIL issues H009 and Z005 related to NW-side data collection. They were shortly discussed during RAN2#131bis and at that time companies felt that RAN2 can wait for RAN1 reply to an LS sent in [1]. RAN1 actually did discuss this LS in their last RAN1 meeting (see [2]), but they made no conclusion. What is more, they did not even discuss the issue as raised by H009/Z005. The reason for this may be that this issue has not been directly raised in this LS and it is unclear whether RAN1 is aware this is something that needs to be discussed, while it causes issues, as described in this Tdoc.

Observation 1: Previous RAN2 LS was not clear enough on the issues raised in H009/Z005, so RAN1 may not address them properly, without further input from RAN2.

Therefore, we propose to discuss the issue in RAN2, make a decision and directly ask RAN1 to take it under consideration.

# Discussion

## [H009, Z005] Interactions with PHY for NW-side data collection

H009 and Z005 both raise the same issue, i.e. in the current procedural text in RRC, only logging actions are described while the interactions with PHY layer (responsible for providing the measurement results to be logged) are absent. The problem with the current description is that it assumes that PHY layer should be aware of when to perform the measurements based on the CSI logging configurations. This is not true for several reasons:

- Whether logging can be performed depends on the UE memory status, in case the UE’s buffer is full, the UE stops logging and it makes no sense for the PHY layer to perform measurements in this case

- Event-based logging is based on events evaluated at Layer 3 which are not visible to PHY layer.

Observation 2: PHY layer is not able to determine when to perform CSI measurements for NW-side data collection solely based on the received logging configuration, e.g. due to unknown memory status of the UE or lack of knowledge of Layer 3 events being met or not.

Observation 3: In case PHY layer would perform measurements just based on the configuration provided by RRC, some effort at PHY layer may be wasted as the RRC layer will not always log the measurements (e.g. when an event is not met or the UE’s memory is full).

At the same time, the general principle with NW-sided data collection was to make it as transparent to PHY layer as possible, and keep all the logic in RRC layer.

Observation 4: In principle, the intention was to make NW-sided data collection as transparent to PHY layer as possible, and keep all the logic in RRC layer.

Hence, to address the issue described above, we believe the simplest approach is to add in the procedural text in RRC an instruction from the UE to the lower layers about when the physical layer is expected to perform measurements which are to be logged. This can be easily done in section 5.5a.3. This section already contains the logic of when the UE is expected to log the measurements which can be reused for this purpose.

Proposal 1 [H009, Z005]: Capture in section 5.5a.3 that the UE should instruct lower layers to perform CSI measurements when the conditions for logging measurements are met (e.g. buffer is not full, events are met) and instruct to stop the measurements when they are no longer met.

H009 and Z005 proposed to solve this issue with slightly different procedure modifications. We have a slight preference for the proposal in H009, because it makes it clearer that the measurements at PHY layer are only performed at the times when logging is performed. With Z005, one may get an impression that periodicity of the measurements and logging is different which could lead to unnecessary measurements at PHY layer which we think needs to be avoided. However, we think the text from H009 can be further simplified and a reference to RAN1 specifications can also be added. The resulting TP is included in the Annex.

Since this change may have an impact on how RAN1 captures NW-side data collection in their specifications, we propose to send an LS to RAN1 informing them about our agreement and a corresponding TP so that they can align their specifications accordingly.

Proposal 2 [H009, Z005]: Send an LS to RAN1 to inform them about how RRC specifications captures RRC-PHY layer interactions for NW-side data collection and asking them to consider this in their work/specifications update.

# Conclusion

Based on the discussion in the Tdoc, we provide the following observations and proposals:

Observation 1: Previous RAN2 LS was not clear enough on the issues raised in H009/Z005, so RAN1 may not address them properly, without further input from RAN2.

Observation 2: PHY layer is not able to determine when to perform CSI measurements for NW-side data collection solely based on the received logging configuration, e.g. due to unknown memory status of the UE or lack of knowledge of Layer 3 events being met or not.

Observation 3: In case PHY layer would perform measurements just based on the configuration provided by RRC, some effort at PHY layer may be wasted as the RRC layer will not always log the measurements (e.g. when an event is not met or the UE’s memory is full).

Observation 4: In principle, the intention was to make NW-sided data collection as transparent to PHY layer as possible, and keep all the logic in RRC layer.

Proposal 1 [H009, Z005]: Capture in section 5.5a.3 that the UE should instruct lower layers to perform CSI measurements when the conditions for logging measurements are met (e.g. buffer is not full, events are met) and instruct to stop the measurements when they are no longer met.

Proposal 2 [H009, Z005]: Send an LS to RAN1 to inform them about how RRC specifications captures RRC-PHY layer interactions for NW-side data collection and asking them to consider this in their work/specifications update.

# References

[1] 	R2-2506545 LS to RAN1 and RAN3 on NW side data collection, RAN2

[2]	R1-2507998	FL summary #0 for AI/ML in beam management, Samsung (Moderator)

# Annex A – Proposed TP

5.5x.3	Measurements logging

5.5x.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_CONNECTED that has a logged measurement configuration for network-side data collection.

5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer for network-side data collection is not full:

3>	instruct lower layers to perform the measurements according to the corresponding CSI logged measurement configuration, as specified in TS 38.214 [19];

3>	perform logging of the measurement results provided by lower layers;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	instruct lower layers to perform measurements according to the corresponding CSI logged measurement configuration, as specified in TS 38.214 [19];

4>	perform logging of the measurement received from lower layers;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	instruct lower layers not to perform measurements for the corresponding CSI logged measurement configuration , as specified in TS 38.214 [19];

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.