3GPP TSG-RAN WG2 Meeting #132					           R2-2508701

Dallas, USA, 17 - 21 November, 2025

Agenda Item:		9.3.2

Source:			Huawei, HiSilicon

Title:			Discussion on RRM measurement prediction

Document for:		Discussion and Decision

# 1   Introduction

The objectives of AI mobility WID were approved in [1]. In this Tdoc, we will discuss some LCM functionality management for RRC measurement prediction with UE-sided model, including UE capability, inference configuration, applicability, inference reporting.

# 2   Discussion

## 2.1 Inference configuration

In Rel-19, following agreements related to inference parameters were made.

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) Agreements - For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping. - For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration. This list is optional.

In our thinking, RAN2 needs to discuss the relationship between inference configuration and the existing RRM measurement configuration. In general, such inference parameters can be included in the existing RRM measurement configuration, i.e. in the existing measObject and/or reportConfig IEs.

Proposal 1: RAN2 to confirm that inference parameters configuration for the RRM measurement prediction can be included in the existing RRM measurement configuration, e.g., as part of measObject and/or reportConfig.

Another thing worth discussing is how to configure the reporting of AI and non-AI RRM measurement reporting. The UE may not obtain the measurement results via prediction all the time, e.g.:

- The UE may be able to obtain predictions only for a subset of neighbouring cells, while for other cells it may need to perform and provide actual measurements

- The UE may estimate the prediction accuracy to be insufficient based on performance monitoring and may provide actual measurements instead of predicted measurements (in case RAN2 agrees for such UE behaviour)

- the UE may need to obtain both actual measurement results and predicted measurement results at same time for e.g. network sided performance monitoring.

Therefore, the key issue becomes whether the measurement ID can at the same time refer to an actual measurement configuration and predicted measurement configuration. If the NW uses different measurement IDs to associate actual measurement configuration and predicted measurement configuration, it means that additional measurement IDs will be consumed, and a larger measurement ID space may be needed. Additionally, both configurations are expected to be very similar, with the main difference being an additional configuration of inference parameters in case the predicted measurement results are to be provided by the UE. Based on this we observe and propose the following:

Observation 1: If different measurement IDs are used to associate actual measurement configuration and predicted measurement configuration, a larger measurement ID space may be needed while both configurations are expected to be very similar.

Proposal 2: It is suggested that the same measurement ID can be associated to both actual measurement configuration and predicted measurement configuration.

## 2.2 Inference reporting

In Rel-19, following agreements related to inference reporting were made.

Agreements - For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message - For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results). - For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

As we discussed in above section 2.1, we think it should be possible to use the same measurement configuration for both actual measurement configuration and predicted measurement configuration. This means that the UE needs to report both actual measurement results and predicted measurement results associated with the same measurement ID. This includes both the case where predicted and actual measurements are included in a single report message and a case where different report messages may include either predicted or actual measurements. However, the network should be at each time aware of whether the reported result is a prediction or an actual measurement, e.g. because they may have different accuracies which can impact network decisions. Hence, an indication is needed in the measurement report to indicate whether a certain result is a predicted or an actual measurement.

Observation 2: The UE may need to report both actual measurement results and predicted measurement results associated with same measurement configuration and the network should be able to distinguish predicted and actual measurements.

Proposal 3: When a measurement ID is associated with both actual measurement configuration and predicted measurement configuration at the same time, the UE needs to indicate whether the corresponding measurement result is actual or predicted.

## 2.3 UE capability

Figure 1 presents overall LCM procedure which is used for AIML for air interface developed in Rel-19.

Figure 1: LCM procedure agreed in AIML interface.

UE capability for RRM measurement prediction and measurement event prediction

Regarding UE capability for AI mobility, in our thinking, the basic issue is to define the granularity. Firstly, RAN2 should discuss whether the UE capability should be defined separately for RRM measurement prediction and measurement event prediction. The main problem is whether direct measurement event prediction is supported or not, as the capability for RMM measurement prediction can be shared for indirect measurement event prediction, since the input and output are the same for these two kinds of prediction.

However, whether direct measurement event prediction is supported depends on RAN2 and RAN4 conclusions with respect to performance monitoring feasibility.

Observation 3: Whether direct measurement event prediction can be supported is not clear for now.

Therefore, at the moment we can focus on indirect event prediction for which RAN2 can agree that the UE capability can be shared for RRM measurement prediction and indirect measurement event prediction.

Proposal 4: The UE capability can be shared for RRM measurement prediction and indirect measurement event prediction.

UE capability for different use cases

In the WID for UE-sided model, we have three use cases for RRM measurement prediction, i.e., temporal domain prediction case A and case B, and frequency domain prediction for co-located cases. It is easy to understand that the UE capability should be defined separately for temporal domain prediction and frequency domain prediction, since one prediction is dependent on the temporal domain relevance and the other one prediction is dependent on frequency domain relevance.

Proposal 5: It is suggested to define separate UE capability for temporal domain prediction and frequency domain prediction.

Furthermore, for temporal domain prediction, there are temporal domain prediction case A and temporal domain prediction case B, as shown in figure 2 to figure 4.

Figure 2: Example of intra-frequency temporal domain case A

Figure 3: Skipping pattern example 1 of intra-frequency temporal domain case B

Figure 4: Skipping pattern example 2 of intra-frequency temporal domain case B

For temporal domain prediction case A and skipping pattern example 1 of temporal domain, the input/output are the same, the difference is that temporal domain prediction case B can skip multiple measurement instances to derive the next prediction result. For skipping pattern example 2 for temporal domain prediction case B, the input within OW is discontinuous, which is different from temporal domain prediction case A and skipping pattern example 1 of temporal domain prediction case B. Also, for Case A the output will normally be a measurement for some time in future, while for Case B, the output will be usually for the current time instance. Since these two cases are quite different, it is suggested to have separate UE capabilities for temporal domain prediction case A and temporal domain prediction case B.

Observation 4: The input of skipping pattern example 2 for temporal domain prediction case B is discontinuous, which is different from temporal domain prediction case A and skipping pattern example 2 of temporal domain prediction case B.

Observation 5: The output for temporal domain Case A will usually be a predicted measurement for a time instance in future, while for temporal domain Case B the output will be usually a predicted measurement for the current time.

Proposal 6: Separate UE capabilities should be defined for temporal domain prediction case A and temporal domain prediction case B.

UE capability for different RRM sub-use cases

For L3 cell level prediction, three RRM sub-use cases were defined in Rel-19. Furthermore, RAN2 agreed that it is up to UE implementation which sub-case is used for L3 cell level prediction. Therefore, it seems there is no need to define different UE capabilities for different sub-cases, although their input/output are different.

Proposal 7: No additional UE capability is needed to distinguish different RRM sub-use cases.

# 3   Summary

In the paper, RRM measurement prediction is discussed, and the following observations and proposals are provided:

Inference configuration

Proposal 1: RAN2 to confirm that inference parameters configuration for the RRM measurement prediction can be included in the existing RRM measurement configuration, e.g., as part of measObject and/or reportConfig.

Observation 1: If different measurement IDs are used to associate actual measurement configuration and predicted measurement configuration, a larger measurement ID space may be needed while both configurations are expected to be very similar.

Proposal 2: It is suggested that the same measurement ID can be associated to both actual measurement configuration and predicted measurement configuration.

Inference reporting

Observation 2: The UE may need to report both actual measurement results and predicted measurement results associated with same measurement configuration and the network should be able to distinguish predicted and actual measurements.

Proposal 3: When a measurement ID is associated with both actual measurement configuration and predicted measurement configuration at the same time, the UE needs to indicate whether the corresponding measurement result is actual or predicted.

UE capability

Observation 3: Whether direct measurement event prediction can be supported is not clear for now.

Proposal 4: The UE capability can be shared for RRM measurement prediction and indirect measurement event prediction.

Proposal 5: It is suggested to define separate UE capability for temporal domain prediction and frequency domain prediction.

Observation 4: The input of skipping pattern example 2 for temporal domain prediction case B is discontinuous, which is different from temporal domain prediction case A and skipping pattern example 2 of temporal domain prediction case B.

Observation 5: The output for temporal domain Case A will usually be a predicted measurement for a time instance in future, while for temporal domain Case B the output will be usually a predicted measurement for the current time.

Proposal 6: Separate UE capabilities should be defined for temporal domain prediction case A and temporal domain prediction case B.

Proposal 7: No additional UE capability is needed to distinguish different RRM sub-use cases.

# 4   References

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR