3GPP TSG-RAN2#132	R2-2508721

Dallas, USA, 17th – 21st November 2025

Agenda Item:	10.4

Source:	Ericsson

Title:	Aspects on 6G mobility and measurements

Document for:	Discussion, Decision

# 1	Introduction

In this document we present our view on mobility aspects of the 6G radio interface. We analyse problems observed in previous generations and suggest high level principles for 6G to avoid the same problems again. The mobility discussion started in RAN2#131bis and the following agreements were taken:

Agreements 1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

# 2	Discussion

## 2.1	Measurements framework

### 2.1.1	Configuring the same reference signals on different layers

In the NR L1 and L3 measurement framework, it is evident that the current signalling allows the network to configure the same type of reference signals on different configuration levels in the RRC structure hierarchy. Considering that the biggest difference between L1 and L3 measurements is basically the L3 filter that the UE applies to the sample which has been measured, in 6G it would be good to make things simple and build a measurement configuration framework where reference signals to be measured by the UE are indicated by the network in a unique configuration.

Such a unified measurement configuration framework would remove redundant signalling and eliminate the current layering complexity, where the same components of the measurement framework are defined separately on different levels. Instead, in 6G, reference signals should be part of a single configuration that encapsulates the necessary parameters, while the network specifies whether the reporting should be based on raw (L1-like) or filtered (L3-like) samples. This approach would simplify both standardization and implementation, reducing ambiguities in interpretation across vendors.

5G NR RRC signalling allows the network to configure the same type of reference signals at different configuration levels in the RRC structure.

### 2.1.2	Reporting the same measurement quantity on different layers

NR defines means for sending the same or similar type of measurement reports using different uplink channels. For instance, L1-RSRP reports are used in legacy CSI measurements for RRM, in MIMO, and also in LTM. Nevertheless, 5G UEs need to send this measurement quantity as L1 UCI on PUCCH for LTM and MIMO, and in MAC CEs (L2) for LTM. Furthermore, beam level measurements can also be reported via L3 on the RRC layer.

This unnecessary signalling complexity occurred partially because different features were standardized in different releases of NR. However, also the fact that in 5G NR the L1 measurements framework and L3 measurements framework have distinct measurement configurations and reporting configurations contributed to the diverse reporting schemes. Considering the fact that the samples which the UE measures are actually the same for both L1 and L3 measurements, we believe that in 6G we can do better and try to unify the way how the UE reports measurements to the network.

NR defines means for sending the same or similar type of measurement reports using different uplink channels which causes unnecessary signalling complexity.

Given this, it is of utmost importance to learn from the mistake made in 5G and make sure that the measurement framework in 6G is lean and less complex. Therefore, there are three main aspects to consider about the mobility framework in 6G:

- how the measurements are configured,

- how a measurement report is triggered and what the UE shall include inside it, and

- how the measurement report is transmitted to the network.

Regarding how the measurement report is transmitted to the network, we have addressed this issue already in one or our contribution submitted in RAN2#131-bis meeting in [1], where the concept was basically that in defining uplink and downlink control messages for 6G, RAN working groups should carefully analyse the reports’ properties and requirements and choose an appropriate control channel taking the entire protocol stack and the impact on e2e protocols into account. This indeed applies also to the reporting of measurements, where UE may need to select the appropriate channel on where to report those based on the requirements (e.g., reliability, latency, or error cases).

About the how the measurements are configured and how a measurement report is triggered and what the UE shall include inside it, the common principle which we want to highlight is that in 6G there should not be any difference for something measured and report for what today we call “L1 measurements” or “L3 measurements”. The approach, indeed, would be that both the reference signals, how the UE measures them, and how a measurement report is triggered and what the UE includes in it would be done according to one common unified configuration from the signalling (RRC ASN.1) perspective. According to this new paradigm, whether measurements are “L1” or “L3” would be just an attribute of such common unified configuration or even according to different set of UE requirements which would be defined by RAN4. This new unified measurement framework will avoid dealing with redundancy, increased signaling complexity, and vendor implementation overhead (which is what has happened in 5G). For these reasons, we propose:

Measurements in 6GR shall be configured and reported according to a unified measurement framework which has the following characteristics:

  - A measurement configuration which defines reference signals to be measured by the UE.

  - A measurement reporting configuration which defines when a UE shall transmit a measurement report to the network and what information to include in it.

  - Characteristics related to current L1 and L3 measurements (e.g., L3 filter) are defined as attributes (of the measurement configuration and measurement reporting configuration).

  - UE should use appropriate channels on where to report measurement reports based on requirements (e.g., reliability, latency, or error cases).

## 2.2	Connected mode mobility

### 2.2.1 Processing delay requirements of RRC messages

The number of parameters that are updated at mobility varies depending on the type of reconfiguration. In NR, delta signalling is rather complex because of the current hierarchical RRC structure, and the network (target cell) often configures a full configuration at handover even if not many of the configured values would need to be changed or updated. Another issue is related to the fact that the RRCReconfiguration message is widely used for many RRC procedures, including mobility, and its size is considerably large. Also, because of the hierarchical RRC structure, even updating a few parameters to indicate to the UE to perform a mobility procedure requires the network to include a lot more of parent IEs and parameters which may not be required for the specific case. The outcome of this is that it is not possible in 5G to achieve a large benefit by sending a (relatively) small RRCReconfiguration message instead of a full RRCReconfiguration message. This is also reflected in section 12 of TS 38.331 where the UE processing delay for the RRCReconfiguration is defined in the order or 10-16 ms (without considering segmentation, which would make the processing delay even longer).

Considering that in NR the processing delay requirement to perform an RRCReconfiguration is typically 10 ms and the processing requirements to perform a beam switch are in the order of 3 ms (since no RRC parameters are reconfigured), in 6G we should rather define small RRC messages for specific procedures, such as mobility, which the UE could process faster.

Especially in intra-xNB mobility scenarios, where the UE configuration typically may not change much, the use of small RRC messages may be helpful for the network to keep most of the UE configuration unchanged and include only new fields or parameters which the UE should apply during the mobility procedure.

If this type of small RRC messages are defined for 6G, the RRC processing delay requirements could be tailored to the specific changes done by such message. For example, a small RRC message which only updates a QCL source at the UE may have RRC processing delay requirements similar to the processing requirements for a beam switch (e.g. 3 ms). Therefore, a good solution would be to define specific RRC message(s) for mobility which, depending on the type of mobility or what the network what to reconfigure at the UE, may have a relatively small size. According to this analysis, we propose:

Define specific RRC message(s) for mobility (which may have a rather small size if needed) and study how the UE may apply different RRC processing delay requirements depending on the content of such RRC message.

### 2.2.2 Early synchronization

Early synchronization mechanisms (for both UL and DL) were introduced in NR Rel-18 within the context of LTM with the main aim to reduce the synchronization delay experienced by UEs prior to the execution of a mobility procedure (which in LTM is called “cell switch”).

Early uplink synchronization enables the UE to achieve uplink time alignment with the serving cell before it needs to send data or control information. This is fundamentally different from conventional NR operations during handover procedures, where a UE becomes uplink synchronized through the random access procedure or via timing advance adjustments after receiving an explicit trigger from the network, thus adding additional delay. With early UL synchronization, this delay can be avoided by allowing the UE to perform a proactive uplink synchronization procedure ahead of time. The UE, or in the case of a network-controlled approach, the gNB, may anticipate the need for uplink transmission and trigger an early synchronization procedure at the UE to acquire a valid timing advance (TA) before the mobility execution procedure takes place.

Similarly, early downlink synchronization aims at ensuring that the UE maintains or re-establishes downlink synchronization with one or more beams or transmission/reception points (TRPs) of a candidate cells before the mobility procedure is performed. Under normal circumstances, a UE achieves downlink synchronization by detecting and decoding synchronization signals when the UE is entering a new cell or when a secondary cell is being activated and this process can take a measurable amount of time, particularly if multiple cells or beams must be scanned or if the UE is operating in a low-power state. Therefore, when network indicates to the UE to initiate the early DL synchronization procedure, the UE can proactively align its receiver timing and frequency with the downlink signals of the potential candidate cells (or secondary cells to be activated) thus significantly reducing the downlink access delay, enabling faster mobility events and quicker secondary cell activations.

To meet the expected 6G latency and reliability requirements, it becomes clear that exploring network-controlled early synchronization is a necessity, since they allow the network to proactively coordinate synchronization actions at the UE in anticipation of mobility or multi-cell operations. Therefore, it is proposed that in 6GR network controlled early synchronization procedures (for both UL and DL) are a fundamental component for mobility in 6G, allowing the network to initiate, configure, and manage synchronization proactively at the UE. Further to this, the network controlled early synchronization procedures should support scenarios like handover and multi-cell operations (like secondary cell activation procedure).

Early synchronization procedures (both for UL and DL) shall be one of the main components of mobility and spectrum aggregation in 6GR, where:

  - The early synchronization procedures apply to all types of mobility procedures (e.g., L3 handover, LTM, CLTM in 5G)

  - Both the early UL and DL synchronization procedure are network controlled as baseline

  - The early UL synchronization procedure shall aim for the UE to establish and maintain in advance uplink timing alignment thus avoiding the need for the random access procedure.

  - The early DL synchronization procedure shall aim for the UE to acquire and store downlink synchronization information for candidate cells prior to handover or spectrum aggregation operation (e.g., activation of a "cell").

### 2.2.3 Robustness

It was agreed in RAN2#131bis that the 6G mobility design will consider robustness of the mobility procedures. It is a known problem that the radio conditions can deteriorate quickly leading to that the Handover Command may not reach the UE or the Measurement Report triggering the Handover Command may not reach the network if the radio conditions are too poor. For those reasons, conditional handover was specified for NR in Rel-16.

There are also other problems that need to be taken into account when improving the robustness. There can be sudden interruptions in the communication between the UE and the network, a base station may experience failure or the interfaces between base stations may experience failure. These are rare events, but for UEs that require highly reliable communication, these kinds of problems need to be taken into account. Also, in those cases, conditional reconfigurations can be used as a tool to improve the robustness and mobility can be used as a tool to avoid RLF.

So, for 6G conditional reconfigurations should be studied as a tool to improve the robustness at mobility.

Study conditional mobility as a solution to improve the robustness.

## 2.3	Idle/Inactive mode mobility

When designing the framework for cell reselection in 6GR we should look at possible improvements compared to the one in NR. There are however some basic principles from the NR cell reselection that we think can be reused:

- Cell reselection is based on cell defining sync signals

- Intra-frequency reselection is based on ranking of cells

- Inter-frequency/inter-RAT reselection is based on absolute priorities, through broadcasted or dedicated signalling.

In NR there is also support for UE (feature) specific cell reselection procedures, such as speed dependent scaling and slicing specific cell reselection priorities.

The speed dependent scaling is based on mobility states, which are determined by the number of cell reselections that the UE has performed during a time period. To have cell reselection procedures that are dependent on the speed/mobility of the UE can be useful, but we think that the mechanism to determine mobility states in NR is imprecise and thereby not so useful as the current procedure does not differentiate between intra-frequency reselections and inter-frequency reselections. This e.g. leads to two co-located UEs being classified to belong to different speed classes if one of the UE performs inter-frequency reselections and the other does not. We therefore think that the NR speed dependent cell reselection procedures should not simply be reused in 6GR, but that it can be studied further.

Differentiation based on UE properties, such as for different slices or PLMNs, can also be studied further in 6GR. With the slicing based cell reselection in NR, the UE camps on the highest priority frequency of the highest priority slice. However the UE might then still need to be handed over to a different frequency if the current Connected mode session is associated to a slice that is different than the highest priority slice as assigned by the core network node. We should then not reuse the slicing based cell reselection in NR, which is rather complex and with unclear usefulness.

6G cell reselection framework should reuse the following aspects from NR cell reselection:

  - Cell reselection is based on cell defining sync signals

  - Intra-frequency reselection is based on ranking of cells

  - Inter-frequency/inter-RAT reselection is based on absolute priorities, through broadcasted or dedicated signalling

When a UE enters an energy saving state (RRC_IDLE or RRC_INACTIVE) in NR, it finds a suitable cell to camp on through the cell selection procedure. The frequency selected at the cell selection procedure is up to UE implementation. This is then followed by a cell reselection procedure where the UE updates the cell, frequency and/or RAT to camp on. The cell reselection procedure is controlled by the network through cell reselection priorities, which influence which frequency and/or RAT that the UE shall prioritize to camp on. The cell reselection priorities should enable the network to steer where UEs camp and then perform access based on e.g. load situations in the network and expected service requirements for the UE. This possibility for the network to control where UEs camp in the network should be supported also in 6G.

For the inter-frequency reselection procedure, the UE is required to perform inter-frequency measurements. However, the time requirements for inter-frequency measurements are scaled by the number of frequencies configured as part of SIB4. This in turn leads to a situation where the UE often transitions to connected state without completing these inter-frequency measurements and the corresponding reselection procedure to a prioritized frequency.

For example, in NR with a configuration of 3 carriers on FR1 (as in Figure 1) a UE shall perform inter-frequency reselection to a previously undetected cell on a higher priority frequency within an allowed maximum time of ~120 seconds (when a suitable cell exists on the higher priority frequency), according to TS 38.133. Even for the case of a known cell, the time it takes a UE to complete cell reselection measurements (according to NR requirements) is significantly longer than the time that a UE typically spends in an energy saving state (RRC_IDLE or RRC_INACTIVE) before transitioning to connected state.

UE time spent in energy saving state is often short compared to the requirements for inter-frequency measurements.

Figure 1: An example with cell reselection priorities configuration with 3 frequencies.

A consequence of time requirements that scale with the number of frequencies is that the provided cell reselection priorities do often not impact where the UEs actually camp and perform their access at the end, even if there are suitable cells for the UE on the higher priority frequencies. As explained above, this can be due to that the UE returns to connected state before completing the cell reselection procedure.

Field observations using measurements performed after entering RRC_CONNECTED (as seen in the Annex) show that most of the UEs accessing a lower priority frequency were actually in good enough coverage of a higher priority frequency. According to the cell reselection priorities those UEs should thus have been camping and performing access on that frequency.

Instead of relying on inter-frequency cell reselection priorities in RRC_IDLE/RRC_INACTIVE the network could use inter-frequency measurements and trigger inter-frequency handovers for the UE when it is in RRC_CONNECTED. However, such measurements and handovers would impact the performance of the ongoing data transfer and create additional control signalling.

UEs often do not reselect to higher priority frequencies even in the cases where coverage is present, as observed by the connected mode measurements.

6G should aim to find solutions for Idle/Inactive mobility that benefits both the UE and network in terms of performance and energy efficiency. The goal is that the UE camps on a frequency intended by the network so that the network can best serve the UE while avoiding unnecessary handovers in RRC_CONNECTED.

The prioritization of frequency/RAT to camp on while in idle/inactive should be under network control. RAN2 should, together with RAN4, study how to ensure that UEs camp on a frequency where the network wants to serve the UE in connected state.

## 2.4	Inter-RAT

According to the 6G study WID [RP-252912], migration from 5G to 6G will be based on 5G/6G MRSS and inter-RAT handover. Since the 6G coverage will be limited in the beginning there is a need for an efficient 5G/6G inter-RAT handover procedure to ensure service continuity.

For RRC CONNECTED UEs, an efficient 5G/6G inter-RAT handover procedure is required to ensure service continuity when moving between 5G and 6G coverage.

In NRFor UEs in RRC_INACTIVE, the network keeps a UE Inactive context for UEs in RRC_INACTIVE, including security context and established bearers to support a fast resume procedure at transition to RRC_CONNECTED when there is data to transmit. Maintaining this UE context also for the case of an inter-RAT re-selection would allow UEs to freely roam between 5G and 6G coverage, with a possibility for a fast resume procedure in any of the RATs. However, itRRC_INACTIVE but would create a dependency between 5G and 6G specifications and severely limit the possibilities for 6G. Another issue is reachability, i.e. paging is performed in the UE serving system where the UE is registered. Assuming that the UE triggers registration when moving to another RAT, a new UE context is created in the new RAT and the UE context in the old RAT would be removed. This was discussed also during early 5G standardization but was dismissed for these reasons. We think the same principle should apply also for 6G, i.e. inter-RAT reselection between 5G and 6G should go via RRC_IDLE.

For UEs in inactive mode, keeping the inactive context when moving between 5G and 6G will introduce dependency between 5G and 6G specs, and additional complexity.

Inter-RAT mobility between 5G and 6G is supported in RRC_CONNECTED. Inter-RAT reselection between 5G and 6G in RRC_INACTIVE should go via RRC_IDLE.

# 3	Conclusion

In this paper we have reached these observations:

Observation 1	5G NR RRC signalling allows the network to configure the same type of reference signals at different configuration levels in the RRC structure.

Observation 2	NR defines means for sending the same or similar type of measurement reports using different uplink channels which causes unnecessary signalling complexity.

Observation 3	UE time spent in energy saving state is often short compared to the requirements for inter-frequency measurements.

Observation 4	UEs often do not reselect to higher priority frequencies even in the cases where coverage is present, as observed by the connected mode measurements.

Observation 5	For RRC CONNECTED UEs, an efficient 5G/6G inter-RAT handover procedure is required to ensure service continuity when moving between 5G and 6G coverage.

Observation 6	For UEs in inactive mode, keeping the inactive context when moving between 5G and 6G will introduce dependency between 5G and 6G specs, and additional complexity.

We propose:

Proposal 1	Measurements in 6GR shall be configured and reported according to a unified measurement framework which has the following characteristics:

a.	A measurement configuration which defines reference signals to be measured by the UE.

b.	A measurement reporting configuration which defines when a UE shall transmit a measurement report to the network and what information to include in it.

c.	Characteristics related to current L1 and L3 measurements (e.g., L3 filter) are defined as attributes (of the measurement configuration and measurement reporting configuration).

d.	UE should use appropriate channels on where to report measurement reports based on requirements (e.g., reliability, latency, or error cases).

Proposal 2	Define specific RRC message(s) for mobility (which may have a rather small size if needed) and study how the UE may apply different RRC processing delay requirements depending on the content of such RRC message.

Proposal 3	Early synchronization procedures (both for UL and DL) shall be one of the main components of mobility and spectrum aggregation in 6GR, where:

a.	The early synchronization procedures apply to all types of mobility procedures (e.g., L3 handover, LTM, CLTM in 5G)

b.	Both the early UL and DL synchronization procedure are network controlled as baseline

c.	The early UL synchronization procedure shall aim for the UE to establish and maintain in advance uplink timing alignment thus avoiding the need for the random access procedure.

d.	The early DL synchronization procedure shall aim for the UE to acquire and store downlink synchronization information for candidate cells prior to handover or spectrum aggregation operation (e.g., activation of a "cell").

Proposal 4	Study conditional mobility as a solution to improve the robustness.

Proposal 5	6G cell reselection framework should reuse the following aspects from NR cell reselection:

a.	Cell reselection is based on cell defining sync signals

b.	Intra-frequency reselection is based on ranking of cells

c.	Inter-frequency/inter-RAT reselection is based on absolute priorities, through broadcasted or dedicated signalling

Proposal 6	The prioritization of frequency/RAT to camp on while in idle/inactive should be under network control. RAN2 should, together with RAN4, study how to ensure that UEs camp on a frequency where the network wants to serve the UE in connected state.

Proposal 7	Inter-RAT mobility between 5G and 6G is supported in RRC_CONNECTED. Inter-RAT reselection between 5G and 6G in RRC_INACTIVE should go via RRC_IDLE.

# 4	References

R2-2507074, Common user and control plane aspects for 6G, Ericsson, RAN2#131-bis, Prague, October 2025.

# 5	Annex

Observations from the field show that UEs often perform access to a cell on a lower priority frequency even though they are in coverage of a cell on a higher priority frequency. An example can be seen in Figure 2 with measurement results from UEs directly after entering RRC_CONNECTED. The UEs have a PCell on a frequency with a lower cell reselection priority and are configured with an SCell on a frequency with higher cell reselection priority and perform measurements on that. The measurement results for the SCell show that very many of those UEs are in good coverage of that cell, with a higher RSRP values than the configured threshold for reselecting to it. Based on the coverage, these UEs should thus have been camping and performing access on that frequency.

Figure 2 Field data with measurement results collected from multiple UEs in a network. The two graphs show the number of received measurement results for different RSRP values, the lower graph for a cell (SCell) on a higher priority frequency and the upper graph for a cell (PCell) on a lower priority frequency. The indicated threshold for the SCell is the threshold for reselection.