# 3GPP TSG-RAN2 Meeting #132	R2-2508759

# Dallas, USA, Nov 17-21, 2025

Agenda Item:	9.2.2

Work Item:	Ambient_IoT_Solutions_Ph2

Source: 	Qualcomm Incorporated

Title: 	Topology 2 aspects for DO-DTT and DT

Document for:	Discussion/Decision

# Introduction

Revised WID for Ambient IoT Ph2 is in [1]. One of the objectives for RAN2 is listed as follows:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: <<skip RAN1 parts>> RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

RAN2#131bis discussed some aspects of Rel-20 A-IoT and made the following agreements:

Agreements

- 1.  	gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

Additionally, after some discussion on Resource Validity, following was captured:

Continue discussion next meeting on how resource allocation looks like.

This document provides our views about RAN2 work needed for Rel-20 A-IoT, taking into account the above agreements. According to the guidelines by the chair, we limit it only to traffic types DO-DTT and DT (i.e. exclude aspects on DO-A, active devices etc.).

# Discussion

## Resource configuration for A-IoT Reader

RAN2#131bis agreed gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.

Now, there are various ways already possible in 5G to configure the resources using dedicated Uu RRC signaling. We think as a baseline, periodic configured resources are needed for A-IoT operation. In addition, one-time resources are also useful.

For A-IoT resource allocation to the UE Reader, both periodic (e.g. RRC-based CG-like resources) and one-time resources configured by dedicated RRC are supported.

  - Uu and A-IoT coexistence and resource utilization

The WID states “Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band” is not supported. Considering that the gNB is in full control of the resource allocation for the UE readers within its coverage, at the first glance, it is natural expectation that the gNB should ensure time overlapping resources are not configured for a UE operating as a Reader for both Uu and A-IoT interfaces.

However, let’s consider the following scenario. Suppose the gNB has configured certain PRBs for certain number of slots for A-IoT operation. There is no guarantee that there will not be any UL or DL traffic for that UE during this time. If such traffic is delay tolerant, that could be less of an issue, but what if such traffic is critical, e.g. related to emergency call, other control information (e.g. periodic CSI, PHR reports). Making sure that allocated resources for Uu and A-IoT never overlap in time (in the same band) is a huge restriction for NW.

Imagine another scenario where the UE reader is configured with a block of resources for A-IoT operation. However, when the UE reader performed the A-IoT operation, all the A-IoT operations are completed well beforehand. In that case, the rest of the resources that are configured would be wasted. It is worthwhile to discuss how to minimize such resource wastage.

Some possible ways to deal with this situation are:

- Option 1: Enable UE to send A-IoT resource release request message. This would need additional specification work to define Release Request message and corresponding procedures. In addition, UL grants on Uu link would be needed to send this new RRC message.

- Option 2: Allow the UE to be configured with the same resources for both A-IoT and Uu operations and allow the UE to switch to Uu operation once A-IoT operations are all completed. Since the A-IoT operations also use FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum (according to WID), this should be possible. This would need specifications to clarify that while A-IoT operations are ongoing the UE would skip the UL on Uu.

It is worth reclarifying that we are not arguing to change the WI objective or to allow time overlapping operation of Uu and A-IoT (in the same band). What we are saying, instead, is there may be situations where the gNB benefits by configuring overlapping resources, but that does not necessarily mean there is time overlapping operations on both A-IoT and Uu link in the same band.

- Simultaneous (time overlapping) operation of Uu and A-IoT at the UE reader in the same band is not supported. However, that does not necessarily mean time overlapping resource allocation for Uu and A-IoT is not allowed.

RAN2 to discuss how to avoid resource wastage if the allocated A-IoT resources are too much (i.e. the UE reader finishes all A-IoT operations earlier than expected), e.g. (option 1) introduce Resource Release Request message to be sent by the UE reader, (option 2) Allow the UE reader to use remaining resources for Uu operation after the A-IoT operations are complete.

Another aspect to be considered for the coexistence of Uu and A-IoT operation at the UE reader is the following. Consider a scenario where the Reader finished certain steps of an A-IoT session, but not all the steps. This could happen for various reasons such as the allocated resources were not enough to finish all the steps, there was higher priority interruption due to e.g. emergency call on the Uu side. Now, should the remaining steps of ongoing A-IoT session be allowed to continue in a future A-IoT resource allocation? It seems reasonable to allow this as long as the future resources is within a reasonable time frame. This is illustrated in the figure below. We would encourage RAN2 to discuss this scenario.

f

t

A-IoT Resource Allocation X

Uu Resource Allocation

A-IoT Resource Allocation X+1

A-IoT Paging with Transaction ID1,

msg1, msg2… up to step M

A-IoT (same session or TX ID),

step M+1, step M+2 … last step.

Uu communication

Figure 1 Uu and A-IoT in the same band for a UE Reader

One A-IoT session (e.g. inventory + command) can span across multiple A-IoT resource allocations (e.g. some steps of A-IoT operation completed in resource X followed by remaining steps in resource X+1).

  - UE reader temporary out of connection

Note that UE reader’s temporary out of connection scenario was heavily discussed during the study item phase as well as in the RAN plenaries during WI scoping.

The following was captured for resource allocation in TR 38.769 section 6.3.6 regarding different RRC states and scenarios (highlights added):

For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase): - Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: - Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or - Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration. - Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations. NOTE 1: The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.) - Scenario 3: In RRC_ INACTIVE, <<skip>> - Scenario 4: In RRC_ IDLE, <<skip>> NOTE 2: The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios) <<skip>>

And following is captured in the WID

- RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios.

In other words, there are valid scenarios where the UE Reader is temporarily out of connection due to HO or RLF.

It is also understood that the WID captures the following “A-IoT radio resource of the UE reader is only valid within the serving cell.” Putting the two together, the intent should be that the A-IoT radio resources of the UE reader is not considered immediately invalid while HO or RLF recovery is ongoing.

We illustrate this with an example as shown in figure below. The command from the AIOTF arrive to the UE Reader at time t1 while it is in coverage; the UE reader starts to perform the actual inventory or command with the A-IoT devices (tags) at t2, but during this time before the A-IoT operation is complete, the UE Reader suffers RLF. In such case, the UE reader should not be required to prematurely stop all the ongoing A-IoT operations as long as the previously provided resource allocation is still valid. In other words, merely going through temporary interruption in Uu should not require the Reader to abandon A-IoT resources already provided to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.). Then, the Reader may come back to the coverage at t3 in the same cell. If the A-IoT operations are completed, the reader can successfully transmit the collected A-IoT data to the AIOTF.

Similar scenario can be considered for HO case: the reader may start a HO procedure but may end up not completing the HO and come back to the same cell. In such case, it is unwise to have to stop all AIOT procedures while the UE is ongoing the HO procedure (until the HO is actually completed and UE has moved to target cell).

Figure 2 UE Reader temporary out of connection

- Temporary interruption in Uu such as HO or RLF are valid scenarios and are explicitly captured as being within the scope of the WI.

To fulfil the above principle (that the temporary out of connection should not necessarily lead to dropping of all ongoing A-IoT operations), RAN2 should design solution(s) to enable uninterrupted A-IoT operation during HO or RLF of the UE reader.

RAN2 aims to enable uninterrupted A-IOT operation during HO or RLF of the UE reader.

Regarding how to avoid the UE reader from wonder around or staying in such temporary state for far too long, we prefer option 2 from the TR, i.e. timer-based resource validity should be introduced.

- Timer based resource configuration validity is simple and fully under NW control.

- UE reader going through a HO or RLF event shall not be the sole reason to consider the A-IoT resources as immediately invalid unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

Therefore, we propose:

The A-IoT resources configured by the serving cell are considered as being valid while the UE is undergoing a HO or RLF event (i.e. until HO complete or end of RLF recovery procedure or until a network-configured validity timer expires).

# Summary

Based on the discussion above, we have following observations and proposals:

Resource configuration for A-IoT reader

Proposal 1:	For A-IoT resource allocation to the UE Reader, both periodic (e.g. RRC-based CG-like resources) and one-time resources configured by dedicated RRC are supported.

Uu and A-IoT coexistence & resource utilization

Observation 1.	Simultaneous (time overlapping) operation of Uu and A-IoT at the UE reader in the same band is not supported. However, that does not necessarily mean time overlapping resource allocation for Uu and A-IoT is not allowed.

Proposal 2:	RAN2 to discuss how to avoid resource wastage if the allocated A-IoT resources are too much (i.e. the UE reader finishes all A-IoT operations earlier than expected), e.g. (option 1) introduce Resource Release Request message to be sent by the UE reader, (option 2) Allow the UE reader to use remaining resources for Uu operation after the A-IoT operations are complete.

Proposal 3:	One A-IoT session (e.g. inventory + command) can span across multiple A-IoT resource allocations (e.g. some steps of A-IoT operation completed in resource X followed by remaining steps in resource X+1).

UE Reader temporary out of connection

Observation 2.	Temporary interruption in Uu such as HO or RLF are valid scenarios and are explicitly captured as being within the scope of the WI.

Proposal 4:	RAN2 aims to enable uninterrupted A-IOT operation during HO or RLF of the UE reader.

Observation 3.	Timer based resource configuration validity is simple and fully under NW control.

Observation 4.	UE reader going through a HO or RLF event shall not be the sole reason to consider the A-IoT resources as immediately invalid unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

Proposal 5:	The A-IoT resources configured by the serving cell are considered as being valid while the UE is undergoing a HO or RLF event (i.e. until HO complete or end of RLF recovery procedure or until a network-configured validity timer expires).

# References

[1] RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2, RAN#109, Sep 2025

[2] TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, Release 19

[3] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18