3GPP TSG-RAN WG2 Meeting #132	R2-2508762

Dallas, USA, Nov 17th- 21st , 2025

Agenda item:		10.3.2.1

Source:	Rakuten

Title:	Discussion on RRC State modelling in 6GR

Document for:		Discussion and Decision

# Introduction

The SID of Study on 6G Radio in RAN #108 includes the following objectives [1]

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we present our analysis and views regarding the Control plane architecture and protocol design for 6GR.

# Discussion

In RAN2#131bis, the below agreement was made.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this contribution, we analyse some of the aspects that were discussed in the RAN2#131bis meeting.

## RRC states

5G specifies three RRC states: RRC_IDLE, RRC_CONNECTED, and RRC_INACTIVE.
The first two were carried over from LTE, while RRC_INACTIVE is a new state introduced in NR. It offers power consumption comparable to RRC_IDLE but allows faster transitions to RRC_CONNECTED with less signalling overhead. Some important features of these states are

- RRC_IDLE:

  - UE is not connected to the gNB but can receive system information and perform cell (re)selection.

  - The UE is known to the network at the level of a Tracking Area (TA).

  - Mobility is UE controlled where the UE transmits a Tracking Area Update message periodically or whenever it changes it TA.

- RRC_INACTIVE:

  - RRC connection and bearers are released but gNB retains UE context at the last serving gNB.

  - UE context at the gNB enables fast resume to RRC CONNECTED.

  - UE is known at the level of a RAN Notification Area (RNA), which is a group of cells or gNBs.

  - Mobility is UE controlled where the UE performs RNA Update periodically or whenever it detects a change of RNA.

  - Though the UE’s bearers are released (between UE  RAN), the PDU sessions still exist i.e RRC Inactive state is not visible to the Core network.

  - UE can perform fast Small Data Transmission (SDT) from RRC Inactive.

- RRC_CONNECTED:

  - UE has an active connection with the gNB; used for UL/DL data transfer, mobility, and control signalling.

  - UE is known to the gNB at the cell level.

  - Mobility is network controlled (eg: L3 HO, LTM) or network-controlled, UE-triggered (eg: CHO, CLTM).

  - PDU sessions and bearers are active all the time.

The objective of the 3 states is different and hence they have different characteristics. RRC Connected is for UL/DL data transfer, RRC_Idle is to allow the UE to remain reachable by the network while minimizing power consumption and radio signalling load and RRC Inactive is a lightweight connected state that keeps the UE reachable but minimizes signalling and power use while allowing the UE to perform SDT in UL.

The 3 RRC States of NR have different objectives and characteristics. Therefore, there is very little commonality between them, especially RRC Idle and RRC Inactive.

There were two proposals submitted by companies to RAN2#131bis.

- Merge RRC_Inactive with RRC_Idle

The main objective behind this is to have the same location tracking in both states and ensure paging load is managed well. However, this is replete with disadvantages. 
-	A larger RNA (as big as TA) implies less signalling and more power savings, but a larger paging load. 
-	A smaller RNA implies effective paging, but slightly more signalling (RNAU) and lesser UE power savings.
- 	A reasonably large TA (as in 5G) is effective for IDLE mode.
-	A smaller TA (as big as an RNA) implies excess signalling and higher power consumption for IDLE UEs also.

The ideal TA size is the smallest area that keeps the average UE registration update rate within acceptable limits,
without overloading paging resources.

The RNA is like a “mini-tracking area” managed by the RAN, not the core. Therefore, the ideal RNA size is the smallest configuration that minimizes RNA Update frequency while keeping paging load manageable in the RAN.

The UE monitors paging differently in Idle and Inactive states, using distinct paging parameters. Specifically, in the IDLE state, the UE monitors core network (CN) paging using DRX parameters negotiated between the UE and the network (CN/RAN) along with the S-TMSI. In the Inactive state, the UE monitors RAN paging using DRX parameters configured by the RAN and the I-RNTI.

Since, the RRC Inactive and RRC Idle have different objectives, it is best that UEs in those states are location tracked at different levels i.e RNA and TA respectively.

RRC Inactive and RRC Idle are not merged and location tracking of UEs in their respective states are at RNA and TA levels respectively.

- Integrate RRC Inactive as a sub-state of RRC Connected

The main objective of this proposal was to reduce paging load, improve energy efficiency and improve state transition latency. The solution proposed included the below aspects
-	UE is location tracked within a very small area, eg: 3 cells.
-	UE is in a long DRX and performs silent mobility during DRX-off period.
-	LP-WUS is used instead of Paging to wake up UEs when there is DL data.


It is true that the state transition time is avoided and Paging load is reduced considerably, but we have to pay for this in other terms elsewhere.
-	The load in RRC Connected will be super-high and it may not be sustainable to retain all UEs in RRC Connected. The number of UEs in RRC Connected is always limited in a gNB.
- 	One of the main benefits of RRC Inactive is that the UE bearers are released and UE context at the gNB-DU is released. This benefit is lost. 
-	A mobile UE has to perform frequent cell changes and therefore, the power savings of the RRC Inactive state is also compromised big time.

Lastly, we think that an RNA in RRC Inactive could be configured intelligently to mimic the solution in the sub-state proposal, except the state-transition latency, which we consider as an enhancement topic for 6G.

RRC Inactive and RRC Connected are kept distinct in 6GR and not integrated.

Based on the above, we think that the RRC state model of NR and can be retained as the baseline in 6GR.

RRC State model of 5G i.e. RRC Idle, RRC Inactive and RRC Connected are considered as the baseline for 6GR.

However, there are a few shortcomings with 5G RRC_INACTIVE that should be addressed in 6GR:

- Xn connectivity in an RNA: All the gNBs associated with an RNA are expected to be directly connected over Xn to facilitate paging and context retrieval in the RNA. This is also required for UE context transfer when a UE undergoes inter-RNA mobility. It is practically infeasible to connect all gNBs over an Xn interface.

  - This could be addressed by having multi-hop context transfer in RNAs. A master node could ensure that RNAs are well connected without the mesh of Xn connections. When this is also infeasible, context fetching could be performed via the CN.

- RNA composition: In 5G, the RNA could be configured in a multitude of ways, as a list of cells, as a list of RAN Areas, as a list of Tracking Areas. Perhaps there is no need to offer so much flexibility and can be simplified.

  - Paging in RRC Inactive is RAN-initiated, as the state is not visible to the CN. Therefore, it is sufficient to configure RNAs as a list of cells or a list of gNB-DUs or a list of gNBs (all cells included).

- I-RNTI: The I-RNTI in 5G NR is a 16-bit identifier assigned by the gNB-CU to a UE in RRC_INACTIVE. There is a mismatch between the gNB ID (32 bits) and the I-RNTI. This causes an issue while identifying the UE’s previous gNB for a context fetch during mobility. This, coupled with the missing Xn interface can cause a lot of trouble.

  - I-RNTI could be the same size as gNB-ID.

- UL access latency: SDT was added to RRC_Inactive later, in Rel 16. But this is not the only reason for lack of commercialisation of RRC Inactive. The UL access latency is also crucial. Random access, Measurements and Configuration are the aspects that needs to be considered for enhancements.

  - A conditional early TA acquisition could be considered where the UE may be able to predict an MO-SDT packet and execute early UL sync with the receiving cell.

Consider Xn connectivity in an RNA, composition of an RNA, structure of an I-RNTI and UL access latency for enhancements in RRC Inactive state for 6GR.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:

Observation 1		The 3 RRC States of NR have different objectives and characteristics. Therefore, there is very little commonality between them, especially RRC_Idle and RRC_Inactive.

Proposal 1			RRC_Inactive and RRC_Idle are not merged and location tracking of UEs in their respective states are at RNA and TA levels respectively.

Proposal 2		RRC_Inactive and RRC_Connected are kept distinct in 6GR and not integrated.

Proposal 3			RRC State model of 5G i.e RRC_Idle, RRC_Inactive and RRC_Connected are considered as the baseline for 6GR.

Proposal 4			Consider Xn connectivity in an RNA, composition of an RNA, structure of an I-RNTI and UL access latency for enhancements in RRC Inactive state for 6GR.

# References

- RP-251881. (n.d.). New SID: Study on 6G Radio. NTT Docomo.

- R2-2507074 - Common user and control plane aspects for 6G, Ericsson