3GPP TSG-RAN WG2 Meeting #132	R2-2508765

Dallas, USA, Nov 17th- 21st,, 2025

Agenda item:		10.3.3.2

Source:	Rakuten

Title:	Discussion on Energy Efficiency aspects of 6GR

Document for:		Discussion and Decision

# Introduction

This contribution discusses the energy efficiency aspects of UE and Network in 6GR.

# Discussion

## UE Power Savings

### Idle and Inactive Mode

In 5G the IoT devices with Low Power Wake-Up Receiver (LR) is unable to perform neighbour cell measurements regardless of its mobility characteristics i.e even if the LR is a low mobility device, it is not permitted to perform neighbour cell measurements in IDLE mode.

We think that devices with low mobility and/or non-cell edge devices may be allowed to perform RRM measurements in IDLE mode. This can bring significant power savings, especially for IoT devices. This helps the UEs in connected mode also.

Devices with low mobility and non-cell edge devices are allowed to perform RRM measurements in IDLE mode.

Small data transmission (SDT) in RRC Inactive is already supported in 5G but comes with quite some limitations including power consumption. Earlier, there have been studies on extending small data transmission (SDT) to RRC_IDLE for low-complexity UEs (RedCap and IoT devices). This could be feasible using configured uplink resources or grant-free transmissions in idle mode. Usage of pre-stored NAS/RRC context could be employed to skip full reconnection to RRC Connected. This enables signalling reduction (compared to Inactive) even further for infrequent, small data bursts.

Study IDLE mode SDT for specific low complexity devices and specific use-cases to benefit from lightweight signaling and reduced power consumption of IDLE mode.

For the power constrained devices (eg: LPWA including narrowband IoT), whenever random access procedure cannot be served successfully by the network (eg: due to resource crunch) then Random Access retries will lead to a lot of power wastage. This could be prevented by the network configuring the UE (explicitly or implicitly) to start a back-off timer before retrying. When we consider the LPWA devices in huge numbers, the resource crunch could be longer and hence the UE re-trying after the back-off timer expiry will be futile.

Therefore, a mechanism where the gNB itself remembers the UE that performed Random Access (eg, using a temporary identifier) and pages the UE to initiate Random Access when the resource condition is better will enable better power savings for LPWA devices.

During resource crunch, when random access requests of LPWA devices cannot be served successfully by the network, the 6G NB shall indicate the UE to go to DRX and remembers the LPWA device using a temporary identifier. Subsequently, the 6G NB pages the UE to trigger Randon Access procedure when the resource condition is better.

### Connected Mode

In 5G, network can adjust the monitoring occasions of individual PDCCH’s in time and frequency domain using features like search space switching, BWP adaptation, etc. This results in more effective PDCCH monitoring.

Therefore, PDCCH switching when coupled with traditional DRX will result in UE power saving. We propose to start with this as baseline for further optimization.

For Idle and Inactive mode UEs, RRM neighbour cell measurement relaxation is permitted, if the UE mobility condition and UE position in the cell is favorable
For Connected mode UEs, PDCCH switching coupled with traditional DRX is considered as baseline.

Further, during PDCCH decoding, the UE is required to perform blind decoding of the PDCCH payload as it is not aware of the detailed control channel structure, including the number of control channels and the number of CCEs to which each control channel is mapped.

When sending Downlink Control Information (DCI) on the PDCCH:

- The gNB computes the CRC over the DCI bits.

- The CRC is then scrambled with the RNTI associated with that DCI (e.g. C-RNTI, SI-RNTI, RA-RNTI, etc.).

- The UE performs blind decoding of the PDCCH and de-scrambles the CRC using its known RNTIs.
If the CRC matches (i.e., passes the check), the UE concludes that the DCI was meant for it.

Multiple PDCCHs can be transmitted by the gNB in a single subframe which may or may not be all relevant to a particular UE. The UE finds the PDCCH specific to it by monitoring a set of PDCCH candidates (a set of consecutive CCEs on which a PDCCH could be mapped) in every subframe. Each UE is told (via RRC signalling) which CORESET(s) and search spaces it must monitor. The UE uses its Radio Network Temporary Identifier (e.g.: C-RNTI) to decode candidates. The C-RNTI is used to demask a PDCCH candidate's CRC. If no CRC error is detected the UE determines that PDCCH carries its own control information.

In 5G, the PDCCH decoding is based on the trial-and-error approach i.e it has to decode all candidates using all valid RNTIs. Therefore, the UE completes the blind decoding of PDCCH regardless of the outcome of decoding i.e., CRC error or no error. This is inefficient as it consumes full power required for decoding even in case of an unsuccessful decoding.

This blind decoding can be enhanced by splitting the DCI into more than one parts where the first part could be very small, e.g.: indicates the UE’s RNTI (in part or full) associated with the DCI. The subsequent parts may contain the DCI message, which has the CRC scrambled with the RNTI appended. The UE may decode the RNTI in part 1 and verify if the DCI is meant for it and can stop the blind decoding of PDCCH, if it is not meant for the UE. This is very efficient compared to the UE performing blind decoding of the entire PDCCH.

DCI contents are split into more than one part with the first part being very small.

UE first decodes part 1 of the DCI message to determine if it is meant for the UE and if the PDCCH decoding should be completed.

## Network Energy Savings

Network energy saving (NES) features were introduced in the later releases of 5G. Therefore, their benefits are limited to only a small amount of UEs which support those NES features. Only a limited number of network vendors have been able to implement the features.

Add table from HW paper (R2-2506855)

| R15 | Cell-(De)Activation On-Demand SI Intra-band SSB-less SCell C-DRX BWP adjustment Overheating UAI |
| --- | --- |
| R16 | PDCCH-based WUS(DCP) UE power saving UAI Scell Dormancy RRM Measurement Relaxation |
| R17 | PEI Assistance TRS RLM/BFD Relaxation PDCCH Skipping/SSSG Switching Inactive SDT |
| R18 | Cell DTX/DRX Intra-band SSB-less SCell Spatial and Power Domain adaptation Inter-node beam activation LP-WUS(Study) |
| R19 | On-Demand SSB(SCell) On-Demand SIB1 Common Channel Adaptation(SSB/PRACH/Paging) LP-WUS(LP-WUS based Paging with RRM offloading/Relaxation, LP-WUS based PDCCH monitoring) |

Cell DTX/DRX can be used to supersede gNB’s PDCCH transmissions enabling the gNB to sleep even when the PDCCH configuration suggests a gNB transmission.

OD-SSB feature can be used to increase the default SSB periodicity beyond the 5G default of 20ms, providing significant network energy savings.

Paging and random-access channel adaptations can achieve improved network energy efficiency.

Network Energy Savings features like Cell DTX/DRX, OD-SSB, Paging and random-access channel adaptations etc. are considered baseline for first 6G release.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:

Proposal 1		Devices with low mobility and non-cell edge devices are allowed to perform RRM measurements in IDLE mode.

Proposal 2		Study IDLE mode SDT for specific low complexity devices and specific use-cases to benefit from lightweight signaling and reduced power consumption of IDLE mode.

Proposal 3		During resource crunch, when random access requests of LPWA devices cannot be served successfully by the network, the 6G NB shall indicate the UE to go to DRX and remembers the LPWA device using a temporary identifier. Subsequently, the 6G NB pages the UE to trigger Randon Access procedure when the resource condition is better.

Proposal 4		For Idle and Inactive mode UEs, RRM neighbor cell measurement relaxation is permitted, if the UE mobility condition and UE position in the cell is favorable
For Connected mode UEs, PDCCH switching coupled with traditional DRX is considered as baseline.

Proposal 5		DCI contents are split into more than one part with the first part being very small.

Proposal 6		UE first decodes part 1 of the DCI message to determine if it is meant for the UE and if the PDCCH decoding should be completed.

Proposal 7	Network Energy Savings features like Cell DTX/DRX, OD-SSB, Paging and random-access channel adaptations etc are considered baseline for first 6G release.

# References

- RP-251881. (n.d.). New SID: Study on 6G Radio. NTT Docomo.