3GPP TSG-RAN WG2 #132					      		 R2-2508770

Dallas, USA – November 17 - 21, 2025

Agenda item:	10.3.2.1

Source: 	Kyocera

Title: 	RRC state modelling for 6GR

Document for:	Discussion

# Introduction

In this contribution, we consider some aspects of RRC state modelling that should be addressed by RAN2 for 6G.

# Discussion

During RAN2#131-bis, there were discussions on RRC state modelling, which concluded with the following points:

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

## 2.1	RRC state modelling options

Based on the agreements, it is already clear that the legacy NR functionalities of the IDLE and CONNECTED states should be kept for 6GR and the main change will be focused on what to do with the INACTIVE state.

- Observation 1	The legacy NR functionalities of the IDLE and CONNECTED states should be kept for 6GR and the main change will be focused on what to do with the INACTIVE state.

While Observation 1 assumes the legacy NR functionalities should be kept for IDLE and CONNECTED states should be kept, it shouldn’t be assumed that additional functionalities on top of the legacy IDLE and/or CONNECTED states are excluded.

- Proposal 1	Enhancements to the existing IDLE and CONNECTED states are allowed for 6GR.

Based on the agreement, it is already clear that some changes to the legacy INACTIVE state is needed and the discussions in RAN2#131-bis centered on the following 3 options:

- Keep the existing INACTIVE state but work on improvements to retain 3 RRC states from NR.

- Merge the INACTIVE state with the IDLE state to form 2 RRC states.

- Merge the INACTIVE state with the CONNECTED state to form 2 RRC states.

With Option 1, the primary objective should be to improve the shortcomings of the INACTIVE state in the handling of the RAN page based on RNA configuration.  It has been discussed in [1] that possible simplification of RNA configuration can be improved, e.g., by configuring RNA for only cells in the same gNB (or intra-CU cells).  Furthermore, coordination between RAN and CN may be possible to improve the paging mechanism such that CN could be responsible for paging both IDLE and INACTIVE UEs. It would be preferable to discuss with SA2 and CT1 to see if they have concerns with the legacy INACTIVE state and if they foresee any enhancement needed for INACTIVE for 6GR. Since one of the main goals for 6GR is for latency reduction, including for connection access, the existing Resume procedure can serve as baseline for this criterion as compared to the IDLE state.  To keep the three RRC states, there should be a greater distinction between INACTIVE and IDLE; therefore, it should be further considered if additional enhancement should be adopted to close the gap between the Connected state and the Inactive state, while the UE remains in two distinctive states, without sacrificing the inherent UE power reduction in the legacy INACTIVE state. This may include enhancements in SIB signaling and the resumption in connection with fewer message exchanges.

- Proposal 2	For the option of keeping all 3 RRC states, the improvements for the INACTIVE state should center on the coordination between RAN and CN on the paging mechanism and the reduction of latency in access attempts for INACTIVE UEs.

- Proposal 3	RAN2 should send an LS to SA2 and CT1 to gain a better understanding of their concerns for the legacy INACTIVE state and the possible enhancements they foresee for INACTIVE in 6GR.

With Option 2, the primary objective is to retain some of the benefits of the INACTIVE state while the UE is officially in the IDLE state.  The main advantage of the existing INACTIVE state is for the RAN and the UE to keep the UE context such that fast resume procedure can be performed with minimal connection setup delay, by using the stored RRC configuration and security contexts.  Under this option, the legacy RRC Connected state would remain unchanged, while the RRC IDLE state will need to be enhanced to support some of the key features from the RRC INACTIVE state, namely with the retention of the UE context in the RAN and at the UE.  The combined RRC states for IDLE/INACTIVE, may be referred to as the RRC non-connected state. Although memory storage has become cheaper over the years, managing the UE contexts for all UEs not in RRC CONNECTED can still be quite complicated for the RAN.  In particular, certain IoT devices, for example, do not require fast resume procedure, esp. if SDT is also supported, although it should be further considered whether certain IoT devices (e.g., C-IoT) will be required to store UE context for the support of signalling overhead reduction Therefore, it is expected that only a subset of UEs will require the benefits that come with stored UE context.  This can be, for example, based on UE types.  The following uses cases may be associated with the RRC non-connected state:

- For UEs performing the initial Attach procedure, the UE will not have any stored UE context and behave the same as in legacy RRC IDLE state.

- For delay-sensitive UE types, when the UE is released to non-connected state, both the RAN and the UE will keep the UE context, similar to the legacy NR RRC INACTIVE state.

- For delay-tolerant UE types, when the UE is released to non-connected state, UE context is not stored at the UE, similar to the legacy NR RRC IDLE state.

- Proposal 4	For the option of merging the IDLE and INACTIVE states, UE context can be optionally kept at the UE and the NW.

With Option 3, the RRC Inactive state would become part of the CONNECTED state, albeit in a power-reduction mode to conserve UE power.  This may also be referred to as a substate of the RRC CONNECTED state.  Since the UEs in this power-reduction mode would still need to be RRC CONNECTED, a number of cases should be considered:

- CDRX in RRC CONN which was supported in LTE could be reused in 6GR.  However, the main intention of C-DRX is to support devices that are mainly stationary.  For the legacy DRX mode for RRC CONNECTED, only 10.24 sec duration is supported.  This may be extended in 6GR to allow for further UE power reduction.

- For services that require very low-latency data traffic deliveries, UEs should not operate in the power-reduction mode, even if infrequent DL/UL data is expected.  Frequent UE measurements and reporting will likely be needed. Devices with such low-latency services will likely not be configured with such power-reduction mode. However, further mobility enhancements should be considered in the mobility agenda item, esp. if UE power consumption can be reduced.

- For services that don’t expect to need very low-latency data traffic deliveries, such UE may be ideal candidate to be configured to such low-power mode. Under the low-power mode, the UE may perform cell reselection similar to the legacy INACTIVE state, within an RNA. However, it may be further assumed that UE’s in this low-power mode will be allocated with UL resources within the RNA for RACH-less access. The UL resource may be dedicated for the UE or shared resource pool.  Furthermore, it will be assumed that UEs will be able to obtain early TA (either UA-based or NW-provided), similar to LTM or C-LTM. Even if the beam/cell threshold condition is satisfied for one of the candidate cells within the RNA, the UE is not expected to send UL message to the candidate cell unless incoming data arrives. The UE will simply reselect and camp on the candidate cell.  In case DL data arrives, the LP-WUS may be largely reused to inform the UE of the available data. However, instead of releasing the UE to INACTIVE state with the LP-WUS configuration, the UE will remain in CONNECTED state. Upon waking up by the LP-WUS signal to monitor potential page message, the UE will monitor PDCCH as in CONNECTED state. Figure 1 provides an overview description of the low-power mode of the connected state for the case on incoming data arrival.

- Proposal 5	For the option of merging the INACTIVE state with the CONNECTED state, if very low-latency access is needed, the UE should remain in the legacy CONNECTED state.

- Proposal 6	For the option of merging the INACTIVE state with the CONNECTED state, consider the addition of low-power mode for UEs that do not require very low latency access.

Table 1 below is a summary of the above options in comparison with the legacy NR INACTIVE.  When compared between the legacy NR INACTIVE and Option 1 (retain 3 RRC states but with CN-RAN coordination), the main improvement with Option 1 is the reduced complexity in paging, which is beneficial to both the UE and the NW.  When compared between legacy NR INACTIVE and Option 2 (2 RRC states with merged IDLE/INACTIVE), the UE complexity should be reduced since the UE only needs to keep two RRC states, while access latency is not reduced for certain device types that require low latency access (device-type specific UE context retention).  When compared between the NR INACTIVE and Option 3 (2 RRC states with merged INACTIVE/CONN), the main benefit with Option 3 is the potential reduction in access latency, while NW resources are increased due to pre-allocation of UL resources for neighbour cells within an RNA.

- Observation 2	Each option above has its distinctive benefit over the legacy NR INACTIVE state, whether it’s access latency improvement, UE complexity/NW complexity reduction and/or UE power consumption reduction, but none of the options can achieve gains in all areas.

- Proposal 7	RAN2 should discuss which factor(s), latency reduction, UE/NW complexity, UE power consumption, NW resources should be prioritized for changes to RRC states.

Table 1 Summary of RRC state options

|  | Access latency | UE power consumption | Mobility control | UE complexity | NW complexity | Signalling load (Uu and CN) | NW resources |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR INACTIVE | moderate | low | moderate | Moderate | High (different paging modes) | moderate | moderate |
| Option 1 3 RRC states w/CN-RAN coordination | moderate | low | moderate | low (unified paging) | moderate | low | moderate |
| Option 2 2 RRC states Merge IDLE with INACTIVE (device-type context retention) | moderate | low | moderate | low -to-moderate (depend on device-type) | moderate | low | moderate |
| Option 3 2 RRC states Merge INACTIVE with CONN (low-power mode) | low | moderate | good | moderate | moderate | moderate | high |

# Conclusion

In this contribution, we addressed some of the issues related to RRC state modeling for 6GR. RAN2 is kindly asked to take into account of the observations and proposals below.

- Observation 1	The legacy NR functionalities of the IDLE and CONNECTED states should be kept for 6GR and the main change will be focused on what to do with the INACTIVE state.

- Proposal 1	Enhancements to the existing IDLE and CONNECTED states are allowed for 6GR.

- Proposal 2	For the option of keeping all 3 RRC states, the improvements for the INACTIVE state should center on the coordination between RAN and CN on the paging mechanism and the reduction of latency in access attempts for INACTIVE UEs.

- Proposal 3	RAN2 should send an LS to SA2 and CT1 to gain a better understanding of their concerns for the legacy INACTIVE state and the possible enhancements they foresee for INACTIVE in 6GR.

- Proposal 4	For the option of merging the IDLE and INACTIVE states, UE context can be optionally kept at the UE and the NW.

- Proposal 5	For the option of merging the INACTIVE state with the CONNECTED state, if very low-latency access is needed, the UE should remain in the legacy CONNECTED state.

- Proposal 6	For the option of merging the INACTIVE state with the CONNECTED state, consider the addition of low-power mode for UEs that do not require very low latency access.

- Observation 2	Each option above has its distinctive benefit over the legacy NR INACTIVE state, whether it’s access latency improvement, UE complexity/NW complexity reduction and/or UE power consumption reduction, but none of the options can achieve gains in all areas.

- Proposal 7	RAN2 should discuss which factor(s), latency reduction, UE/NW complexity, UE power consumption, NW resources should be prioritized for changes to RRC states.

# References

- R2-2507072, “Controlling the 6G access stratum”, Ericsson