3GPP TSG-RAN WG2 Meeting #132	R2-2508776

Dallas, USA, Nov 17th- 21st , 2025

Agenda item:		10.4

Source:	Rakuten

Title:	Discussion on mobility aspects of 6G

Document for:		Discussion and Decision

# Introduction

The SID of Study on 6G Radio in RAN #108 includes the following objectives [2]:

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

We present our analysis and views regarding the mobility for 6GR in RRC Connected state below.

# Background and Motivation

The 6G Study Item aims to investigate deployment scenarios and define technical performance requirements (TPRs) based on ITU-R M.2160. Mobility support must evolve to meet the demands of:

- High-speed mobility (e.g., >500 km/h for High speed Rail (HSR) and aerial platforms)

- Multi-layer coverage (terrestrial, aerial, satellite)

- AI-native mobility management

- Energy-efficient handover mechanisms

- Mobility-aware service continuity for XR, ISAC, and industrial automation

Therefore, the mobility solution framework in 6GR must address all the requirements posed by the above.

## Mobility in RRC CONNECTED State

### 6G Requirements impacting connected mode mobility

From [1], we see two important requirements connected to service interruption time permitted during mobility in RRC Connected.

- Immersive media service via AAM/UAM imposes a Service interruption time of 10ms.

- Dynamic HO between TN and NTN imposes a Service interruption time of 50ms.

These are the service requirements that are expected to be met by 6GR. Therefore, the mobility solutions have to take this into account.

The baseline 5G handover procedure from 3GPP Release 15 effectively supported enhanced Mobile Broadband (eMBB) but was limited to basic mobility scenarios. Later 5G releases introduced improvements to enhance robustness, minimize service interruption, and support new use cases. To address reliability and low latency, Release 16 added advanced mobility features like conditional handover (CHO) that improves mobility robustness, dual active protocol stack (DAPS) that reduces interruption time. In Release 18, layer 1/layer 2 triggered mobility (LTM) was introduced to reduce service interruption further and also integrated the RACH-less aspect. Also in Release 18, specifically for mobile integrated access and backhaul (mIAB) and non-terrestrial network (NTN) deployments, a Release 15 HO was enhanced with RACH-less possibility. The CLTM feature in Rel 19 added the network controlled, UE autonomously executed solution to the list.

However, the proliferation of distinct mobility procedures has created complexity and interoperability challenges for device and network vendors, potentially slowing technical progress and raising doubts about the cost-benefit of such intricate designs within 3GPP.

| Mobility Solution | 3GPP Release | Objective | Key Benefits |
| --- | --- | --- | --- |
| Baseline Handover (HO) | Release 15 | Provide basic, reliable mobility for eMBB services | Ensures seamless connectivity during movement; sufficient for standard mobility scenarios |
| Conditional Handover (CHO) | Release 16 | Improve handover robustness and reduce failures | UE autonomously triggers handover based on predefined conditions → lower failure rates and improved reliability |
| Dual Active Protocol Stack (DAPS) | Release 16 | Minimize service interruption during handover | Maintains simultaneous connections with source and target cells → reduces latency and packet loss |
| Layer 1/Layer 2 Triggered Mobility (LTM) | Release 18 | Integrate beam management with mobility for faster handover | Enables L1/L2-based handover triggers → more responsive mobility decisions and improved performance in beamforming scenarios |
| RACH-less Handover | Release 18 | Reduce access delay and signaling overhead during handover | Eliminates RACH procedure during HO → lower latency, especially useful for mIAB and NTN deployments |
| Mobility Enhancements for Private Networks | Release 16+ | Support ultra-reliable, low-latency mobility in enterprise/private environments | Enhanced reliability and responsiveness for industrial and private networks |
| Conditional LTM | Release 19 | To reduce handover latency and improve robustness. | Combine the advantages of CHO with those of LTM |

An integrated mobility solution framework, which absorbs all the important features of the advanced mobility solutions should be considered as baseline for 6GR. Some use-case specific features could be considered optional and may not be needed in all scenarios and for all devices.

Below, we analyse the individual features of all the above mobility solutions of 5G, which can be included in the integrated mobility solution framework.

### Mobility features related to Handover interruption time reduction

Handover interruption time reduction is one of the important objectives of any mobility solution.

Below is a break-up of HO delay.

- Handover Command Processing Time,

- Uplink/Downlink Suspension in Source Cell

- Random Access Procedure (RACH) in Target Cell

- Target gNB Processing and Path Setup

- Core Network Path Switching (if applicable)

- UE Synchronization and RLC/MAC Re-establishment

Some features were specified in 5G to avoid/minimize the above delays.

- Early UL sync

  - This is realized by performing early RACH to acquire TA of the candidate/target cell, while the UE is still in serving cell. This ensures that the UE can perform RACH-less HO, which implies that the UE can skip performing the RACH procedure and waiting for the RACH-occasions during the HO. Approximately 30 ms can be reduced here.

- Early DL sync

  - This is realized by activating the TCI states of the candidate/target cell while the UE is still in serving cell. Approximately 2 ms can be reduced here.

- Early decoding and processing

  - Separation of handover preparation and execution phases using different separate commands ensures that the candidate/target cell configuration can be received, ASN.1 decoded and processed for validity/compliance check in advance to HO execution. Therefore, RRC processing delay is avoided during the HO. Approximately 30ms can be reduced here.

- Early CSI acquisition

  - Early CSI acquisition was introduced to enable immediate data transmission and reception after an LTM cell switch. The UE is configured to acquire and report CSI parameters (CQI, PMI, RI, and/or LI) of the candidate/target cell and this enables appropriate link adaptation (e.g., modulation and coding scheme, number of layers, and precoding weights) when data transmission is resumed at the target cell, after LTM cell switch.

Early UL Sync, Early DL Sync, Early decoding & processing of target cell configuration and Early CSI acquisition of contribute significantly towards minimizing handover interruption time.

Early UL Sync, Early DL Sync, Early decoding & processing of target cell configuration and Early CSI acquisition are adopted as part of the integrated mobility solution framework for 6GR, to reduce Handover interruption time.

- Probability of a RACH-less HO:

  - In 5G, the TA is delivered from the candidate cell to the serving cell. This is in turn delivered to the UE. Quite some time is lost during this. This implies that the TAT validity is shortened at the UE, which forces the network to set aggressive thresholds for UL sync making it sub-optimal. In addition to this, delivery of TA directly using RAR during early sync could shorten the TA delivery time and enhance the probability of a RACH-less HO, as the TA validity time at the UE is extended.

Timing Advance delivery directly using RAR is adopted to enhance the probability of a RACH-less HO in the 6G mobility solution framework.

### Mobility features related to mobility robustness

Some features were specified in 5G to enhance mobility robustness.

- Network configured - UE autonomously executed HO

  - A candidate configuration prepared in advance is executed based on pre-defined condition, in this type of HO. Conditional Handover and Conditional LTM are two examples for this. Both CHO & CLTM are known to improve HO success rate and reduce RLF. This is primarily because of the fact that the HO command is not delivered by the network, but the UE triggers the HO execution when the radio condition is appropriate.

  - Any network preconfigured candidate cell configuration implies resource reservation and blocking. Given that the lead time between candidate cell preparation and execution is uncertain, delayed resource reservation i.e delaying the reservation of some important resources until the TA acquisition is performed can greatly help optimize resources. The probability of HO to a candidate cell is more at the time of TA acquisition than candidate cell preparation.

- Subsequent mobility

  - When multiple candidate cell configurations are prepared, ensuring the rest of the candidate cells (excluding the target cell) are usable by both network and the UE is called subsequent mobility. By retaining all the candidate cells, the configuration preparation, decoding, validation, processing etc can be avoided.

Network configured - UE autonomously executed HO and Subsequent mobility are adopted as part of the integrated mobility solution framework for 6GR to provide mobility robustness.

- Mobility robustness and HO service interruption reduction using mTRP/ICBM

  - Advanced physical layer features like Multi-TRP and Inter Cell Beam Management features could be used to enhance mobility robustness by configuring the “most likely target cell” (among the list of candidate cells) with those features. For eg, by configuring the UE in a mTRP configuration with most likely target cell as the other TRP, mobility robustness for control plane can be enhanced, as control plane signaling could be delivered using both TRPs during the handover.

  - mTRP could also be intelligently configured during the handover time to reduce the handover interruption time considerably, as the target cell can be one of the TRPs and can be active with data transmission even when the cell switch is being performed.

Integrate advanced physical layer features like mTRP and ICBM to the mobility solution framework to enhance mobility robustness and reduce handover interruption time for 6G UEs.

.

## Measurements

In 5G, we have Layer 3 RRM measurements and Layer 1 measurements. In L3 measurements, UE measures each detected beam of the serving cell and neighbour cells, and these beam measurement results are averaged to derive the cell quality. L3 measurements report the cell quality but can be optionally configured to report the L3 filtered beam quality as well.

With the introduction of LTM in Rel 18, 5G L1 measurements. Here, the cell quality is not used for target cell selection and only the beam level measurements are used for mobility decisions. L1 measurements are based on CSI measurement/report configuration which are configured dedicatedly to the UE. This is performed during LTM candidate cell preparation for a UE.

It can be observed that the L3 and L1 measurement framework is quite distinct because they were designed at different points in time of 5G. In 6G, they could be integrated and a single framework based on beam measurements could be used in both L1 and L3 measurements.

In fact, this could be extended to include CSI acquisition as well.

An integrated measurement framework based on beam measurements is adopted for L3 and L1 measurements, CSI acquisition by harmonizing both, signaling and configuration.

In 5G, L1 measurements cannot be used for discovery of cells that are not configured. This is a limitation in some scenarios. For eg, if a candidate cell is getting prepared from the same cell group (or DU) as the serving cell, the L3 configuration of the candidate cell remains the same as serving cell, only the L2 and L1 configuration maybe different. In such a case, the candidate cell could be prepared based on L1 measurements. L3 measurements could be suppressed in some scenarios.

Therefore, L1 based cell discovery using enhanced L1 measurements could be supported in 6G to enable a simplified candidate cell preparation in applicable and feasible scenarios.

L1 based cell discovery is supported in 6G to enable a simplified candidate cell preparation in feasible scenarios.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals:

Observation 1		Early UL Sync, Early DL Sync, Early decoding & processing of target cell configuration and Early CSI acquisition of contribute significantly towards minimizing handover interruption time.

Proposal 1			Early UL Sync, Early DL Sync, Early decoding & processing of target cell configuration and Early CSI acquisition are adopted as part of the integrated mobility solution framework for 6GR, to reduce Handover interruption time.

Proposal 2		Timing Advance delivery directly using RAR is adopted to enhance the probability of a RACH-less HO in the 6G mobility solution framework.

Proposal 3		Network configured - UE autonomously executed HO and Subsequent mobility are adopted as part of the integrated mobility solution framework for 6GR to provide mobility robustness.

Proposal 4	Integrate advanced physical layer features like mTRP and ICBM to the mobility solution framework to enhance mobility robustness and reduce handover interruption time for 6G UEs.

Proposal 5	An integrated measurement framework based on beam measurements is adopted for L3 and L1 measurements, CSI acquisition by harmonizing both, signaling and configuration.

Proposal 6	L1 based cell discovery is supported in 6G to enable a simplified candidate cell preparation in feasible scenarios.

# References

- 3GPP TR22.870

- RP-251881. (n.d.). New SID: Study on 6G Radio. NTT Docomo.