3GPP TSG-RAN WG2 Meeting #132	R2-2508781

Dallas, USA, Nov 17th- 21st

Agenda item:		10.3.2.2

Source:	Rakuten

Title:	Discussion on Radio Protocol Architecture – Control Plane

Document for:		Discussion and Decision

# Introduction

The SID of Study on 6G Radio in RAN #108 includes the following objectives [1]

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we present our analysis and views regarding the Control plane architecture and protocol design for 6GR.

# Discussion

## Control Plane Protocol Architecture

### Layer 3 (RRC) functionality in control plane

The control plane protocol architecture in 5G NR is a key component of the 5G RAN architecture that manages the connection between the UE (User Equipment) and the gNB (Next Generation Node B).

RRC is the Layer 3 control-plane protocol between the User Equipment (UE) and the base station. Its main purpose is to handle end-to-end communication, mobility management, and signalling. Basically, it manages how data and control information move between devices and mobile network.

RRC protocol is a key component of this stack which operates above the PDCP (Packet Data Convergence Protocol) layer and interacts with both NAS (Non-Access Stratum) and lower layers like MAC and PHY.

Below provided is the list of important RRC functions.

#### RRC Layer Functions

- Connection Management: Establish, modify, and release RRC connections.

- Mobility Management: Handover, cell reselection, and beam management.

- Security Configuration: Ciphering and integrity protection setup.

- QoS Management: Configuration of QoS flows and DRBs.

- Measurement Control: Configure and report radio measurements.

- System Information Broadcast: MIB and SIB delivery.

- UE Capability Exchange: Communicate UE features to the network.

The 5G RRC protocol architecture is shown below

Figure 1: 5G control plane protocol architecture

### Signalling information exchange mechanisms

One of the concerns pointed out in [2] is that control plane signalling transmission is scattered in 5G. For eg, messages introduced by control plane (RRC) are by default sent via an SRB. User plane map their control messages to a MAC CE. And RAN1 chooses DCI/UCI by default for L1 messages.

  - In 5G, we have RRC (layer 3 protocol) which is used to setup initial connection, mobility, measurement control and reporting, security control and system information broadcast. SRBs are used for this this purpose.

  - We have MAC (Layer 2 protocol) which is also used to for exchange of some specific control signalling, mobility execution and measurement reporting (L1 measurements). We also have control-PDUs in user plane.

  - In 5G NR Rel-19, RAN1 introduced the feature of UE-initiated/Event-driven beam Reporting (UEIBR), whose main purpose is to reduce measurement reporting overhead in intra-cell and inter-cell beam management and facilitate fast beam switching. This is transmitted over PUCCH.

  - Layer 1 uses PDCCH, PUCCH for sending DCI and UCI.

  - In summary, the gNB sends an RRC Reconfiguration message through an SRB, i.e., as PDCP SDU. MAC Activation/Deactivation control elements and Buffer Status Reports are injected directly into the payload of the PxSCH transport blocks. CSI reports and HARQ feedback are encoded separately and mapped into physical resources at the edge of the carrier (PUCCH) or multiplexed into resource elements that would otherwise have been used for PUSCH.

While some independency in layer1 control signaling is unavoidable, we think that there is a need to provide a common layer3 framework to transmit all signaling information in both UL and DL.

Different signalling information in UL and DL is transmitted using different protocols in different ways and hence there is transmission framework is not unified.

### Issues with Layer3 signalling information exchange in 5G

The 5G RAN architecture has proven itself to be future compatible by being flexible to support all requirements of all the new services that were introduced during the 5G timeline. Therefore, we consider 5G RAN architecture as a good starting point for 6GR as well. We also note that the split-RAN deployment is under discussion in RAN3.

In 5G, RLC, MAC and PHY protocols are controlled by the gNB-DU and hence has a big role in encoding RRC parameters necessary for any UE configuration. Though RRC parameter encoding is performed by the gNB-DU, the gNB-DU is not eligible to send RRC messages i.e there is no layer3 protocol at the gNB-DU for communication with the UE.

This implies the following issues exist

- Limited Control Over UE Signalling: Even when the functionality is limited to the scope of gNB-DU and no gNB-CU involvement is required, the signalling cannot be performed independently without the involvement of gNB-CU. This means the gNB-DU is more a passive entity, relying heavily on the Central Unit (CU) for control plane signalling and decisions, despite some of them being in the realm of gNB-DU managed functionalities. Some examples of this are RRC Reconfiguration involving parameters of gNB-DU only, like in an intra-cell or intra-DU scenario.

- Increased Latency: Without a layer3 protocol at the DU, the handover decision-making is centralized, potentially slowing down the process and increasing the risk of dropped connections. For eg, in an intra-DU cell change based on L3 mobility, all the decisions have to be performed by the gNB-CU which adds additional delays. Similarly, in an intra-DU LTM candidate cell addition, gNB-CU involvement causes additional delays. In both scenarios, there is no impact to gNB-CU parameters, but its involvement is unavoidable.

- Security issues: RRC handles security key exchanges and authentication procedures. Delaying these processes by routing them through the CU can expose the network to timing vulnerabilities or security risks. For eg, in LTM, MAC CE carries control signalling which isn’t secure. Presence of a layer3 protocol would naturally bring security together which can also be used for MAC CE signalling.

- Limited Local intelligence: DUs without a layer3 protocol cannot make autonomous decisions based on local radio conditions. This limits the ability to implement edge intelligence. For eg, when UE sends an UL RRC message, even if the message is meant for gNB-DU, it has to be first decoded at gNB-CU and then forwarded as an F1 message to the gNB-DU. Depending on the scenario, the decision making could be localized at gNB-DU instead of always seeking permission from gNB-CU.

Absence of a layer3 signalling protocol at the gNB-DU over the air interface has several disadvantages.

An alternative to this is a control plane protocol architecture design where RRC (layer3) is split into RRC-High(H) and RRC-Low(L) and is associated with gNB-CU and gNB-DU respectively. In a split architecture, RRC-H could be located at CU and RRC-L located at DU, with a proper segregation of functionality based on the requirements and other functions performed by CU and DU.

This addresses all the issues mentioned above

- RRC-L could be used by gNB-DU (MAC and PHY layers) for lightweight control signalling in UL/DL.

- Intra-cell and intra-DU (re)-configuration scenarios could be handled autonomously using RRC-L, without involvement of the gNB-CU which greatly enhances the latency KPIs.

- Security for MAC CE signalling comes free of cost, as RRC-L needs to be complimented with security.

- Back and forth signalling between gNB-CU and gNB-DU are avoided.

Even in a non-split centralized gNB, RRC-L protocol could be designed in such a way that it can be used explicitly for lightweight signalling of layer 2 and layer 1 signalling messages, described in Sec 2.1.2.

RRC-L could be designed to suit the requirements of both split and non-split RAN architecture deployments.

Figure 2: Illustrative 6G Control plane protocol architecture

A split RRC architecture with RRC-L protocol being used for all control plane signalling between UE  gNB-DU enables customization of message size as per the requirement. All RRC-L signalling can be mapped to a dedicated SRB and all control signalling can be consolidated under RRC-L, preventing fragmentation.

All control plane signaling between UE  gNB-DU could be consolidated under a layer 3 protocol and mapped to an SRB.

Study the split RRC architecture for 6GR over the air interface with the objective of addressing the disadvantages of 5G control plane protocol architecture.

Send an LS to RAN3 to study the impacts of a split-RRC architecture on the network side.

# Conclusion and Proposals

Based on the discussion above, we conclude this contribution with the following observations and proposals.

Observation 1		Different signalling information in UL and DL is transmitted using different protocols in different ways and hence there is transmission framework is not unified.

Observation 2		Absence of a layer3 signalling protocol at the gNB-DU over the air interface has several disadvantages.

Observation 3		RRC-L could be designed to suit the requirements of both split and non-split RAN architecture deployments.

Observation 4		All control plane signaling between UE  gNB-DU could be consolidated under a layer 3 protocol and mapped to an SRB.

Proposal 1			Study the split RRC architecture for 6GR over the air interface with the objective of addressing the disadvantages of 5G control plane protocol architecture.

Proposal 2			Send an LS to RAN3 to study the impacts of a split-RRC architecture on the network side.

# References

- RP-251881. (n.d.). New SID: Study on 6G Radio. NTT Docomo.

- R2-2507074 - Common user and control plane aspects for 6G, Ericsson