3GPP TSG RAN WG2 #132	R2-2508787

Dallas, USA, 17th – 21st November, 2025

Agenda item	: 10.4

Source	: Jio Platforms Limited

Title	: 6G Mobility Procedures with Efficient Resource Reservation

Document for	: Discussion and Decision

# Introduction

In RAN2#131bis meeting, the following has been agreed on 6G Mobility:

Agreements - Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW - Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

Following these agreements, it is noted that several mobility mechanisms introduced in 5G, such as Conditional Handover (CHO), L1/L2 Triggered Mobility (LTM), and Conditional LTM (C-LTM), rely on in-advance preparation and resource reservation at candidate cells to enhance mobility robustness and reduce interruption time. While these mechanisms have proven effective in ensuring seamless service continuity, they also introduce challenges related to prolonged resource reservation and inefficient utilization of network resources at the prepared target cells. Considering the ongoing studies on 6G mobility design, where both robustness and resource efficiency are key requirements, this contribution discusses the resource reservation aspects of existing 5G mobility mechanisms and highlights the importance of studying resource-efficient mobility solutions for future 6G systems.

# Discussion

In 5G, several mobility mechanisms have been introduced to enhance mobility robustness and reduce handover interruption time. Among these, certain mechanisms rely on in-advance preparation of candidate cells, which inherently involve resource reservation at the target cells. The following contribution describes these mobility mechanisms, CHO, LTM, and C-LTM with a focus on their resource reservation aspects.

Conditional Handover (CHO)

In 3GPP Release 16, CHO mechanism was introduced to enhance mobility robustness and reduce handover failures, particularly in scenarios with unreliable or delayed handover signalling. In CHO, the source cell prepares up to eight candidate target cells, each with its own handover configuration and execution condition. These preparation configurations are delivered to the UE, enabling it to execute the handover autonomously when the predefined condition for a particular target cell is met.

While CHO improves responsiveness and reliability, it introduces a resource reservation inefficiency: each prepared target cell must reserve the necessary radio and core network resources for the UE during the preparation phase. These resources remain reserved until either the UE executes the CHO to one of the candidate cells or the CHO configurations are explicitly released. Consequently, in cases where the UE remains connected to the source cell for an extended period or when the prepared cells are not selected, the reserved resources in those target cells remain unused, leading to long resource reservation durations and reduced spectral efficiency.

Release 18 – L1/L2 Triggered Mobility (LTM)

Building on CHO, Release 18 introduced the LTM framework to further improve mobility robustness and interruption time, particularly in low-latency and deterministic communication scenarios. In LTM, similar to CHO, the network can prepare up to eight candidate cells, and the corresponding preparation configurations are provided to the UE. The key difference is that handover execution in LTM is triggered by the network via Layer 1 or Layer 2 signalling, rather than by the UE autonomously.

As with CHO, each prepared target cell must reserve resources for the UE after LTM preparation. The resource reservation problem becomes more significant in Release 18 with the introduction of subsequent LTM procedures. In this enhancement, once the UE executes an LTM handover towards one prepared cell, the other prepared candidate cells are not automatically released. This means that the reserved resources in those non-executed candidate cells remain occupied for a longer period, potentially persisting until an explicit release command is issued by the network. This prolonged reservation exacerbates the inefficiency, especially in dense mobility scenarios with frequent or overlapping LTM preparations.

Release 19 – Conditional LTM (C-LTM)

In Release 19, the C-LTM mechanism was introduced, combining the benefits of CHO’s conditional execution and LTM’s low-latency network-based triggering. With C-LTM, the network provides the UE with multiple prepared LTM configurations, each associated with an execution condition. Up to eight candidate cells can be prepared, and the UE executes the C-LTM handover toward one of these target cells when the defined condition is met.

However, C-LTM inherits and amplifies the resource reservation challenge observed in previous releases. Since each C-LTM candidate cell must reserve the necessary radio and control plane resources between the time of preparation and the actual handover execution, these resources can remain unused for extended durations. Furthermore, C-LTM allows for subsequent C-LTM configurations, where new preparations are added before previous ones are released. In such cases, both the initial and subsequent C-LTM preparations may coexist, and if not properly managed, can lead to cascading resource reservation, where multiple sets of target cells maintain reserved resources unnecessarily. This persistent reservation behaviour introduces scalability and efficiency concerns, especially in high-mobility or ultra-dense network scenarios.

In CHO, LTM, and C-LTM, the mobility mechanisms are subject to high-cost resource reservation between the time that the candidate cells are prepared and the handover is completed.

In LTM and C-LTM, the resource reservation problem is more pronounced as they can be configured with subsequent cell change, leading to retention of network resources for a longer period of time.

- Network resources are among the most critical and valuable assets for mobile network operators, as their efficient utilization directly impacts both operational cost and user experience. When a handover is prepared at candidate cells, such as in CHO, LTM, or C-LTM, the network must reserve several types of resources to ensure seamless mobility and low-latency execution. These include radio resources (e.g., Physical Resource Blocks (PRBs) in the target cell’s scheduling domain), control-plane resources (such as Radio Network Temporary Identifiers (RNTIs), Context Identifiers, and signaling channels), and core network resources (for example, session contexts and GTP-U tunnel endpoints established for data forwarding). Additionally, buffer and admission control resources are held to guarantee quality-of-service continuity during the handover event. Because these reserved resources remain unavailable for other users until the handover is either executed or released, inefficient reservation management can lead to capacity underutilization, increased signaling load, and ultimately reduced spectral and operational efficiency across the network.

Inefficient management of reserved radio, control-plane, and core network resources during handover preparation leads to underutilization of network capacity and degraded efficiency, which is undesirable for network operators.

While the existing mobility mechanisms such as CHO, LTM, and C-LTM introduce certain inefficiencies in resource reservation, they remain highly beneficial as they significantly enhance mobility robustness, which is essential for services requiring high reliability and continuity. However, as network resources are valuable and directly tied to the operator’s cost structure and business efficiency, it is equally important to ensure that mobility enhancements do not compromise resource utilization. Therefore, to support sustainable evolution towards 6G, it is proposed that RAN2 studies mobility frameworks with a balanced focus on both robustness and resource efficiency, ensuring that future mobility designs, especially those involving pre-configurations, are optimized to minimize unnecessary resource reservation and maximize network efficiency.

To support sustainable evolution towards 6G, RAN2 studies mobility frameworks with a balanced focus on both robustness and resource efficiency, ensuring that future mobility designs, especially those involving pre-configurations, are optimized to minimize unnecessary resource reservation and maximize network efficiency.

# Conclusion

Below is the list of observations and proposal of this contribution:

Observation 1: In CHO, LTM, and C-LTM, the mobility mechanisms are subject to high-cost resource reservation between the time that the candidate cells are prepared and the handover is completed.

Observation 2: In LTM and C-LTM, the resource reservation problem is more pronounced as they can be configured with subsequent cell change, leading to retention of network resources for a longer period of time.

Observation 3: Inefficient management of reserved radio, control-plane, and core network resources during handover preparation leads to underutilization of network capacity and degraded efficiency, which is undesirable for network operators.

Proposal 1: To support sustainable evolution towards 6G, RAN2 studies mobility frameworks with a balanced focus on both robustness and resource efficiency, ensuring that future mobility designs, especially those involving pre-configurations, are optimized to minimize unnecessary resource reservation and maximize network efficiency.