3GPP TSG-RAN WG2 #132	R2-2508796

Dallas, United States, 17th – 21st November 2025

Source:	NTT DOCOMO, INC.

Title:	On user plane functionalities for 6G

Document for:	Discussion

Agenda Item:	10.3.1.1

- Introduction

In this contribution, we present a general analysis of the functionalities that the 6G user plane should support, along with our views on hardware friendliness.

- Discussion

  - User plane functionalities

In the previous meeting, it was agreed to identify the functionalities that the 6G user plane should support from the perspective of addressing issues to be resolved in 6G and meeting the agreed requirements for 6G [1].

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

In this section, we evaluate the necessity of various functionalities that were supported in the NR user plane and propose their applicability for 6G.

| NR sublayer | Functionality | Analysis |
| --- | --- | --- |
| SDAP | Mapping between QoS Flow and DRB | Up to discussion under Agenda 10.3.1.2. |
|  | Reflective QoS | Up to discussion under Agenda 10.3.1.2. |
| PDCP | Header compression | Necessary. Baseline is to support ROHC for 6G Voice. EHC depends on whether Industrial IoT for 6GR is supported. |
|  | Uplink Data Compression | Necessary for uplink data. Open to discuss whether to perform for UE capability signaling. |
|  | Security protection | It should be assumed to be necessary at least for UP data and CP data (i.e., dedicated RRC messages and NAS messages). It would be good to send an LS to SA3 requesting confirmation from RAN2. FFS for new plane (e.g., for AI/ML or sensing) if we have it. FFS for MAC CE and system information security (discussed under Agenda 10.3.3.3). |
|  | In-sequence delivery | Necessary. Unless we stop operating HARQ processes in parallel at the MAC layer, the transmitted data will continue to be received in a completely different order. Control plane data must be re-ordered to avoid decoding failures. For user plane data, depending on the transport layer protocol (e.g., TCP), it might be possible to delegate re-ordering; however, at least for UDP, re-ordering needs to be performed at Layer 2. |
|  | Discarding of outdated data | Necessary. |
|  | PDCP retransmission | It depends on the RAN2 discussions on Mobility under Agenda 10.4. If 6G still supports loss-less handover, it will be necessary to perform data forwarding from the source BS to the target BS. |
|  | Split bearer and routing | Until the RAN plenary decides otherwise, it was already agreed in the previous meeting that RAN2 will conduct discussions under the assumption that Dual Connectivity will not be supported in 6G. Therefore, there is currently no need for further consideration. |
|  | Duplicate discarding | Necessary. Our understanding is that the reasons for the receiver receiving the same SN PDU multiple times are PDCP duplication and data forwarding during handover. Since we are currently discussing under the assumption that Dual Connectivity is not used, PDCP duplication will not occur; however, duplicate detection is still necessary for data forwarding. |
| RLC | ARQ | Necessary to ensure loss-less delivery by air interface. Note: We would like to selectively perform ARQ depending on upper layer configuration. Details are discussed under Agenda 10.3.1.3. |
|  | Segmentation | Necessary to fit the packet size when UL grant is smaller than data packet size. |
|  | Re-segmentation | Necessary because it is still reasonable to reduce the size of packet when it fails. |
| MAC | Multiplexing | Necessary. |
|  | HARQ | Necessary. Note: We would like to discuss modification of HARQ operation. Details are discussed under Agenda 10.3.1.3. |
|  | MAC CE | Necessary since NR legacy features (most of which are expected to be reused in 6G) rely on MAC CE. |

For each function identified as necessary in the above analysis, it is appropriate to use the implementation in NR as a baseline and to discuss potential optimizations or modifications for 6GR as needed.

Proposal 1.	6G Layer 2 supports the following functionalities, using NR as a baseline and applying modifications or optimizations as needed.

- Header compression (ROHC)

- Uplink Data Compression

- Security protection for UP/CP data

- In-sequence delivery

- Discarding outdated data

- ARQ

- Segmentation and re-segmentation

- Multiplexing

- HARQ

- MAC CE

  - Hardware friendliness

During the previous meeting, several companies pointed out that it is desirable for the user plane design to be hardware-processing friendly, and the following agreement was reached.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

One of the motivations presented by the proponents was the issue that, when the headers of various protocols in the user plane are of variable length, the decoding process in hardware becomes more complex.

Here is our analysis of the header lengths of each protocol stack in NR.

| NR sublayer | Analysis on header or subheader length |
| --- | --- |
| SDAP | The header length of a Data PDU is 0 or 1 byte and depends on a parameter semi-statically configured for the DRB through RRC reconfiguration; therefore, it does not vary dynamically. Consequently, we believe it does not impose a burden on the decoding process. |
| PDCP | The header length of a Data PDU is 2 or 3 bytes and depends on a parameter semi-statically configured for the DRB through SN length settings in RRC reconfiguration; therefore, it does not vary dynamically. Consequently, we believe it does not impose a burden on the decoding process. |
| RLC | It depends on RLC entity type. - TM: The RLC header is fixed as 0 byte, so there is no impact on decoding. - UM: When transmitting a complete RLC SDU, the header is 1 byte. If the SN length configured by RRC is 6 bits, the first segment transmission uses 1 byte, while the middle or last segment uses 3 bytes. If the SN length is 12 bits, the first segment transmission uses 2 bytes, and the middle or last segment uses 4 bytes. - AM: If the SN length configured by RRC is 12 bits, the first segment transmission uses 2 bytes, while the middle or last segment uses 4 bytes. If the SN length is 18 bits, the first segment transmission uses 3 bytes, and the middle or last segment uses 5 bytes. In other words, the length of the RLC header is determined by three factors: the RLC entity type, the SN length, and whether segmentation is performed. Since the RLC entity type and SN length are semi-statically configured by RRC, it does not affect decoding complexity; however, whether segmentation occurs can cause the header length to change dynamically, which we believe could impose a burden on the decoding process from a hardware perspective. |
| MAC | The length of the subheader is determined by the value of the LCID and the value of the F field. The shortest MAC subheader length is 2 bytes; however, it varies as follows. - LCID: When LCID indicates 33, additional 2 bytes are added as eLCID. When LCID indicates 34, an additional 1 byte is added as eLCID. - F: When F field is set to 1, an additional 1 byte is added as L. In other words, since the MAC subheader length changes dynamically depending on the values of LCID and the F field, we believe this increases the decoding burden from a hardware perspective. |

Observation 1.	The factors that cause the header length to change dynamically are as follows: the presence or absence of segmentation in RLC, and the LCID and L field in MAC.

Therefore, we are open to discuss minimizing dynamic changes in header length for the elements mentioned above.

However, our potential concern is the approach of unifying to a longer header. For example, in the case of the F field in MAC layer, if we eliminate the F field and always set the L field length to 2 bytes, the header length would no longer vary dynamically. But this would mean allocating 2 bytes for the L field even when transmitting short MAC SDUs of 256 bytes or less. Our particular concern is with critical messages such as Message 3, which are short and represent coverage bottlenecks. For such messages, unnecessarily increasing the header length, even by just 1 byte, could degrade overall system coverage.

Observation 2.	A simple way to unify header lengths is to always adopt the longer header; however, this should be avoided as it would increase the size of messages such as Message 3, which are coverage bottlenecks.

Proposal 2.	RAN2 studies to minimize dynamic changes of header length focusing on segmentation in RLC, and the LCID and L field in MAC while avoiding unifying to a longer header.

- Conclusions

User plane functionalities

Proposal 1.	6G Layer 2 supports the following functionalities, using NR as a baseline and applying modifications or optimizations as needed.

- Header compression (ROHC)

- Uplink Data Compression

- Security protection for UP/CP data

- In-sequence delivery

- Discarding outdated data

- ARQ

- Segmentation and re-segmentation

- Multiplexing

- HARQ

- MAC CE

Hardware friendliness

Observation 1.	The factors that cause the header length to change dynamically are as follows: the presence or absence of segmentation in RLC, and the LCID and L field in MAC.

Observation 2.	A simple way to unify header lengths is to always adopt the longer header; however, this should be avoided as it would increase the size of messages such as Message 3, which are coverage bottlenecks.

Proposal 2.	RAN2 studies to minimize dynamic changes of header length focusing on segmentation in RLC, and the LCID and L field in MAC while avoiding unifying to a longer header.

- References

[1]	Chair notes, RAN2#131bis.