3GPP TSG-RAN WG2 #132	R2-2508798

Dallas, United States, 17th – 21st November 2025

Source:	NTT DOCOMO INC.

Title:	On scheduling and retransmission for 6G

Document for:	Discussion

Agenda Item:	10.3.1.3

- Introduction

This contribution presents our views on scheduling and retransmission functions aimed at reducing user data transmission delays in 6G.

- Discussion

  - Dynamic grant and configured grant

In last meeting, RAN2 reached the agreement that we “support” dynamic grant and configured grant in 6G [1].

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

However, we would like to provide clarification that RAN2 should be aware of decision on scheduling schemes including dynamic grant and configured grant should be left totally up to RAN1.

On the other hand, RAN2 can discuss enhancements on scheduling schemes for our upcoming meetings, assuming there would be dynamic grant and configured grant in 6G functionalities, even though final decision is up to RAN1.

Proposal 1.	Final decision is up to RAN1 whether to support both dynamic grant and configured grant in 6G. In contrast, RAN2 can discuss both dynamic grant and configured grant, assuming that they would be supported for future discussions.

  - Contention based BSR resource

Regarding L2 scheduling in 6G, the following agreement has been reached [1]:

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

Some companies support “contention-based (CB) BSR resource” as 6G Day1 feature to reduce SR/BSR latency.

With respect to a typical procedure based on dynamic grant in NR, before the UE can transmit UL data, there are 4 steps after UL data is generated:

- The UE transmits Scheduling Request (SR)

- The network allocates UL grant for BSR

- The UE transmits BSR

- The network allocates UL grant for UL data

As you can see, the round trip before the UE transmits UL data leads to additional delay. To deal with this delay, a potential solution would be to begin with BSR transmission, without SR transmission.

Observation 1.	Since scheduling requests and resource allocation using dynamic grants require multiple round trips before UL data transmission, starting with the transmission of a BSR can be considered as a solution.

However, if the network allocates UL grants for BSR to UEs semi-persistently, the radio resource could be exhausted, because UL grant for BSR would be UE dedicated in this case. Then CB-BSR (common) resources would be valid approach, because a fixed number of resources for BSR would be allocated even if there are many UEs in the cell.

Observation 2.	It is possible to prevent the exhaustion of UL resources by introducing contention-based BSR resource.

Proposal 2.	RAN2 study to introduce contention based BSR resource.

  - Scheduling request

On the other hand, SR should be supported as 6G day1 feature as well as CB-BSR resource. There are 2 key reasons for it.

- The behavior following CB-BSR resource contention has not yet been defined. In other words, the extent of delay caused by such contention is unknown. Therefore, retaining the existing SR mechanism as an operational fallback option is essential.

- Functionalities should not be evaluated from a single perspective. While the CB-BSR procedure may enable the network to provide UL grants faster than the dynamic grant procedure involving SR, SR offers greater robustness at the physical channel compared to BSR, which consists of multiple bytes. From a coverage standpoint, SR is an indispensable option.

Observation 3.	To avoid the additional delay caused by contention in CB-BSR resources and to prevent impact on coverage, it is beneficial to keep the option of initiating UL data transmission via a scheduling request.

Proposal 3.	Scheduling Request is supported as 6G day1 feature.

We would like to enhance SR mechanism in 6GR to make it more flexible than in NR. The current issue regarding SR in NR is periodicity of PUCCH for SR is reconfigured via RRC signaling, even though the radio environment is highly dynamic and RRC signaling is not frequent enough. From an operational perspective, enabling dynamic reconfiguration through alternative methods would be both effective and beneficial.

Here are some scenarios where more dynamic adjustment of periodicity of PUCCH for SR is needed.

- Low UL congestion: Since enough UL resource is left unused, the periodicity should be shorter to allow faster UL data transmission upon data generation.

- High UL congestion: Since UL resource is depleting, the periodicity should be longer to reduce the PUCCH for SR usage per UE.

Hence, it is effective for the network to switch periodicity of PUCCH for SR more dynamically than RRC reconfiguration for minimizing delay from UL data generation to transmission and relaxing UL resource depletion.

Observation 4.	Network operations can achieve a trade-off between reducing UL data transmission delay and relaxing depletion of UL radio resource.

Proposal 4.	Dynamic adjustment of periodicity of PUCCH for SR is studied to minimize delay for UL data transmission without UL resource depletion.

  - Enhancements in retransmission

In the previous meeting, it was agreed to study the coordination between ARQ and HARQ as follows [1].

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

As pointed out by several companies in the contributions submitted at the previous meeting, the magnitude of the additional delay caused by ARQ retransmissions in NR has become a concern.

In NR, the transmitting RLC entity performs ARQ retransmission upon receiving status reporting from the receiving RLC entity. The receiving RLC entity sends status reporting only when the t-Reassembly timer, started after detecting a missing RLC PDU, expires, or when it receives a polling bit from the transmitting RLC entity. We consider this to be one of the main reasons for the significant delay caused by RLC ARQ retransmissions.

Observation 5.	The limited triggers for sending RLC ARQ retransmission requests are one of the main reasons for the significant delay caused by retransmissions.

On the other hand, if ARQ functionality is implemented at the MAC layer, it would be possible to consider increasing the triggers for sending status reports. For example, the MAC layer could proactively decide to request ARQ retransmission and re-segmentation based on the retransmission status of HARQ processes, potentially reducing the delay caused by ARQ retransmissions.

Proposal 5.	RAN2 studies implementing ARQ functionality at the MAC layer, expanding the triggers for sending retransmission requests, and reducing delays caused by retransmissions.

- Conclusions

Dynamic grant and configured grant

Proposal 1.	Final decision is up to RAN1 whether to support both dynamic grant and configured grant in 6G. In contrast, RAN2 can discuss both dynamic grant and configured grant, assuming that they would be supported for future discussions.

Contention based BSR resource

Observation 1.	Since scheduling requests and resource allocation using dynamic grants require multiple round trips before UL data transmission, starting with the transmission of a BSR can be considered as a solution.

Observation 2.	It is possible to prevent the exhaustion of UL resources by introducing contention-based BSR resource.

Proposal 2.	RAN2 study to introduce contention based BSR resource.

Scheduling request

Observation 3.	To avoid the additional delay caused by contention in CB-BSR resources and to prevent impact on coverage, it is beneficial to keep the option of initiating UL data transmission via a scheduling request.

Proposal 3.	Scheduling Request is supported as 6G day1 feature.

Observation 4.	Network operations can achieve a trade-off between reducing UL data transmission delay and relaxing depletion of UL radio resource.

Proposal 4.	Dynamic adjustment of periodicity of PUCCH for SR is studied to minimize delay for UL data transmission without UL resource depletion.

Enhancements in retransmission

Observation 5.	The limited triggers for sending RLC ARQ retransmission requests are one of the main reasons for the significant delay caused by retransmissions.

Proposal 5.	RAN2 studies implementing ARQ functionality at the MAC layer, expanding the triggers for sending retransmission requests, and reducing delays caused by retransmissions.

- References

[1]	Chair notes, RAN2#131bis.