3GPP TSG-RAN WG2 Meeting #132                                    	R2-2508810

Dallas, USA, November 17th-21st, 2025

Agenda item:	8.9.2

Source:	Qualcomm Incorporated

Title:	MPDCCH narrowband configuration for CB-Msg3-EDT

Document for:	Discussion and Decision

- Introduction

RAN2 has received LS from RAN1 in [1] with the following action.

Support narrow band MPDCCH configuration for CB-Msg3-EDT

RAN2 ask RAN1 to check the specification impact in RAN1 to support 2 MPDCCH narrowbands for CB-Msg4 monitoring, and update the corresponding specification if needed.

RAN1 response for support MPDCCH narrowband configuration for CB-Msg3-EDT:

- From RAN1 perspective, introducing 2 MPDCCH narrowbands requires additional RAN1 standard effort. RAN1 would like to inform RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

In this document, we provide details on solution to select a narrowband for Msg4 monitoring.

- Discussion

Currently CB Msg3 configuration can configure UE with two different narrowbands for monitoring the response (i.e., Msg4) of the CB Msg3 transmission.

However, a UE won’t be able to monitor both narrowbands at the same time. This is why RAN1 has asked RAN2 to define a mechanism to select a narrowband to monitor.

In case of 4 step RACH, network can also configure a set of narrowbands for RAR but there is a mechanism defined in RAN1 specification (as shown in table) based on the preamble ID, i.e., the preamble index is used to map the response to the narrowbands so that a UE monitors only the narrowband corresponding to its preamble choice.

However, for CB Msg3 transmission, there is no preamble ID and there is also no resource index that can be used as network wouldn’t know in which time/frequency resource did UE transmit the CB Msg3 replica within transmission window.

In the case of CB Msg3 EDT, some of the solutions that can be considered can be based on

- Contention resolution ID or UE ID

- CB-RNTI

- Msg3 transmission window index

Since the Msg4 monitoring window (i.e., response window) can be as large as 10.24s and windows may likely to be overlapped for the two different CB Msg3 transmission, it could be a good option to select the narrowband based on CB-RNTI or Msg3 transmission window index. Since the granularity of CE level is not needed (as in CB-RNTI), the narrowband can be based on Msg3 transmission window index, i.e., the Msg3 transmission window index can be determined from the SFN where the transmission window starts.

Narrowband index = floor (SFN_id/WP) modulo (2)

Where SFN_id is the first SFN of the selected CB-Msg3 transmission window and

WP is the configured CB-Msg3 transmission window periodicity in unit of 10 ms provided by

windowPeriodicity.

However, as all replica transmissions include the contention resolution identifier or UE ID, one bit or a hash of this can be used to select the narrowband. This distributes the UEs transmitting in the same window across the two narrowbands. As both narrowbands can be used at the same time for a transmission window, network can transmit 2 MPDCCH and correspondingly 2 PDSCH (e.g., for different UEs) to deliver the response message in a shorter duration resulting in reduced latency. In case of backoff, the network can send it in both narrowbands. Therefore, we suggest that narrowband is selected based on UE ID.

For Msg4 monitoring, the UE monitors the first MPDDCH narrowband if UE ID modulo (2) = 0 and second MPDCCH narrowband if UE ID modulo (2) = 1.

- Conclusion

Following observation and proposals are made:

Proposal 1	For Msg4 monitoring, the UE monitors the first MPDDCH narrowband if UE ID modulo (2) = 0 and second MPDCCH narrowband if UE ID modulo (2) = 1.

- Reference

# [1] R2-2508014, “Reply LS on power ramping and RRC configuration for CB-Msg3-EDT”, Dallas, USA, 17th - 21st, November 2025.