3GPP TSG-RAN WG2 Meeting #132                                    	R2-2508813

Dallas, USA, November 17th-21st, 2025

Agenda item:	10.2.2

Source:	Qualcomm Incorporated

Title:	Views on TN NTN integration for 6G

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis meeting, initial discussion on 6G NTN started and following guideline was captured on how to support NTN from 6G day 1.

- For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this document, we provide details on the basic NTN features and requirements that can be integrated in the 6G design from day 1.

- Discussion

Top priority requirements

Due to large RTT and time variant service link and feeder link, the requirements for the successful transmission and reception of data/signal are different in NTN compared terrestrial networks. For 6G design, essential requirements to support NTN can be considered from the first release. For this purpose, we think simple following two existing UE capabilities should be taken into account, one is nonTerrestrialNetwork-r17 and the other is uplinkPreCompensation-r17.

| nonTerrestrialNetwork-r17 Indicates whether the UE supports NR NTN access. If the UE indicates this capability the UE shall support the following NTN essential features, e.g., timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT, acquiring NTN specific SIB and more than one TAC per PLMN broadcast in one cell. | UE | No | No | No |
| --- | --- | --- | --- | --- |

| uplinkPreCompensation-r17 Indicates whether the UE supports the uplink time and frequency pre-compensation and timing relationship enhancements comprised of the following functional components: - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of pre-compensation of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of UE receiving cell-specific K_offset/K_mac in system information Support of this feature in NTN bands is mandatory for UE supporting nonTerrestrialNetwork-r17. This field is only applicable for bands in Table 5.2.2-1 and Table 5.2.3-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

As we can see above, both nonTerrestrialNetwork-r17 and uplinkPreCompensation-r17 are mandatory for UEs to access NTN cell. These should be prioritized first as essential requirements for TN NTN integration.

Basic requirements for NTN access can be considered to be same as in 5G NTN requirements as specified by in nonTerrestrialNetwork-r17 and uplinkPreCompensation-r17.

In addition, some optional NTN features can also be considered for the first 6G release. For example, HARQ feedback disabling, UL HARQ mode and 32 HARQ processes have been popular and even being considered for other features (e.g., MBS multicast, FR1/FR2 CA etc.) in Terrestrial network.

In 5G, providing satellite information (e.g., ephemeris, epoch time, common TA etc) for the NTN cell via system information and dedicated signaling is supported and we think this is important feature for the support of cell selection/reselection, carrier redirection, measurements and handover. For handover case, the handover command (intra-system or inter-system) can include the satellite information if the target cell is an NTN cell.

Additionally, HARQ feedback disabling, UL HARQ mode and 32 HARQ processes can be considered as important feature to support NTN.

Mechanism of delivering satellite information (e.g., ephemeris, common TA etc.) for the NTN cell via system information and dedicated signalling is also considered an important feature.

From RANs perspective, the transparent payload vs regenerative payload architecture may not need to be differentiated and similarly moving cell vs fixed cell scenarios may not need to be differentiated. However, support of feature for NGSO and GSO would need to be differentiated.

RAN2 assume both transparent and regenerative payloads and both moving and fixed cell scenarios can be considered assuming differentiation between NGSO and GSO scenario support.

Other priority NTN specific features:

We think it is also reasonable to leverage other features to bring additional value for NTN support. Some examples include followings:

- UE assistance information reporting

  - Due to movement of satellite and UE mobility, measurement configuration becomes challenging. The UE coarse location information reporting, propagation delay reporting and timing advance reporting mechanisms can also be useful features.

- Coverage enhancements

  - Coverage enhancements such as Msg4 PDSCH and Msg4 HARQ ACK feedback repetition levels can be important to support for initial access success.

- Spectral efficiency enhancements

  - New features such as support of OCC to improve capacity can also be considered.

- GNSS-less support

  - 5G Rel-20 is already studying the GNSS-resilient operation. It is important due to temporary loss of GNSS capability, it can be due to NLOS of GNSS satellites or jamming of GNSS satellites.

- Beam hopping

  - Initial cell search with default larger SSB periodicity such as 160ms and support of 7 SMTC offsets is also useful for NTN as a satellite may be capable of several beams but only be capable using few of them simultaneously.

- TN NTN mobility

  - Basic UE capability to support NTN, receiving HO command with NTN cell information and basic mechanism to handle the handover interruption due to GNSS acquisition and satellite synchronization can also be considered.

- PWS/MBS broadcast

  - Support of ETWS and CMAS over NTN cell is also important to consider. In NTN specific way, ETWS/CMAS geofencing can also be considered. Similarly, MBS broadcast with intended service area can also be considered (if MBS is to be supported in 6GR).

- Power saving features

  - TN Measurement relaxation based on location information can also be useful for saving power.

- Network verified UE location and RAT dependent positioning:

  - RAT dependent positioning methods such as TDOA can be defined based on moving satellite TRPs. Network may need to verify if the UE is present in the area where satellite access is authorized.

Other features to consider with lower priority to leverage other features to bring additional value to NTN deployments are UE assistance information reporting, Coverage enhancements, spectral efficiency enhancements,	GNSS-less support, Beam hopping, TN NTN mobility and Power saving features.

Lower priority features:

Some NTN specific features can also bring additional values to NTN deployments, but that can be considered non-essential for the first 6G solution as TN NTN integration can work without them. For example,

- Mobility enhancements

  - Handover enhancements such as time/location-based CHO, RACH-less HO and satellite switch enhancements can also be considered. However, these may not be essential in the first release.

- Position, Navigation, Tracking (PNT) with Low Earth Orbit (LEO) Satellites

  - With many satellites being deployed in LEO, the positioning based on LEO satellites can be faster and reliable. Development of this feature may depend on input from operators and also whether positioning as a whole is supported in 6G first release.

- CA and DC

  - Motivation for Carrier aggregation and dual connectivity is not strong for the initial 6G solution as NTN might be needed for supplementary coverage when TN coverage is not available

- Robust paging notification

  - Failure to respond to paging due to poor UL link budget can be an issue which could be resolved with robust paging notification or user cooperative solution. However, this will require coordination across different working groups including SA1 requirements.

- NTN TDD

  - At this stage, it is unclear if there is a strong motivation to define 6G TDD NTN in day 1. Operator spectrum for this should be identified. Support of TDD may offer challenges due to large RTT and issues such as inter-satellite cross-satellite interference.

- Voice over GEO

  - Due to extremely long RTT over GEO and poor UL link budget, support of voice over GEO may need more work for handheld smart UEs.

Based on the above description, we have categorized the features in Table 1 that can be considered as part of TN NTN unified 6G design and essential NTN specific feature for initial 6G solutions. We also provide features that should be excluded from initial 6G solutions. The essential requirements to support NTN should be the steps taken towards maximizing TN NTN commonality.

| Requirements to consider in common TN NTN 6G design | Leveraging NTN specific features for additional value | Non-essential NTN features |
| --- | --- | --- |
| timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT | transparent and regenerative payloads and both moving and fixed cell scenarios can be considered assuming differentiation between NGSO and GSO scenario | Mobility enhancements e.g., time/location-based CHO, RACH-less HO, satellite switch etc. |
| satellite specific information via SIB and dedicated signaling e.g., SIB/RRC design can support signaling of satellite information | UE assistance information reporting. e.g., The UE coarse location information reporting, propagation delay reporting and timing advance reporting | Position, Navigation, Tracking (PNT) with Low Earth Orbit (LEO) Satellites |
| Multiple Tracking Area Codes per PLMN e.g., SIB signaling design can support signaling of multiple TACs | spectral efficiency enhancements e.g., OCC | Carrier aggregation and dual connectivity |
| UE specific TA calculation and common TA calculation | Beam hopping e.g., larger SSB periodicity such as 160ms and support of 7 SMTC offsets | Voice over GEO |
| Timing adjustment e.g., pre-compensation of the calculated TA in its uplink transmissions and determining timing of the scheduling of PUSCH with cell-specific K_offset | TN NTN mobility e.g., Basic UE capability to support NTN, receiving HO command with NTN cell information and basic mechanism to handle the handover interruption due to GNSS acquisition and satellite synchronization | Robust paging notification |
| HARQ feedback disabling, UL HARQ mode and 32 HARQ processes | GNSS-less support e.g., access without GNSS or temporary loss of GNSS | NTN TDD |
| Coverage enhancements e.g., Msg4 PDSCH and Msg4 HARQ ACK feedback repetition levels | Power saving features e.g., location-based TN cell measurement relaxation |  |
| PWS/MBS broadcast e.g., ETWS/CMAS geofencing, MBS broadcast intended service area (if MBS is supported) | Network verified UE location or RAT dependent positioning |  |
| Multiple measurement gaps per frequency layer |  |  |

Based on above, we propose followings:

For the common design of TN NTN integration and to maximize the commonality, consider followings as essential requirements:

  - timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT

  - satellite specific information via SIB and dedicated signaling

  - Multiple Tracking Area Codes signaling per PLMN

  - UE specific TA calculation and common TA calculation

  - pre-compensation of the calculated TA and cell specific K_offset in its uplink transmissions

  - HARQ feedback disabling, UL HARQ mode and 32 HARQ processes

  - PWS/MBS broadcast

  - Coverage enhancements

  - Multiple measurement gaps per frequency layer

Consider following NTN specific features based on need to bring additional value to the system:

  - UE assistance information reporting, e.g., TA report, coarse UE location report

  - transparent and regenerative payloads and both moving and fixed cell scenarios can be considered assuming differentiation between NGSO and GSO scenario

  - spectral efficiency enhancements

  - Beam hopping

  - TN to NTN mobility

  - GNSS-less support

  - Power saving features

  - Network verified UE location or RAT dependent positioning

Consider following features as non-essential for TN NTN integration and they can be considered in later releases:

  - Mobility enhancements, e.g., time/location-based CHO, RACH-less HO, satellite switch etc.

  - Positioning, e.g., Position, Navigation, Tracking (PNT) with Low Earth Orbit (LEO) Satellites,

  - Carrier aggregation and dual connectivity

  - Voice over GEO

  - NTN TDD

  - Robust paging notification

- Conclusion

Following observations are made:

Observation 1.	Basic requirements for NTN access can be considered to be same as in 5G NTN requirements as specified by in nonTerrestrialNetwork-r17 and uplinkPreCompensation-r17.

Observation 2.	Additionally, HARQ feedback disabling, UL HARQ mode and 32 HARQ processes can be considered as important feature to support NTN.

Observation 3.	Mechanism of delivering satellite information (e.g., ephemeris, common TA etc.) for the NTN cell via system information and dedicated signalling is also considered an important feature.

Observation 4.	RAN2 assume both transparent and regenerative payloads and both moving and fixed cell scenarios can be considered assuming differentiation between NGSO and GSO scenario support.

Observation 5.	Other features to consider with lower priority to leverage other features to bring additional value to NTN deployments are UE assistance information reporting, Coverage enhancements, spectral efficiency enhancements, GNSS-less support, Beam hopping, TN NTN mobility and Power saving features.

Following proposals are made:

Proposal 1	For the common design of TN NTN integration and to maximize the commonality, consider followings as essential requirements:

-	timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT

-	satellite specific information via SIB and dedicated signaling

-	Multiple Tracking Area Codes signaling per PLMN

-	UE specific TA calculation and common TA calculation

-	pre-compensation of the calculated TA and cell specific K_offset in its uplink transmissions

-	HARQ feedback disabling, UL HARQ mode and 32 HARQ processes

-	PWS/MBS broadcast

-	Coverage enhancements

-	Multiple measurement gaps per frequency layer

Proposal 2	Consider following NTN specific features based on need to bring additional value to the system:

-	UE assistance information reporting, e.g., TA report, coarse UE location report

-	transparent and regenerative payloads and both moving and fixed cell scenarios can be considered assuming differentiation between NGSO and GSO scenario

-	spectral efficiency enhancements

-	Beam hopping

-	TN to NTN mobility

-	GNSS-less support

-	Power saving features

-	Network verified UE location or RAT dependent positioning

Proposal 3	Consider following features as non-essential for TN NTN integration and they can be considered in later releases:

-	Mobility enhancements, e.g., time/location-based CHO, RACH-less HO, satellite switch etc.

-	Positioning, e.g., Position, Navigation, Tracking (PNT) with Low Earth Orbit (LEO) Satellites,

-	Carrier aggregation and dual connectivity

-	Voice over GEO

-	NTN TDD

-	Robust paging notification