3GPP TSG-RAN-WG2 Meeting #132	                                       R2-2508818

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	9.3.2

Source: 	Sharp

Title:  	Discussion on RRM Measurement Inference Reporting Configuration

Document for: 	Discussion

# Introduction

In RAN2#130 [1], the following agreements were made:

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain - PW length (for temporal case A) - Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

For the applicability report “inference parameters”, in the functionality-based LCM, the UE should jointly examine the inference performance for the applicability. Hence, RAN2 should further consider some performance-related parameters or criteria for applicability reporting on the self-perspective of the UE. Further agreements were made during RAN2#131 [2]

Agreements

- For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

- For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration.  This list is optional.

- For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

- For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results).

- For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

We will discuss the issues mentioned previously in detail in the next section.

# Discussion

## Inference/reporting configuration

In the RAN2#129bis meeting, the following agreement was reached [3].

Agreement The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework.

In the legacy framework, a measurement object specifies the frequency/time location and subcarrier spacing of reference signals to be measured, while a reporting configuration defines the triggering conditions and content of measurement reporting. A measurement identity associates a measurement object with a reporting configuration, such that the UE applies the reporting rules indicated in the reporting configuration to the measurement results obtained from the corresponding measurement object.

For RRM measurement prediction, the same principle can be applied. A measurement object configures what to predict (i.e., inference-related parameters), and a reporting configuration specifies when and how to report the predicted measurement results. A given pair of measurement object and reporting configuration may therefore represent a prediction type (i.e., a specific AI/ML functionality for mobility). For example, a pair of measurement object and reporting configuration may represent temporal domain prediction Case A/B, frequency-domain prediction, or spatial-domain prediction.

In addition, we think it is considered reasonable to configure a UE with multiple prediction types for the same target frequency, since the prediction types serve different objectives. Specifically, temporal domain prediction Case A generates predicted measurement results for future time instances, targeting handover performance improvement. Frequency domain prediction generates predicted measurement results for the current time instance, targeting reduction of measurement effort.

However, TS 38.331 currently specifies that, for all SSB-based measurements, the network shall configure at most one measurement object per frequency (i.e., ssbFrequency). Assuming a measurement object is used to configure inference related parameters, this restriction prevents the configuration of multiple measurement objects for different prediction types for the same frequency. To address this, the following options may be considered.

- Option 1: Allow configuration of multiple measurement objects for different prediction types for the same frequency (with associated specification impact).

- Option 2: Introduce a new prediction object, without the above restriction, to configure inference-related parameters for a certain prediction type.

Based on the analysis above, we then have the following proposals:

Proposal 1: RAN2 to clarify whether multiple prediction types can be configured for the same SSB frequency.

Proposal 2: RAN2 to discuss the following configuration options for inference-related parameters for a prediction type:

- Option 1: Use the existing measurement object.

- Option 2: Introduce a new prediction object.

## Inference parameter configurations

The study on AI/ML Mobility [4] considered measurement reduction as one of the main goals (case B). It is noted in the WID [5] that no degradation in SLS handover performance is observed with up to 50% measurement reduction. For discussions on measurement reductions, the TR [6] defines MRRT (Measurement Reduction Rate in Temporal domain) and MRRS (Measurement Reduction Rate in Spatial domain) as listed below.

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

Measurement reduction may be considered, for example, for the following sub-use cases [6] for cell-level RRM measurement predictions.

-	Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated;

-	Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s);

-	Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

It is observed that certain use cases (sub-use cases 1 and 3) require actual L1 measurement results directly for generating L1/L3 beam/cell-level measurement predictions, although filtering may be left to implementation. Therefore, it is important to consider the actual L1 measurements available to the AI/ML mobility functionality, especially when considering the measurement reduction scenarios.

Observation 1: Actual L1 measurements are used as inputs for generating L1/L3 beam/cell-level measurement predictions for several sub-use cases.

It was discussed during RAN2#130, whether the inference parameters provided by the network to the UE may include the skipping pattern (e.g. SSB config to measure), and MRRT (for temporal case B). Whereas the SSB measurement configuration indicates to the UE which subset of SSBs are to be measured, the MRRT parameter may potentially be used to further determine the SSBs to be measured (or skipped) within the configured subset of SSBs. The actual measurement results can then be used as the inputs to the configured AI/ML mobility functionality. Since both SSB measurement configuration and MRRT might play an important role in providing inputs to the AI/ML mobility functionality, it is important to consider them as potential inference parameters for the AI/ML model. RAN2 is requested to discuss the topics in further detail.

Observation 2: SSB measurement configuration and MRRT both impact the available inputs to the AI/ML mobility functionality, which consist of actual L1 measurements performed by the UE.

Proposal 3: RAN2 is requested to discuss SSB measurement configuration and MRRT in further detail in the context of the inference parameters to be used for AI/ML mobility functionalities.

In this context, two general approaches can be considered for measurement reduction.

- Network-controlled (with SSB configuration and skipping pattern)

- UE-flexible (with MRRT/MRRS configuration)

Network-controlled (with SSB configuration, and skipping pattern): In this case, the network can provide the relevant SSB configuration to the UE in the context of AI/ML mobility, which can be used by the UE to perform measurements as configured. The actual MRRT can be derived by the UE from the provided SSB configuration. RAN2 can consider using the existing SSB measurement configuration mechanisms as the baseline, and discuss further enhancements required for AI/ML functionalities.

UE-flexible (with MRRT/MRRS configuration): In this case, the network may provide only a target value for MRRT/MRRS to the AI/ML functionality configured to the UE. Since MRRT/MRRS only measure time averages for the number (or location) of measurements, the UE can be provided the flexibility to choose the SSBs to measure (or skip) based on the provided MRRT/MRRS by the network, while satisfying the measurement reduction requirements. Therefore, RAN2 is requested to consider the MRRT/MRRS parameters in the context of inference configuration.

Observation 3: MRRT and MRRS only consider time-averages for the measurements to be skipped due to the AI/ML functionality.

Observation 4: In case only SSB configurations with relevant skipping patterns are provided to the UE by the network, the MRRT/MRRS can be derived by the UE based on the provided skipping configuration.

Proposal 4: RAN2 is requested to consider using the existing SSB measurement configuration mechanisms as the baseline for considering inference parameter configuration.

Proposal 5: RAN2 to discuss enhancements required for SSB measurement configuration procedures such as skipping patterns for realizing measurement reduction under network-provided configurations.

Proposal 6: Consider MRRT and MRRS as candidate parameters for inference configuration for the case where the UE is provided flexibility for SSB measurement reduction.

Since both options of providing skipping patterns for SSB measurements and providing MRRT/MRRS configurations to the UE can be useful under different use cases, RAN2 is requested to consider both options during the WI stage and not deprioritize either option.

Proposal 7: RAN2 to consider discussing both SSB measurement skipping patterns and MRRT/MRRS configurations as inference parameters for appropriate use cases during the WI.

# Conclusions

Based on the discussion above, we have the following observations:

Observation 1: Actual L1 measurements are used as inputs for generating L1/L3 beam/cell-level measurement predictions for several sub-use cases.

Observation 2: SSB measurement configuration and MRRT both impact the available inputs to the AI/ML mobility functionality, which consist of actual L1 measurements performed by the UE.

Observation 3: MRRT and MRRS only consider time-averages for the measurements to be skipped due to the AI/ML functionality.

Observation 4: In case only SSB configurations with relevant skipping patterns are provided to the UE by the network, the MRRT/MRRS can be derived by the UE based on the provided skipping configuration.

We also make the following proposals:

Proposal 1: RAN2 to clarify whether multiple prediction types can be configured for the same SSB frequency.

Proposal 2: RAN2 to discuss the following configuration options for inference-related parameters for a prediction type:

- Option 1: Use the existing measurement object.

- Option 2: Introduce a new prediction object.

Proposal 3: RAN2 is requested to discuss SSB measurement configuration and MRRT in further detail in the context of the inference parameters to be used for AI/ML mobility functionalities.

Proposal 4: RAN2 is requested to consider using the existing SSB measurement configuration mechanisms as the baseline for considering inference parameter configuration.

Proposal 5: RAN2 to discuss enhancements required for SSB measurement configuration procedures such as skipping patterns for realizing measurement reduction under network-provided configurations.

Proposal 6: Consider MRRT and MRRS as candidate parameters for inference configuration for the case where the UE is provided flexibility for SSB measurement reduction.

Proposal 7: RAN2 to consider discussing both SSB measurement skipping patterns and MRRT/MRRS configurations as inference parameters for appropriate use cases during the WI.

# References

[1] Chair notes, RAN2 130 3GPP TSG-RAN WG2 Meeting #130, St.Julians, Malta, 2025

[2] Chair notes, RAN2 131 3GPP TSG-RAN WG2 Meeting #131, Bengaluru, India, 2025

[3] Chair notes, RAN2 3GPP TSG-RAN WG2 meeting #129bis, Wuhan, China, 2025

[4] RP-242393 SID AI/ML Mobility

[5] RP-251864 WID AI Mobility

[6] TR 38.744 Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR; (Release 19)