3GPP TSG-RAN WG2 Meeting #132	R2-2508840

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.2.2

Source:	CMCC

Title:		Considerations on TN/NTN integration for 6GR

WID/SID:		FS_6G_Radio

Document for:	Discussion

Introduction

As in the 6G RAN SID [1] , the following objective is supported:

j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

And in the latest TR 38.914 [2], the following requirement for architecture and migration is introduced:

- The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

Then in last RAN2 meeting, there were some discussion on TN/NTN integration, and the following agreement is reached [3]:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this contribution, we would like to provide some considerations on the TN/NTN integration for 6GR.

Discussion

2.1 NTN features supported in 5G

As known the typical characteristics of the NTN system are mainly larger coverage, longer propagation delay and high speed movement of satellite compared to TN network. And In R17~R19 as well as the ongoing 5GA R50, there are plenty of enhancements introduced for NTN. Therefore the NTN specific solutions introduced in 5G could be the starting point to for TN/NTN integration considerations in 6GR. And in the subsequent discussions, we would illustrate the main NTN features supported in 5G firstly and then identify essential requirements for NTN and which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

The main user plane enhancements and control plane enhancements supported in 5G NTN are illustrated in Table 1 and Table 2 respectively.

Table 1 User plane enhancements in 5G NTN

| RACH | - TA (timing advance) pre-compensation - TA reporting |
| --- | --- |
| HARQ | - DL: HARQ feedback disabled - UL: HARQ mode A/B |
| UP timer | - offset UE-gNB RTT to start some UP timers (e.g. ra-ContentionResolutionTimer) - Extension the timer value range (e.g. t-ReassemblyExt-r17, discardTimerExt-r17) |

Table 2 Control plane enhancements in 5G NTN

| Idle/Inactive mode | - Support broadcast more than one TACs per PLMN in a cell - Time-based and location-based neighbour cells measurements initiation - Multiple SMTCs configuration, and UE can adjust SMTCs based on its location and assistance information in SIB19. - Location-based SMTC selection - Provide TN coverage information in NTN cells for NTN->TN cell recelection and NTN assistance information in TN cell for TN->NTN cell recelection |
| --- | --- |
| Connected mode | - RRM measurement-based event A4, time-based and location-based for CHO trigger condition - PCI unchanged solution for quasi-Earth fixed scenario under transparent payload - RACH-less HO - Multiple SMTCs configuration, and network control the adjustment of SMTCs based on UE assistance information (i.e. service link propagation delay difference(s) between serving the cell and neighbour cell(s).) reporting - UE assisted SMTC configuration |

In addition to the above functions, network verified UE location and MBS broadcast, etc. are also supported in 5G NTN.

2.2 6G NTN/TN integration consideration

### 2.2.1 User plane enhancements analysis

- TA pre-ompensation

Due to the longer propagation delay, to achieve synchronisation, before and during connection to an NTN cell, UE needs to perform TA pre-compensation based on the network assistance information including ephemeris and Common TA and its own GNSS location. And UE could report Timing Advance during RACH procedure and connected mode based on network configuration. If without TA pre-compensation mechanism, there would be significant access issues for the UE in an NTN cell, therefore, TA pre-compensation is essential for 6G NTN. However, in TN system, the propagation delay is not so long, such TA pre-compensation solution is not needed.

Observation 1: Due to longer propagation delay in NTN, the TA pre-compensation enhancements essential for 6G NTN, but not for 6G TN.

- HARQ enhancements

HARQ Round Trip Time in NR is of the order of several milliseconds. The propagation delays in NTN are much longer, ranging from several milliseconds to hundreds of milliseconds depending on the satellite orbit. To adapt to the long propagation delay in NTN, HARQ feedback disabled for downlink and two HARQ modes for uplink are supported in 5G NTN. Although the number of HARQ processes has increased from 16 to 32 in 5G NTN, the HARQ feedback disabled for DL and HARQ modes mechanism for UL are still important to help improve the transmission delay as well as stop-and-wait issue. Further, considering the such HARQ enhancements could also be considered in 6G TN, especially for delay-sensitive services, which could help improve the user experience.

Observation 2: For transmission delay reduction, HARQ feedback disabled for DL and HARQ modes mechanism for UL could be supported for both 6G NTN and some services in TN, e.g., XR, MBS.

- UP timers enhancements

In order to match the too much long UE-gNB RTT in NTN, using UE-gNB RTT as the offset to start some UP timers and extension the value range solution are introduced in 5G NTN. Considering that such enhancements are used to adapt to the long transmission delay in NTN, which are also needed to be supported for 6G NTN. Then, for the timer value range extension solution, what needs to do is just introducing some new values for NTN system, for 6GR system, the corresponding timer value range could be configured commonly for both NTN and TN, and up to network configuration for different scenarios. For example, current T-Reassembly with value range {ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms120, ms130, ms140, ms150, ms160, ms170,ms180, ms190, ms200, spare1}, T-ReassemblyExt-r17 with value range {ms210, ms220, ms340, ms350, ms550, ms1100, ms1650, ms2200}, in 6G day1, the T-Reassembly could be designed commonly with one IE to cover all the possible values, and up to network configuration for different scenarios.

Observation 3: UP timers value range configuration could be designed commonly for both 6G TN and NTN while offset UE-gNB RTT to start some UP timers only for 6G NTN.

### 2.2.2 Control plane enhancements analysis

- Tracking Area update enhancements

Larger coverage and high speed movement of the satellites in NTN lead to the TAU issue, small tracking Areas would lead to massive TAU signaling and wide tracking Areas would lead to high paging load, which needs compromise between the TAU and the paging signaling load. To alleviate the issue, both hard TAC update and soft TAC update solutions are supported in 5G NTN, and which solution is actually performed is up to network implementation. As shown in figure 1, for hard TAC update, network broadcast only one TAC per PLMN in a cell, which help reduce the paging overhead but needs frequent update. On the contrary, for soft TAC update, network broadcast multiple TACs per PLMN in a cell, which help reduce the signaling overhead of frequent update but increase the paging overhead. In order to improve the TAU performance, such TAC update mechanism could be supported for 6G NTN but not for 6G TN network, due to that there is no so larger cell coverage and high speed network node.

Figure 1 Hard TAC update (left) and soft TAC update (right)

Observation 4: For Idle/inactive mode, both hard and soft TAC update mechanism could be supported for 6G NTN but not for 6G TN network, due to that there is no so larger cell coverage and high speed network node.

- Time-based and location-based for both idle/inactive and connected mode

Due to unobvious near-far effect in NTN network, time-based and location-based neighbour cells measurements initiation mechanism for cell reselection are supported in 5G NTN, which would help to initiate neighbour cell measurement properly in NTN. And if just rely on the RSRP/RSRQ, neighbour cell measurement may only be initiated when the satellite coverage is completely lost, which is unexpected. Thence, the time-based and location-based neighbour cells measurements initiation mechanism are demanded to support in 6G NTN.

Figure 2 Near-far effect in TN (left) vs. NTN (right)

Further for connected mode, in addition to unobvious near-far effect, frequent and unavoidable handover (as in figure 3) due to the high speed movement of the satellite is another critical issue, 5G NTN introduced event A4, time-based (CondEvent T1) and location-based (Event D1/D2) for CHO trigger condition. If the time-based and location-based CHO trigger condition are not supported, there would be a problem for the CHO trigger occasion in NTN. Therefore, to better support 6G NTN, both time-based and location-based for CHO trigger condition are indispensable. Then considering GNSS free and GNSS resilient cases, UE may not have available location information or just obtain location information with decreased accuracy, which would lead to that location-based solution for both idle/inactive and connected mode are not feasible or not work well.

Figure 3 Frequent and unavoidable handover in NTN system

Observation 5: Both time-based and location-based neighbour cells measurements initiation as well as CHO trigger condition are indispensable for 6G NTN. Then considering GNSS free and GNSS resilient cases, location-based solution is not feasible or does not work well.

Further, for 6G TN, location-based solution is also useful for some cases, for example, for UEs preparing to enter elevator and underground garage, solely rely on RSRP/RSRQ measurement may not enable timely handover due to weaker signal of the candidate cell, and location-based could be considered for such cases. Network provide reference location information and distance threshold, if the distance between UE and the neighbour cell reference location is smaller than the distance threshold, CHO could be triggered.

Figure 4 Weaker signal of the candidate cell in some TN cases

Observation 6: For 6G TN, location-based solution could also be considered due to weaker signal of the candidate cell in some cases (e.g. UEs preparing to enter elevator and underground garage).

- Multiple SMTCs configuration

As shown in figure 5, the transmission distance difference between serving and neighbor satellites may be too long(e.g. about 0~several hundreds of kilometres for LEO and 0~several thousands of kilometres for GEO) leading to quite large propagation delay difference in NTN scenario. And such issue would result in that the SSB window from one or more neighbour satellite(s) may not fall in the measurement window configured by the serving satellite (as in figure 6). Therefore, to avoid missing the SSB window of neighbour satellites, multiple SMTCs configuration is supported in 5G NTN. Network could configure multiple SMTCs for the UE, and UE can adjust SMTCs based on its location and assistance information in SIB19. It can be seen that this is a crucial enhancements to avoid missing the measurement window of neighbour satellites, which is needed to support in 6G NTN. For 6G TN, the variable propagation delay difference problem between serving and neighbour cell is not exist, therefore multiple SMTCs configuration seems not needed.

Figure 5 Distance difference between two satellites     Figure 6 Mismatch between the measurement configuration corresponding to S1 and SSB from S2

Different UE adaptive SMTC adjustment in idle/inactive mode, in RRC_CONNECTED, network could configure multiple SMTCs based on propagation delay difference assistance information reported by UE to avoid missing the SSB/CSI-RS measurement window of neighbour satellites. Likewise the NW-controlled adjustment of SMTCs in connected mode should also be supported in 6G NTN at least. Further, considering that gap enhancements (e.g. gap-less) and SSB design is being discussed by RAN4 and RAN1 respectively, for 6GR measurement configuration, coordination with RAN1/4 should be considered.

Observation 7: Multiple SMTCs configuration with UE adaptive adjustment solutions in idle/inactive mode and with NW-controlled adjustment of SMTCs solutions in connected mode are important for 6G NTN at least. And NTN/TN common design may have some possibility, which need coordination with RAN1/4.

- SMTC enhancements in beam hopping scenario

Moreover, location-based SMTC selection for idle/inactive mode is introduced in R19 to accommodate‌ beam hopping scenario with scattered active cells. Network configure multiple SMTCs, each SMTC is associated with a reference location of the intended neighbor cells that need to be measured by the UE, and UE select the appropriate SMTC based on the distance between UE and the intended neighbor cell corresponding to the reference location information. Then UE assisted SMTC configuration in connected mode in beam hopping scenario is also supported, network configure multiple SMTCs based on the N closest reference locations information reported from UE.

- For 6G NTN, to enhance the coverage, ‌beam hopping is a benefit method, such location-based SMTC selection solution in idle/inactive and UE assisted SMTC configuration in connected mode are necessary for 6G NTN. Similarly, whether such measurements enhancements could be considered together for both NTN and TN needs coordination with RAN1/4.

- In addition, for 6G NTN, DL coverage enhancements due to extended SSB periodicity will be served as a baseline, wherein the maximum of the additional default value (apart from the existing 20ms value) is 160 ms by beam hopping. Moreover, different SSB periodicity can be configured for different beam position in the same satellite cell coverage according to the number of UE or service type in the beam position as shown in Figure 7. For 6G TN, certain network energy saving solutions apply deep sleep to common channels/signals, for example, involving long periodicity SS and on-demand short periodicity SS. Therefore, 6G TN and 6G NTN have the same case with long periodicity SS and short periodicity SS which may impact UE measurement. Similarly, whether such measurements enhancements could be considered together for both NTN and TN needs coordination with RAN1/4.

- Observation 8: Location-based SMTC selection solution in idle/inactive and UE assisted SMTC configuration in connected mode are necessary for 6G NTN at least to support ‌beam hopping. In addition, 6G TN and 6G NTN have the same case with long periodicity SS and short periodicity SS which may impact UE measurement. And whether such measurements enhancements could be considered together for both NTN and TN needs coordination with RAN1/4.

Figure 7  Beam positions with different SSB periodicity

- NTN coverage information broadcast in NTN cell/NTN assistance information provided in TN cell

Considering that NTN has large coverage and there are multiple TN neighbour cells for a NTN cell (as shown in figure 8), to skip unnecessary TN measurement for power consumption reduction, NTN cell could broadcast TN coverage information which consists in a list of geographical TN areas. Further network could provide NTN assistance information to help recelection from TN to NTN. Obviously, the enhancements are beneficial for energy saving and needs to support for 6G NTN-TN reselection.

Figure 8 Multiple TN neighbour cells for a NTN cell

Observation 9: TN coverage information broadcast in NTN cell and NTN assistance information provided in TN cell are beneficial for energy saving and needs to support for 6G NTN-TN reselection.

- PCI unchanged

- In order to reduce the HO signaling overhead due to the frequent (seconds, tens of seconds or hundreds seconds of HO frequency) and unavoidable handover for a large number of UEs, PCI unchanged solution for quasi-Earth fixed scenario is introduced in 5G NTN. Then for 6G NTN, such solution is still necessary for HO signaling overhead reduction. .

Figure 9 PCI unchanged

Further, for TN/NTN integration, NTN-TN seamless handover is an significant feature to discuss, and PCI unchanged solution could be the baseline. Considering that PCI unchanged solution is just supported in quasi-Earth fixed scenario under transparent payload architecture in 5G NTN, it could be further discussed in regenerative payload for intra-NTN and NTN-TN scenarios in 6G.

Figure 10 PCI unchanged for intra-NTN regenerative payload and NTN-TN scenario

Observation 10: PCI unchanged solution is important for 6G NTN and NTN-TN seamless handover.

- RACH-less HO

Moreover, RACH HO and combination with Time-based CHO are also supported in 5G NTN for HO signaling optimization. Considering that RACH-less HO is already supported in 5G TN, therefore it could also be supported for 6G TN. Therefore, RACH-less (C)HO could be common designed for both 6G NTN and TN

Observation 11: RACH-less (C)HO could be common designed for both 6G NTN and TN.

- Multi-orbit mobility

For multi-orbit mobility, there is no dedicated enhancements in 5G. Then for 6GR, multi-dimensional matching of orbital characteristics, positioning requirements, transmission power, and other factors could be considered for multi-orbit mobility. For example, if the service is time-delay insensitive, GEO could be used to perform the data transmission, and once the latency requirements increase, the UE switches to the lower orbit (e.g. LEO).

Figure 11 Multi-orbit mobility

Observation 12: Multi-dimensional matching of orbital characteristics, positioning requirements, transmission power, and other factors could be considered for 6G NTN multi-orbit mobility.

- Whether integration of SIB 19 and SIB 1

In 5G NTN, SIB 19 is introduced to provide satellite assistance information for NTN access. And for 6GR, in order to support TN/NTN integration, maybe the dedicated SIB19 used for NTN access could be designed together with SIB1. With such system information integration mechanism, once UE obtain the SIB1 which including both access information for TN and NTN access, it could access the network directly based on the network scenario where the UE is located without waiting for another SIB19. However, whether to design a unified minimum SI for both NTN an TN or separate SIs depends on the study on the size limitation of SIB1 in RAN2 and RAN1. Therefore whether integration of SIB 19 and SIB 1 could be discussed for 6GR.

Observation 13: RAN2 needs study whether to design a unified minimum SI for both NTN an TN or separate SIs considering the size limitation of a given SIB.

### 2.2.3 Other enhancements analysis

- Network verified UE location

To be able to check the UE reported location, 5G NTN introduced network verified UE location feature in a single satellite in view. And specify NTN specific enhancements based on the multi-RTT positioning mechanism. Further resolve the mirror point ambiguity issue via properly configuring neighbor cell measurement. Then there is a ongoing SI: Study on GNSS resilient NR-NTN operation discussed in RAN1. Meanwhile, for 6G NTN, reducing the reliance on GNSS such as GNSS resilient or even GNSS free needs to be considered as well. Considering that multi-RTT and other positioning solutions are already supported in TN network, for 6GR, positioning and possible enhancements could be considered for both NTN and TN. However, UE with GNSS may be a common case in usual, whether to introduce such positioning and possible enhancements in 6G day1 needs further discussion.

Figure 12 Network verified UE location in a single satellite in view

Observation 14: Positioning and possible enhancements could be considered for both 6G NTN and TN And further discussion is needed to determine whether to introduce such positioning and possible enhancements in 6G day1.

- MBS broadcast

5G NTN supports MBS broadcast feature, which provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. And introduce some enhancement, i.e. specify SIB signaling to indicate the intended service area information, due to that some NTN cases the intended service area is expected to be smaller than the coverage of a Uu cell. Then both MBS broadcast and multicast are supported in 5G TN, for 6GR, MBS broadcast as well as multicast could be considered for NTN and TN together to better enable MBS feature everywhere.

Figure 13 Intended service area information indication in NTN scenario

Observation 15: For 6GR, MBS broadcast as well as multicast could be considered for NTN and TN together to better enable MBS feature everywhere.

Based on the above analysis, regarding TN/NTN common design feature and NTN specific feature for 6GR, we provide the following 2 proposals respectively.

Proposal 1: For 6G TN/NTN common design, the following features could be considered in Day1:

- User plane

- DL HARQ feedback disabled and UL HARQ modes for NTN and some services in TN, e.g., XR, MBS

- UP timers value range configuration

- Control plane

- TN coverage information broadcast in NTN cell for 6G NTN-TN mobility, NTN assistance information provided in TN cell for 6G TN-NTN reselection

- Location-based CHO trigger condition (impacts of GNSS free and GNSS resilient may be needed to consider) for NTN and for TN use cases, e.g., entering elevator and underground garage

- Coordination with RAN1/4 is needed for possible measurements configuration common design

- PCI unchanged solution for 6G intra-NTN, NTN-TN, intra-TN handover

- RACH-less (C)HO

- Whether to design a unified minimum SI for both NTN an TN (i.e., integration of SIB 19 and SIB 1) or separate SIs considering the size limitation of a given SIB

- Other

- MBS broadcast and multicast

- Positioning and possible enhancements (whether support in 6G day1 needs further discussion)

Proposal 2: 6G NTN needs to support the following NTN specific features:

- User plane:

- TA pre-compensation enhancements

- Using UE-gNB RTT as the offset to start some UP timers

- Control plane:

- Hard and soft TAC update

- Time-based neighbour cells measurements initiation and time based CHO trigger condition

- Multiple SMTCs configuration for both idle/inactive and connected mode

- Location-based SMTC selection for idle/inactive mode and UE assisted SMTC configuration for connected mode for beam hopping scenario

- Multi-orbit mobility

Conclusion

Based on the discussions mentioned above, in this contribution we provide some considerations on TN/NTN integration for 6GR and have the following observations and proposals:

Observation 1: Due to longer propagation delay in NTN, the TA pre-compensation enhancements essential for 6G NTN, but not for 6G TN.

Observation 2: For transmission delay reduction, HARQ feedback disabled for DL and HARQ modes

mechanism for UL could be supported for both 6G NTN and some services in TN, e.g., XR, MBS.

Observation 3: UP timers value range configuration could be designed commonly for both 6G TN and NTN while offset UE-gNB RTT to start some UP timers only for 6G NTN.

Observation 4: For Idle/inactive mode, both hard and soft TAC update mechanism could be supported for 6G NTN but not for 6G TN network, due to that there is no so larger cell coverage and high speed network node.

Observation 5: Both time-based and location-based neighbour cells measurements initiation as well as CHO trigger condition are indispensable for 6G NTN. Then considering GNSS free and GNSS resilient cases, location-based solution is not feasible or does not work well.

Observation 6: For 6G TN, location-based solution could also be considered due to weaker signal of the candidate cell in some cases (e.g. UEs preparing to enter elevator and underground garage).

Observation 7: Multiple SMTCs configuration with UE adaptive adjustment solutions in idle/inactive mode and with NW-controlled adjustment of SMTCs solutions in connected mode are important for 6G NTN at least. And NTN/TN common design may have some possibility, which need coordination with RAN1/4.

- Observation 8: Location-based SMTC selection solution in idle/inactive and UE assisted SMTC configuration in connected mode are necessary for 6G NTN at least to support ‌beam hopping. In addition,6G TN and 6G NTN have the same case with long periodicity SS and short periodicity SS which may impact UE measurement. And whether such measurements enhancements could be considered together for both NTN and TN needs coordination with RAN1/4.

Observation 9: TN coverage information broadcast in NTN cell and NTN assistance information provided in TN cell are beneficial for energy saving and needs to support for 6G NTN-TN reselection.

Observation 10: PCI unchanged solution is important for 6G NTN and NTN-TN seamless handover.

Observation 11: RACH-less (C)HO could be common designed for both 6G NTN and TN.

Observation 12: Multi-dimensional matching of orbital characteristics, positioning requirements, transmission power, and other factors could be considered for 6G NTN multi-orbit mobility.

Observation 13: RAN2 needs study whether to design a unified minimum SI for both NTN an TN or seperate SIs considering the size limitation of a given SIB.

Observation 14: Positioning and possible enhancements could be considered for both 6G NTN and TN And further discussion is needed to determine whether to introduce such positioning and possible enhancements in 6G day1.

Observation 15: For 6GR, MBS broadcast as well as multicast could be considered for NTN and TN together to better enable MBS feature everywhere.

Proposal 1: For 6G TN/NTN common design, the following features could be considered in Day1:

- User plane

- DL HARQ feedback disabled and UL HARQ modes for NTN and some services in TN, e.g., XR, MBS

- UP timers value range configuration

- Control plane

- TN coverage information broadcast in NTN cell for 6G NTN-TN mobility, NTN assistance information provided in TN cell for 6G TN-NTN reselection

- Location-based CHO trigger condition (impacts of GNSS free and GNSS resilient may be needed to consider) for NTN and for TN use cases, e.g., entering elevator and underground garage

- Coordination with RAN1/4 is needed for possible measurements configuration common design

- PCI unchanged solution for 6G intra-NTN, NTN-TN, intra-TN handover

- RACH-less (C)HO

- Whether to design a unified minimum SI for both NTN an TN (i.e., integration of SIB 19 and SIB 1) or separate SIs considering the size limitation of a given SIB

- Other

- MBS broadcast and multicast

- Positioning and possible enhancements (whether support in 6G day1 needs further discussion)

Proposal 2: 6G NTN needs to support the following NTN specific features:

- User plane:

- TA pre-compensation enhancements

- Using UE-gNB RTT as the offset to start some UP timers

- Control plane:

- Hard and soft TAC update

- Time-based neighbour cells measurements initiation and time based CHO trigger condition

- Multiple SMTCs configuration for both idle/inactive and connected mode

- Location-based SMTC selection for idle/inactive mode and UE assisted SMTC configuration for connected mode for beam hopping scenario

- Multi-orbit mobility

Reference

- RP-251881_6G_WG_SID

- 3GPP TR 38.914 V0.2.0 Study on 6G Scenarios and Requirements; (Release 20)

- R2_131bis_ChairNotes_25-10-17_17-00_final