3GPP TSG-RAN WG2 Meeting #132	R2-2508842

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	9.3.2

Source:	CMCC

Title:	Discussion on UE sided RRM measurement prediction

WID/SID:				NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

Based on WID [1], the following objectives are associated to RRM measurement prediction:

- Specify support for RRM measurement prediction for both UE and network sided models [RAN2]:

- Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction

- Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction

- At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported

Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI.

Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY)

- Inference configuration

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline

Note 3: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation.

- For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]:

- Network sided data collection, at least

    - UE is configured to log RRM measurement results.

    - UE supports periodical logging and L3 event triggered logging.

    - UE can indicate logged data availability.

Note 4: The above objectives to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

This contribution will focus on UE capability, applicability reporting procedure, inference configuration/reporting for UE sided RRM measurement prediction.

# 2 Discussion

Regarding to RRM measurement prediction, the following description about the LCM functionality management were captured in TR 38.744 [2].

  - Only functionality-based LCM is considered.

  - Scenarios including intra-frequency temporal domain case A, intra-frequency temporal domain case B, intra-frequency spatial domain prediction and inter-frequency prediction are considered.

  - Both L3 cell level prediction and L3 beam level prediction are considered.

  - RRM measurement prediction can be performed using either UE-sided model or network-sided model.

The applicability reporting procedure for AI/ML based beam management is specified in section 22.2.3 of TS 38.300 and illustrated as in the following Figure [3], and RAN2 agreed to use the similar procedure as baseline for AI Mob including RRM measurement prediction and measurement event prediction. We could take this procedure as the baseline for the discussion on LCM functionality management for UE sided RRM measurement prediction in WI phase.

Fig 1. Initial applicability and applicability status change reporting

2.1 UE capability

For AI/ML for NR air interface, the legacy UE capability framework serves as the baseline to report UE’s supported AI/ML-enabled Feature/FG. We think the legacy UE capability framework could also be reused for AI/ML for Mobility. Therefore, for RRM measurement prediction and Measurement events prediction use cases, this information is indicated in UE AS capability in RRC (e.g., UECapabilityEnquiry/UECapabilityInformation).

Proposal 1: UE capability framework (i.e. UECapabilityEnquiry/UECapabilityInformation) could be used to report UE’s supported functionalities for UE-sided RRM measurement prediction and measurement event prediction.

During the discussion in R19 AI/ML PHY, RAN1 determined the following feature groups (i.e. supported functionalities) for beam management use case [4], and further defined the components for each feature group, including inference related parameters (e.g. RS type and maximum number of Set A, Set B), and reporting related parameters (e.g. support predicted beam index, predicted RSRP), etc..

  - UE-side beam prediction for BM Case1 for inference (support of beam prediction with reporting of predicted beam index)

  - UE-side beam prediction for BM Case1 with predicted RSRP for inference (support of beam prediction with reporting of predicted beam index and predicted RSRP)

  - UE-side beam prediction for BM Case2 for inference (support of beam prediction with reporting of predicted beam index)

  - UE-side beam prediction for BM Case2 with predicted RSRP for inference (support of beam prediction with reporting of predicted beam index + predicted RSRP)

It can be observed that the granularity of supported functionality is dependent on the inference configuration and reporting. Therefore, we think the output data of UE sided model and the scenarios identified for RRM measurement prediction could be considered for the definition of functionalities for RRM measurement prediction. Here is an example for the AI/ML functionalities for UE-sided RRM measurement prediction:

| Output data | Scenarios |
| --- | --- |
| predicted L3 cell level results | intra-frequency temporal domain case A |
|  | intra-frequency temporal domain case B |
|  | inter-frequency prediction |
| predicted L3 filtered beam level results | intra-frequency temporal domain case A |
|  | intra-frequency temporal domain case B |
|  | intra-frequency spatial domain prediction |
|  | inter-frequency prediction |

Proposal 2: The output data of AI/ML model and the scenarios should be considered in the definition of functionalities for RRM measurement prediction. Here is an example for the AI/ML functionalities for UE-sided RRM measurement prediction:

| Output data | Scenarios |
| --- | --- |
| predicted L3 cell level results | intra-frequency temporal domain case A |
|  | intra-frequency temporal domain case B |
|  | inter-frequency prediction |
| predicted L3 filtered beam level results | intra-frequency temporal domain case A |
|  | intra-frequency temporal domain case B |
|  | intra-frequency spatial domain prediction |
|  | inter-frequency prediction |

During the study phase in Rel-19, RAN2 agreed that UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters in a RRCReconfiguration message. It is the same as the mechanism of AI/ML for NR air interface. Regarding to AI/ML for NR air interface, the fields applicabilityReportingCSI-r19 and applicabilityReportingOther-r19 [5] are introduced as part of UE capabilities associated to applicability reporting for AI-based beam management and CSI prediction in R19. These two fields are used to indicate whether the UE supports applicability reporting and/or its updates based on the full inference configuration and partial inference configuration, respectively. We propose to introduce the similar parameters into UE capabilities for UE-sided RRM measurement prediction.

Proposal 3: The two new UE capability parameters similar to applicabilityReportingCSI-r19 and applicabilityReportingOther-r19 could be introduced into UE capabilities for UE-sided RRM measurement prediction, to indicate whether the UE supports applicability reporting based on inference configuration or inference related parameters.

2.2 Applicability procedure

In Rel-19, the following statement on the procedure for applicability reporting of UE sided RRM measurement prediction is captured in [2]:

UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters defined in section 6.1.2.1.2 in a RRCReconfiguration message.

Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message. If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to release it. When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported. For an inapplicable full inference configuration, UE continues to perform the inference. It is up to network implementation what to do with reported result after UE indicates the corresponding full inference configuration is inapplicable. If a full inference configuration is applicable, UE applies it. No dynamic lower layer signalling is needed to active a full inference configuration.

Applicability can be updated via UAI. A flag in OtherConfig is introduced to indicate whether applicability reporting via UAI is enabled or not. When an inference configuration becomes inapplicable UE shall report its inapplicability via UAI.

Considering that the above procedure for applicability reporting is the same as that of AI/ML for NR air interface specified in [3], we propose to take the applicability reporting framework of AI/ML for the NR air interface as the baseline for UE-side RRM measurement prediction and introduce the necessary IEs for UE-side RRM measurement prediction at stage 3.

Observation 1: The procedure for applicability reporting of UE sided RRM measurement prediction captured in 38.744 is the same as that of AI/ML for NR air interface specified in 38.300.

Proposal 4: RAN2 to take the applicability reporting framework of AI/ML for the NR air interface as the baseline for UE-side RRM measurement prediction and introduce the necessary fields/IEs for UE-side RRM measurement prediction at stage 3.

2.3 Inference configuration and reporting

In Rel-19, RAN2 agreed that the existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report. And the following inference parameters can be configured to UE for inference and assessing applicability [2]:

  - PW length for intra-frequency temporal domain case A;

  - Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction;

  - Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement);

  - Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available);

  - Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction;

  - Optional associated ID.

For RRM measurement prediction with UE sided model, besides the above potential inference parameters that are used for UE to examine the applicability, we think RAN2 should further consider whether the inference input and output related information needed for UE to examine the applicability. Because the input data is related to the measurement/inference configuration from the network; and the network could determine which type of prediction information (the output data) to use based on its own needs.

Observation 2: The input data is related to the measurement/inference configuration from the network. Furthermore, the network could determine which type of prediction information (the output data) to use based on its own needs.

Proposal 5: RAN2 to further consider whether the inference input and output related information needed for UE to examine the applicability, for example:

  - Inference output related info: predicted L3 cell level results or predicted L3 filtered beam level results

  - Inference input related info: L3 cell-level measurement results, L1 beam-level measurement results and L3 beam-level measurement results

In Rel-19, RAN2 discussed on the condition when to perform the inference for UE sided RRM measurement prediction and agreed that the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements for RRM measurement prediction with UE sided model. But there was no consensus on the details. From our perspective, for temporal domain case A, the inference could not provide additional gain for the UE in cell centre and it is not friendly for UE complexity and power saving. On the other hand, when the UE approaches cell edge with a high probability of handover, the functionality should be activated and start performing inference quickly to provide the assistance information for handover decision.

Observation 3: For temporal domain case A, the inference could not provide additional gain for the UE in cell centre and it is not friendly for UE complexity and power saving. When the UE approaches cell edge with a high probability of handover, the functionality should be activated and start performing inference quickly to provide the assistance information for handover decision.

Therefore, we think condition information/event triggers are needed for AI Mob since it could make UE sided model start/stop performing inference more quickly based on UE status, that is beneficial to improve HO performance and reduce UE complexity.

Observation 4: Condition information/event triggers could make UE sided model start/stop performing inference more quickly based on UE status, at least for temporal domain case A, that is beneficial to improve HO performance and reduce UE complexity.

Proposal 6: RAN2 to consider supporting condition information/event triggers provided by the network to UE, which are used to activate/deactivate UE sided model or trigger UE to start/stop performing inference. Once the prediction is generated, it could be reported to the network.

We think the following potential condition information could be considered as part of inference configuration or inference related information, which is used for event triggers to trigger the UE sided model to perform inference and/or report RRM measurement prediction, at least for Temporal domain case A:

  - The threshold of actual RRM measurement results, e.g., RSRP, it could be used to examine whether UE is near the cell edge and has high probability of handover;

  - The threshold of UE speed; When UE’s mobility is high, more unintended events e.g., handover failure, radio link failure etc. may occur. In this case, UE sided model inference could provide assistant information to network to avoid the unintended events.

Proposal 7: The following potential condition information could be considered as part of inference configuration or inference related information, which is used for event triggers to trigger the UE sided model to perform inference and/or report RRM measurement prediction, at least for Temporal domain case A:

  - The threshold of actual RRM measurement results

  - The threshold of UE speed

In Rel-19, RAN2 discussed the reporting content of UE sided RRM measurement prediction, and the following texts are captured in [2]:

For each reported cell in the measurementReport message:

-	For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported;

-	For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported;

-	For inter-frequency prediction, the latest predicted measurement result is reported.

If it is necessary to distinguish the predicted measurement results from the actual measurement results, a new separate IE or indicator for the predicted measurement results could be introduced in the measurementReport message.

Proposal 8: If it is necessary to distinguish the predicted measurement results from the actual measurement results in the measurementReport message, a new separate IE or indicator for the predicted measurement results could be introduced in the measurementReport message.

# 3 Conclusion

In this paper, we have the following observations and proposals on UE capability, applicability reporting procedure, inference configuration/reporting for UE sided RRM measurement prediction.

UE capability

Proposal 1: UE capability framework (i.e. UECapabilityEnquiry/UECapabilityInformation) could be used to report UE’s supported functionalities for UE-sided RRM measurement prediction and measurement event prediction.

Proposal 2: The output data of AI/ML model and the scenarios should be considered in the definition of functionalities for RRM measurement prediction. Here is an example for the AI/ML functionalities for UE-sided RRM measurement prediction:

| Output data | Scenarios |
| --- | --- |
| predicted L3 cell level results | intra-frequency temporal domain case A |
|  | intra-frequency temporal domain case B |
|  | inter-frequency prediction |
| predicted L3 filtered beam level results | intra-frequency temporal domain case A |
|  | intra-frequency temporal domain case B |
|  | intra-frequency spatial domain prediction |
|  | inter-frequency prediction |

Proposal 3: The two new UE capability parameters similar to applicabilityReportingCSI-r19 and applicabilityReportingOther-r19 could be introduced into UE capabilities for UE-sided RRM measurement prediction, to indicate whether the UE supports applicability reporting based on inference configuration or inference related parameters.

Applicability procedure

Observation 1: The procedure for applicability reporting of UE sided RRM measurement prediction captured in 38.744 is the same as that of AI/ML for NR air interface specified in 38.300.

Proposal 4: RAN2 to take the applicability reporting framework of AI/ML for the NR air interface as the baseline for UE-side RRM measurement prediction and introduce the necessary fields/IEs for UE-side RRM measurement prediction at stage 3.

Inference configuration and reporting

Observation 2: The input data is related to the measurement/inference configuration from the network. Furthermore, the network could determine which type of prediction information (the output data) to use based on its own needs.

Proposal 5: RAN2 to further consider whether the inference input and output related information needed for UE to examine the applicability, for example:

  - Inference output related info: predicted L3 cell level results or predicted L3 filtered beam level results

  - Inference input related info: L3 cell-level measurement results, L1 beam-level measurement results and L3 beam-level measurement results

Observation 3: For temporal domain case A, the inference could not provide additional gain for the UE in cell centre and it is not friendly for UE complexity and power saving. When the UE approaches cell edge with a high probability of handover, the functionality should be activated and start performing inference quickly to provide the assistance information for handover decision.

Observation 4: Condition information/event triggers could make UE sided model start/stop performing inference more quickly based on UE status, at least for temporal domain case A, that is beneficial to improve HO performance and reduce UE complexity.

Proposal 6: RAN2 to consider supporting condition information/event triggers provided by the network to UE, which are used to activate/deactivate UE sided model or trigger UE to start/stop performing inference. Once the prediction is generated, it could be reported to the network.

Proposal 7: The following potential condition information could be considered as part of inference configuration or inference related information, which is used for event triggers to trigger the UE sided model to perform inference and/or report RRM measurement prediction, at least for Temporal domain case A:

  - The threshold of actual RRM measurement results

  - The threshold of UE speed

Proposal 8: If it is necessary to distinguish the predicted measurement results from the actual measurement results in the measurementReport message, a new separate IE or indicator for the predicted measurement results could be introduced in the measurementReport message.

# Reference

- RP-251864 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR

- TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2

- R1-2506627 Updated RAN1 UE features list for Rel-19 NR after RAN1 #122

- TS 38.331, NR; Radio Resource Control (RRC); Protocol specification

- TS 38.306, NR; User Equipment (UE) radio access capabilities