3GPP TSG-RAN WG2#132		      					R2-2508849

Dallas, USA, November 17 – 21, 2025

Agenda item:	10.4

Source: 	Kyocera

Title: 	Consideration for 6G connected mode mobility

Document for:	Discussion

Introduction

The following statements for 6G mobility were agreed in the RAN2#131bis meeting [1].

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

Based on the agreements, further considerations on intra-RAT connected mode mobility for 6G study are provided in this paper.

Discussion

Minimization of interruption time

In RAN2#131bis, it was agreed to study the minimization of interruption time and to ensure service continuity as one of the requirements of the 6G mobility design [1]. In 5G, the extremely stringent requirement of 0ms mobility interruption time was defined [8]. To meet this requirement, DAPS (Dual Active Protocol Stack) Handover was specified as additional connected mode mobility procedure in Rel-16. However, due to its high complexity, there were challenges in being deployed in commercial networks. In addition, LTM (L1/L2-triggered mobility) was specified in Rel-18 to reduce interruption time; nevertheless, the legacy L3 handover has continued to be the primary approach in practice. For 6G, it is necessary to study the connected mode mobility procedures with minimum U-plane interruption time, taking into account the balance between the complexity and the performance gain provided by candidate solutions.

Table 1 shows the components of mobility latency derived from TS38.133, section 6.1.1.2 “NR FR1-NR FR1 Handover” [4]. Note that the values on each component are examples with certain assumptions. Based on these values, the interruption times for L3 Handover and RACH-less LTM can be calculated. The interruption time refers, according to TS38.300 Annex G [2], as the time period from the UE receives the handover command/cell switch command to the UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

Table 1　Components of Mobility Latency

| Components | [ms] | Remarks |
| --- | --- | --- |
| TRRC | 10 | Processing time for RRCReconfiguration carrying candidate configurations [7] |
| TMAC | over 3 | Time for processing the LTM cell switch command MAC CE: THARQ + 3ms |
| Tprocessing | 20 | Time for UE processing |
| TLTM-RRC-processing | 10 | Time for ASN.1 decoding and validity/compliance check for the RRC configuration of the target cell |
| TLTM-processing | 20 | Time for UE processing, consisting of applying the target cell parameters and L1/L2 change |
| Tsearch | 0 | Time required to search the target cell: Target cell is known |
| TΔ ( = Trs) | (5) | Time for fine time tracking and acquiring full timing information of the target cell: SSB transmission periodicity = Assume 5ms |
| Tmargin | 2 | Time for SSB processing |
| TIU | 20 | Time until RA transmission. Delay for first available PRACH occasion; TS38.213, Table 8.1-1 plus 10ms [5] |
| TRAR | 1~10 | Time until RAR reception: the network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum [7] |
| Tfirst-data | (4) | Time for UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell, after RAR: Assume 4slots |

Figure 1 shows the mobility latency for L3 Handover. For L3 Handover, after the target cell has been determined, the NW sends the handover command (RRC Reconfiguration) to the UE. The transmission time (TRRC) is not included in the interruption time. Upon reception of the handover command, the UE detaches from the source cell, applies the provided configuration and performs the preparation to access the target cell (Tprocessing).

The UE performs DL synchronization with the target cell. This process involves the time for fine time tracking and acquiring full timing information of the target cell (TΔ) and the time for SSB processing (Tmargin), with a total duration of approximately 7ms. In this paper, TΔ is assumed to be approximately 5ms, based on the SSB transmission periodicity. For Handover scenarios, since the target cell has already been determined, the target cell search time (Tsearch) is considered to be 0ms.

As the final step, the UE performs UL synchronization with the target cell. The time until the UE transmits the RA preamble (TIU) is assumed as 20ms, according to TS38,213 Table 8.1-1 [7] plus 10ms, whereby It should be noted that the exact time to transmit the RA preamble depends on the situation. Furthermore, the time to receive the Random Access Response (RAR) is approximately 1 to 10ms, as specified in TS38.331 [7]. Consequently, the total time required for UL synchronization is approximately 21 to 30ms.

After completing DL and UL synchronizations, the UE performs its first DL/UL reception and transmission on the indicated beam of the target cell. For the purposes of this estimation, Tfirst-data is assumed as 4 slots, which is 4ms. Therefore, the interruption time for L3 Handover (Tinterruption) is estimated to be at least approximately 51ms.

Figure 1 Mobility Latency for L3 Handover

Figure 2 shows the mobility latency for RACH-less LTM. In LTM, the NW configures one or more the LTM candidate configurations to the UE prior to the cell switch execution (i.e., pre-configuration). After the UE receives the LTM candidate configurations, the UE performs DL and UL synchronization. Since DL and UL synchronization are performed before the cell switch execution, this is called Early DL/UL synchronization. For LTM, the LTM Cell Switch Command MAC CE (LTM CSC MAC CE) provides the TA value for the target cell, so in Early UL synchronization the UE does not need to wait for the RAR after the preamble transmission triggered by PDCCH-order.

Subsequently, based on the measurement report from the UE, the NW triggers the UE to execute the LTM cell switch by the LTM CSC MAC CE. The UE processes the received LTM CSC MAC CE and initiates to access to the target cell (TMAC). The UE decodes the ASN.1 and checks the validity/compliance for the RRC configuration of the target cell (TLTM-RRC-processing). Then, the UE applies the target cell parameters and performs the L1/L2 change (TLTM-processing).

Since the UE has already completed DL/UL synchronization in advance, it immediately performs DL/UL reception and transmission on the indicated beam of the target cell just after the completion of the UE reconfiguration (Tfirst-data). Therefore, the interruption time for RACH-less LTM (TLTM-interruption) is estimated to be at least approximately 34ms.

In addition, if the UE supports the ltm-FastProcessingConfig-r18 capability [3], it can perform pre-processing of the LTM candidate configurations before receiving the LTM CSC MAC CE. Therefore, TLTM-RRC-processing component of the interruption time can be considered as 0ms, which allows the interruption time (TLTM-interruption) to be further reduced to approximately 24ms.

Figure 2 Mobility Latency for RACH-less LTM

Based on the above, in LTM, the Early DL/UL synchronization, which completes the DL and UL synchronizations prior to the cell switch, contributes to reduce the interruption time approximately 27ms. In addition, depending on the UE’s capability, the pre-configuration allows the UE to perform the pre-processing of the LTM candidate configuration before reception of the LTM CSC MAC CE, thereby enabling the additional reduction of approximately 10ms in interruption time.

In total, RACH-less LTM can reduce the interruption time by approximately 37ms compared to L3 Handover. Therefore, the LTM-specific features of Early DL/UL synchronization and pre-processing enabled by LTM candidate cell pre-configuration are effective in reducing the interruption time.

RAN2 should confirm the Early DL/UL synchronization including RACH-less mobility and pre-processing as effective to reduce the interruption time.

RAN2 should discuss whether the 24ms interruption time is sufficient as the “minimization of interruption time” in terms of service continuity.

Robustness of mobility procedures

In RAN2#131bis, it was agreed to study mobility robustness as the second requirement of the 6G mobility design [1]. In 5G, to improve mobility robustness, RAN2 introduced Conditional Handover (CHO) in Rel-16, LTM in Rel-18, and Conditional LTM (CLTM) in Rel-19.

Layer 3 signalling vs Layer 1/Layer 2 signalling

In L3 Handover, the NW decides the handover execution based on the L3 measurement reports from the UE and indicates the handover command via L3 signalling. However, L3 signalling is relatively slow, so it cannot keep up with rapidly changing radio conditions, which may cause Handover Failure (HOF). This situation could become particularly significant at higher frequency bands (e.g., FR1 upper band, FR3, FR2) and in 3D coverage scenarios (e.g., aerial environments). In LTM introduced in Rel-18, the NW decides the cell switch execution based on either L3 measurement reports or L1 measurement reports, and it indicates the cell switch execution via L2 signalling (i.e., with MAC CE). The use of L1/L2 signalling enables a faster delivery of the handover command and enhances robustness in tracking rapidly changing radio conditions.

RAN2 should confirm that L1/L2 signalling based mobility is more robust than L3 signalling based one, as L2 is quicker compared to L3.

NW-controlled mobility vs UE-based (Conditional) mobility

Rel-16 and Rel-19 introduced the conditional mobility mechanisms, i.e., CHO and CLTM respectively. The conditional mobility eliminates the need for handover command signalling from the NW; instead, the handover is executed based on the measurements and event evaluation by the UE. Since these allow the UE itself to measure the radio condition and decide on the handover execution, the handover can be performed even if the signalling from the NW is no longer received. This approach is particularly effective to avoid HOF caused by “Too late Handover”. Consequently, it can provide higher robustness than LTM, which requires a cell switch command from the NW (i.e., NW-controlled mobility).

RAN2 should confirm that Conditional mobility is more robust than NW-controlled mobility, since it can execute handover stably even if the radio conditions between the UE and the source cell degrade rapidly.

Subsequent mobility

In Rel-18, subsequent mobility is supported for LTM. According to TS38.300 [2], Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without the need to release, reconfigure or add other LTM candidate configurations after each LTM cell switch completion. In other words, the UE continue to store the other LTM candidate configurations even after completing each LTM cell switch. This allows the UE, when a “HO to wrong cell” occurs [14] – meaning immediately after executing a handover to a target cell, triggering another handover to different target cell – if this different target cell is one of the LTM candidate cells, the UE can avoid the risk of Radio Link Failure (RLF) since it keeps the LTM candidate configurations. Moreover, in subsequent mobility, the reconfiguration for each cell switch is unnecessary, thereby it can minimize the cost when a “HO to wrong cell” occurs. As a result, subsequent mobility is considered effective in mitigating the impact of “HO to wrong cell” occurs.

RAN2 should discuss whether subsequent mobility could contribute to robustness in terms of minimizing the impact of “HO to wrong cell” if it occurs.

Fast recovery

When a HOF occurs, the UE initiates the RRC re-establishment procedure. In the fast recovery mechanism, if the cell selected during the cell selection phase of the RRC re-establishment procedure is among the candidate cells pre-configured in the UE, the UE can perform the handover or cell switch execution procedure [2]. This approach helps minimize the impact of the HOF, thereby enhancing robustness. Fast recovery is considered effective for three HOF scenarios: Too early HO, Too late HO, and HO to wrong cell.

RAN2 should discuss whether fast recovery could contribute to robustness in HOF cases, with respect to avoiding RRC Reestablishment for three HOF scenarios (e.g., Too early HO, Too late HO, HO to wrong cell).

Energy efficiency for both UE and NW

As the third requirement for 6G mobility design agreed at RAN2#131bis [1], Energy Efficiency for both the UE and the NW should be considered. In 5G NR, the features for energy efficiency, such as network energy saving (NES) and UE power saving (e.g., LP-WUS) have been investigated to reduce energy/power consumption for the IDLE mode procedures (e.g., cell reselection) and the Connected mode operations (e.g., Carrier Aggregation). On the other hand, the energy efficiency for connected mode mobility has not been well studied so far.

Regarding the energy efficiency for connected mode mobility, the definition and KPI should be considered first as it’s a new concept. For example, if it’s assumed that the energy efficiency is improved by maximizing the spectral efficiency in the handover procedure, the three possible factors can be considered as follows:

The handover procedure should;

- Minimize dedicated resource reservations

  - The target base station needs to reserve the dedicated resources for the UE, when it receives the Handover Request from the source cell. The reserved resource is wasted until the UE accesses the target cell.

  - In general, the resource reservation becomes worse by the pre-configuration, the conditional mobility, the RACH-less mobility and the subsequent mobility, compared to L3 handover (baseline).

- Avoid throughput degradation (esp. just after HO completion)

  - The throughput degradation just after the handover completion happens by a couple of known causes; the improper MCS until the target cell receives the first CSI feedback from the UE, and the deactivations of SCells upon the handover execution for the UE configured with CA.

  - This issue is observed in all the existing handover mechanisms, i.e., L3 handover, LTM and so on. In general, the former cause would be expected to be resolved by the early CSI acquisition which is being discussed for Rel-20 5G-Advaonced. The latter cause may be resolved if the SCells can continue to be activated during the handover execution or if the SCells can be quickly activated after the handover completion.

  - In case of throughput is low, transmitting the same amount of data as in high-throughput conditions requires more time, which can result in increased power consumption. Therefore, throughput degradation occurring with each handover causes higher power consumption, highlighting the need to mitigate throughput loss associated with handover procedure.

- Minimize signalling exchanges between NW and UE

  - The transmission and reception of signalling require power consumption by transceivers at both the NW and the UE. Therefore, from the energy efficiency perspective, it is important to minimize the number of signalling exchanges between the NW and the UE.

  - For instance, compared to NW-controlled mobility, UE-based mobility does not require sending/ receiving measurement reports and the handover command. Therefore, it is considered superior in terms of the signalling count indicator.

The energy efficiency, as interpreted as spectrum efficiency for example, may include: (1) To minimize dedicated resource reservations, (2) To avoid throughput degradation, and (3) To minimize the number of signalling exchanges between the NW and the UE.

However, energy efficiency in mobility is a new concept, RAN2 should have a common understanding. Therefore, RAN2 should discuss and clarify the definition and KPIs of the energy efficiency for connected mode mobility.

RAN2 should discuss and clarify the definition and KPIs of the energy efficiency for connected mode mobility.

Other consideration for connected mode mobility

AI mobility aspects

The AI/ML for mobility was studied in Rel-19, and the normative work to specify the RRM measurement prediction and the measurement event prediction has been approved for Rel-20 5G-Advanced [12]. The study result showed the considerable gains for the reductions of measurement processing and for the improvements of handover performance [13]. In addition, the AI-enabled radio is one of key requirements for 6GR [9]. Therefore, it’s worth considering whether these AI mobility features should be supported in 6G from Day 1.

From the mobility framework point of view, the expected impact to support AI mobility is mainly in the measurement report framework with regards to the RRM measurement prediction and the measurement event prediction, since the other big impact is LCM which should be handled by AI/ML agenda (unseen). It may be further expected that 6GR would support additional use cases, e.g., RLF prediction, AI-based CHO decision, etc., but such new use cases should be discussed later.

On the other hand, the duplicated efforts with Rel-20 5G-Advanced should be avoided. So, RAN2 should consider supporting AI mobility features and wait for 5G-Advanced normative work for now.

AI mobility features, e.g., the RRM measurement prediction and the measurement event prediction, would be considered based on Rel-20 5G-Advanced progress.

MIMO harmonization aspects

The inter-cell multi-TRP operation is supported in Rel-17, which allows the DL transmission from two TRPs belonging to different cells [2]. It may be seen as the Joint Transmission in Coordinated Multi Point transmission (JT-CoMP) in LTE [14]. It may be also seen as a pathway towards the distributed MIMO or the cell-free MIMO operations. From RAN2 point of view, it’s almost seen as the cell switch, if the context transfer to the target cell is added, which, in particular, has commonalities with LTM.

In Rel-19, RAN1 introduced the UE-initiated beam management with the event conditions in PHY layer to trigger the CSI feedback transmissions [15]. On the other hand, RAN2 introduced the event-triggered L1 measurement reporting, whereby the event condition and L1 measurement reporting are defined in MAC layer [11]. These are, of course, for the different purposes (MIMO or LTM), but it’s also true that these functionalities look similar.

Since simplicity is one of the key requirements for 6GR, similar functionalities like these should be harmonized as part of the study, i.e., aiming to integrate these into a single/simple framework. On the other hand, such a harmonized framework may increase the complexity, which is a risk and is not aligned with the requirement for simplicity. So, RAN2 should study whether some MIMO features should be integrated within the mobility framework (or kept independently), involving RAN1.

The mobility functions and the MIMO functions would be harmonized as a unified framework, e.g., with regards to the lower layer measurement report framework and the inter-cell multi-TRP operation.

Conclusion

In this contribution, the initial consideration for supporting 6G mobility are discussed. RAN2 is kindly asked to take into account the observations and proposals below:

Proposal 1	RAN2 should confirm the Early DL/UL synchronization including RACH-less mobility and pre-processing as effective to reduce the interruption time.

Proposal 2	RAN2 should discuss whether the 24ms interruption time is sufficient as the “minimization of interruption time” in terms of service continuity.

Proposal 3	RAN2 should confirm that L1/L2 signalling based mobility is more robust than L3 signalling based one, as L2 is quicker compared to L3.

Proposal 4	RAN2 should confirm that Conditional mobility is more robust than NW-controlled mobility, since it can execute handover stably even if the radio conditions between the UE and the source cell degrade rapidly.

Proposal 5	RAN2 should discuss whether subsequent mobility could contribute to robustness in terms of minimizing the impact of “HO to wrong cell” if it occurs.

Proposal 6	RAN2 should discuss whether fast recovery could contribute to robustness in HOF cases, with respect to avoiding RRC Reestablishment for three HOF scenarios (e.g., Too early HO, Too late HO, HO to wrong cell).

Observation 1	The energy efficiency, as interpreted as spectrum efficiency for example, may include: (1) To minimize dedicated resource reservations, (2) To avoid throughput degradation, and (3) To minimize the number of signalling exchanges between the NW and the UE.

Proposal 7	RAN2 should discuss and clarify the definition and KPIs of the energy efficiency for connected mode mobility.

Observation 2	AI mobility features, e.g., the RRM measurement prediction and the measurement event prediction, would be considered based on Rel-20 5G-Advanced progress.

Observation 3	The mobility functions and the MIMO functions would be harmonized as a unified framework, e.g., with regards to the lower layer measurement report framework and the inter-cell multi-TRP operation.

References

- “Chair’s notes”, RAN2 Chair (InterDigital), RAN2#131bis

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- TS38.306, “User Equipment (UE) radio access capabilities”, 3GPP

- TS38.133, “Requirements for support of radio resource management“, 3GPP

- TS38.213, “Physical layer procedures for control“, 3GPP

- TS38.321, “Medium Access Control (MAC) protocol specification”, 3GPP

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP

- TR38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPP

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), RAN#108

- RP-251152, “Correction on S-CPAC and L3 measurements-based LTM”, RAN3, RAN#108

- RP-252777, “NR mobility enhancements Phase 4”, RAN2, RAN#109

- RP-251864, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, CATT (Moderator), RAN#108

- RP‑252876, “TR 38.744 v2.1.0 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, Guangdong OPPO Mobile Telecom., RAN#109

- TS36.300, “E-UTRA and E-UTRAN Overall description; Stage 2”, 3GPP

- RP‑251580, “Introduction of Rel-19 NR MIMO Phase 5”, RAN1, RAN#108