3GPP TSG-RAN WG2 Meeting #132	R2-2508855

Dallas, USA, 17th - 21th November 2025

Agenda Item:	8.13.2

Source: 	Huawei, HiSilicon

Title: 	Remaining issues for Multi-hop Relay

Document for:	Discussion and Decision

# Introduction

RAN2#131 meeting agreed to complete the Rel-19 WI for the multi-hop relay.

Rel-19 relay WI is complete from RAN2 point of view (pending closure of the CR email discussions).

This paper discusses remaining issues that need to be addressed to ensure reliable operation of Rel-19 multi-hop relay. The details are provided below.

# Discussion

## 2.1 Remaining issues for Discovery

### 2.1.1 Discovery Model B with no PC5 Link in multi hop scenario

In the previous meeting there were discussions on Discovery Model B, specifically for scenarios where no PC5 link is established between a parent Relay UE and the Remote UE. It was agreed that in Discovery Model B, when there is no established PC5 link between the Remote UE and the Intermediate Relay UE, the Intermediate Relay UE should forward the solicitation message only if the PC5 RSRP between the Remote UE (or First Relay UE) and the Intermediate Relay UE is above a defined threshold.

One possible scenario involves a UE that is capable of functioning as an intermediate relay UE (i.e., it supports the Rel-19 multi-hop relay feature through the path gNB <-> relay UE2 <-> the current intermediate relay UE as shown in Figure 2), but is not currently operating as a relay in a multi-hop configuration (i.e., it is now connected through the single-hop relay UE, the relay UE1, as illustrated in Figure 2). In this figure, the UE is capable of acting as an intermediate relay UE but is currently connected to a Rel-17 single-hop relay. In such a scenario, if a multi-hop remote UE attempts to discover a valid multi-hop path, the intermediate relay UE can participate in Model B discovery by forwarding the discovery message by evaluate the AS conditions [1]. This enables the remote UE to identify potential relay paths.

Figure 1. Intermediate Relay UE participating in Model B multi-hop relay discovery.

Observation 1: The scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, and the intermediate relay UE sends/forwards the discovery solicitation message received from a nearby Remote UE before establishing itself as a Remote UE could happen when A UE that is capable of functioning as an intermediate relay UE, but not currently operating as a relay in a multi-hop configuration, participates in Model B discovery by forwarding the discovery Solicitation message.

In above case in Figure 1, we should allow the remote UE to select the multi-hop relaying path through the candidate intermediate relay UE, since the remote UE may not find any other available candidate path. In this case, when the remote UE selects a path through an intermediate relay UE, the intermediate relay UE should be allowed to change the path form the single-hop relaying to multi-hop relaying.

Proposal 1: When the remote UE selects a path through an intermediate relay UE, the intermediate relay UE should be allowed to release the legacy single-hop relaying path and establish the candidate multi-hop relaying path, as indicate in the text proposal in the Annex 1.

### 2.1.2 PC5 Link information in multi hop scenario

Figure 2. Path selection for a remote UE among multiple candidate paths.

As shown in Figure 2, a remote UE may discover several candidate paths, in which some can be established while other may not. Based on current information in the discovery message, the remote UE can determines the hop number and the UEs information along the path, but have no idea whether the path is already established or not.

- Information contained in the discovery message [2]

------------------------------------------------------------ TS 23.304 starts---------------------------------------------------------

##### 6.3.2.5.2	Procedure for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model A

Figure 6.3.2.5.2-1 presents the procedures for 5G ProSe Multi-hop UE-to-Network Relay Discovery using Model A discovery.

<omitted>

3.	The 5G ProSe Intermediate UE-to-Network Relay creates a corresponding discovery entry based on the received 5G ProSe UE-to-Network Relay Discovery Announcement message, if the discovery message meets the AS layer criteria and the Hop-Count is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

The discovery entry includes the following information:	RSC, (optional) Root Relay Info, Announcer Info, Hop-Count, (optional) Accumulated QoS for PC5 link, (optional) NCGI, (optional) RRC Container, (optional) Hop-Limit.

The Announcer Info is set to the Announcer Info of the received message. The Root Relay Info is set to the Root Relay Info of the reived message. The Hop-count is set to the value of the received message. The Accumulated QoS for PC5 link, if present in the received discovery message, will be updated based on the QoS of the PC5 link between the 5G ProSe Intermediate UE-to-Network Relay and the sender of the message (e.g. sum of delay of each hop).

------------------------------------------------------------ TS 23.304 ends---------------------------------------------------------

Observation 2: Based on the current information in the discovery message, the remote UE does not know whether the candidate path is already established or not.

Based on the above observation, the remote UE may select a path which is not established yet even there is a better one that is already established. The access time to the network can be much longer if the remote UE select a path that is not established yet. Therefore, we think it is better for the remote UE to be aware that whether the candidate path is established or not. One good way is to contain an indication in the discovery message (e.g., in the RRC container) form the candidate intermediate relay UE to the remote UE to indicate that the path is established.

Proposal 2: The intermediate relay UE can provide an indication in the Discovery Message RRC Container to indicate whether the PC5 path with parent is established, so that the remote UE can prioritize a candidate path that is already established and access the network faster.

# Conclusion

In this paper, we discuss the remaining issues and have the following observation and proposals.

Discovery Model B with no PC5 Link in multi hop scenario

Observation 1: The scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, and the intermediate relay UE sends/forwards the discovery solicitation message received from a nearby Remote UE before establishing itself as a Remote UE could happen when A UE that is capable of functioning as an intermediate relay UE, but not currently operating as a relay in a multi-hop configuration, participates in Model B discovery by forwarding the discovery Solicitation message.

Proposal 1: When the remote UE selects a path through an intermediate relay UE, the intermediate relay UE should be allowed to release the legacy single-hop relaying path and establish the candidate multi-hop relaying path, as indicate in the text proposal in the Annex 1.

Observation 2: Based on the current information in the discovery message, the remote UE does not know whether the candidate path is already established or not.

Proposal 2: The intermediate relay UE can provide an indication in the Discovery Message RRC Container to indicate whether the PC5 path with parent is established, so that the remote UE can prioritize a candidate path that is already established and access the network faster.

# References

[1] 3GPP TS 38.300 V19.0.0

[2] 3GPP TS23.304, V19.5.0.

# Annex 1 – TP for 38.300

### 16.12.3	Relay Discovery

Model A and Model B discovery models as defined in TS 23.304 [48] are supported for U2N/U2U Relay discovery. The protocol stack used for discovery is illustrated in Figure 16.12.3-1.

Figure 16.12.3-1: Protocol Stack of Discovery Message for UE-to-Network/UE-to-UE Relay

The U2N Remote UE can perform Relay discovery message (i.e., as specified in TS 23.304 [48]) transmission and may monitor the sidelink for Relay discovery message while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The network may broadcast or configure via dedicated RRC signalling a Uu RSRP threshold, which is used by the U2N Remote UE to determine if it can transmit Relay discovery messages to U2N Relay UE(s).

The U2N Relay UE can perform Relay discovery message (i.e., as specified in TS 23.304 [48]) transmission and may monitor the sidelink for Relay discovery message while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The network may broadcast or configure via dedicated RRC signalling a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, which are used by the U2N Relay UE to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

In multi-hop U2N relay, one U2N Relay UE establishes a PC5 connection with each of its child UE(s). One intermediate U2N Relay UE or U2N Remote UE maintains a single PC5 connection with its parent UE.

The U2U Remote UE and U2U Relay UE can perform Relay discovery message transmission or DCR/DCA message with integrated discovery transmission and may monitor for Relay discovery message or DCR/DCA message with integrated discovery while in coverage (i.e. RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED) or out-of-coverage.

The network may provide the Relay discovery configuration using broadcast or dedicated signalling. In addition, the U2N/U2U Remote UE, L3 U2N Relay UE and U2U Relay UE may use pre-configuration for Relay discovery. If the candidate intermediate U2N Relay UE is out of coverage without a PC5 connection to a parent UE, it can forward discovery messages based on pre-configuration. If the candidate intermediate U2N Relay UE is in coverage without a PC5 connection to a parent UE or out of coverage with a PC5 connection to a parent UE, it can forward discovery message based on SIB12 or dedicated configuration.

After the discovery procedure, when the U2N Remote UE selects a path via an intermediate U2N Relay UE that has an existing path through a single-hop U2N Relay UE, the intermediate Relay UE may release the existing single-hop relay path and establish the candidate multi-hop relay path via another Last U2N Relay UE to serve the Remote UE.

The resource pool(s) used for NR sidelink communication can be used for Relay discovery or the network may configure resource pool(s) dedicated for Relay discovery. Resource pool(s) dedicated for Relay discovery can be configured simultaneously with resource pool(s) for NR sidelink communication in system information, dedicated signalling and/or pre-configuration. Whether dedicated resource pool(s) for Relay discovery are configured is based on network implementation. If resource pool(s) dedicated for Relay discovery are configured, only those resource pool(s) dedicated for Relay discovery shall be used for Relay discovery. If only resource pool(s) for NR sidelink communication are configured, all the configured resource pool(s) can be used for Relay discovery and NR sidelink communication. Only the resource pool for NR sidelink communication is used for the DCR/DCA message with integrated discovery.

For U2N Remote UE (including both in-coverage and out of coverage cases) that has been connected to the network via single/multi-hop U2N Relay UE(s), only resource allocation mode 2 is used for Relay discovery message transmission.

For in-coverage U2N Relay UE, and for both in-coverage and out of coverage U2N Remote UEs, NR sidelink resource allocation principles are applied for Relay discovery message transmission.

For U2U Remote UE and U2U Relay UE, NR sidelink resource allocation principles, both mode 1 and mode 2, can be applied for Relay discovery message transmission.

The sidelink power control for the transmission of Relay discovery messages is same as for NR sidelink communication.

No ciphering or integrity protection in PDCP layer is applied for the Relay discovery messages.

The U2N/U2U Remote UE and U2N/U2U Relay UE can determine from SIB12 whether the gNB supports Relay discovery, or Non-Relay discovery, or both.

# Annex 2 – TP for 38.331

#### –	SL-AccessInfo-L2U2N

The IE SL-AccessInfo-L2U2N includes the radio information included in Discovery Message used for L2 U2N relay operation.

SL-AccessInfo-L2U2N information elements

-- ASN1START

-- TAG-SL-ACCESSINFO-L2U2N-START

NR-Sidelink-DiscoveryMessage DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

CellAccessRelatedInfo,

SL-ServingCellInfo-r17,

SL-RelayIndicationMP-r18

FROM NR-RRC-Definitions;

SL-AccessInfo-L2U2N-r17 ::=             SEQUENCE {

cellAccessRelatedInfo-r17               CellAccessRelatedInfo,

sl-ServingCellInfo-r17                  SL-ServingCellInfo-r17,

...,

[[

sl-RelayIndication-r18                  SL-RelayIndicationMP-r18                  OPTIONAL

]],

[[

relayUE-RRCState-r19                    SL-RelayUE-RRCState-r19	  	  			   OPTIONAL,

pC5ConnectionWithParent-r19             ENUMERATED {established, spare1}           OPTIONAL

]]

}

END

-- TAG-SL-ACCESSINFO-L2U2N-STOP

-- ASN1STOP