3GPP TSG-RAN WG2 Meeting #132	R2-2508863

Dallas, U.S.A., 17th – 21st November 2025

Agenda Item:	10.3.3.2

Source:	MediaTek Inc.

Title:	Design considerations for the energy efficiency framework

Document for:	Discussion and decision

# Introduction

In the previous meeting [1], we reached the following design goal on energy efficiency for the UE and NW.  This document discusses additional design considerations.

Design goal - Strive to develop solutions that consider network energy efficiency and UE power saving

# Discussion

As part of 6G design, energy efficiency must be treated as a core system objective. Operators are increasingly focused on green and energy-aware RAN evolution, not only to meet sustainability commitments but also to mitigate the impact of rising electricity costs. With potentially higher-density deployments on new 6G frequency bands, base station energy consumption will constitute a significant share of operators’ operational expenditure, creating a clear need for an energy efficient design. At the same time, the growing diversity of UEs, including wearables, XR, and massive IoT amplifies the importance of low power UE operation to extend battery life and enable wider application reach. Consequently, 6G should incorporate energy efficiency as a key metric for both network and UE design, while maintaining necessary performance.

In NR, we have several mechanisms for energy saving such as DRX, DTX, WUS and LP-WUS introduced over several releases. DRX enables the UE to go to sleep by only requiring discontinuous monitoring of PDCCH . DTX reduces network power and interference by muting UE and gNB transmissions when no data is present. LP-WUS reduces standby consumption while increasing reachability by using a low-power wake-up receiver to detect a simplified wake-up signal. DRX/LP-WUS address UE-side power (periodic vs event-triggered), while DTX addresses network-side power. Among these features, LP-WUS performance and usage highly depend on its physical channel/signal design which could lead to different coverage, mobility and FAR characteristics.

Before diving into the detailed design of the energy efficiency framework for 6G, we need to wait for an outline of the LP-WUS design from RAN1 to gauge its performance. Once its performance targets and limitations are understood, RAN2 can design the energy efficiency framework for different RRC states that can incorporate this LP-WUS signal.

Proposal 1: RAN2 designs the framework for LP-WUS usage in different RRC states once its performance targets are determined by RAN1

When designing the energy efficiency framework for 6G, a principle that should not be forgotten is that the design needs to ensure that the UE and NW can remain in sync regardless of whether one or both nodes go to sleep. While arguments have been raised that the existing NR energy efficiency framework is very rigid with fixed DRX patterns, such a rigid scheme ensures that there are known/fixed occasions that the gNB knows that the UE will monitor if there is DL data to be transmitted to the UE. For 6G, the energy efficiency framework needs to be designed to jointly consider DRX/DTX and potentially LP-WUS to further improve power saving gain for both the NW and the UE. While doing so, it is important that the proposed scheme considers potential cases where the UE or NW mis-detects or fails to receive dynamic signalling due to noise or poor SNR at the cell-edge and deliver a resilient solution.

Proposal 2: The framework for energy savings is designed to be resilient to ensure that the UE and network are in sync to enable communications when needed.

# Conclusion

In this contribution we have the following proposals:

Proposal 1: RAN2 designs the framework for LP-WUS usage in different RRC states once its performance targets are determined by RAN1

Proposal 2: The framework for energy savings is designed to be resilient to ensure that the UE and network are in sync to enable communications when needed

# References

- R2-2508002 - RAN2#131bis Meeting Report (ETSI MCC)