3GPP TSG-RAN WG2 Meeting #132	R2-2508868

Dallas, USA, November 17-21, 2025

Source: 			Qualcomm Incorporated

Title: 	Considerations on UE capability framework in 6G

Document for: 	Decision

Agenda Item: 	10.2.1

# Introduction

This document provides further analysis on the 6G UE capability framework for the discussion points we identified in [1].

# Discussion

  - UE capability signalling overhead

In [1], we looked at the high-level breakdown of UE-NR-Capability content as reported by devices in the field and made the following observations on the UE capability signalling overhead.

Observation 1: Band combination related UE capability parameters are the main contributor to UE capability size increase. Observation 2: The size of featureSetCombinations (merely containing pointers to feature sets) becomes prominent factor when the number of supported band combination increases. Observation 3: The Feature Set scheme serves the intended purpose to reuse the same set of UE capabilities across different band combinations. Observation 4: Roughly the half of RF-parameters is baseband related UE capability parameters, which would be reuseable across band combinations. Observation 5: “Multi-dimensional” aspect of UE capabilities, such as CC BW and MIMO-layers, leads to the advertisement of a large number of feature sets within each featureSetCombination (corresponding to one featureSetCombinationId).

From the observation 3 and observation 4, what we should be doing in 6G are rather obvious:

- Continue using the Feature Set signalling method (feature sets and feasure set combinations) in 6G.

- Signal baseband related capabilities per band combination at the feature set combination level for better reuse across different band combinations.

On the observation 5, we point out that BCS4 and BCS5 were introduced to address the exact issue. They were however introduced in Rel-17 and therefore not all band combinations defined in RAN4 specification can benefit from the solution. To look at the potential gain of BCS5 (signalling of maximum aggregated bandwidth across CCs of the band combination) if introduced from day-one of 6G standard, we analysed how much signalling overhead reduction the BCS5 signalling could achieve if applicable to all band combinations.

The following figure shows the UE capability signalling overhead reduction in % when applicable to all band combinations, compared to the baseline where BCS5 signalling is not used at all. The “queries” represent different band filters in UE Capability Enquiry that we observe in the field.

Figure-1: UE capability signalling reduction by BCS5 usage

The gain is promising enough for us to consider applying a similar approach in 6G. It can be further studied whether the same approach can be extended to other UE capability parameters, such as MIMO capability. Such study however cannot be conducted solely by RAN2.

Proposal 1:	RAN2 to work with RAN1 and RAN4 to study the UE capability signalling where the UE indicates the envelope limitations across the CCs of the band combinations.

  - Dynamic UE capability change during connected mode

In [1], we have shown the UAI solutions standardized in NR with the associated use cases and UE capability restrictions. The summary table is reproduced below. Note that the in-device coexistence use cases are struck out for this discussion because they do not pertain to temporary UE capability limitations.

| Use Case | Release | UE Indications |
| --- | --- | --- |
| Overheating | Rel-15 | - Maximum bandwidth (FR1/FR2, UL/DL) - Maximum MIMO layers (FR1/FR2, UL/DL) - Maximum number of CCs (UL/DL) |
| In-Device Coex. | Rel-16 | - Affected frequencies - Affected frequency combinations. |
| Power Saving | Rel-16 | - Maximum bandwidth (FR1/FR2, UL/DL) - Maximum MIMO layers (FR1/FR2, UL/DL) - Maximum number of CCs (UL/DL) |
| Overheating (FR2-2) | Rel-17 | - Maximum bandwidth (UL/DL) - Maximum MIMO layers (UL/DL) |
| In-Device Coex. (FDM) | Rel-17 | - Affected frequency ranges. |
| Power Saving (FR2-2) | Rel-17 | - Maximum bandwidth (UL/DL) - Maximum MIMO layers (UL/DL) |
| MUSIM Temporary capability restrictions | Rel-18 | Per serving cell - Maximum bandwidth (UL/DL) - Maximum MIMO layers (UL/DL) Per affected band - Maximum bandwidth (UL/DL) - Maximum MIMO layers (UL/DL) - Maximum number of CCs (FR1/FR2/FR2-2, UL/DL) |

We believe the table above already covers key use cases and it can be used as a reference for the RAN2 study, while the details of UE capabilities the UE can indicate to the network can be discussed in the normative work.

For the signalling procedure, we identified the following major shortcoming of the existing UAI procedure in NR.

- The UE is unable to indicate its temporary UE capability restriction unless configured by the network.

- Even if UAI is configured, there is no guarantee that the network will respect the UE indication and change the RRC configuration accordingly.

- As a result, the UE may be forced to resort to UE implementation specific solutions, which often results in waste of radio resources, e.g. unused ports in MIMO operation, hence system performance losses.

RAN2 should aim for a solution that can avoid system performance losses and waste of radio resources. The solution therefore must be such that it incentivizes the network to honour the UE capability change. We believe the solution standardized for MUSIM operation in NR is a good baseline.

Proposal 2:	RAN2 to study a solution of dynamic UE capability change as a basic 6G feature - the UE can send the indication without network configuration, while a prohibit timer for the UE indication to avoid too frequent UE capability changes can be considered.

Proposal 3:	RAN2 to study a solution where step 1) the UE indicates preference for UE capability change, step 2) the UE waits for radio resource configuration by the network during a “wait time” and step 3) the UE applies the UE capability change as indicated in the step 1 if the network does not respond and the wait time expires.

As discussed in NR MUSIM work, we should also consider the cases where the UE experiences temporary UE capability restriction during RRC_IDLE, RRC_INACTIVE and RRC re-establishment. The applicability of those states depends on the separate RAN2 study on protocol states.

Proposal 4:	RAN2 to further study solutions by which the UE indicates its UE capability restrictions upon RRC connection establishment, resume and re-establishment procedures.

  - IODT availability vs inter-operability

In [1], we provided our view on an issue with UE capability reporting in relation to IODT availability.

The UE indicates its support for a given feature only if the feature is sufficiently verified through IODT. This applies to both optional features with UE capability signalling and mandatory features with UE capability signalling. There are however shortcomings with this approach, according to our field experiences. - The UE has to wait for the IODT with the “slowest” network vendor before activating a capability in operator’s network. - Even after activating a capability, the UE may encounter inter-operability issues when a new version of network equipment or network equipment of a new vendor is deployed into operator’s network.

The discussion in RAN2#131bis hinted at some potential solutions, such as providing the network name/ID to the UE or providing the UE vendor name/ID to the network. It is meant to allow the receiving entity (UE or network) to adjust its behaviour depending on the IODT status with the connecting network or UE.

Solutions in such direction in nature allow the receiving entity to differentiate the treatment of different vendors providing its name/ID. The risk due to the abuse of the vendor name/ID information is very high in particular when the network is made aware of the UE vendor name/ID, e.g. preferential treatment of one vendor over another in terms of radio resource configurations and/or scheduling. On the other hand, what the UE can do by knowing the network vendor name/ID is quite limited because the UE anyway needs to rely on and maximize the use of the network coverage that’s available at the UE location.

Proposal 5:	To address the “IODT issue”, solution where the network is provided with the served UE vendor name/ID should be avoided.

Proposal 6:	RAN2 to study solutions where the UE is provided with the serving network vendor name/ID.

Another potential solution is that the UE reports “IOT-ed” network vendor names/IDs per main feature within the UE radio capability. The network uses the information to decide whether to activate a given feature or to use a given set of configuration parameters that would work.

Proposal 7:	RAN2 to study solutions where the UE reports “IOT-ed” network vendor names / IDs per main feature, within the UE radio capability.

  - Inter-WG coordination

UE capability signalling complexity:

The UE signalling complexity and signalling overhead often go hand in hand. Detailed representations of the UE capability, such as per CC per band per band combination, is meant to maximize the usage of actual UE capability. At the same time, it increases the complexity for the network to go through the reported UE capabilities and decide on the RRC configuration tailored for the UE. The associated specification complexity is yet another concern. So the trade-off here is to reduce the signalling complexity while avoiding excessive under-reporting of UE capability.

As discussed in the proposal 1 in this document, we may want to explore new ways to signal UE capabilities. The UE capability signalling based on envelope limitations is a promising direction as we have already seen compelling gain with BCS5 framework.

In NR RRC, the UE capability signalling allows the UE to report an unsupported superset band combination in the RF band combination capability and report actually supported fallback band combinations by means of Feature Sets. This approach essentially reduces the number of reported band combinations, hence repeated signalling of same bands there.

#### – FeatureSetCombination […] NOTE 2: The UE may advertise a FeatureSetCombination containing only fallback band combinations. That means, in a FeatureSetCombination, each group of FeatureSets across the bands may contain at least one pair of FeatureSetUplinkId and FeatureSetDownlinkId which is set to 0/0.

This approach has the limitation that the reported superset band combination must be one of the band combinations defined in the RAN4 specification. This is due to the fact that any reported band combination needs to be associated with a valid BCS. For example, the UE cannot report undefined 8-CC band combination which contains various fallback 4-CC band combinations that the UE supports. This limitation can be alleviated if the UE is allowed to report undefined superset band combinations with BCS4/5.

In summary, RAN2 could consider communicating the following points with RAN1 and RAN4 regarding the UE capability signalling complexity:

- Aim for reduced UE capability signalling complexity, while avoiding excessive under-reporting of UE capability.

- BCS4 and BCC5 as baseline BCS framework in 6G.

- More usage of UE capability signalling based on envelope limitations, such as the maximum aggregated bandwidth of BCS5 in NR, could be beneficial.

- It is beneficial to allow the UE to signal a superset band combination which is not defined by RAN4 (this is not possible in NR because of UE capability signalling for band combination required signalling of a valid BCS for the reported band combination).

RACS:

RACS (Radio Capabilities Signalling Optimizations) was introduced in Rel-16. The overall goal was to reduce the signalling over Uu and network interfaces.​ There are two variants in RACS.

- Manufacturer assigned UE Radio Capability ID​

- PLMN assigned UE Radio Capability ID

We believe RACS (and its potential enhancements) is a promising day-one 6G feature to address many shortcomings of UE capability signalling today, for example;

- No OTA signalling of UE capability is necessary in case of Manufacturer-assigned UE Radio Capability ID.

- “Full” UE capability can be stored in UCMF (UE Capability Management Function), which can avoid OTA re-enquiry of UE capability upon mobility across RAN nodes of different network vendors.

- UE capability filtering can be applied within the network to reduce the size of UE capability delivered to the RAN node, without disclosing the capability of or features supported by the RAN node in the form of UE capability filters.

- A single UE can have multiple sets of UE radio capabilities by means of multiple UE Radio Capability IDs, and the UE can select an appropriate ID depending on the UE situation.

However, SA2 currently does not have a Work Task corresponding to RACS. RAN2 input may be necessary to trigger their study on RACS.

# Conclusion

Based on the analysis in this document, we made the following proposals.

Proposal 1:	RAN2 to work with RAN1 and RAN4 to study the UE capability signalling where the UE indicates the envelope limitations across the CCs of the band combinations.

Proposal 2:	RAN2 to study a solution of dynamic UE capability change as a basic 6G feature - the UE can send the indication without network configuration, while a prohibit timer for the UE indication to avoid too frequent UE capability changes can be considered.

Proposal 3:	RAN2 to study a solution where step 1) the UE indicates preference for UE capability change, step 2) the UE waits for radio resource configuration by the network during a “wait time” and step 3) the UE applies the UE capability change as indicated in the step 1 if the network does not respond and the wait time expires.

Proposal 4:	RAN2 to further study solutions by which the UE indicates its UE capability restrictions upon RRC connection establishment, resume and re-establishment procedures.

Proposal 5:	To address the “IODT issue”, solution where the network is provided with the served UE vendor name/ID should be avoided.

Proposal 6:	RAN2 to study solutions where the UE is provided with the serving network vendor name/ID.

Proposal 7:	RAN2 to study solutions where the UE reports “IOT-ed” network vendor names / IDs per main feature, within the UE radio capability.

We also touched upon the need of inter-WG coordination.

# Reference

[1]	R2-2506988	Considerations on UE capability signalling in 6G	Qualcomm Incorporated