3GPP TSG-RAN WG2 Meeting #132			         R2-2508869

Dallas, USA, Nov. 17th-21st

Agenda Item:	10.2.2

Source: 	LG Electronics Inc.

Title:         	Discussion on TN/NTN integration for 6G

Document for: 	Discussion and Decision

Introduction

A WID for study on 6G Radio was approved [1]. In the SID, one of the objectives of the study item is that “aim at a harmonized 6G Radio design for TN and NTN, including their integration”. According to this SID, RAN2 discussed harmonized design for TN and NTN at the last meeting and reached the following agreements:

- For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

In this proposal, we examine the minimal essential sets of requirements for 6GR NTN. We believe that 6GR NTN should incorporate additional functionalities beyond the 5G NR NTN. Based on the minimal essential sets of requirements for 6GR and the expected additional capabilities for 6G NTN, we present our perspective on the functionalities that should be discussed from the day-1 to ensure a harmonized design with TN.

Discussion

2.1 Minimal essential set of requirements

Lessons from 5G NR NTN

In 5G NR NTN, to simplify specification work, it is assumed that all UEs have a valid UE location. Based on this assumption, the specification works for NTN support are simplified especially for physical channels and procedure design to cover the high Doppler shift and large propagation delay in NTN scenario. Moreover, in NTN scenario, the RSRP measurement results can be similar within a cell area. To mitigate this issue, the location-based handover and cell managements were introduced instead of the RSRP-based procedures. However, in 6GR, it is important to support the UE without GNSS capability to extend the applicable UE types for NTN. In this case, the location-based procedures in NR NTN would need to be revisited while the measurement-based procedures may not work properly.

The NTN UE experiences frequent satellite switching due to satellite mobility. During this switching, the UE should perform handover depending on satellite payload type, as well as DL/UL re-synchronization. Consequently, the UE may suffer from service interruption during the satellite switching. Furthermore, in earth-moving cells, the NTN UE undergoes more frequent handover due to cell movement. In 6GR NTN, VLEO can be further considered for the relatively high throughput and low latency. However, due to the lower altitude of the VLEO (e.g., 300km) and the higher satellite speed, satellite switching is expected to occur more frequently. Depending on the target scenario (e.g., voice call or emergency alert), service continuity would be critical, and then service interruption needs to be resolved or further mitigated.

In NR NTN, due to the large propagation delay and the limited HARQ buffer at the UE side, it is supported that the NW disables HARQ feedback for some HARQ processes. In this case, the data associated with HARQ process without HARQ feedback relies on the RLC ACK mechanism for the reliable communication. However, it can cause huge end-to-end latency. Moreover, in 6GR, it is currently unclear whether the RLC ACK mechanism will be introduced or not. In such situation, achieving more reliable communication over 5G NR NTN will need further consideration in 6GR. The reliable transmission can save the resources for the retransmissions, and then the overall latency could be further enhanced. The enhancements for reliability and latency will lead to increase throughput. These kinds of enhancements would be useful for the successful commercialization of NTN. In 6GR, it will be necessary to study how to improve reliability/throughput for NTN UE.

Observation 1: Since NR NTN was introduced after TN standardization had already progressed significantly, it was defined by reusing TN-related functions as much as possible without sufficient consideration of its performance optimization and UE implementation complexity. This means that simply reusing NR NTN functions for 6G NTN would negatively impact its necessity and commercialization.

GNSS independent operation

In 5G NR NTN, NTN UE is generally required to have GNSS capability since many NTN functions rely on the GNSS-based location information. The satellite for the 3GPP communication is different from the satellite for providing GNSS information. The NTN UE receives communication data via 3GPP link and GNSS information via non-3GPP link. This requirement inevitably increases both the cost and design complexity of NTN UE implementation. GNSS-based operation also introduces potential vulnerabilities. GNSS signals can be jammed, spoofed, degraded or leading to incorrect positioning. Consequently, in this case, the NTN UE may be restricted from transmitting or receiving data via NTN, which can impact service reliability.

Considering those restrictions, we need to consider GNSS-independent operation for 6GR NTN. By removing the dependency on GNSS information, 3GPP NTN can get lower hardware implementation cost and more flexible deployment scenarios.

UE mobility management between TN and NTN or between NTN and NTN

One of the motivations for adopting NTN communication is to enable a ubiquitous connectivity environment, where users can access service seamlessly regardless of its location. From this perspective, mobility management between TN and NTN, as well as between different NTNs in the same/different orbits can be an essential requirement for 6GR NTN system.

Unlike TN environment, mobility management in NTN poses unique challenges because satellites are constantly in motion relative to the Earth. Even when the UE remains stationary, the serving satellite will eventually move out of coverage. It requires a handover to another satellite, i.e., HO between NTN and NTN. Or when UE moves from TN coverage to out-of-TN coverage or vice versa, the UE requires HO from TN to NTN or vice versa. Another case, even the UE becomes belonging in the TN coverage, the UE may want to have connection with NTN instead of TN if some services are dedicated in NTN path. There can be various scenarios on the mobility. So, 6GR needs to support mobility management between TN and NTN and/or between NTN and NTN.

Also, in the NTN environment, the difference in the received signal strength between the cell centre and the cell edge is marginal compared to TN, which is why location-based measurement triggering and CHO were introduced in 5G NR NTN. If the 6G NTN accepts the GNSS-independent operation, the UE may need to be triggered other conditions at the NTN cell edge instead of location-based measurement triggering.

- Reduction of service interruption time

As mentioned above, since NTN link can be changed frequently, making the reduction of service interruption time is another important issue. For the realization of a ubiquitous environment, it is essential that UE experiences no noticeable service disruption during these link changes. One of the key factors influencing service interruption time is the long propagation delay between UE and source/target path. In conventional HO, the NW commands the UE to perform HO when the NW decides HO, then the UE initiates synchronization with the target cell. In this process, the UE receives and transmits signals through the source/target path. As the propagation delay is long in NTN, the reduction of service interruption time is critical.

- Avoiding frequent HO occurrences

In the NTN communication, frequent HOs are expected due to the continuous movement of satellites. This frequent HO is inevitable in NTN system. It can lead to undesirable effects such as service interruptions and increased energy consumption at the UE side. Whenever the measurement report is triggered, the UE has to spend energy for the measurement and the reporting. If the measurement report is triggered too frequently, the energy consumption of the UE and can be increased and the service interruption can occur frequently. Therefore, minimizing unnecessary HO events and optimizing HO triggering conditions can be beneficial for maintaining service continuity and reducing UE energy consumption. For instance, if the UE can select a satellite that is capable of maintaining a stable link for the potentially longest duration, overall HO frequency can be reduced. This requires a satellite selection mechanism that considers factors such as orbital trajectory, coverage duration, link quality, and predicted movement patterns.

- Support of simultaneous HO for large number of UEs

In NTN systems, satellite coverage areas are extremely large. As a satellite moves across the Earth in an earth-moving cell, a large number of UEs may need to perform handover simultaneously. If the available bandwidth or radio resources are limited, this simultaneous HO can lead to RACH congestion. It makes difficult for all UEs to complete the HO successfully. Such congestion can result in service interruptions or even HO failures. Even in a quasi-earth-fixed cell, RACH congestion can also occur during satellite switching events, particularly in GNSS-independent operation scenarios. Without GNSS assistance, UEs may not know precise timing and location information. If large number of UEs initiate HO procedure simultaneously when switching to a new satellite, the RACH load can spike. As the result, the access delay can be increased and the connection to the new satellite can be failed.

Measurement procedure considering satellite beam hopping

Due to the power limitations of satellites, satellite beam hopping has been introduced to improve energy efficiency. In 5G NR NTN, a longer SSB periodicity—up to 160ms—is adopted, and this may be further extended in 6G NTN. When neighbor cells have different SSB periodicities, especially in scenarios where they are deployed across multiple satellites, measurement periodicity and measurement timing offset among neighbor cells can vary significantly. Moreover, if the UE operates without GNSS information, the measurement procedure should be carefully designed to account for satellite beam hopping.

Initial access with low latency

In NTN, the duration for which a UE can maintain a connection with a satellite is inherently limited due to satellite mobility. This connection window becomes even shorter for satellites operating at lower altitudes, such as those in LEO, where orbital periods are faster and the satellite quickly moves out of the UE’s line of sight. Under this short connection duration, fast initial access can become an essential requirement for 6G NTN. By minimizing the time needed for a UE to establish a link with a satellite, the effective usable connection time can be extended.

Maintain stable/reliable connection

In NTN system, satellite moves fast over the Earth’s surface. As the result, the NTN connection has a relatively short duration for maintaining a continuous connection with a UE. Furthermore, due to the long distance between the satellite and UE, the received signal strength is often week. It makes the NTN link vulnerable. Therefore, ensuring a stable connection is essential to delivering consistent service quality to the UE.

One of the approaches to enhance connection stability is for the UE to maintain multiple links with multiple satellites. In this configuration, if one link is interrupted or degraded, the UE may receive service seamlessly via another link. Also, this multi-link configuration can improve reliability, reduce the impact of handover events, and provide resilience against link failures caused by satellite movement.

Reduction of energy consumption of UE and NW

Energy saving of UE and NW is one of the essential themes in the all parts of the 6GR. Especially, in the NTN, once the satellite is launched, the satellite relies on solar panels and batteries. Efficient energy usage can extend the operational lifespan of the satellite system and reduce the cost for replacements. For the NTN UE, the NTN links requires higher transmission power due to the long distance between UE and satellite. Also, the frequent HOs, RACH procedures for targeting long distance, and continuous link measurement can cause UE energy consumption. To reduce energy consumption of the UE, configuring proper monitoring window, DRX operation, and timer configuration needs to be considered. To reduce energy consumption of the NW, beam scheduling optimization and energy efficient routing using inter-satellite links need to be considered.

Increased throughput/reliability

The range of services supported by NTN is extensive, for example, encompassing voice communication, messaging, remote sensing, and high-bandwidth streaming applications. Each of these services may impose various requirements, such as high throughput and strong reliability. However, NTN operates over vast coverage areas, and the propagation distance between satellites and UE is significantly longer than in TN. This implies that a large number of UEs may need to share the same carrier resources simultaneously. This can reduce potentially per UE resources. The long distance causes large path loss and week signal strength. To meet the diverse service requirements in 6GR NTN, throughput and/or reliability increment should be considered. One of the approaches for throughput and/or reliability increment can be the use of multi-carrier via same satellite or across multiple satellites.

Improved UL/DL coverage

In NTN scenario, since the distance between the satellite and UE is too high, the path loss is also high. Especially for supporting handheld device with small antenna gain, the coverage enhancement scheme needs to be considered as a basis. For the coverage enhancement, the repetition can be adopted, and the repetition can be used from the initial access procedure. For the measurement-based operation including RLF or RRM, the status of the repetition may need to be further considered.

Proposal 1: The followings can be considered as a minimal essential set of requirements that should be satisfied by 6G NTN:

- GNSS independent operation

- UE mobility management between TN and NTN or between NTN and NTN, including at least the following aspects:

- Reduction of service interruption time

- Avoiding frequent HO occurrences

- Simultaneous HO of large numbers of UEs

- Measurement procedure considering satellite beam hopping

- Initial access with low latency

- Maintaining stable/reliable connection

- Reduction of energy consumption for UE and NW

- Increased throughput/reliability

- Improved UL/DL coverage

2.2 Technical functionalities for harmonized design with TN

Based on the above-mentioned essential set of requirements and NTN characteristics, we can list up the functionalities for harmonized design with TN at Day-1.

Fast and/or power-efficient (re)acquiring of NTN-related SI

The Acquisition of NTN-related system information should be addressed in collaboration with TN from day-1. In the 5G NR NTN, for the initial access, the NTN UE obtains both SIB1 and SIB19. Among these, SIB19 includes satellite related information, which changes periodically as the satellite moves. These changes are not signalled through SIB1. The NTN UE has to reacquire the satellite related information within valid duration.

If IDLE/INACTIVE NTN UE acquire satellite related information before receiving paging, it can be helpful to fast initial access. However, since the UE cannot predict when the paging will occur, the UE has to acquire satellite related information periodically based on valid duration. This approach can lead to unnecessary energy consumption. To achieve both fast initial access, and power efficient acquisition of the NTN related information, the harmonized design could involve restructuring the information elements or minimizing the wasteful reacquisition.

Measurement management due to increased SSB periodicity

In 5G NR TN, the SSB periodicity is typically configured at 20ms, enabling frequent synchronization opportunities and fast measurement cycles. In contrast, 5G NR NTN adopt a significantly longer SSB periodicity—up to 160ms—to reduce energy consumption for both the satellite and the UE. In the future, this periodicity may be extended even further, potentially reaching 320ms, as part of ongoing efforts to optimize energy efficiency in NTN deployments.

The increase in SSB periodicity compared to the legacy TN configuration introduces measurement management. The handling of timing offsets and the configuration of SMTC has to be carefully designed to ensure seamless operation across both TN and NTN environments.

Beam-specific SSB/SI transmission on/off and maintenance in beam hopping operation

In NTN, beam hopping is one way to achieve flexible and efficient resource allocation for uneven traffic demands. Considering that beam hopping will lead to transmission on/off for common signals/channels, NTN Beam hopping operation can be considered for common TN/NTN design of SSB/SI transmissions in 6GR, considering that beam/TRP based SSB/SI transmission on/off is beneficial for network energy in TN as well as NTN. Both SSB beam hopping for NTN and beam/TRP based SSB transmission on/off can be designed for both TN and NTN with commonality.

In NR, system information is assumed to be same across different beams at a cell. Such restriction can be reconsidered for 6GR. For example, considering a single beam for NTN covers a large area, different common channel configurations can be provided for different beams. We think that different common channel configurations can be beneficial for even for TN e.g. for multiple TRPs of a cell.

Timer extension (e.g., DRX related timers, RLC reassembling timer)

In 5G NR NTN, certain protocol timers—such as those related to DRX cycles and RLC reassembling—have already been extended or adjusted to account for the inherently long propagation delays in satellite communication. These modifications ensure that UEs can maintain reliable operation despite the significantly greater RTT (round-trip times) compared to terrestrial networks.

In the 6G Day-1, if GNSS- independent operation is adopted, further discussion will be required on how to adjust both the start points and durations of these timers to reflect NTN-specific characteristics. While TN UEs already support GNSS- independent operation under the assumption of short propagation delays, NTN UEs operating without GNSS will face much longer delays, making the existing TN timer configurations potentially unsuitable. For the harmonized design between TN and NTN, a unified or simplified timer configuration can be considered.

HARQ feedback disabled operation

In 5G NR NTN, due to the large RTT, for the efficient utilization of the resources, there are a need to increase the number of HARQ processes. Meanwhile, even with the increased HARQ processes, due to the limited soft buffer at the UE side, it was difficult to keep the HARQ combining for all the HARQ process numbers. Moreover, due to the large RTT, HARQ-feedback-based retransmission could be inefficient in terms of the latency. In those points of views, HARQ-disabled operation was adopted in 5G NR NTN.

In 6GR, for FR2 or FR3, when the large SCS is used, the RTT can be high while the slot duration is still small. In this case, the similar situation can happen in TN. Moreover, depending on the application such as HRLLC, the HARQ-disabled operation can be further considered even in TN.

Seamless TN-NTN/NTN-NTN mobility

For seamless mobility between TN and NTN, or between different NTN nodes, the source node should be properly configured to trigger measurement reporting and transmit the handover command at the right time. For the seamless mobility management, long propagation delay or propagation delay difference between source path and target path can be considered. In addition, the RAN components of both the source and target networks can exchange supporting messages to facilitate a seamless HO process. Such coordination may include sharing context information, pre-configuring resources, and optimizing signaling to minimize latency and packet loss during the transition. These coordination messages can be related to the HO procedures. To achieve unified and simplified mobility management, it is important to address seamless mobility between TN and NTN, or between NTN and NTN.

UE mobility management in GNSS independent operation

Many 5G NR NTN operations have been designed to rely on precise location information, typically obtained by the UE through GNSS. This location information is critical for enabling legacy GNSS-based operations, e.g., location/time based (conditional) HO, STMC adjustment, timer expansion, and other location-dependent optimizations. However, if a UE does not use GNSS information, these legacy GNSS-dependent operations should be revisited.

One possible alternative approach is for the UE to use a reference location instead of the GNSS information. The reference location can be broadcasted on the SIB that may be associated with beam or cell. In this case, how to broadcast the reference location can be discussed with how to compose SIBs.

Mobility handling for large numbers of UEs

When the large number of UEs perform HO simultaneously under limited time/frequency resources, problems such as increased RACH collision probability and signalling overhead may occur. It can lead to increased latency until HO completion, or even HO failures. This phenomenon is not exclusive to NTN. It can also occur in TN under similar conditions. Therefore, the ways to distribute HO events that mitigates congestion or to reduce HO signalling overhead can be considered, regardless of whether the environment is NTN or TN.

Multi-carrier operation for increased throughput/reliability and/or improved connection stability

When a UE supports multi-connection, it can maintain service continuity by receiving data through at least one active connection, even if other connections are temporarily interrupted due to HO events, degraded signal strength, or link failures.

One potential approach to enabling multi-connection is through multi-carrier aggregation across different satellites. For example, several carriers could be aggregated via one satellite path, while other carriers are aggregated via a separate satellite. If a UE can establish multiple paths through different satellites, each with its own carrier group, both throughput and reliability can be enhanced. This multi-path, multi-carrier operation should be discussed with TN to ensure alignment with NTN-specific requirements.

However, each satellite path may exhibit significantly different propagation delays due to varying distances and orbital positions. These delay differences can impact scheduling, synchronization, and HARQ timing. Therefore, scheduling algorithms and synchronization mechanisms may need to be revisited to accommodate the propagation delay difference of multi-path NTN connections.

Provision of required UL/DL coverage

In NTN scenario, as mentioned before, due to the large pathloss, it would be necessary to consider that the coverage enhancement scheme will be used for DL/UL transmissions. Even in TN scenario, it was discussed that the coverage for FR3 needs to be enhanced due to the increased carrier frequency. Moreover, in general, the IoT use case usually consider the harsh condition (e.g., underground). In those cases, the coverage enhancement scheme can be used from the initial access or all the time.

In this case, the measurement-based operation such as RLF and RRM would need to consider this situation. Moreover, the management of measurement resources or gap, and other L2-operation need to be modified accordingly.

Proposal 2: In terms of common design with TN, at least the following set of functionalities affected by NTN characteristics should be considered:

- Fast and/or power-efficient (re)acquiring of NTN-related SI

- Measurement management due to increased SSB periodicity

- Beam-specific SSB/SI transmission on/off and maintenance in beam hopping operation

- Timer extension (e.g., DRX related timers, RLC reassembling timer)

- HARQ feedback disabled operation

- Seamless TN-NTN/NTN-NTN mobility

- UE mobility management in GNSS independent operation

- Mobility handling for large numbers of UEs

- Multi-carrier operation for increased throughput/reliability and/or improved connection stability.

- Provision of required UL/DL coverage

Conclusion

In this contribution, we present our views on harmonized design between TN and NTN for 6GR that it should be addressed:

Observation 1: Since NR NTN was introduced after TN standardization had already progressed significantly, it was defined by reusing TN-related functions as much as possible without sufficient consideration of its performance optimization and UE implementation complexity. This means that simply reusing NR NTN functions for 6G NTN would negatively impact its necessity and commercialization.

Proposal 1: The followings can be considered as a minimal essential set of requirements that should be satisfied by 6G NTN:

- GNSS independent operation

- UE mobility management between TN and NTN or between NTN and NTN, including at least the following aspects:

- Reduction of service interruption time

- Avoiding frequent HO occurrences

- Simultaneous HO of large numbers of UEs

- Measurement procedure considering satellite beam hopping

- Initial access with low latency

- Maintaining stable/reliable connection

- Reduction of energy consumption for UE and NW

- Increased throughput/reliability

- Improved UL/DL coverage

Proposal 2: In terms of common design with TN, at least the following set of functionalities affected by NTN characteristics should be considered:

- Fast and/or power-efficient (re)acquiring of NTN-related SI

- Measurement management due to increased SSB periodicity

- Beam-specific SSB/SI transmission on/off and maintenance in beam hopping operation

- Timer extension (e.g., DRX related timers, RLC reassembling timer)

- HARQ feedback disabled operation

- Seamless TN-NTN/NTN-NTN mobility

- UE mobility management in GNSS independent operation

- Mobility handling for large numbers of UEs

- Multi-carrier operation for increased throughput/reliability and/or improved connection stability.

- Provision of required UL/DL coverage

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”