3GPP TSG-RAN WG2 Meeting #132	R2-2508913

Dallas, USA,  Nov. 17th - 21st

Agenda item:	8.10.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RILZ305, Z312 and stage 2 corrections

Document for:	Discussion and Decision

1. Introduction

In the post RAN2 #131bis meeting 38.331 ASN.1 review procedure, some RILs are indicated "Todo", which need to be brought for online decision.

In this contribution, we discuss RILs and stage-2 TS 38.300 corrections.

2. Discussion

2.1 RILs

RIL: Z305

During the ASN.1 review discussion, the status of RIL: Z305 is that the Rapporteur is inclined to agree, but further discussion is needed from other companies. The primary issue addressed by Z305 is that in the highlighted procedure below, the scenario involving multiple condRRCReconfig is not clearly defined.

38331-j00 5.3.10.5 RLF report content determination The UE shall determine the content in the VarRLF-Report as follows: ... 2> if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration including condRRCReconfig for the same target candidate PCell as set in pCellId: 3> set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

In the CHO with candidate SCG procedure, the network may configure multiple CHO configurations containing condRRCReconfig for the same target PCell. During failure analysis, it is necessary to uniquely identify the "last configuration received by the UE before the failure"; otherwise, the value of fulfilledConfigWhenChoOnly cannot be correctly traced back. Therefore, we think that in cases where the network may update condRRCReconfig multiple times, the specs need to define "which one" serves as the baseline for failure analysis.

Proposal 1: Clarify in the NR RRC specs that for the CHO with candidate SCG procedure, the CHO configuration is the last one received by the UE before the failure, and agree to the associated TP (First change).

RIL: Z312

In the ASN.1 review discussion, the measResultLastServPSCell-r19 field within the RLF-Report message is used to report key measurement information related to PSCell failure. However, this parameter lacks the necessary field description. The absence of a field description may lead to confusion among readers regarding the precise meaning and application scenario of this field. Therefore, for the measResultLastServPSCell-r19 field, we suggest adding the following description in the field description section of the RLF-Report:

This field refers to the log measurement results taken in the PSCell upon SCG failure (in case of no PSCell change) or the source PSCell upon PSCell change failure (in case of PSCell change).

Proposal 2: In the NR RRC specs, add the following description for the measResultLastServPSCell-r19 in RLF-Report, and agree to the associated TP (Second change)：

measResultLastServPSCell

This field refers to the measurement results taken in the PSCell upon SCG failure (in case of no PSCell change) or the source PSCell upon PSCell change failure (in case of PSCell change).

2.2 stage-2 TS 38.300 corrections

At the RAN2 #131bis meeting, the correction CR for successful LTM cell handover in R2-2507235 was discussed. The primary issue addressed was that the definition of successful completeness of a RACH-less LTM in section 15.5.2.2.2 is not aligned with the original definition in section 9.2.3.5.2.

38300-j00 9.2.3.5.2 C-Plane Handling ... 8. The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data. ##### 15.5.2.2.2 Connection failure due to intra-system mobility ... In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

The conclusion at the RAN2 #131bis meeting was to postpone the above discussion to the next meeting. After the meeting, we further discussed the above issue via email, and most companies agreed that the current problem needs to be resolved. Additionally, during the meeting, some companies expressed that the protocol should further clarify that the LTM cell switch only applies to the MCG, in order to align with the description of MCG LTM in the current TS 38.331. Furthermore, there are some editorial issues, such as the need to italicize "SCGFailureInformation" and capitalize "SCG Failure Indication".

Proposal 3: Revise the following description in the TS 38.300 specs and agree to the associated TP (Third change):

- Align the success criteria in Section 15.5.2.2.2 with the relevant content in Section 9.2.3.5.2,

- Clarify that the LTM cell switch only applies to the MCG,

- Correct some editorial issues.

A TP has been provided in Annex for P1, P2 and P3.

3. Conclusion

In this contribution, we analyzed the remaining issue in MAC. The following proposals were made:

Proposal 1: Clarify in the NR RRC specs that for the CHO with candidate SCG procedure, the CHO configuration is the last one received by the UE before the failure, and agree to the associated TP (First change).

Proposal 2: In the NR RRC specs, add the following description for the measResultLastServPSCell-r19 in RLF-Report, and agree to the associated TP (Second change)：

measResultLastServPSCell

This field refers to the log measurement results taken in the PSCell upon SCG failure (in case of no PSCell change) or the source PSCell upon PSCell change failure (in case of PSCell change).

Proposal 3: Revise the following description in the TS 38.300 specs and agree to the associated TP (Third change):

- Align the success criteria in Section 15.5.2.2.2 with the relevant content in Section 9.2.3.5.2,

- Clarify that the LTM cell switch only applies to the MCG,

- Correct some editorial issues.

4. Reference

[1]. 3GPP TS 38.300-j00: "NR; Overall description; Stage 2".

[2]. 3GPP TS 38.331-j00: "NR; Radio Resource Control (RRC); Protocol specification".

Annex TP

First change

5.3.10.5	RLF report content determination

The UE shall determine the content in the VarRLF-Report as follows:

...

2>	if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration including condRRCReconfig for the same target candidate PCell as set in pCellId and the conditional handover was the last one UE received before the failure:

3>	set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

End of first change

Second change

RLF-Report field descriptions

Omitted....

measResultLastServPSCell This field is used to log the measurement results taken in the PSCell upon SCG failure (in case of no PSCell change) or the source PSCell upon PSCell change failure (in case of PSCell change).

End of Second change

Third change

<Start of modified section>

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of MCG LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or when the UE determines that the network has successfully received its first UL data in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

<Next modified section>

#### 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different M-NG-RAN node, the serving M-NG-RAN node sends SCG FAILURE INDICATION message to the M-NG-RAN node that caused the failure. If it is an SN that caused the failure, the M-NG-RAN node sends SCG FAILURE INFORMATION REPORT message to the SN as defined in TS 37.340 [21].

<End of modified section>

End of Third change