3GPP TSG RAN WG2 #132                                                                                   R2-2508917

Dallas, USA, Nov. 17th - 21st, 2025

Source:	CATT

Title:	Discussion on RILC074

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

During ASN.1 review phase 1, we proposed RIL [C074] which is related to network side data collection. And currently it is marked with “ToDo” for further discussion in the ASN.1 review file [1][2].

[Comments]: C074 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C074; AIML; 1; Duplicate release/discard UE behaviour; R2-25xxxx; Tangxun; V003; ToDo. [Description]: in case of MCG RLF, UE will initiate RRC re-establishment procedure, and the CSI logged measurement configuration will also be released due to “2> release spCellConfig, if configured;”, as it’s a part of spCellConfig. For the similar reason, it’s also unnecessary to add duplicate description of “release loggedDataCollectionAssistanceConfig,” and “discard the logged measurement entries”, since the same contents have been added in RRC re-establishment procedure. [Proposed Change]: update the procedural text as below: 3> else: 4> consider radio link failure to be detected for the MCG, i.e. MCG RLF; 4> discard any segments of segmented RRC messages stored according to 5.7.6.3; [Comments]: [Huawei-Dawid-v004] Agree with CATT’s comment and proposal. During the CR review, rapporteur mentioned MCG failure case. However, this case results in MCG recovery procedure being triggered which can result in the following outcome: - Connection release configuration and data discard is already covered in a dedicated section - Handover already covered in a dedicated section - Re-establishment already covered in a dedicated section [Lenovo-Congchi-v011]: Also agree with CATT [Xiaomi-Xing-v012]: We also suppor to delete these parts. If UE is configured with CHO, UE may perform CHO recovery successfully. Therefore, UE should not release UAI and data logging configuration upon RLF. [Samsung-Beom-v028]: Agree [Nokia-Sakira-ASN.1-review-v052]: Disagree because if we remove these from the RLF detection, then UE behavior is undefined when UE is configured with both attemptCondReconfig and attemptLTM-Switch.

In this contribution, we further discuss why the proposed change in RIL [C074] is needed.

- Discussion

According to current latest TS38.331 spec, the UE removes network side data collection related configuration (i.e., configuration in CSI-LoggedMeasurementConfig and loggedDataCollectionAssistanceConfig) and stored logged measurement (UE variable VarCSI-LogMeasReport) when UE detects MCG RLF as follow (highlighted in yellow).

After detecting the MCG RLF, the UE performs one of the following action depending on different conditions:

- 1) going to RRC_IDLE (highlighted in blue);

- 2) perform re-establishment procedure (highlighted in red);

- 3) perform MCG failure recovery procedure (highlighted in green).

5.3.10.3 Detection of radio link failure …… 3> else: 4> consider radio link failure to be detected for the MCG, i.e. MCG RLF; 4> discard any segments of segmented RRC messages stored according to 5.7.6.3; 4> release CSI-LoggedMeasurementConfig, if configured; 4> release loggedDataCollectionAssistanceConfig, if configured; 4> discard the logged measurement entries included in VarCSI-LogMeasReport, if any; NOTE 1: Void. 4> if AS security has not been activated: 5> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';- 4> else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, have not been setup: 5> store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5; 5> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; 4> else: 5> store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5; 5> if MP is configured: 6> if T316 is configured, and MP indirect path transmission is not suspended; and 6> if neither MP indirect path change nor MP indirect path addition is ongoing: 7> initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure. 6> else: 7> initiate the connection re-establishment procedure as specified in 5.3.7. 5> else: 6> if the UE supports RLF-Report for fast MCG recovery procedure and if T316 is configured: 7> if the SCG is deactivated at the moment of detecting RLF in the MCG: 8> set the mcg-RecoveryFailureCause in the VarRLF-Report to scg-Deactivated; 8> set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell; 7> else if SCG transmission is suspended at the moment of detecting RLF in the MCG: 8> set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell; 8> set the scg-FailureCause value in the VarRLF-Report according to 5.7.3.5; 8> set the elapsedTimeSCG-Failure in the VarRLF-Report to the time elapsed between SCG failure and the MCG failure; 6> if T316 is configured; and 6> if SCG transmission is not suspended; and 6> if the SCG is not deactivated; and 6> if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC): 7> initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure. 6> else: 7> initiate the connection re-establishment procedure as specified in 5.3.7.

1) Upon going to RRC_IDLE after detecting MCG RLF, according to the spec, the UE removes network side data collection related configuration and stored logged measurement as follow, which cause duplicate UE action for removing network side data collection related configuration and stored logged measurement.

- 5.3.11 UE actions upon going to RRC_IDLE The UE shall: …… 1> release CSI-LoggedMeasurementConfig, if configured; 1> release loggedDataCollectionAssistanceConfig, if configured; 1> discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

Observation 1: In current spec, upon going to RRC_IDLE after detecting MCG RLF, UE removes network side data collection related configuration and stored logged measurement twice.

2) If UE performs re-establishment procedure after detecting MCG RLF, there are the following cases:

Case a: UE is not configured with attemptCondReconfig and attemptLTM-Switch;

Case b: UE is configured with attemptCondReconfig or attemptLTM-Switch but does not perform CHO recovery or LTM recovery (e.g., due to selected cell is not candidate cell);

Case c: UE is configured with attemptCondReconfig or attemptLTM-Switch and performs CHO recovery or LTM recovery successfully;

Case d: UE is configured with attemptCondReconfig or attemptLTM-Switch and performs CHO recovery or LTM recovery failed.

In Case a, after detecting MCG RLF, the UE removes network side data collection related configuration and stored logged measurement as follow, which cause duplicate UE action for removing network side data collection related configuration and stored logged measurement.

5.3.7.2 Initiation The UE initiates the procedure when one of the following conditions is met: …… 1> if UE is not configured with attemptCondReconfig; and 1> if UE is not configured with attemptLTM-Switch: 2> reset MAC; 2> release spCellConfig, if configured; 2> suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE or for L2 Last U2N Relay UE, except SRB0 and broadcast MRBs; 2> release the MCG SCell(s), if configured; 2> if MR-DC is configured: 3> perform MR-DC release, as specified in clause 5.3.5.10; 2> perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7; 2> release delayBudgetReportingConfig, if configured and stop timer T342, if running; 2> release overheatingAssistanceConfig, if configured and stop timer T345, if running; 2> release idc-AssistanceConfig, if configured; 2> release btNameList, if configured; 2> release wlanNameList, if configured; 2> release sensorNameList, if configured; …… 2> release loggedDataCollectionAssistanceConfig, if configured; 2> discard the logged measurement entries included in VarCSI-LogMeasReport, if any; ……

In Case b, after detecting MCG RLF, the UE removes network side data collection related configuration and stored logged measurement as follow, which cause duplicate UE action for removing network side data collection related configuration and stored logged measurement.

5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: …… 1> else: 2> if UE is configured with attemptCondReconfig; or 2> if UE is configured with attemptLTM-Switch: 3> reset MAC; 3> release spCellConfig, if configured; 3> release the MCG SCell(s), if configured; 3> release delayBudgetReportingConfig, if configured and stop timer T342, if running; 3> release overheatingAssistanceConfig , if configured and stop timer T345, if running; …… 3> release loggedDataCollectionAssistanceConfig, if configured; 3> discard the logged measurement entries included in VarCSI-LogMeasReport, if any; ……

In Case c, after detecting MCG RLF, the UE performs CHO recovery or LTM recovery successfully, the UE does not need to remove network side data collection related configuration and stored logged measurement automatically unless receiving explicit network indication considering that the RRC connection has been recovered, the UE action for how to handle network side data collection related configuration and stored logged measurement for Case c is similar as the corresponding UE action for handover successfully. However, based on 5.3.10.3 section (highlighted in yellow) in the current spec, the UE removes network side data collection related configuration and stored logged measurement even if the subsequent CHO recovery or LTM recovery performed successfully, which is not RAN2’s intention.

Observation 2: In current spec, the UE removes network side data collection related configuration and stored logged measurement if MCG RLF is detected even if the subsequent CHO recovery or LTM recovery performed successfully.

In Case d, after detecting MCG RLF, the UE performs CHO recovery or LTM recovery failed, UE performs re-establishment procedure. If UE performs CHO recovery failed, UE removes network side data collection related configuration and stored logged measurement as Case a as CHO configuration will be removed after performing CHO recovery; if UE performs LTM recovery failed, UE removes network side data collection related configuration and stored logged measurement as Case b as LTM configuration will be maintained after performing LTM recovery, but no LTM recovery again.

Observation 3: In current spec, in case of UE performs re-establishment procedure after detecting MCG RLF, UE removes network side data collection related configuration and stored logged measurement twice (for Case a, Case b, and Case d).

3) If UE performs MCG failure recovery procedure after detecting MCG RLF, there are the following cases:

Case a: UE receives RRCReconfiguration message and performs MCG failure recovery successfully;

Case b: UE receives RRCRelease message and going to RRC_IDLE or RRC_INACTIVE;

Case c: UE receives MobilityFromNRCommand and executes MobilityFromNRCommand successfully;

Case d: UE performs MCG failure recovery failed;

In Case a, the UE does not need to remove network side data collection related configuration and stored logged measurement automatically unless receiving explicit network indication considering that the RRC connection has been recovered, the UE action for how to handle network side data collection related configuration and stored logged measurement for MCG failure recovery successfully is similar as the corresponding UE action for handover successfully. However, based on 5.3.10.3 section (highlighted in yellow) in the current spec, the UE removes network side data collection related configuration and stored logged measurement even if the subsequent MCG failure recovery performed successfully, which is not RAN2’s intention.

Observation 4: In current spec, the UE removes network side data collection related configuration and stored logged measurement if MCG RLF is detected even if the subsequent MCG failure recovery performed successfully.

In Case b, if UE receives RRCRelease message and going to RRC_IDLE, UE removes network side data collection related configuration and stored logged measurement twice which is similar as UE going to RRC_IDLD directly when detects MCG RLF. If UE receives RRCRelease message and going to RRC_INACTIVE, according to the spec, the UE removes network side data collection related configuration and stored logged measurement as follow, which cause duplicate UE action for removing network side data collection related configuration and stored logged measurement.

5.3.8.3 Reception of the RRCRelease by the UE The UE shall: …… 1> if the RRCRelease includes suspendConfig: 2> reset MAC and release the default MAC Cell Group configuration, if any; 2> apply the received suspendConfig except the received nextHopChainingCount; …… 2> release CSI-LoggedMeasurementConfig, if configured; 2> release loggedDataCollectionAssistanceConfig, if configured; 2> discard the logged measurement entries included in VarCSI-LogMeasReport, if any; 1> else: 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'. NOTE 3: Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation. NOTE 4: It is left to UE implementation whether to stop T430, if running, when going to RRC_INACTIVE.

Observation 5: In current spec, in case of UE going to RRC_IDLE or RRC_INACTIVE after performing MCG failure recovery, UE removes network side data collection related configuration and stored logged measurement twice.

For Case c, if UE receives MobilityFromNRCommand, and executes MobilityFromNRCommand successfully, according to the spec, the UE removes network side data collection related configuration and stored logged measurement as follow, which cause duplicate UE action for removing network side data collection related configuration and stored logged measurement.

5.4.3.4 Successful completion of the mobility from NR Upon successfully completing the handover, at the source side the UE shall: …… 1> release the AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if stored; 1> release all radio resources, including release of the RLC entity and the MAC configuration; 1> release the associated PDCP entity and SDAP entity for all established RBs; NOTE : PDCP and SDAP configured by the source RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-system handover when fullConfig is not present) is used, are not released as part of this procedure. 1> if the UE is configured with any application layer measurement configuration: 2> inform upper layers about the release of all application layer measurement configurations; 2> release all application layer measurement configurations including their fields in UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig; 2> discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission; 2> consider itself not to be configured to send application layer measurement reports; 1> discard any logged measurement entries included in VarCSI-LogMeasReport; 1> if the targetRAT-Type is set to eutra and the nas-SecurityParamFromNR is included: or 1> if the targetRAT-Type is set to utra-fdd: 2> indicate the release of the RRC connection to upper layers together with the release cause 'other'.

Observation 6: In current spec, in case of UE executes MobilityFromNRCommand successfully after performing MCG failure recovery, UE removes network side data collection related configuration and stored logged measurement twice.

In case d, if UE performs MCG failure recovery failed, UE performs re-establishment procedure, the UE’s action of handling network side data collection related configuration and stored logged measurement is similar as 2) UE performs re-establishment procedure directly when UE detects MCG RLF.

Observation 7: In current spec, if UE performs MCG failure recovery failed, UE performs re-establishment procedure, the UE’s action of handling network side data collection related configuration and stored logged measurement is similar as UE performs re-establishment procedure directly when UE detects MCG RLF.

Based on Observation 1~Observation 7 above, we propose to delete the procedure description of removing network side data collection related configuration and stored logged measurement in  5.3.10.3 section. A TP is provided in Annex.

Proposal 1: To agree RIL [C074], i.e., delete the procedure description of removing network side data collection related configuration and stored logged measurement in  5.3.10.3 section.

- Conclusion

According to the discussion in section 2, we propose:

Observation 1: In current spec, upon going to RRC_IDLE after detecting MCG RLF, UE removes network side data collection related configuration and stored logged measurement twice.

Observation 2: In current spec, the UE removes network side data collection related configuration and stored logged measurement if MCG RLF is detected even if the subsequent CHO recovery or LTM recovery performed successfully.

Observation 3: In current spec, in case of UE performs re-establishment procedure after detecting MCG RLF, UE removes network side data collection related configuration and stored logged measurement twice (for Case a, Case b, and Case d).

Observation 4: In current spec, the UE removes network side data collection related configuration and stored logged measurement if MCG RLF is detected even if the subsequent MCG failure recovery performed successfully.

Observation 5: In current spec, in case of UE going to RRC_IDLE or RRC_INACTIVE after performing MCG failure recovery, UE removes network side data collection related configuration and stored logged measurement twice.

Observation 6: In current spec, in case of UE executes MobilityFromNRCommand successfully after performing MCG failure recovery, UE removes network side data collection related configuration and stored logged measurement twice.

Observation 7: In current spec, if UE performs MCG failure recovery failed, UE performs re-establishment procedure, the UE’s action of handling network side data collection related configuration and stored logged measurement is similar as UE performs re-establishment procedure directly when UE detects MCG RLF.

Proposal 1: To agree RIL [C074], i.e., delete the procedure description of removing network side data collection related configuration and stored logged measurement in  5.3.10.3 section.

- References

- 38331 Comments file v077

- 38331 Review file v077

- TS38.331-j00

Annex

The suggested change is based on the TS38.331 version of “TS38.331-j00” [3].

FIRST CHANGE

5.3.10.3	Detection of radio link failure

The UE shall:

1>	if any DAPS bearer is configured and T304 is running:

2>	upon T310 expiry in source SpCell; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon consistent uplink LBT failure indication from source MCG MAC:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

3>	suspend the transmission and reception of all DRBs and multicast MRBs in the source MCG;

3>	reset MAC for the source MCG;

3>	release the source connection.

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

2>	upon T312 expiry in PCell; or

2>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running and SDT procedure is not ongoing; or

2>	upon indication from MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is not ongoing; or

2>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the MCG; or

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

3>	if the indication is from MCG RLC and CA duplication is configured and activated for MCG, and for the corresponding logical channel allowedServingCells only includes SCell(s):

4>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

3>	else:

4>	consider radio link failure to be detected for the MCG, i.e. MCG RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

NOTE 1:	Void.

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	if MP is configured:

6>	if T316 is configured, and MP indirect path transmission is not suspended; and

6>	if neither MP indirect path change nor MP indirect path addition is ongoing:

7>	initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure.

6>	else:

7>	initiate the connection re-establishment procedure as specified in 5.3.7.

5>	else:

6>	if the UE supports RLF-Report for fast MCG recovery procedure and if T316 is configured:

7>	if the SCG is deactivated at the moment of detecting RLF in the MCG:

8>	set the mcg-RecoveryFailureCause in the VarRLF-Report to scg-Deactivated;

8>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

7>	else if SCG transmission is suspended at the moment of detecting RLF in the MCG:

8>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

8>	set the scg-FailureCause value in the VarRLF-Report according to 5.7.3.5;

8>	set the elapsedTimeSCG-Failure in the VarRLF-Report to the time elapsed between SCG failure and the MCG failure;

6>	if T316 is configured; and

6>	if SCG transmission is not suspended; and

6>	if the SCG is not deactivated; and

6>	if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC):

7>	initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure.

6>	else:

7>	initiate the connection re-establishment procedure as specified in 5.3.7.

A L2/L3 U2N Relay UE in case of single hop or the L2 Last U2N Relay UE shall:

1>	upon detecting radio link failure:

2>	either indicate to upper layers (to trigger PC5 unicast link release with its child UE(s)) or send NotificationMessageSidelink to the connected L2/L3 U2N Remote UE(s) or to the child UE(s)) in accordance with 5.8.9.10.

A N3C Relay UE shall:

1>	upon detecting radio link failure:

2>	indicates to the associated N3C remote UE via the Non-3GPP Connection.

NOTE 2:	How the N3C Relay UE indicates Uu RLF on the Non-3GPP Connection is left to implementation.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon T312 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached; or

1>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the SCG; or

1>	upon consistent uplink LBT failure indication from SCG MAC:

2>	if the indication is from SCG RLC and CA duplication is configured and activated for SCG, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	if the SCG is deactivated:

4>	stop radio link monitoring on the SCG;

4>	indicate to lower layers to stop beam failure detection on the PSCell;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

3>	else:

4>	if the UE is in NR-DC:

5>	if the UE supports RLF-Report for fast MCG recovery procedure and if the UE detected SCG failure while the timer T316 was running:

6>	set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell;

6>	set the scg-FailureCause in the VarRLF-Report value according to 5.7.3.5;

6>	set the elapsedTimeSCG-Failure in the VarRLF-Report to the time elapsed between MCG failure and the SCG failure;

6>	include scg-FailedAfterMCG;

5>	initiate the connection re-establishment procedure as specified in 5.3.7;

4>	else (the UE is in (NG)EN-DC):

5>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7;

CHANGE END