3GPP TSG-RAN WG2 Meeting #132	R2-2508918

Dallas, TX, USA, Nov 17th – 21st, 2025

Agenda item:		10.3.3.1 Data transfer, model transfer, and AIML

Source:		Futurewei

Title:		Discussion on data transfer, model transfer, and AI/ML framework

Document for:		Discussion and Decision

# Introduction

Regarding data collection/transfer, during RAN2 meeting #131bis [1], the discussions were focused on the following topics.

- Core Principles and Requirements for AI/ML Data Framework

- Requirements for Minimizing UE Impact

- Data Transfer Volume and Efficiency

Discussions on model transfer requirements and architecture aspect were also on the meeting plan but were not discussed due to lack of time.

The discussion resulted the following agreement.

Agreements

1.  	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

- This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

- The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

- UE data collection should minimize impact to user traffic transmission and power saving features

These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.   Study model transfer/delivery requirements/functions

For the next meeting (i.e., RAN2 #132, Nov 2025), companies agreed to consider at least the following aspects on data collection.

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

- For each type of data, study:

  - Applicable use case(s)

  - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

  - Data size in a single reporting and total data size

  - Frequency of data reporting;

  - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

Regarding AI/ML framework, LCM and requirements were discussed. [2] was the only TDoc treated and the base for discussion. The theme of the proposals is to design a generic and extensible 6G AI/ML framework, enabling new AI/ML use cases to be easily introduced and integrated, and supporting both one-sided models (including UE-sided and NW-sided models) and two-sided models.

The discussion resulted the following agreement.

Agreements

Study 6G AI/ML LCM framework that supports at least the following functions/procedures:

- UE AI/ML Capabilities Exchange;

- Applicable Functionality Reporting;

- Inference Configuration and Reporting;

- Performance Monitoring Configuration and Reporting;

- Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In this paper, we discuss the diverse types of data related to AI/ML, model transfer requirements, and general AI/ML framework consideration for 6G RAN.

# AI/ML Data Collection and Transfer

In this sections, we summarize model input and output data types for all RAN1 use cases submitted to Rel-20 6GR. For each data type, we subtract data characteristics and requirements for data collection and transfer.  Note that RAN2 and RAN3 have not determined the use cases to be studied in 6G so RAN1 use cases are the only ones available for analysis at the moment.

## Data Types Used in RAN1 AI/ML Rel-20 Use Cases

The following table (Table 1) shows input and output data of all 37 use cases (after consolidation) that have been proposed to Rel 20 6GR in RAN1 discussions. Please note that these use cases are not very stable yet and most will be excluded from 6G study in future meetings. However, they do provide a snapshot of the use cases and their related input and output data types, and hence worth the study. Please keep in mind that these use cases are subject to change in subsequent meetings.

Also note that at least some inputs/outputs for/from the identified sub-cases will not have spec impact. Even if there is, there may not have RAN2 impact, e.g., internally generated and used by gNB/UE only.

More details of these use cases can be found in the Chair’s notes in [5].

Table 1: RAN1 Use Cases for Rel-20 6GR Study Item and Input/Output Data

| Use Case # | Use Case | Model Input | Model Output |
| --- | --- | --- | --- |
| 1 | Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML | 1. Measurement of channel with sparse/low overhead CSI-RS (majority) 1a. Additional long-term multi-path power, angle, and/or delay info information as assistance information 2. Reported CSI for NW-sided model | 1. Full channel matrix (majority) 2. Eigenvector for NW-sided model 3. Channel matrix/ eigenvector with different/targeted antenna on/off patterns |
| 2 | CSI time domain prediction (as Rel-19 CSI prediction or extension) | 1. Channel matrix over K CSI-RS occasions 2. Measurements of interference over K CSI-RS occasions1 3. Channel matrix over K CSI-RS occasions with >20ms periodicity 4 Channel matrix with one P CSI-RS with 20ms periodicity and K-1 AP CSI-RS | 1. Channel matrix of future instances 2. Interference in future instances |
| 3 | CSI prediction cross carrier/band/frequency band | Channel matrix of carrier/band/frequency block A | Channel matrix of carrier/band/frequency block B |
| 4 | CSI prediction across analog beams | Channel matrix of Set B of beams | Channel matrix of Set A of beams |
| 5 | CSI prediction with linear projection as pre-processing | K past CSI information after linear projecting | Predicted CSI information after linear projecting at a future time instance |
| 6 | Sparse orthogonal DMRS in frequency and/or time domain | 1. Received signal/estimated channel at DMRS and received signal on data 1a. additionally noise variance 2. Received signal/estimated channel at DMRS | 1. Estimated channel at target data and/or DMRS REs 1a. Estimated noise variance 2. LLRs 3. Filtering coefficients for channel estimation |
| 7 | Superimposed pilot for low-overhead DMRS | 1. Received signal and DMRS sequence (superimposed signal) (Majority) 2. Estimated channel (in delay doppler domain) from the received signal of target REs (superimposed signal) For Tx side of two-sided model: modulated symbols and DMRS symbol | 1. Estimated channel at target data REs 2.LLR 3.Estimated modulation symbols For Tx side of two-sided model: superimposed signal |
| 8 | DMRS-free approach | Received signal For Tx side of two-sided model: coded bit | 1.LLR (majority) 2.Estimated channel For Tx side of two-sided model: modulated data symbols |
| 9 | CSI compression with JSCC | 1. Compressed CSI bits 1a. additionally estimated channel based on SRS 1b. (for training), assuming the model input via error bits caused by in UL transmission after legacy channel decoding | 1. (Reconstructed) Eigenvectors 2. (Reconstructed) Explicit H |
| 10 | CSI compression with JSCM | 1. Compressed CSI complex values via UE-sided model 2. Compressed CSI complex values via a projection matrix 3. Received signal at sparse CSI-RS and CSI-RS sequence | 1. (Reconstructed) Eigenvectors 2. (Reconstructed) Explicit H |
| 11 | DLable basis/codebook | 1.Amplitudes and phases obtained by a look up table based on feedback CSI bits 2. Selected basis | Reconstructed Eigenvectors |
| 12 | CSI reconstruction with CSI feedback with SRS (assuming SSCC) | 1. Compressed CSI bits 2. Estimated channel based on SRS | Reconstructed Eigenvectors |
| 13 | AI-based (de)modulation | For constellation design: 1. Coded bits 2. Channel characterization and modulation order For AI receiver: 1.Received signal | For constellation design: 1. Learned constellation 2. Probability of constellation points For receiver: 1. LLR |
| 14 | AI-based modulation and precoding | Encoder: Coded bits Decoder: Estimated symbols | Encoder: modulated symbols after layer mapping Decoder: Soft LLR |
| 15 | AI-based DPoD/None-linearity compensation | 1. Received signal | 1. Compensated signal in time domain 2. Soft bits |
| 16 | AI-based DPD | Time domain samples before pre-distortion | Time domain samples after pre-distortion |
| 17 | Low overhead SRS with AI/ML | Measurement of channel with low overhead SRS of frequency/temporal domain | Estimated channel |
| 18 | Low PAPR SRS sequence design | Sequence index | Learn sequences |
| 19 | AI-enabled UL precoder indication | UL precoder indicator /compressed UL precoder | (Reconstructed) eigenvectors of UL channel |
| 20 | AI/ML based waveform for PAPR reduction | Symbols in frequency domain | For model output of encoder for UE-sided/NW-part of two-sided model: transformed/ precoded symbols in frequency domain For output of decoder for NW-part of two-sided model: 1. LLR 2. Symbols in frequency domain |
| 21 | AI/ML based HARQ-ACK feedback | HARQ ACK/NACK bit sequence | Learned sequences/modulated symbols |
| 22 | Improved scheduling/HARQ for token traffic | Input for tokenizer model: Raw data (e.g., image/video/audio, etc.) Input for de-tokenizer model: Tokens | Output for tokenizer model: Tokens (e.g., tokenized image/video/audio) Output for de-tokenizer model: Inference results for downstream tasks/Raw data (e.g., image/video/audio, etc.) |
| 23 | Beam Management： Inter-Cell/M-TRP DL Tx beam prediction and management | Measurements from Set B of one or more TRPs/Cells | Predicted best beam information and/or predicted measurements from Set A of target cell/TRP(s) [of current or future time instance] |
| 24 | Beam Management：Cross frequency DL Tx beam prediction | Measurements in frequency A | Predicted cell/beam related information of frequency B [of current or future time instance] |
| 25 | Beam Management：Tx-Rx beam pair prediction | Measurements from Set B DL Tx-Rx beam pairs. | Predicted best DL Tx-Rx beam pairs information from Set A DL Tx-Rx pairs. |
| 26 | Beam Management：Beam prediction for initial access | Measurements from Set B of SSB | Predicted best DL Tx beam information (and/or predicted measurements from Set A [of current or future time instance] |
| 27 | Beam Management：DL Tx beam prediction for spatial and/or temporal domain with additional local UE information | Measurements from Set B And additional local UE information (moving direction and speed) as UE side model input | Predicted Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] |
| 28 | Beam Management：reinforcement learning-based approach beam selection | Measurements from a set of DL Tx beam scheduling stats (at the NW), Cross correlation among DL Tx beams | Selected beam index for scheduling UE(s) |
| 29 | Prior-Information-Aided DCI Decoding | LLR after demodulation at current transmission, and historical AI/ML based predicted LLR | Decoded DCI payloads, and predicted LLR for next transmission |
| 30 | Lossless DCI Compression | Historical DCI payload | Predicted DCI payload |
| 31 | UL closed-loop power control with an NW-sided AI/ML model, where the model predicts the optimal power adjustment (or TPC command index) for the UE. | UL SINR measurement, UE Tx power estimate (derived from Pcmax, P0, PL alpha, pathloss measurement), and PUSCH allocation size | Predicted TPC command index |
| 32 | Pathloss prediction in the spatial, temporal, and/or frequency domain, to use the predicted pathloss in UL(PUSCH/PUCCH/PRACH/SRS) power control. | L1-RSRP measurements from a sub-set/set of RSs/beams (Set B). input can consider history of measurements | Predicted pathloss value(s) (or predicted L1-RSRP(s)) for a set of RSs/beams (Set A). output can consider future instances |
| 33 | Early contention resolution in RACH | Received PRACH signal (e.g., preamble waveform) | Predicted number of UEs that transmitted the same preamble for given PRACH resources |
| 34 | Sensing based RAN digital twin construction with NW-side AI/ML model | Point cloud sensed by the BS with mono-static sensing and sensed/reported by UEs with bi-static sensing | 3D point cloud representing the static environment |
| 35 | AI/ML-enabled RAN digital twin with distributed model | UE-part models: local sparse point cloud NW-part model: latent space information from multiple UEs | UE-part models: compressed latent space information NW-part model: global point cloud |
| 36 | AI/ML based SRS power imbalance compensation | UL measured channel matrix from SRS with IL imbalance | DL channel matrix with IL compensated |
| 37 | Site Specific Learning for AI/ML and RAN Digital Twin | Received signal/estimated channel at DMRS and received signal on data, and the channel information generated by digital twin | Decoded bit |

## Model Input Data Types

Table 2 re-presents the extraction of model input data from Table 1, organizing the data into different categories and use case numbers from Table 1.

Table 2: Input Data Type Category for RAN1 Rel-20 Use Cases

| Input Data Type Category | Detailed Description (Includes examples and variations) | Supporting Use Case Sources |
| --- | --- | --- |
| Channel State Information (CSI) / Channel Measurements | Measurement of channel with sparse/low overhead CSI-RS; Reported CSI for NW-sided model; Long-term multi-path power, angle, and/or delay information; K past CSI information after linear projecting; Measurement of channel with low overhead SRS. | 1, 5, 17 |
| Channel Matrix / Interference | Channel matrix over K CSI-RS occasions; Measurements of interference over K CSI-RS occasions; Channel matrix over K CSI-RS occasions with >20ms periodicity; Channel matrix with one P CSI-RS and K-1 AP CSI-RS; Channel matrix of carrier/band/frequency block A; Channel matrix of Set B of beams; UL measured channel matrix from SRS with IL imbalance. | 2, 3, 4, 36 |
| Received Signal / Estimated Channel / Noise | Received signal (e.g., received PRACH signal/preamble waveform, received signal on data); Estimated channel (at DMRS or in delay doppler domain); Additionally noise variance; Received signal at sparse CSI-RS and CSI-RS sequence. | 6, 7, 8, 10, 13, 15, 33, 37 |
| Coded Bits / Symbols | Coded bits (for constellation design or encoder input); DMRS symbol (for Tx side); Symbols in frequency domain; Estimated symbols (for decoder input); Coded bit (for Tx side). | 7, 8, 13, 14, 20 |
| Compressed / Feedback Data | Compressed CSI bits; Compressed CSI complex values (via UE-sided model or projection matrix); Amplitudes and phases obtained by a look up table based on feedback CSI bits; Compressed UL precoder. | 9, 10, 11, 12, 19 |
| Measurements (L1-RSRP / Beam Management) | L1-RSRP measurements from a sub-set/set of RSs/beams (Set B); Measurements from Set B of one or more TRPs/Cells; Measurements in frequency A; Measurements from Set B DL Tx-Rx beam pairs; Measurements from Set B of SSB; Measurements from a set of DL Tx beam scheduling stats. | 23, 24, 25, 26, 27, 28, 32 |
| Sensing Data / Point Cloud | Point cloud sensed by the BS with mono-static sensing; Point cloud sensed/reported by UEs with bi-static sensing; Local sparse point cloud (UE-part models). | 34, 35 |
| System Status / Control Information | UL precoder indicator; HARQ ACK/NACK bit sequence; UL SINR measurement; UE Tx power estimate; PUSCH allocation size; Channel characterization and modulation order. | 13, 19, 21, 31 |
| Raw/Payload Data | Raw data (e.g., image/video/audio, etc.); Tokens (input for de-tokenizer model); Historical DCI payload; Time domain samples before pre-distortion. | 16, 22, 30 |
| AI/ML Internal Data | Error bits caused by in UL transmission after legacy channel decoding (for training); Selected basis; Latent space information from multiple UEs (NW-part model input); Historical AI/ML based predicted LLR. | 9, 11, 29, 35 |
| Contextual/Metadata Information | Additional local UE information (moving direction and speed); Cross correlation among DL Tx beams; Sequence index; Channel information generated by digital twin. | 18, 27, 28, 37 |
| Log-Likelihood Ratio (LLR) | LLR after demodulation at current transmission. | 29 |

Observation 1: For RAN1 Rel-20 6GR SI, model input data for the proposed use cases can be classified into the following categories.

- Channel State Information (CSI) / Channel Measurements

- Channel Matrix / Interference

- Received Signal / Estimated Channel / Noise

- Coded Bits / Symbols

- Compressed / Feedback Data

- Measurements (L1-RSRP / Beam Management)

- Sensing Data / Point Cloud

- System Status / Control Information

- Raw/Payload Data

- AI/ML Internal Data

- Contextual/Metadata Information

- Log-Likelihood Ratio (LLR)

## Model Output Data Types

Table 3 re-presents the extraction of model output data from Table 1, organizing the data into different categories and use case numbers from Table 1.

Table 3: Output Data Type Category for RAN1 Rel-20 Use Cases

| Output Data Type Category | Detailed Description (Includes examples and variations) | Supporting Use Case Sources |
| --- | --- | --- |
| Channel Matrix / Eigenvectors / CSI | Full channel matrix; Eigenvector for NW-sided model; Channel matrix/eigenvector with different/targeted antenna on/off patterns; Channel matrix of future instances; Channel matrix of carrier/band/frequency block B; Channel matrix of Set A of beams; Predicted CSI information; (Reconstructed) Eigenvectors; (Reconstructed) Explicit H; (Reconstructed) eigenvectors of UL channel; DL channel matrix with IL compensated. | 1, 2, 3, 4, 5, 9, 10, 11, 12, 19, 36 |
| Estimated Channel / Noise | Estimated channel (at target data and/or DMRS REs, or general); Estimated noise variance. | 6, 7, 8, 17 |
| Log-Likelihood Ratio (LLR) | LLRs; Soft LLR; Predicted LLR for next transmission. | 6, 7, 8, 13, 14, 20, 29 |
| Beam Information / Measurements (Predicted) | Predicted best beam information; Predicted measurements (from Set A); Predicted cell/beam related information of frequency B; Predicted best DL Tx-Rx beam pairs information; Predicted Best beam indexes (probability of each Tx beam); Selected beam index for scheduling UE(s). | 23, 24, 25, 26, 27, 28 |
| Symbols / Signal | Estimated modulation symbols; Superimposed signal (for Tx side); Modulated data symbols (for Tx side); Modulated symbols after layer mapping; Compensated signal in time domain; Transformed/precoded symbols in frequency domain; Symbols in frequency domain; Modulated symbols. | 7, 8, 14, 15, 20, 21 |
| Sequences / Coefficients | Filtering coefficients for channel estimation; Learned constellation; Probability of constellation points; Learned sequences. | 6, 13, 18, 21 |
| Data / Decoded Information | Soft bits; Decoded DCI payloads; Predicted DCI payload; Decoded bit. | 15, 29, 30, 37 |
| Tokens / Raw Data / Inference | Tokens (e.g., tokenized image/video/audio); Inference results for downstream tasks; Raw data (e.g., image/video/audio, etc.). | 22 |
| Sensing Data / Point Cloud (3D) | 3D point cloud representing the static environment; Global point cloud (NW-part model). | 34, 35 |
| Prediction Outcomes (Non-Channel) | Interference in future instances; Predicted TPC command index; Predicted pathloss value(s) (or predicted L1-RSRP(s)); Predicted number of UEs that transmitted the same preamble. | 2, 31, 32, 33 |
| AI/ML Internal Data | Compressed latent space information (UE-part model output). | 35 |
| Time Domain Samples | Time domain samples after pre-distortion. | 16 |

Observation 2: For RAN1 Rel-20 6GR SI, model output data for the proposed use cases can be classified into the following categories.

- Channel Matrix / Eigenvectors / CSI

- Estimated Channel / Noise

- Log-Likelihood Ratio (LLR)

- Beam Information / Measurements (Predicted)

- Symbols / Signal

- Sequences / Coefficients

- Data / Decoded Information

- Tokens / Raw Data / Inference

- Sensing Data / Point Cloud (3D)

- Prediction Outcomes (Non-Channel)

- AI/ML Internal Data

- Time Domain Samples

## Discussion of Data Endpoints

Based on the use cases submitted to RAN1 for 6G SI, end points are the gNB and UE for all types of data, except for the one-sided models. Table 4 breaks down RAN1 use cases by model types.

Table 4: Use Cases by Model Types

| Model Type | Count | Use Case Numbers |
| --- | --- | --- |
| One-sided (NW-side) | 25 | 1, 2, 3, 4, 5, 6, 7*, 8*, 15, 16, 17, 18, 19, 23, 24, 25, 26, 27, 28, 31, 33 |
| One-sided (UE-side) | 2 | 29, 30 |
| Two-sided | 10 | 7, 8, 13, 14, 20, 21, 22, 34, 35, 37 |

Note: UC7 and UC8 are hybrid: majority NW-side, but two-sided variant explicitly mentioned.

Observation 3: Two-sided models are utilized for many RAN1 use cases for 6G (10 out of 37).

While the end points for RAN1 use cases are mostly between the NW and the UE, use cases of other work groups (e.g., RAN2 and RAN3, which have not decided their use cases for 6G) may have different endpoints, such as CN, OAM, and outside server etc. In fact, as we move into the era of 6G, AI/ML use cases can be a lot richer than in 5G, which implies collecting data from wireless networks may face challenges from both the business side and the technical side.

From the business side, 6G's inherent perception and intelligence, enabled digital twins, holographic communications, industrial IoT and other new applications bring massive perception data and AI data, as well as network operation data. These non-communication session data contain huge value and require efficient data control mechanisms to realize data monetization. As a core asset, 6G networks will drive network intelligence, data sharing and monetization, improve the network's own operational efficiency.

From the technical side, 6G's new data types bring challenges to the existing user plane; the existing communication network lacks a normalized data service framework and cannot process the newly added perception data and AI model data of 6G. In addition, the data collection mechanism and processing functions in the existing architecture are rigid and lack flexibility and cannot meet the needs of "on-path computing" (i.e., processing functions exist in the relaying nodes when delivering data from one end point to another) and "arbitrary topology" (i.e., any node in the network can be the data producer/source and data consumer/destination).

Proposal 1. Study traffic distribution of 6G and whether existing session-centric framework for data collection and transfer (i.e., using UP tunnels and PDU sessions) needs to be enhanced/changed (e.g., to session-less framework) to accommodate the new traffic distribution (if there is one) and diverse types of data/services.

Proposal 2. Study the necessity of on-path computing/processing (e.g., functions such as data fusion and de-privacy) for data collection and transfer in 6G.

Proposal 3. Study the necessity of supporting arbitrary topology for data collection/delivery and service in 6G.

## Session-Less Framework for Data Collection/Transfer

Under the data collection framework of 5G/5G-A, the CP handles tasks such as establishing connections and controlling the forwarding of UP data (commonly referred to as session management within the 5G core). The UP is designed to forward network packets to their intended destinations efficiently. Both the CP and UP operate within a communication session-centric framework; the CP sets up UP data tunnels for each session, facilitating signaling message exchange among various entities, including UEs, BSs, and CN functions, to maintain state synchronization.

Figure 1. The architecture of 5G

As shown in Figure 1, the SBI acts as a communication bus, using the HTTP/2 protocol for communication across all the CP functions. HTTP/2 connections are peer-to-peer connections, with messages flowing in both directions between NFs.

Figure 2. Data transfer in 5G

As shown in Figure 2, data traffic on the UP is carried by protocol data unit (PDU) sessions established between the UEs and the user plane function (UPF) deployed in the core network (CN) across the radio access network (RAN).

This existing CP/UP framework/collaboration for data collection may not be enough to support data collection needs in 6G.

Proposal 4. If session-less framework is necessary for 6G, study the changes/enhancements needed for CP and UP, or whether a new plane dedicated to data collection/management (i.e., data plane) is needed.

Proposal 5. Study necessary enhancements to network elements (e.g., NB, UE and CN) to enable session-less transmission and support of arbitrary topology (if supported) for data collection/delivery and service in 6G.

# Model Transfer Requirements

In our view the discussion regarding Model Transfer/Delivery (MT/D) in 6G is primarily driven by the limitations encountered in 5G-A and the necessity of establishing a unified, scalable framework to handle the complex requirements of future AI-native systems and models, particularly two-sided models.

## Background of 5G/5G-A Study on Model Transfer

In 5G-A discussions, MT/D was studied in Release 18/19 but faced significant challenges on the following two aspects:

- Lack of Specification: Model transfer/delivery procedures (including request, transfer, and update) were discussed for both one-sided and two-sided models in 5G, but were generally not specified.

- Lack of Standardization Motivation: RAN2 spent substantial time discussing model transfer, but standardization was not pursued due to a perceived lack of motivation; for instance, UE-side models could often be transferred transparently via the application layer (Over-The-Top, or OTT) without 3GPP specification impact.

For 6G, Model Transfer is considered an essential LCM activity that must be standardized, especially because it is an important feature for supporting two-sided models.

## Problems and Limitations of Model Transfer in 5G-A

The existing 5G radio protocol architecture (Control Plane and User Plane) is deemed inadequate for handling the requirements of AI/ML model transfer due to limitations regarding data volume, routing, and complexity.

### Limitations on Large Data Volume

AI/ML model transfer/delivery often involves models or model parameters that are characterized as a large model with large number of parameters. Existing 5G transport mechanisms fail to support this:

- Control Plane (CP) Limitations: The existing Signaling Radio Bearers (SRBs) used by the Control Plane cannot transmit large models or large volumes of data. This is because RRC signaling has a limited maximum size (e.g., 144kbytes uplink / 45kbytes downlink). Attempting to use multiple RRC messages for large transfers leads to signaling overload.

- User Plane (UP) Limitations: Existing Data Radio Bearers (DRBs) are associated with PDU sessions that terminate at the UPF. Therefore, the large models trained at the RAN node (gNB) cannot be transferred directly to the UE via DRB.

- Lack of Convergence: In 5G discussions, neither the CP nor the UP schemes converged into a widely accepted solution for downlink AI model transfer/delivery.

### Implementation Complexity and Management

- UE Burden: Model transfer becomes critical when a UE has memory limitations that prevent it from storing all related models, necessitating frequent downloads.

- Multicast/Broadcast Complexity: Schemes utilizing MBS multicast/broadcast to simultaneously deliver dataset/model parameters from the RAN node to a group of UEs may introduce additional complexity in scheduling and handling.

## Proposals

Based on the discussion above, we have the following proposals for model transfer.

Proposal 6: 6G to study unifying data collection and model/dataset transfer in the same framework to achieve efficiency and eliminate redundancy.

Proposal 7: 6G to study handling large volume of model/dataset for transfer. Consider the following two solutions,

Proposal 8: 6G to study the mechanism to guarantee different QoS requirements for model/dataset transfer, depending on characteristics of the learning mechanisms.

- This includes QoS requirements for online training and federated learning, which may impose higher QoS requirements than traditional offline training.

# AI/ML Framework

In our view the discussions regarding the 6G AI/ML framework and Lifecycle Management (LCM) procedures should focus on overcoming the fragmentation and limitations inherited from 5G-Advanced, aiming for a unified, extensible, and efficient system in 6G.

## Challenges and Problems with the 5G AI/ML Framework and LCM

The prevailing challenge identified across the sources is that the initial integration of AI/ML in 5G was largely retroactive and siloed.

### Lack of Unified LCM and Scalability

The fundamental problem in 5G is that AI/ML capabilities were introduced following a use-case-specific design paradigm. This approach resulted in some significant problems:

- Fragmentation and Duplication: Each AI/ML use case (e.g., beam management, mobility, positioning) often developed its own specific framework, procedures, data pipeline, and computational resources. This fragmentation leads to specifications being fragmented and duplicated, requiring redundant discussions across Working Groups (WGs).

- Operational Complexity: Network operators must manage countless individual models, each having separate KPIs, update cycles, and failure modes, leading to an OPEX explosion.

- Limited Extensibility: The use-case-specific approach consumes significant time when a new AI/ML use case is introduced, requiring re-discussion for integration. The current design is not conducive to rapid and scalable deployment.

### Limitations in Model Training and Control

Current mechanisms also present limitations concerning the lifecycle elements themselves:

- Model Burden and Complexity: UEs are tasked with managing AI/ML models, necessitating avoidance of scenarios that require UEs to maintain an excessive number of models or model parameters, which consumes significant UE storage and complicates management.

- Training Limitations: 5G primarily focused on offline training. This approach is inherently limited in generalization capabilities when facing time-varying wireless channel conditions, often requiring fallbacks to non-AI solutions.

- Limited UE Autonomy: The 5G LCM framework mostly operates under "Network decision, network-initiated AI/ML management". Although discussions covered "UE-initiated" management solutions, they were not concluded due to limited resources, despite the justification that the UE has better local observation and internal device information that cannot be shared with the Network.

- Switching Overhead: Fallback or switching between AI/ML and non-AI/ML functions currently involves latency due to the exchange of multiple RRC messages, which impacts overall network performance. An efficient/light-weight mechanism to guarantee seamless switching is required.

- Applicability Reporting Inefficiency: The optimization offered by signaling "inference-related parameters" for applicability determination is minimal, as it still requires an additional round of messages and configuring most parameters.

## Discussions and Proposals

### Two-sided models

A two-sided model is defined as a paired AI/ML model(s) over which joint inference is performed. This joint inference process occurs jointly across the UE and the NW. Specifically, the inference workflow involves the first part of inference being performed by the UE and then the remaining part being performed by the gNB, or vice versa.

As discussed in Section 2.4, some options/sub-cases in the identified potential 6GR use cases leverage two-sided model mechanism (10 out of 37 use cases), e.g., “frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML”, “low overhead DMRS with AI/ML receiver with superimposed pilot”.

Generally speaking, two-sided models have the potential to achieve global optimization and deliver enhanced performance gains that are challenging to realize with traditional or one-sided models trained and updated independently..

- Enhanced system performance and joint optimization (of models at both sides)

- Handling dynamic propagation environment changes effectively

- Continuous learning/update at both sides supports fine-tuning and adaptation of UE and NW AI/ML-based algorithms to respond to changing network conditions

Despite their benefits, two-sided models introduce challenges that need to be studied and addressed. These challenges include, but not limited to,

- synchronization, including model pairing/alignment,

- increased signaling overhead,

- complex inter-vendor collaboration compared to one-sided models,

- LCM (e.g. performance monitoring, root cause identification, fallback procedures on both sides, switching models for one or both sides.)

- incurred additional signaling overhead due to the above

Some of these challenges, such as inter-vendor collaboration, have been discussed in 5G/5G-A. However, more aspects related to two-sided models need to be further studied and concluded to ensure their successful applications in wireless networks.

Proposal 9. RAN2 to further study how to provide optimized/simplified support of two-sided models in 6G.

### AI/ML techniques beyond supervised learning

More AI/ML use cases are expected to be developed in 6G time frame with new AI/ML learning types.

An enhanced AI/ML framework needs to support diverse use cases, both 1-sided or 2-sided, including model (parameters) and dataset exchange when applicable. In addition, as most AI/ML models belong to vendor implementation and AI/ML technology is continuously evolving, the framework should not be limited to supporting only some types of model structures.

We believe the 6G AI/ML framework needs to support various learning techniques. In addition to supervised learning which is leveraged in all 5G NR AI/ML use cases, other learning techniques to consider may include (but not limited to):

- Online or continuous learning, e.g., reinforcement learning.

- Decentralized learning, e.g., federated learning.

- Distributed learning

Reinforcement Learning (RL) and Deep RL have the potential to be one of the major AI/ML approaches for improving high-layer functions/features. Digital Twin, as a powerful tool for RL/DRL model training/inference, is often an integrated part of a RL/DRL-based system for AI/ML applications in wireless networks. Because of this, ITU-T has adopted the following reference architecture of digital twin for 6G, as shown in Figure 2.

Figure 2. A reference 6G digital twin network architecture based on ITU-T recommendation Y.3090

The necessity of supporting AI/ML approaches other than supervised learning and digital twin has also reflected in the use cases submitted to 6G. In the RAN1 use cases listed in Table 1, multiple use cases adopted approaches other than supervised learning and the integration of digital twin. For example,

- Use case #28: Reinforcement learning-based approach beam selection

- Use case #34: AI/ML-enabled RAN digital twin with distributed model

- Use case #35: AI/ML-enabled RAN digital twin with distributed model

- Use case #37: Site-specific learning for AI/ML and RAN digital twin

Proposal 10: Support more AI/ML techniques beyond supervised learning, as well as digital twin.

# Conclusions

Observation 1: For RAN1 Rel-20 6GR SI, model input data for the proposed use cases can be classified into the following categories.

- Channel State Information (CSI) / Channel Measurements

- Channel Matrix / Interference

- Received Signal / Estimated Channel / Noise

- Coded Bits / Symbols

- Compressed / Feedback Data

- Measurements (L1-RSRP / Beam Management)

- Sensing Data / Point Cloud

- System Status / Control Information

- Raw/Payload Data

- AI/ML Internal Data

- Contextual/Metadata Information

- Log-Likelihood Ratio (LLR)

Observation 2: For RAN1 Rel-20 6GR SI, model output data for the proposed use cases can be classified into the following categories.

- Channel Matrix / Eigenvectors / CSI

- Estimated Channel / Noise

- Log-Likelihood Ratio (LLR)

- Beam Information / Measurements (Predicted)

- Symbols / Signal

- Sequences / Coefficients

- Data / Decoded Information

- Tokens / Raw Data / Inference

- Sensing Data / Point Cloud (3D)

- Prediction Outcomes (Non-Channel)

- AI/ML Internal Data

- Time Domain Samples

Observation 3: Two-sided models are utilized for many RAN1 use cases for 6G (10 out of 37).

Proposal 1. Study traffic distribution of 6G and whether existing session-centric framework for data collection and transfer (i.e., using UP tunnels and PDU sessions) needs to be enhanced/changed (e.g., to session-less framework) to accommodate the new traffic distribution (if there is one) and diverse types of data/services.

Proposal 2. Study the necessity of on-path computing/processing (e.g., functions such as data fusion and de-privacy) for data collection and transfer in 6G.

Proposal 3. Study the necessity of supporting arbitrary topology for data collection/delivery and service in 6G.

Proposal 4. If session-less framework is necessary for 6G, study the changes/enhancements needed for CP and UP, or whether a new plane dedicated to data collection/management (i.e., data plane) is needed.

Proposal 5. Study necessary enhancements to network elements (e.g., NB, UE and CN) to enable session-less transmission and support of arbitrary topology (if supported) for data collection/delivery and service in 6G.

Proposal 6: 6G to study unifying data collection and model/dataset transfer in the same framework to achieve efficiency and eliminate redundancy.

Proposal 7: 6G to study handling large volume of model/dataset for transfer. Consider the following two solutions,

Proposal 8: 6G to study the mechanism to guarantee different QoS requirements for model/dataset transfer, depending on characteristics of the learning mechanisms.

- This includes QoS requirements for online training and federated learning, which may impose higher QoS requirements than traditional offline training.

Proposal 9. RAN2 to further study how to provide optimized/simplified support of two-sided models in 6G.

Proposal 10: Support more AI/ML techniques beyond supervised learning, as well as digital twin.

# References

Chair’s notes, RAN2 meeting #131bis, Oct 2025, Prague, Czech Republic

R2-2506800, Considerations on 6GR AI framework, vivo, submitted to RAN2 meeting #131bis, Oct 2025

3GPP TR 38.843 V19.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,  2025-09

Chair’s notes, RAN1 meeting #122bis, Oct 2025, Prague, Czech Republic

3GPP TS 23.501, System architecture for the 5G System (5GS)