3GPP TSG-RAN WG2 Meeting #132	R2-2508925

Dallas, USA, 17 - 21 November, 2025

Agenda item:	8.10.2

Source:	Huawei, HiSilicon

Title:	Discussion on the NTN area configuration for MDT

Document for:	Discussion and Decision

Introduction

For this RAN2 meeting, there is an incoming LS [1]. The LS is sent to both RAN3 and RAN2, and we expect that RAN3 will also have some discussions on their specification impacts in the coming RAN3 meeting.

In this paper, we discuss possible RAN2 impacts due to this LS.

Discussion

In the LS [1], SA5 mentions the followings:

SA5 has discussed and agreed on the proposed changes related to Geographical Area. - Geographical Area is agreed and added as a new choice for area scope for MDT. - Reuse PLMN ID list in configuration parameters which are specific for MDT.

In the latest 38.331 CR [2], the NTN area configuration for MDT has been defined as below:

LoggedMeasurementConfiguration-v1900-IEs ::= SEQUENCE { areaConfigurationNTN-List-r19 AreaConfigurationNTN-List-r19 OPTIONAL, -- Cond logAreaNTN nonCriticalExtension SEQUENCE {} OPTIONAL } AreaConfigurationNTN-List-r19 ::= SEQUENCE (SIZE (1..maxNrofAreaNTN-r19)) OF AreaConfigurationNTN-r19 AreaConfigurationNTN-r19 ::= SEQUENCE { areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER (1..65535) } } }

For the 1st bullet in the LS, it has been supported in the current TS 38.331, i.e. the AreaConfigurationNTN-List-r19 IE (including Geographical area information) is optional in DL RRC message.

For the 2nd bullet in the LS, our view is that PLMN ID list should be added into the AreaConfigurationNTN-r19 IE, and one possible change is as below:

AreaConfigurationNTN-r19 ::= SEQUENCE { areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER (1..65535) } }, plmn-IdentityList-r19 PLMN-IdentityList2-r16 } PLMN-IdentityList2-r16 ::= SEQUENCE (SIZE (1..16)) OF PLMN-Identity

We note that this LS is sent to both RAN3 and RAN2, and then we suggest to wait for RAN3 progress before RAN2 discussing detailed solutions as well as possible RAN2 impacts. However, for now there is no ASN.1 extension marker "..." in the AreaConfigurationNTN-r19 IE, and thus it will be a bit challenging to extend this IE in future RAN2 meetings.

In summary, there are the following options regarding how RAN2 can progress on the discussion of the 2nd bullet in the LS:

- Option 1: RAN2 can decide on solutions as well as RAN2 impacts via CR in this RAN2#132 meeting, and co-ordinate with RAN3/SA5

- Option 2: RAN2 can do nothing in this RAN2#132 meeting, and just wait for RAN3 progress. If RAN2 can immediately receive reply LS from RAN3, RAN2 can progress on RAN2 CR accordingly. Otherwise, RAN2 will discuss this topic in future RAN2 meetings

- Option 3: RAN2 can discuss whether to add an ASN.1 extension marker "..." in the AreaConfigurationNTN-r19 IE or not for this RAN2#132 meeting. If RAN2 will discuss this topic in future RAN2 meetings (e.g. based on reply LS from RAN3), it will be straightforward to do the extension

We propose RAN2 to discuss the above three options. For Option 3, one possible change is listed below:

AreaConfigurationNTN-r19 ::= SEQUENCE { areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER (1..65535) } }, ... }

Later, if RAN2 will introduce PLMN ID list in configuration parameters for NTN area configuration, one possible change is also listed below:

AreaConfigurationNTN-r19 ::= SEQUENCE { areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER (1..65535) } }, ..., [[ plmn-IdentityList-r19 PLMN-IdentityList2-r16 ]] }

Conclusion

In this contribution, we discuss possible RAN2 impacts due to the LS [1] on the NTN area configuration for MDT. We have the following proposal:

Proposal 1: Regarding how RAN2 should progress on the discussion of the LS S5-254790, it is proposed RAN2 to discuss the following three options:

- Option 1: RAN2 can decide on solutions as well as RAN2 impacts via CR in this RAN2#132 meeting, and co-ordinate with RAN3/SA5

- Option 2: RAN2 can do nothing in this RAN2#132 meeting, and just wait for RAN3 progress. If RAN2 can immediately receive reply LS from RAN3, RAN2 can progress on RAN2 CR accordingly. Otherwise, RAN2 will discuss this topic in future RAN2 meetings

- Option 3: RAN2 can discuss whether to add an ASN.1 extension marker "..." in the AreaConfigurationNTN-r19 IE or not for this RAN2#132 meeting. If RAN2 will discuss this topic in future RAN2 meetings (e.g. based on reply LS from RAN3), it will be straightforward to do the extension

For Option 1 and option 3, possible RAN2 changes to TS 38.331 are also listed in section 5.

Reference

S5-254790, Reply LS on geographical area scope MDT, Source: SA5, To: RAN3, RAN2

R2-2507667, Corrections on RRC for Rel-19 SONMDT features, Ericsson, CR

Possible changes to TS 38.331 due to Option 1

- Introdcution of the PLMN ID information in the AreaConfigurationNTN-r19 IE.

AreaConfigurationNTN-r19 ::= SEQUENCE { areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER (1..65535) } }, plmn-IdentityList-r19 PLMN-IdentityList2-r16 }

Possible changes to TS 38.331 due to Option 3

- Introdcution of the ASN.1 extension marker "...".

AreaConfigurationNTN-r19 ::= SEQUENCE { areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER (1..65535) } }, ... }

- Introdcution of the PLMN ID information in the AreaConfigurationNTN-r19 IE.

AreaConfigurationNTN-r19 ::= SEQUENCE { areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { referenceLocation-r19 ReferenceLocation-r17, distanceRadius-r19 INTEGER (1..65535) } }, ..., [[ plmn-IdentityList-r19 PLMN-IdentityList2-r16 ]] }