3GPP TSG-RAN WG2 Meeting #132	R2-2508928

Dallas, USA, Nov. 17th – 21st, 2025

Agenda item:	10.3.3.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on energy saving of 6GR

Document for:	Discussion and Decision

1 Introduction

It was agreed in RAN2#131bis [1] to support energy saving for UE and network with following design goal and guideline:

- Strive to develop solutions that consider network energy efficiency and UE power saving. - For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we will share our views on 6G UE and network energy saving.

2 Discussion

## 2.1 General

In 5G, many power saving technologies had been introduced for UE and NW respectively as shown in Table 1, including time/frequency/spatial/power domain. These technologies have been verified in 5G and have significant power saving gain.

Table 1 5G power saving technologies

|  | UE power saving | NW power saving |
| --- | --- | --- |
| Time domain | Idle/inactive mode I-DRX, SDT, RRM measurement relaxed, PEI, TRS availability indication | Idle/inactive mode on-demand SIB1, PRACH adaptation, PO adaptation |
|  | Connected mode C-DRX, DCP(PDCCH-based WUS), cross-slot scheduling adaptation, PDCCH skipping, SSSG switching, LP-WUS/SS | Connected mode Cell DTX/DRX , SSB adaptation on SCell |
| Frequency domain | Connected mode BWP, SCell dormancy | Connected mode SSB-less for SCell, on-demand SSB for SCell |
| Spatial domain | Connected mode Maximum MIMO layer switching | Connected mode Spatial elements adaptation |
| Power domain | / | Connected mode PDSCH transmission power reduced |

However, despite the benefit of these features for power saving, most of them are not deployed with many reasons. One reason may be that most of the features were not 5G Day-1 features, but were gradually introduced in the later releases. Due to the late introduction of these energy-saving features, there is less support from UEs and networks, thus reducing the power-saving gains. In addition, some energy-saving technologies have overlapping functions, such as DCP and LP-WUS, PEI and LP-WUS.

In our view, some essential power saving technologies in Table 1 should be considered in 6G day-1, such as LP-WUS, cell DTX/DRX, on-demand SSB/SIB1.

Observation 1: 6G power saving can be based on some essential techniques of 5G power saving in day-1.

## 2.2 UE power saving in 6GR

In addition, 6G NW energy efficiency can be further improved by enhancing UE power saving technologies, such as LP-WUS.

Idle/Non-connected mode wake up mechanism

In 5G, I-DRX and wake up signals (PEI and LP-WUS) are supported for idle/inactive mode UE power saving. For the case that a UE supports PEI and PEI is configured by the NW, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. It can be seen that PEI and LP-WUS are overlapped. 6GR should avoid overlapping functionalities.

The LP-WUS technology offers better energy-saving benefits than PEI based on the evaluation results of RAN1 [2]. Therefore, we prefer LP-WUS as WUS in idle/non-connected mode for 6G day-1.

Proposal 1: Study LP-WUS for idle/non-connected mode UE in 6G day-1.

In 5G, for idle/inactive mode UE, serving cell RRM measurement relaxing/offloading from MR to LP-WUR is only supported due to limited coverage. LP-WUR operation is almost disabled at serving cell edges, and normal MR based operations are resumed. Limited coverage performance limits better power saving gain. In 6G, enhancement of the coverage of LP-WUS (i.e. OFDM-based sequence) and capabilities of LP-WUR can be studied. With coverage enhancement, LP-WUR can work at cell edges and neighbour cell RRM measurement can be relaxed or offload to LP-WUR so that MR has more time to sleep.

Proposal 2: Study serving cell and neighbour cell RRM measurement relaxed/offloaded to LP-WUR in 6G day-1.

Connected mode wake up mechanism

In 5G, C-DRX and wake up signals (DCP and LP-WUS) are support for connected mode UE. Similar as idle/inactive mode, to avoid overlapping functionalities, we prefer only LP-WUS as wake up signal in connected mode. 5G support two options of LP-WUS operation (i.e. option 1-1 and option 1-2) in connected mode and they can’t work simultaneously.

For option 1-1, LP-WUS replaces DCP to trigger drx-onDurationTimer. For option 1-2, LP-WUS monitoring occasion is located at any time outside C-DRX active time to indicate UE to enter into active time, which enables more flexible LP-WUS monitoring occasions and better power saving gain. Both Option 1-1 and Option 1-2 are assuming that UE is configured with C-DRX, and LP-WUS monitoring occasion is determined based on timers related C-DRX.

In 6G, LP-WUS could be used at any time regardless of whether C-DRX is configured or not. Thus, LP-WUS configuration is more flexible than 5G option 1-1 and 1-2. Power Saving gain is expected compared to current C-DRX mechanism and DCP since LP-WUS monitoring consumes less power than PDCCH monitoring. DL data transmission latency could be reduced compared to current C-DRX mechanism. So we think it is possible to study the decoupling LP-WUS and C-DRX in 6G day-1.

Proposal 3: Study the decoupling of LP-WUS and C-DRX for connected mode in 6G day-1.

## 2.3 Network power saving in 6GR

Idle/Non-connected mode

In 5G, cell DTX/DRX is only support for connected mode UE, and does not affect procedures such as RACH, SSB transmission, paging, and system information broadcasting. For further NW power saving, Cell DTX/DRX for idle/non-connected mode UE can be studied in 6G day-1.

Proposal 4: Study cell DTX/DRX for idle/non-connected UEs in 6G day-1.

Connected mode

In 5G, cell DTX/DRX operation allows network discontinuous transmission and reception. The cell DTX operation controls UE's PDCCH monitoring and configured downlink assignments, while the cell DRX operation controls scheduling request and configured uplink grant transmission activity.

Cell DTX is configured only when C-DRX is configured, and it only restrict UE to do not monitor PDCCH/DL SPS PDSCH/CSI-RS for CSI reporting only in non-active time of cell DTX, but it does not restrict UE to  receive any other reference signals, so gNB can not enter sleep state completely either. As there are no issues of the back-compatibility in 6GR day1, more reference signals can be “muted” in non-active time of cell DTX, such as CSI-RS for tracking and RRM management, so both UE and gNB can enter sleep state completely in non-active time of cell DTX.

Proposal 5: Study enhanced monitor behaviour during non-active time of cell DTX in 6G day-1.

From NW perspective, there is no idle/non-connected/connected state indeed. If gNB enters sleep state, it means cell DTX/DRX for all state UEs. Therefore, if we want gNB to enter sleep state completely, maybe cell DTX/DRX for idle/non-connected UEs and connected UEs needs to be aligned, e.g. some reference signal are muted for idle/non-connected UEs and connected UEs. It can be up to gNB implementation, but there could be some limitations of related configurations which will impact the spec.

Proposal 6: Study alignment of cell DTX/DRX for idle/non-connected UEs and connected UEs in 6G day-1.

## 2.4 Joint UE and network power saving

Although there are various techniques for UE and network energy saving in 5G, but they are motivated and specified independently. Networks have little incentive to enable features that benefit only the UE, as they can increase network complexity and degrade network performance.

Thus, 6GR should support native energy savings features via proper designs that consider both networks and UEs. One obvious direction is to align UE DRX and cell DTX. Alignment of UE DRX and cell DTX can reduce UE and gNB implementation complexity. R18 tried this mechanism but finally it was up to gNB implementation. In 6GR day-1, joint design of UE C-DRX and cell DTX can be further studied (such as aligned or unified), since at least both have function about PDCCH transmission and reception.

Proposal 7: Study joint design of UE C-DRX and cell DTX in 6G day-1.

3 Conclusion

In this contribution we discuss energy saving for UE and network, with the following proposals:

Observation 1: 6G power saving can be based on some essential techniques of 5G power saving in day-1.

Proposal 1: Study LP-WUS for idle/non-connected mode UE in 6G day-1.

Proposal 2: Study serving cell and neighbour cell RRM measurement relaxed/offloaded to LP-WUR in 6G day-1.

Proposal 3: Study the decoupling of LP-WUS and C-DRX for connected mode in 6G day-1.

Proposal 4: Study cell DTX/DRX for idle/non-connected UEs in 6G day-1.

Proposal 5: Study enhanced monitor behaviour during non-active time of cell DTX in 6G day-1.

Proposal 6: Study alignment of cell DTX/DRX for idle/non-connected UEs and connected UEs in 6G day-1.

Proposal 7: Study joint design of UE DRX and cell DTX/DRX in 6G day-1.

- References

- Draft 131bis meeting report, RAN2 chair (InterDigital), Prague, Czech Republic, Oct. 13 - 17, 2025.

- TR 38.869, “Study on low-power wake-up signal and receiver for NR (Release 18)”.