3GPP TSG-RAN WG2 Meeting #132			         R2-2508944

Dallas, USA, Nov. 17th-21th

Agenda Item:	10.3.2.1

Source: 	LG Electronics Inc.

Title:         	Discussion on RRC state and connection management for 6G

Document for: 	Discussion and Decision

Introduction

In the last meeting, RAN2 made agreement for Control Plane as follows.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this contribution, we present our view on RRC_INACTIVE state in various aspects that need to be considered in designing 6GR.

Discussion

Deployable RRC_INACTIVE state

In NR, the RRC_INACTIVE state was introduced as an intermediate mode between RRC_CONNECTED and RRC_IDLE. The objective was to reduce network management overhead for UEs with ongoing sessions while enabling greater UE power savings compared to RRC_CONNECTED. Specifically, when data transmission is temporarily suspended, the network can place the UE into RRC_INACTIVE and quickly resume the connection when new data arrives. In practice, however, commercial deployments have rarely adopted RRC_INACTIVE. While there are multiple pain points to be addressed in RRC_INACTIVE, two issues are outstanding from our point of view: a) long latency of the RRC resume procedure and b) necessity of transmitting paging messages over a wide RAN paging area when downlink data arrives. With these two pain points, operators seem to perceive limited benefits over RRC_IDLE and have little incentive to manage the additional complexity of supporting RRC_INACTIVE.

Pain Points of NR RRC_INACTIVE

- Considerable resume latency from RRC_INACTIVE to RRC_CONNECTED.

- Signaling overhead due to paging across a large RAN paging area.

For 6G, it is important to revisit the design of RRC_INACTIVE to ensure tangible performance gains. One potential enhancement is to enable faster RRC resume procedure, e.g. via RACH-less resume, so that network can transition UEs to RRC_INACTIVE without being constrained by resume latency. Beyond faster RRC resume, enhancements of network control over UE mobility and paging optimization could also be considered. The primary goal should be to make RRC_INACTIVE a practical and deployable feature in 6G, rather than a theoretically defined but seldom used state.

For UEs in NR RRC_INACTIVE, UE context is stored in the gNB side. The stored UE context can reduce initial access delay i.e. UE state transition latency and gNB-CN initial setup latency. Since the UE context (like security keys and configurations) has been already stored in both the UE and the gNB, the UE can start sending data almost immediately after RRC resume procedure with RACH or even with RACH-less operation based on CG-SDT. Thus, one of the benefits that gNB/UE can achieve from RRC_INACTIVE is lower setup delay required before actual data transfer, thanks to quick initial setup, compared to RRC_IDLE.

Observation 1	UE context stored in the gNB side for inactive UEs can reduce initial access delay i.e. UE state transition latency and gNB-CN initial setup latency and support fast data transfer with low latency, even without RACH if CG-SDT configuration is available.

Moreover, considering UEs in RRC_CONNECTED should perform more tasks, such as PDCCH monitoring on USS, measurement task/reporting and TA alignment, than UEs outside RRC_IDLE/INACTIVE, we can say that RRC_INACTIVE helps UE to save UE energy, compared to RRC_CONNECTED. Besides, since gNB does not need to reserve periodic signal resources and channel resources for UEs in RRC_CONNECTED e.g. for BFD/RLM and scheduling, it is obvious that RRC_INACTIVE also help gNB to reduce network energy consumption and the amount of resource required for connected UEs, compared to RRC_CONNECTED.

Besides, NR RRC_INACTIVE can support fast data transfer and quick release, i.e. short duration of a RRC connection for data transfer, especially when UE does not need to stay in RRC_CONNECTED for long-term services. Short duration of a RRC connection allows both gNB and UE to reduce their energy consumption and the amount of resource required to transfer data.

Furthermore, NR RRC_INACTIVE allows gNB to transmit paging in a smaller paging area, which leads to another benefit of network energy saving because paging need not be broadcast in a large area.

Observation 2	Support of RRC_INACTIVE helps gNB/UE to reduce the amount of resource required to transfer data and gNB/UE energy consumption, compared to RRC_CONNECTED.

Observation 3	Support of RRC_INACTIVE helps gNB/UE to reduce initial setup delay required before actual data transfer and gNB/UE energy consumption, compared to RRC_IDLE.

Proposal 1	Study enhanced RRC_INACTIVE to provide clear operational benefits over RRC_IDLE and RRC_CONNECTED, with a focus on enabling low-latency RRC resume, optimizing paging and enhancing network control over UE mobility.

Considerations for 6GR RRC INACTIVE Paging Simplification.

During the RAN2#131bis meeting, there was some discussion regarding RRC_INACTIVE state and some companies consider that paging for RRC_INACTIVE state needs to be simplified in 6G. We would like to share our views on paging simplification that should be considered in 6G design, which will have better radio performance than 5G NR.

One of the things that RAN2 need to consider is that RRC state mismatch problem is still inevitable problem in 6G. When we design RRC_INACTIVE in 5G NR, RAN2 fully understand that RRC state mismatch problem is unavoidable. And UE in RRC_INACTIVE state monitors paging channels for both RAN-initiated paging and CN-initiated paging. This is because RRC_INACTIVE and RRC_IDLE state are not controlled by the network system and there are some scenarios that UE cannot update its state change to network. For example, if the RNA update procedure failure occurs, UE may go to RRC_IDLE state without informing RRC state transition to the network. Also, there is the scenario where UE miss paging for RRC_INACTIVE state due to the temporal out of radio coverage and resulting in the RAN node releasing the UE configuration. As these RRC state mismatch scenarios will still occur in 6G, we need to consider RRC state mismatch problem in simplifying RRC_INACTIVE paging.

Another thing needs to be considered in RRC_INACTIVE paging simplification is that UE-specific paging DRX cycle for the RRC_INACTIVE state is still important in 6G. In our understanding, the purpose of introducing a UE-specific paging DRX cycle for RRC_INACTIVE state is to support paging DRX cycle appropriate for the RRC_INACTIVE state. For instance, RAN can configure shorter paging DRX cycle for RRC_INACTIVE state UEs to reduce paging latency and enable faster RRC state transition. In addition, since the MT-SDT introduced in Release 18 is also performed through paging messages, a separate paging DRX cycle for the RRC_INACTIVE state, which is suitable for on-going session, may still be required.

Therefore, we think that paging simplification for RRC_INACTIVE state should consider key principles of RRC_INACTIVE paging procedure in 5G NR.

Proposal 2     Study paging simplification with following considerations in 6G.

- RRC state mismatch problem is inevitable. RRC_INACTIVE UE should monitor paging for RRC_IDLE state and RRC_INACTIVE state.

- UE specific paging DRX cycle for RRC_INACTIVE state is essential to support paging DRX cycle suitable for on-going session of RRC_INACTIVE UE.

6GR RRC INACTIVE state enhancements for data transfer

In 5G NR, Small Data Transmission (SDT) was introduced to allow data transmission in RRC_INACTIVE. In NR Rel-17, MO-SDT (Mobile Originated SDT) was introduced targeting UL data, and in NR Rel-18, MT-SDT (Mobile Terminated SDT) was introduced targeting DL data.

In our view, RRC_INACTIVE is more efficient than RRC_IDLE from connection setup latency and signalling overhead perspective and more efficient than RRC_CONNECTED from power saving perspective. Thus, we think RRC_INACTIVE should be main state in 6G, and SDT should be basic function for data transmission in RRC_INACTIVE. However, some pain points are observed in 5G NR SDT, which makes the SDT feature unattractive.

First, the SDT procedure is restricted to initial BWP, where initial BWP has narrow bandwidth. Due to the narrow bandwidth of initial BWP, only very small data can be transmitted in the SDT procedure, which is not suitable for practical application. In some cases, data transmission may not be completed during SDT procedure, which leads to frequent RRC resume/release. Moreover, since both SDT and RACH procedure are performed on initial BWP, the collision probability is increased in the RACH procedure. In order to resolve above problem, we think that SDT specific BWP should be introduced from 6G Day-1. Similarly, SDT resources can be configured only on one or a few carriers where normal RACH resources are not configured in single cell multi-carrier deployment.

Another pain point is that the maximum duration of SDT procedure is limited to 4s. We think that this short time contributes to making SDT less efficient.

One of scenario using SDT is the chat application, e.g. whatsapp. The chat application does not require large size of data, but once a chat starts, the text may be sent continuously, and the chat may not end in 4s. However, since the SDT procedure should be terminated in 4s, RRCResumeRequest message and RRCRelease message should continue to be signalled and this leads to signalling overhead. Thus, to reduce signalling overhead, the SDT duration needs to be extended in 6G.

If the SDT duration becomes longer, it also needs to be considered how to reduce the PDCCH monitoring during SDT procedure. In 5G NR, the UE continuously monitors PDCCH during SDT procedure, which may not be acceptable if the SDT duration becomes longer, e.g. 1 hour.

Figure 1. SDT enhancement in 6G

One more thing to consider is to use shared CG resource for CG-SDT. In 5G NR, dedicated CG resource is configured for CG-SDT, which is good for one UE but bad for network overall resource utilization. Moreover, if massive IoT devices, which is considered in 6G scenarios, utilizes CG-SDT, the signaling overhead to configure dedicated CG resource to numerous IoT devices would be unbearable. Therefore, we think shared CG resource should be introduced for CG-SDT in 6G.

Proposal 3	Introduce efficient small data transmission from 6G Day-1.

- Introduce SDT specific BWP and carrier.

- Support longer SDT duration: longer than 4s, e.g. up to 1 hour.

- Reduce PDCCH monitoring in SDT procedure.

- Introduce shared CG-SDT resource.

6GR RRC INACTIVE state enhancements for fast transitions

In 5G NR, the RRC_INACTIVE state was introduced to reduce signaling overhead and power consumption for UEs with intermittent data activity, while enabling faster transition to the RRC_CONNECTED state. However, a non-negligible transition latency remains in the RRC Resume procedure. Considering that one of main contributors to transition latency is the Random Access (RACH) procedure, 6G should study methods to perform the RRC Resume procedure without performing RACH procedure, so that RRC_INACTIVE state is more effectively utilized and commercially deployed.

In NR, with the introduction of SDT in Rel-17, RACH-less resume procedure is already supported under limited conditions through SDT. SDT allows data and/or signalling transmission while the UE remains in the RRC_INACTIVE state. When the UE has SDT CG resources and maintains valid UL timing alignment (i.e., the cg-SDT-TimeAlignmentTimer has not expired) and the RSRP variation is within the configured threshold (e.g., not exceeding cg-SDT-RSRP-ChangeThreshold), the UE can transmit RRC resume message on the CG without performing RACH procedure for SDT.

In 6G, if the use of SDT CG resources can be extended to support the transmission of RRC resume request message for RRC connection establishment, it would enable a fast RRC state transition. In fact, even in NR SDT, when non-SDT data arrives during an ongoing SDT session, the network can establish RRC connection without performing RACH procedure. And this SDT CG resources based RRC state transition does not require extra work to support it. However, the proposed RACH-less RRC Resume based on SDT is limited to the scenario where the UE stays in the anchor cell (=last PCell from which RRC release was received).

Observation 4	During the CG-based SDT, if non-SDT data occurs, 5G NR already supports UE state transition from RRC_INACTIVE to RRC_CONNECTED without requiring a RACH procedure.

To generalize beyond the anchor cell, 6G RRC_INACTIVE state can leverage the mobility configuration and information, which were configured during the RRC_CONNECTED state, to resume a suspended RRC connection. In detail, the 6GR network may instruct the UE—through an RRC Release message—to retain certain mobility configurations of target candidate cells, such as cell configurations, TA information and pre-configured UL grant resources i.e, CG, for certain candidate cells, that were configured for mobility procedures such as CLTM. By keeping the valid mobility configurations, an RRC_INACTIVE UE could perform a RACH-less RRC Resume toward a target cell other than the anchor cell. Since this approach uses the configurations configured for mobility purpose during the RRC_CONNECTED state, we think that this RACH-less Resume is supportable for fast state transition with little input from RAN2. And this approach would enable a fast RRC state transition even in scenarios where the cell change occurs.

Observation 5	If a UE has valid TA and CG for a candidate cell and their use is permitted in RRC_INACTIVE, the UE reselected to the candidate cell can perform a RACH-less resume while camped on it.

Based on above analysis, we propose to study RACH-less resume for 6G.

Proposal 4	Study RACH-less resume for fast transition as follows:

- Consider CG based SDT as a baseline for RACH-less resume for UEs camp on the Anchor cell (=last PCell)

- Reuse early TA and CG configured for cell switch (e.g., candidate cell for CLTM) as a baseline for RACH-less resume after cell re-selection to non-anchor cell.

Conclusion

Observation 1	UE context stored in the gNB side for inactive UEs can reduce initial access delay i.e. UE state transition latency and gNB-CN initial setup latency and support fast data transfer with low latency, even without RACH if CG-SDT configuration is available.

Observation 2	Support of RRC_INACTIVE helps gNB/UE to reduce the amount of resource required to transfer data and gNB/UE energy consumption, compared to RRC_CONNECTED.

Observation 3	Support of RRC_INACTIVE helps gNB/UE to reduce initial setup delay required before actual data transfer and gNB/UE energy consumption, compared to RRC_IDLE.

Proposal 1	Study enhanced RRC_INACTIVE to provide clear operational benefits over RRC_IDLE and RRC_CONNECTED, with a focus on enabling low-latency RRC resume, optimizing paging and enhancing network control over UE mobility.

Proposal 2     Study paging simplification with following considerations in 6G.

- RRC state mismatch problem is inevitable. RRC_INACTIVE UE should monitor paging for RRC_IDLE state and RRC_INACTIVE state.

- UE specific paging DRX cycle for RRC_INACTIVE state is essential to support paging DRX cycle suitable for on-going session of RRC_INACTIVE UE.

Proposal 3	Introduce efficient small data transmission from 6G Day-1.

- Introduce SDT specific BWP and carrier.

- Support longer SDT duration: longer than 4s, e.g. up to 1 hour.

- Reduce PDCCH monitoring in SDT procedure.

- Introduce shared CG-SDT resource.

Observation 4	During the CG-based SDT, if non-SDT data occurs, 5G NR already supports UE state transition from RRC_INACTIVE to RRC_CONNECTED without requiring a RACH procedure.

Observation 5	If a UE has valid TA and CG for a candidate cell and their use is permitted in RRC_INACTIVE, the UE reselected to the candidate cell can perform a RACH-less resume while camped on it.

Proposal 4	Study RACH-less resume for fast transition as follows:

- Consider CG based SDT as a baseline for RACH-less resume for UEs camp on the Anchor cell (=last PCell).

- Reuse early TA and CG configured for cell switch (e.g., candidate cell for CLTM) as a baseline for RACH-less resume after cell re-selection to non-anchor cell.

Reference