3GPP TSG-RAN WG2 Meeting #132	R2-2508957

Dallas, USA, Nov 17th – 21th, 2025

Agenda Item:	9.2.2

Source:			ASUSTeK

Title:	Discussion on A-IoT resources for UE reader

Document for: 	Discussion and Decision

- Introduction

In the Rel-20 WI for Ambient IoT [1] updated in RAN#109, the objectives for D2T2 are specified as below:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN1 - The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS. - No physical layer impact for signaling A-IoT resources to the intermediate UE. - No additional specification change on R2D and D2R for Device 1 in Rel-20 RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility RAN4 - Specify RF requirements for intermediate UE towards the A-IoT device. - Specify necessary updates to RF requirements for A-IoT device 1. - Specify A-IoT active device(s) RF requirements in D2T2 - Specify RRM requirements for intermediate UE due to A-IoT operation, if any. Combination with existing RRM procedures/requirements is excluded. - Specify RRM requirements for Device(s) in D2T2 if necessary - Use band n8 as an example band.

In this contribution, we would like to discuss the A-IoT resources for the intermediate UE as reader.

- Discussion

2.1. Background

In release 19, the work item was completed leading to the specification of Device 1 for use in Deployment Scenario 1 with Topology 1. Based on the release 20 work item [1], with Deployment Scenario 2 with Topology 2 as Figure 1, the ambient IoT devices would communicate bidirectionally with an intermediate node (e.g., a reader) between the A-IoT devices and the base station. The reader, which can also operate as a legacy NR UE, sends R2D signals to the devices and receives D2R signals from the devices.

Figure 1: Topology 2 (Figure 4.2.1.2-1 in TR 38.848 [2])

The diagrams for Topology 2 are shown as Figure 2 for Device 1 and Figure 3 for active devices. The CW node is either outside the topology or absent, and the reader for R2D and D2R is the same.

Figure 2: D2T2-B (from Table 4.2.1-1 in TR 38.769 [3])

Figure 3: D2T2-C (from Table 4.2.1-1 in TR 38.769 [3])

The NW would be in control of A-IoT resources for an A-IoT interface. Based on TR 38.769 [3] as following, the NW would provide the A-IoT resources to the reader and transmit the inventory request to the reader via RRC message. In response to receiving the inventory request, the reader transmits a response to the NW and performs the A-IoT procedure (e.g., transmits an A-IoT paging message to the device(s) as indicated by the inventory request). The reader performs A-IoT procedure on the A-IoT interface if the A-IoT resources are valid. Then, the reader transmits inventory results (e.g., upper layer data from the device(s)) to the NW.

### 6.5.3 Signaling and Procedures for Topology 2 #### 6.5.3.1 Candidate procedures for A-IoT Inventory for Topology 2 ##### 6.5.3.1.1 RRC based solution Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution 1a. The A-IoT CN sends an Inventory request message to the A-IoT enabled gNB 1b. The A-IoT enabled gNB allocates and coordinates usage of A-IoT radio resources. 1c/2a RRC communication with the A-IoT enabled UE takes place. NOTE 1: RRC based communication is only depicted schematically, details subject to RAN2. 2. The A-IoT-enabled gNB sends an Inventory response message to the A-IoT CN. NOTE 2: In step 2, the A-IoT-enabled gNB may instead send an Inventory failure message to the A-IoT CN indicating that the inventory procedure could not be initiated towards the A-IoT device(s). 3. The A-IoT-enabled gNB requests the A-IoT-enabled UE(s) to trigger inventory procedure towards the A-IoT device(s). 4a/4b. After receiving inventory result reported from the A-IoT enabled UEs, the A-IoT-enabled gNB may send one or multiple Inventory reports towards the A-IoT CN including the received inventory result. NOTE 3: Steps 4a/4b may happen in parallel with Step 3 for different A-IoT devices. NOTE 4: Step 4a/4b between A-IoT-enable UE and A-IoT-enabled gNB subject to RAN2.

2.2. Resource Allocation

For the RRC-based solution, the NW should provide the A-IoT resources and the inventory request to the reader before the reader triggers the A-IoT procedure. The NW could provide both the A-IoT resources and inventory request in a single RRC message, which may reduce latency of initiating the A-IoT procedure. On the other hand, it would be more flexible for the NW to provide the A-IoT resources and the inventory request in separate messages. Recall the agreements on A-IoT resources validity in the SI phase, the A-IoT resources may remain valid in the same cell in which the resources were configured, and the A-IoT resources may become invalid if the NW releases them explicitly or after a time period. Thus, the NW could provide the A-IoT resources one time and request the reader to perform multiple A-IoT procedures if the A-IoT resources are still valid.

Proposal 1: The UE reader could receive the A-IoT resources and the inventory request in a single RRC message or separate RRC messages.

Since both the R2D transmission and D2R transmission would be in the UL spectrum, how to control the usage of UL spectrum for A-IoT procedure should be discussed. Firstly, to efficiently allocate the frequency resources in the serving cell and reduce interference to the guard band, it is proposed to support that the A-IoT resources for D2R and R2D could be in-band to NR. Similar to RA resources, the NW could configure the A-IoT resources on some specific UL BWPs to prevent from waste of radio resources. It could be further discussed that on which UL BWP(s) the A-IoT resources could be configured.

Proposal 2: The A-IoT resources could be in-band to NR. FFS on which specific UL BWP(s) the A-IoT resources could be configured.

To perform the A-IoT procedure, the reader should switch the active UL BWP to the UL BWP configured with the A-IoT resources. The UE could perform UL BWP switching to use the A-IoT resources in response to receiving an inventory request from the NW. As shown in Figure 6.5.3.1.1-1 of TR 38.769 [3], the reader would transmit a RRC inventory response to the NW after receiving the RRC inventory request. As a result, the reader could switch the UL BWP after transmitting the inventory response, if there are no A-IoT resources on the active UL BWP. Then, the reader could transmit the A-IoT paging message to trigger the A-IoT procedure on the UL BWP with the A-IoT resources.

Proposal 3: In response to receiving the inventory request, the UE reader performs BWP switching to the UL BWP with A-IoT resources after transmitting the inventory response, if there are no A-IoT resources on the active UL BWP.

Secondly, as agreed in the last meeting, the CN can trigger request of A-IoT resources allocation, and the reader may provide assistance information related to A-IoT transmissions. Based on the above, it is up to the gNB to allocate A-IoT resources to the reader.

Agreements

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

However, as discussed during the last meeting, it is beneficial to allow the reader to provide assistance information related to utilization of resources on the A-IoT interface. For example, the gNB could not be aware of the collision between devices, while the reader has a better understanding of it. The reader may need to transmit more paging messages to the devices due to the collision, and the A-IoT resources allocated for the reader can be insufficient to complete the inventory procedure.

In another example, the reader could provide the assistance information based on radio condition. If the interference for current A-IoT resources is heavy, the reader could request release of the A-IoT resources or request other A-IoT resources. On the other hand, it is also possible that the gNB would over-allocate the A-IoT resources to a reader. If the gNB receives a release request from the reader and there would not be another A-IoT procedure for it, the gNB could release the A-IoT resources on the reader.

As a result, the reader should be able to provide the assistance information to the gNB regarding need for more resources or indication for unused resources. This assistance information can provide efficiency and flexibility on resource allocation for several readers and A-IoT procedures. Moreover, the resource request could also be useful for DO-A procedure.

Proposal 4: The UE reader could provide assistance information for A-IoT resource allocation.

Proposal 5: The assistance information for A-IoT resource allocation comprises resource request and release request.

2.3. Coexistence of RA and A-IoT procedure

Furthermore, the reader also operates as a legacy NR UE, and therefore the reader may trigger an RA procedure during an A-IoT procedure. For example, if the UL synchronization is lost, the reader needs to perform the RA procedure to regain TA. Since the RA procedure and A-IoT procedure are both performed on UL spectrum, there may be interference on each other. Also, the reader may not be able to handle both the procedures simultaneously. There would be two options for coexistence of RA procedure and A-IOT procedure:

- Option 1: When the UE triggers the RA procedure in Uu interface, the UE suspends the ongoing A-IoT procedure and performs the RA procedure. Then the UE resumes the A-IoT procedure after completion of the RA procedure.

- Option 2: When the UE triggers the RA procedure in Uu interface, the UE continues the ongoing A-IoT procedure and performs the RA procedure after completion of the A-IoT procedure.

It is suggested to discuss the case that an RA procedure is triggered during an A-IoT procedure.

Proposal 6: RAN2 discuss the following options for coexistence of RA procedure and A-IoT procedure:

-	Option 1: When the UE triggers the RA procedure during the A-IoT procedure, the UE suspends the A-IoT procedure and resumes the A-IoT procedure after completion of the RA procedure;

-	Option 2: When the UE triggers the RA procedure during the A-IoT procedure, the UE performs the RA procedure after completion of the A-IoT procedure.

- Conclusion

We discuss A-IoT resources handling for UE reader in D2T2 and have the following proposals:

Proposal 1: The UE reader could receive the A-IoT resources and the inventory request in a single RRC message or separate RRC messages.

Proposal 2: The A-IoT resources could be in-band to NR. FFS on which specific UL BWP(s) the A-IoT resources could be configured.

Proposal 3: In response to receiving the inventory request, the UE reader performs BWP switching to the UL BWP with A-IoT resources after transmitting the inventory response, if there are no A-IoT resources on the active UL BWP.

Proposal 4: The UE reader could provide assistance information for A-IoT resource allocation.

Proposal 5: The assistance information for A-IoT resource allocation comprises resource request and release request.

Proposal 6: RAN2 discuss the following options for coexistence of RA procedure and A-IoT procedure:

-	Option 1: When the UE triggers the RA procedure during the A-IoT procedure, the UE suspends the A-IoT procedure and resumes the A-IoT procedure after completion of the RA procedure;

-	Option 2: When the UE triggers the RA procedure during the A-IoT procedure, the UE performs the RA procedure after completion of the A-IoT procedure.

- Reference

- RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2.”

- 3GPP TR 38.848 V18.0.0 (2023-09) 3GPP; TSG RAN; Study on Ambient IoT (Internet of Things) in RAN (Release 18).

- 3GPP TR 38.769 V19.0.0, “Study on solutions for Ambient IoT in NR.”