3GPP TSG-RAN WG2 Meeting #132	R2-2508961

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	10.4

Source: 	ASUSTeK

Title:  	Discussion on 6G Mobility design

Document for:	Discussion and Decision

- Introduction

In 6GR mobility discussion in the previous meeting [1], the following agreement has been made:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this contribution, we’d like to provide analysis for baseline Mobility design in 6G.

- discussion

6G Mobility in RRC_CONNECTED

Before studying 6GR mobility design in RRC_CONNECTED state, we first look into current 5G mobility. Currently, in addition to traditional RRC-signaling-initiated handover, 5G supports various alternatives to perform mobility in RRC_CONNECTED, which were established from different work items for different services:

  - DAPS handover

  - Conditional handover (CHO)

  - L1/L2-triggered Mobility (LTM) or conditional LTM

  - RACH-based and RACH-less handover

In the previous RAN1 meeting[2], it was agreed to study a scalable 6G design and study supported device types:

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

From a scalability point of view, it is undesirable to introduce various mobility mechanisms since not all supported devices may contain the capability to support all mechanisms, and it is also not encouraged to provide multiple options to one functionality. Therefore, we propose not to support all mobility mechanisms from NR in 6G.

Proposal 1: For CONNECTED state mobility, considering the complexity of supporting various mobility mechanisms, not all mobility mechanisms specified in NR should be supported for 6G.

Regarding which mobility mechanisms to consider as a starting point for 6G, based on the agreement, we should prioritize mechanisms which consumes less interruption time. For example, an LTM-like procedure and conditional mobility procedure (e.g., CHO and LTM) are beneficial to enhance mobility procedure time and reduce interruption. In addition, we propose to continue supporting RACH-less mechanisms besides RACH-based mobility to further reduce time consumption.

Proposal 2: Aiming for minimizing interruption time, (RACH-less/RACH-based) CHO and LTM specified in NR are considered as a starting point for 6G CONNECTED state mobility.

Proposal 3: Early uplink synchronization specified in LTM and conditional LTM in NR should be considered for minimizing mobility interruption time in 6G.

6G Mobility design in CHO and LTM enhancement

To further reduce latency and enhance accuracy and efficiency for 6G, first we propose to leverage AI for pre-configuration and resource reservation for CHO and LTM in 6G. This can help the UE to store or help the NW to configured more precise or essential candidate configurations so that the UE can save energy on evaluating/measuring candidate Cells.

Proposal 4: Using AI to enhance the accuracy of pre-configuration and reserved resources for CHO and LTM should be studied in 6G.

Second, with UE-initiated L1/L2 measurement reporting, measurement triggering and reporting can be more efficient and accurate for NW to provide a more dynamic and suitable response to achieve efficient mobility procedures. In addition, AI can also be leveraged to enhance efficiency of measurement and reporting for mobility, which should be studied in 6G as well.

Proposal 5: Aiming for triggering mobility efficiently, UE initiated L1/2 measurement reporting should be considered as starting point for 6G.

Proposal 6: Using AI to enhance efficient measurement and reporting for mobility should be studied in 6G.

- Conclusion

We have the following proposals for 6GR protocol design:

Proposal 1: For CONNECTED state mobility, considering the complexity of supporting various mobility mechanisms, not all mobility mechanisms specified in NR should be supported for 6G.

Proposal 2: Aiming for less interruption time, (RACH-less/RACH-based) CHO and LTM specified in NR are considered as a starting point for 6G CONNECTED state mobility.

Proposal 3: Early uplink synchronization specified in LTM and conditional LTM in NR should be considered for minimizing mobility interruption time in 6G.

Proposal 4: Using AI to enhance the accuracy of pre-configuration and reserved resources for CHO and LTM should be studied in 6G.

Proposal 5: Aiming for triggering mobility efficiently, UE initiated L1/2 measurement reporting should be considered as starting point for 6G.

Proposal 6: Using AI to enhance efficient measurement and reporting for mobility should be studied in 6G.

- Reference

[1] Chairman note RAN2#131bis

[2] Draft Report of 3GPP TSG RAN WG1 #122 v0.1.0