3GPP TSG-RAN WG2 Meeting #132		                                    R2-2508965

Dallas, USA,  Nov. 17th - 21st

Source:	TCL

Title:	Discussion on RRC Modelling for 6GR

Agenda item:	10.3.2.1

Document for:	Discussion and decision

- Introduction

At the 3GPP RAN2 #131bis meeting, the following agreements have been reached for RRC Modelling in 6GR.

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

This contribution focuses on discussion of the RRC Modelling for 6GR .

- Discussion

  - RRC Modelling in 5G

5G NR introduced a three-state RRC model – RRC_IDLE, RRC_CONNECTED, and the newer RRC_INACTIVE state – to balance latency, signaling and power consumption. While this was a step forward, several limitations have been observed in practice:

- Signaling Overhead: To reduce the UE’s power consumption, there would be frequent transitions between RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE which can incur significant signaling. UEs in RRC_IDLE must perform a full connection establishment (RRC connection setup) to send or receive data, which involves multiple message exchanges. Even with RRC_INACTIVE, resuming to RRC_CONNECTED requires a resume procedure. These state transitions for sporadic or small data transmissions add control-plane overhead on both UE and network side.

- Multi-level paging in RRC_INACTIVE: There are two levels of paging triggers for UEs in RRC_INACTIVE in 5G NR: RAN-initiated paging performed by the NG-RAN (gNB) and core network-initiated paging by the 5GC. The mechanism of two-level of paging could increase UE complexity and power overhead if not managed well.

- Implementation Complexity and Adoption: In practice, use of RRC_INACTIVE has been limited in early 5G deployments. Reasons include higher implementation complexity for network vendors, inconsistent UE support (RRC_INACTIVE is only defined for 5G SA and must be explicitly signaled as supported in UE capability), and the fact that for some real-world traffic patterns the gains of RRC_INACTIVE (in terms of saved signaling or battery) can be marginal relative to its costs. In line with the principle of avoiding rarely-used features, the industry has recognized that simplifying 6G RAN by removing low-utilization optional modes could be beneficial. RRC_INACTIVE in its current form might not justify its complexity for certain device types: for example, devices with very infrequent small data transfers (many massive IoT devices) or those with extremely tight latency requirements (URLLC) might prefer a different approach.

Observation 1: For the three-state RRC model in 5G, there are limitations on significant signaling overhead due to frequent state transition, multi-level paging in RRC_INACTIVE and limited commercial adoption for RRC_INACTIVE.

  - Requirements and Challenges for RRC Modelling in 6G

The RRC state model in 6G must accommodate an even more diverse set of use cases and device categories. On one end of the spectrum, for massive IoT devices that transmit only tiny payloads infrequently (e.g. billions of sensors, smart meters), ultra-low power consumption is paramount – the device should remain in a dormant state as long as possible, waking only briefly to send or receive data. On the other end, we have ultra-reliable low-latency communications (URLLC) and emerging applications like extended reality (XR) that require instant network access with minimal delay. Such applications cannot tolerate the latency of state transitions. A 6G RRC design must reconcile these conflicting needs – providing instantaneous access for latency-critical services, yet lower-overhead dormancy for power-critical devices. Several requirements and challenges for 6G RRC modelling have become clear:

- Minimize Number of RRC States: A consensus is forming that the 6G RRC state machine should be simplified to reduce complexity. Removing rarely used states and optional modes can make the RAN behavior more predictable and efficient.

- Use Sub-States or Modes for Power Savings: Instead of a separate third state, 6G could introduce sub-states or modes within the Connected state to handle what RRC_INACTIVE did in 5G. The  Sub-States or Modes should provide at least the functions of UE-based mobility, context retention, and energy efficiency for both UE and network. By keeping the UE formally in Connected state (from the core network perspective) but in a reduced-activity sub-state, the system can avoid the full release/re-establishment cycle yet let the UE sleep and conserve energy.

- Tailor to Diverse Device Capabilities: 6G devices will range from extremely simple to very sophisticated. A one-size-fits-all state model may not be optimal. There is an expectation that 6G RRC might allow some flexibility per device type. For instance, a simple IoT device might operate with only Idle and Connected, whereas an advanced device might support additional sub-states for enhanced performance. The design should be scalable and not force every UE to support every feature.

Observation 2: 6G RAN will face diverse use cases requiring state management that minimizes signaling and power consumption while allowing some flexibility per device type.

  - RRC Modelling framework for 6GR

Given the above requirements and challenges, we propose a simplified yet flexible RRC state framework for 6G that builds on the 5G experience. The high-level approach is to retain only two top-level RRC states – RRC_CONNECTED and RRC_IDLE – while introducing sub-states (modes) to handle different activity levels and power-performance trade-offs. Figure 1 conceptually illustrates the model.

Figure 1

- RRC_CONNECTED state: This state encompasses all situations where the UE has an established RRC context with the RAN. Within RRC_CONNECTED, we define multiple sub-states:

- Active (traditional RRC_CONNECTED): This sub-states act as the . The UE has an active connection with dedicated resources. It can send/receive data continuously, with the network scheduling resources dynamically. All features of 5G RRC_CONNECTED apply (e.g. continuous downlink control monitoring, measurements, beam management). Power-saving mechanisms like connected-mode DRX (cDRX) are employed to balance availability and energy use.

- Inactive (suspended connectivity, similar to RRC_INACTIVE in 5G): The UE’s connection is suspended with context retained in the serving RAN. No dedicated radio bearers or active channel resources are kept, but the UE remains registrable in the RAN for quick reactivation. In this sub-state, the UE performs behavior akin to idle mode (e.g. sleeps with DRX and does cell reselection), but the network can directly reach it via a lightweight paging without involving the core network. The UE can resume to Active sub-state with a short RRC Resume procedure instead of a full connection setup. We enhance this sub-state by allowing ultra-lean communication: if the UE has only infrequent small data, it could transmit those in Inactive sub-state via a quick uplink signaling (bypassing the need to fully transition to Active).

- Dormant (deep suspension): This is a new sub-state for extended dormancy with deeper power savings than Inactive. In Dormant, the UE’s RRC context is still kept in the network (as in Inactive)nto remain known to the RAN. However, the UE ceases all radio monitoring – it does not perform periodic paging reception, measurements, or system information acquisition. Essentially, the UE is “temporarily offline” from the RAN perspective: it does not respond to normal paging, and the network buffers any downlink data or notifications for the UE while Dormant. The Dormant sub-state is designed for scenarios where the UE can afford to be unreachable for longer durations to save power (much like a device in Power Saving Mode, but at the RRC layer).

- Fast transitions between sub-states: The Connected sub-states are designed to allow fast transitions. Active ↔ Inactive should occur with minimal signaling (possibly a single message to suspend or resume); Inactive ↔ Dormant might be triggered by timers or an RRC message and would similarly require minimal overhead since context is already stored. Importantly, no core network signaling (NAS) is needed when toggling between these sub-states, since the UE remains registered and in CM-connected state throughout.

- RRC_IDLE state (equivalent to RRC_IDLE in 5G): The UE is not connected; it only listens to infrequent core-network paging (with NAS-level identifiers) within its registration area, and performs cell selection/reselection autonomously. The UE periodically wakes according to its idle-mode DRX cycle to receive paging or broadcast messages. It also updates the network on its location when required (e.g. tracking area updates) and acquires system information as needed when camping on new cells.

However, to tailor to diverse device capabilities and keep the devices lean, not all UEs need to support all sub-states. The gNB (or network) can configure which sub-states are applicable to a given UE, based on the UE’s radio capability, battery class, or subscription/service profile. For example, a high-power, latency-sensitive device might be configured to use an Inactive sub-state (to enable quick resumes) but not a Dormant sub-state, whereas a low-end IoT device might use a Dormant sub-state for maximum sleeping and not require an Inactive sub-state at all. By allowing per-UE tailoring of the RRC sub-state machine (within a standardized framework), the network can optimize signaling and power consumption on a per-device basis. This addresses the shortcoming of a uniform state machine that may be suboptimal for some device types.

Proposal 1: to retain only two top-level RRC states – RRC_CONNECTED and RRC_IDLE – while introducing sub-states (modes) to handle different activity levels and power-performance trade-offs, and allow per-UE tailoring of the RRC sub-state machine to keep the UE lean.

- Conclusion

In this contribution, we discussed the following aspect of RRC Modelling for 6GR and provided our proposals as follows:

Observation 1: For the three-state RRC model in 5G, there are limitations on significant signaling overhead due to frequent state transition, multi-level paging in RRC_INACTIVE and limited commercial adoption for RRC_INACTIVE.

Observation 2: 6G RAN will face diverse use cases requiring state management that minimizes signaling and power consumption while allowing some flexibility per device type.

Proposal 1: to retain only two top-level RRC states – RRC_CONNECTED and RRC_IDLE – while introducing sub-states (modes) to handle different activity levels and power-performance trade-offs, and allow per-UE tailoring of the RRC sub-state machine to keep the UE lean.

- References

- TR 22.870: "Study on 6G Use Cases and Service Requirements".

- TR 38.914: "Study on 6G Scenarios and Requirements".

- RP-252912: " Revised SID: Study on 6G Radio“NTT DOCOMO, CMCC, AT&T, Vodafone Beijing, China, September 15-18, 2025.