3GPP TSG-RAN WG2 Meeting # 132	R2-2508969

# Dallas, USA, 17th – 21st November 2025

Source:	Google Inc.

Title:	Discussion on inactive state

Agenda Item:	10.3.2.1

Document for:	Discussion and Decision

- Introduction

To meet the demanding Key Performance Indicators (KPIs) of 6G, such as 20ms Control Plane Latency for state transitions, a "light sleep" state with a stored UE context is essential.

The RRC_INACTIVE state was designed for this purpose in 5G. Despite it has been deployed, its adoption is limited. In the previous RAN2 meeting, several key problems with the 5G RRC_INACTIVE mode were identified. This contribution discusses these problems and proposes a simpler, more commercially viable baseline for the 6G inactive mode.

- Discussion

In the previous RAN2 meeting, it was discussed that 5G RRC_INACTIVE state was not widely deployed. The major problems identified for this lack of adoption were:

- RNA Complexity: The introduction of the RAN-based Notification Area (RNA) created significant network planning complexity.

- Paging Complexity: The associated RAN-based paging mechanism was also considered complex to implement and manage.

- Implementation Failures: The state was optional for the network, leading to inconsistent support

An alternative, proven "light sleep" model already exists from LTE: the RRC_IDLE with suspend indication state. The key differences between these two models are detailed in below Table 2-1.

| Feature | RRC_IDLE with Suspend Indication | RRC_INACTIVE State |
| --- | --- | --- |
| Context Storage | Both UE and eNodeB store the AS context. | Both UE and eNodeB store the AS context. |
| Transition to RRC_CONNCTED | Uses the RRC Connection Resume procedure. | Uses the RRC Connection Resume procedure. |
| Mobility Management | Relies on standard idle-mode cell reselection and Tracking Area Updates (TAUs) to the core network. | RAN-based Notification Area (RNA). The UE only contacts the network if it moves outside this smaller, gNodeB-defined area. |
| Paging | Initiated by the Core Network via the eNodeB across a Tracking Area. | Can be initiated directly by the RAN (gNodeB) within the UE's last known RNA, which is faster and more localized. |
| NAS layer state | EMM-IDLE with suspend indication, 5GMM-IDLE with suspend indication, | 5GMM-CONNECTED mode with RRC inactive indication |

Table 2-1 The RRC_IDLE with suspend indication and RRC_INACTIVE

As the table shows, the RRC_IDLE with suspend indication mode achieves the same fundamental goal (storing UE context for a fast resume) but avoids the primary architectural blockers that led to the failure of RRC_INACTIVE:

- Mobility: It does not use the complex RNA, relying instead on the CN-based Tracking Areas.

- Paging: It does not use RAN-based paging, relying instead on the simpler CN-initiated paging procedure.

This makes the RRC_IDLE with suspend indication mode a much simpler and more commercially viable architectural baseline for 6G.

Proposal: 	The 6G inactive mode should use the RRC_IDLE with suspend indication mode as the baseline.

- Conclusion

In conclusion, we have following proposals for 6G:

Proposal: 	The 6G inactive mode should use the RRC_IDLE with suspend indication mode as the baseline.

- References

[1] 3GPP TR 38.914 V0.2.0, "Study on 6G Scenarios and Requirements

[2] RAN2#131bis Chair note